3GPP TSG-RAN WG2 Meeting #118-e	R2-2205190

# Electronic, May 9th – 20th 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR |  | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Overheating assistance info for FR2-2 in (NG)EN-DC - RIL E801 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A new OverheatingAssistance-r17 IE for FR2-2 was added to UEAssistanceInformation. Since the legacy procedure supports reporting of overheating information for SCG (overheatingAssistanceForSCG) in (NG)EN-DC, the corresponding UEAssistanceInformation can be extended to also cover FR2-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new field overheatingAssistanceForSCG-FR2-2 to provide overheating assistance information for SCG in (NG)EN-DC for FR2-2 and update the procedural text. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Reporting of overheatingAssistance information is not supported for FR2-2 in (NG)EN-DC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.10.3, 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of change >

#### 5.6.10.3	Actions related to transmission of UEAssistanceInformation message

The UE shall set the contents of the UEAssistanceInformation message for power preference indications:

1>	if configured to provide power preference indication and if the UE prefers a configuration primarily optimised for power saving:

2>	set powerPrefIndication to lowPowerConsumption;

1>	else if configured to provide power preference indication:

2>	set powerPrefIndication to normal;

The UE shall set the contents of the UEAssistanceInformation message for SPS assistance information:

1>	if configured to provide SPS assistance information:

2>	if there is any traffic for V2X sidelink communication which needs to report SPS assistance information:

3>	include trafficPatternInfoListSL in the UEAssistanceInformation message;

2>	if there is any traffic for uplink communication which needs to report SPS assistance information:

3>	include trafficPatternInfoListUL in the UEAssistanceInformation message;

The UE shall set the contents of the UEAssistanceInformation message for bandwidth preference indications:

1>	set bw-Preference to its preferred configuration;

The UE shall set the contents of the UEAssistanceInformation message for delay budget report:

1>	if configured to provide delay budget report:

2>	if the UE prefers an adjustment in the connected mode DRX cycle length:

3>	set delayBudgetReport to type1 according to a desired value;

2>	else if the UE prefers coverage enhancement configuration change:

3>	set delayBudgetReport to type2 according to a desired value;

The UE shall set the contents of the UEAssistanceInformation message for the RLM report:

1>	if configured to provide RLM report:

2>	if T314 has expired:

3>	set rlm-event to earlyOutOfSync;

2>	if T315 has expired:

3>	set rlm-event to earlyInSync;

3>	if configured to report rlmReportRep-MPDCCH:

4>	set excessRep-MPDCCH to the value indicated by lower layers;

The UE shall set the contents of the UEAssistanceInformation message for overheating assistance indication:

1>	if configured to provide overheating assistance indication:

2>	if the UE experiences internal overheating:

3>	if the UE prefers to temporarily reduce its DL category and UL category:

4>	include reducedUE-Category in the OverheatingAssistance IE;

4>	set reducedUE-CategoryDL to the number to which the UE prefers to temporarily reduce its DL category;

4>	set reducedUE-CategoryUL to the number to which the UE prefers to temporarily reduce its UL category;

3>	if the UE prefers to temporarily reduce the number of maximum secondary component carriers:

4>	include reducedMaxCCs in the OverheatingAssistance IE;

4>	set reducedCCsDL to the number of maximum SCells the UE prefers to be temporarily configured in downlink;

4>	set reducedCCsUL to the number of maximum SCells the UE prefers to be temporarily configured in uplink;

3>	if configured to provide overheating assistance indication for NR SCG:

4>	include overheatingAssistanceForSCG in the OverheatingAssistance IE;

4> if configured with serving cells operating on FR2-2 for NR SCG

5> include overheatingAssistanceForSCG-FR2-2 in the OverheatingAssistance IE;

4>	set overheatingAssistanceForSCG and if applicable, overheatingAssistanceForSCG-FR2-2, in accordance with clause 5.7.4.3a as specified in TS 38.331 [82];

2>	else (if the UE no longer experiences an overheating condition):

3>	do not include reducedUE-Category, reducedMaxCCs, overheatingAssistance-v1610 (if configured to provide overheating assistance indication for NR SCG) and overheatingAssistance-v17xy (if configured to provide overheating assistance indication for NR SCG and FR2-2 serving cells in NR SCG) in OverheatingAssistance IE;

The UE shall:

1>	if the procedure was triggered to provide SPS assistance information and the related configuration was provided by an RRCConnectionReconfiguration message that was received embedded within an NR RRCReconfiguration message:

2>	submit the UEAssistanceInformation message via SRB1 embedded in NR RRC message ULInformationTransferIRAT as specified in TS 38.331 [82];

1>	else:

2>	submit the UEAssistanceInformation message to lower layers for transmission.

NOTE 1:	It is up to UE implementation when and how to trigger SPS assistance information.

NOTE 2:	It is up to UE implementation to set the content of trafficPatternInfoListSL and trafficPatternInfoListUL.

NOTE 3:	Traffic patterns for different Destination Layer 2 IDs are provided in different entries in trafficPatternInfoListSL.

NOTE 4:	Although not recommended, UE may start or restart the following timers whenever it sends the UEAssistanceInformation message (i.e. even if the message was not triggered for the concerned feature): T340, T341, T342, T343, T344 and T345.

< Unchanged parts omitted >

### 6.2.2	Message definitions

< Unchanged parts omitted >

#### –	UEAssistanceInformation

The UEAssistanceInformation message is used for the indication of UE assistance information to the eNB.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

UEAssistanceInformation message

-- ASN1START

UEAssistanceInformation-r11 ::=		SEQUENCE {

criticalExtensions					CHOICE {

c1									CHOICE {

ueAssistanceInformation-r11			UEAssistanceInformation-r11-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

UEAssistanceInformation-r11-IEs ::=		SEQUENCE {

powerPrefIndication-r11				ENUMERATED	{normal, lowPowerConsumption}	OPTIONAL,

lateNonCriticalExtension			OCTET STRING								OPTIONAL,

nonCriticalExtension				UEAssistanceInformation-v1430-IEs			OPTIONAL

}

UEAssistanceInformation-v1430-IEs ::=	SEQUENCE {

bw-Preference-r14						BW-Preference-r14						OPTIONAL,

sps-AssistanceInformation-r14			SEQUENCE {

trafficPatternInfoListSL-r14			TrafficPatternInfoList-r14			OPTIONAL,

trafficPatternInfoListUL-r14			TrafficPatternInfoList-r14			OPTIONAL

}			OPTIONAL,

rlm-Report-r14							SEQUENCE {

rlm-Event-r14							ENUMERATED {earlyOutOfSync, earlyInSync},

excessRep-MPDCCH-r14					ENUMERATED {excessRep1, excessRep2}	OPTIONAL

}																				OPTIONAL,

delayBudgetReport-r14					DelayBudgetReport-r14					OPTIONAL,

nonCriticalExtension					UEAssistanceInformation-v1450-IEs		OPTIONAL

}

UEAssistanceInformation-v1450-IEs ::=	SEQUENCE {

overheatingAssistance-r14				OverheatingAssistance-r14				OPTIONAL,

nonCriticalExtension					UEAssistanceInformation-v1530-IEs		OPTIONAL

}

UEAssistanceInformation-v1530-IEs ::=	SEQUENCE {

sps-AssistanceInformation-v1530			SEQUENCE {

trafficPatternInfoListSL-v1530			TrafficPatternInfoList-v1530

}			OPTIONAL,

nonCriticalExtension					UEAssistanceInformation-v1610-IEs						OPTIONAL

}

UEAssistanceInformation-v1610-IEs ::=	SEQUENCE {

overheatingAssistance-v1610				OverheatingAssistance-v1610			OPTIONAL,

nonCriticalExtension					UEAssistanceInformation-v1700-IEs		OPTIONAL

}

UEAssistanceInformation-v1700-IEs ::=	SEQUENCE {

uplinkData-r17							ENUMERATED { true }						OPTIONAL,

scg-DeactivationPreference-r17			ENUMERATED { scgDeactivationPreferred,

noPreference }				OPTIONAL,

nonCriticalExtension						UEAssistanceInformation-v17xy-IEs 							OPTIONAL

}

UEAssistanceInformation-v17xy-IEs ::=	SEQUENCE {

overheatingAssistance-v17xy				OverheatingAssistance-v17xy			    OPTIONAL,

nonCriticalExtension					SEQUENCE {}							    OPTIONAL

}

BW-Preference-r14 ::= SEQUENCE {

dl-Preference-r14		ENUMERATED	{mhz1dot4, mhz5, mhz20 }				OPTIONAL,

ul-Preference-r14		ENUMERATED	{mhz1dot4, mhz5}						OPTIONAL

}

TrafficPatternInfoList-r14 ::= SEQUENCE (SIZE (1..maxTrafficPattern-r14)) OF TrafficPatternInfo-r14

TrafficPatternInfo-r14 ::=	SEQUENCE {

trafficPeriodicity-r14			ENUMERATED {

sf20, sf50, sf100, sf200, sf300, sf400, sf500,

sf600, sf700, sf800, sf900, sf1000},

timingOffset-r14				INTEGER (0..10239),

priorityInfoSL-r14				SL-Priority-r13								OPTIONAL,

logicalChannelIdentityUL-r14	INTEGER (3..10)								OPTIONAL,

messageSize-r14					BIT STRING (SIZE (6))

}

TrafficPatternInfoList-v1530 ::= SEQUENCE (SIZE (1..maxTrafficPattern-r14)) OF TrafficPatternInfo-v1530

TrafficPatternInfo-v1530 ::=	SEQUENCE {

trafficDestination-r15			SL-DestinationIdentity-r12					OPTIONAL,

reliabilityInfoSL-r15			SL-Reliability-r15							OPTIONAL

}

DelayBudgetReport-r14::=	CHOICE {

type1							ENUMERATED {

msMinus1280, msMinus640, msMinus320, msMinus160,

msMinus80, msMinus60, msMinus40, msMinus20, ms0, ms20,

ms40, ms60, ms80, ms160, ms320, ms640, ms1280},

type2							ENUMERATED {

msMinus192, msMinus168,msMinus144, msMinus120,

msMinus96, msMinus72, msMinus48, msMinus24, ms0, ms24,

ms48, ms72, ms96, ms120, ms144, ms168, ms192}

}

OverheatingAssistance-r14 ::=	SEQUENCE {

reducedUE-Category			SEQUENCE {

reducedUE-CategoryDL		INTEGER (0..19),

reducedUE-CategoryUL		INTEGER (0..21)

}		OPTIONAL,

reducedMaxCCs				SEQUENCE {

reducedCCsDL				INTEGER (0..31),

reducedCCsUL				INTEGER (0..31)

}		OPTIONAL

}

OverheatingAssistance-v1610 ::=	SEQUENCE {

overheatingAssistanceForSCG-r16			OCTET STRING

}

OverheatingAssistance-v17xy ::=	                 SEQUENCE {

overheatingAssistanceForSCG-FR2-2-r17	    OCTET STRING

}

-- ASN1STOP

UEAssistanceInformation field descriptions

delayBudgetReport Indicates the UE-preferred adjustment to connected mode DRX or coverage enhancement configuration.

dl-Preference Indicates UE's preference on configuration of maximum PDSCH bandwidth. The value mhz1dot4 corresponds to CE mode usage in 1.4MHz bandwidth, mhz5 corresponds to CE mode usage in 5MHz bandwidth, and mhz20 corresponds to CE mode usage in 20MHz bandwidth or normal coverage.

excessRep-MPDCCH Indicates the excess number of repetitions on MPDCCH. Value excessRep1 and excessRep2 indicate the excess number of repetitions defined in TS 36.133 [16].

logicalChannelIdentityUL Indicates the logical channel identity associated with the reported traffic pattern in the uplink logical channel.

messageSize Indicates the maximum TB size based on the observed traffic pattern. The value refers to the index of TS 36.321 [6], table 6.1.3.1-1.

overheatingAssistanceForSCG Includes the NR OverheatingAssistance IE as specified in TS 38.331 [82]. The field indicates UE's preference on reduced configuration for NR SCG to address overheating in FR1 and/or FR2-1.

overheatingAssistanceForSCG-FR2-2 Includes the NR OverheatingAssistance-r17 IE for FR2-2 as specified in TS 38.331 [82]. The field indicates UE's preference on reduced configuration for NR SCG to address overheating in FR2-2.

powerPrefIndication Value lowPowerConsumption indicates the UE prefers a configuration that is primarily optimised for power saving. Otherwise the value is set to normal.

priorityInfoSL Indicates the traffic priority (i.e., PPPP) associated with the reported traffic pattern for V2X sidelink communication.

reducedCCsDL Indicates the UE's preference on reduced configuration corresponding to the maximum number of downlink SCells indicated by the field, to address overheating. This maximum number includes both SCells of E-UTRA and PSCell/SCells of NR in (NG)EN-DC.

reducedCCsUL Indicates the UE's preference on reduced configuration corresponding to the maximum number of uplink SCells indicated by the field, to address overheating. This maximum number includes both SCells of E-UTRA and PSCell/SCells of NR in (NG)EN-DC.

reducedUE-CategoryDL, reducedUE-CategoryUL Indicates that UE prefers a configuration corresponding to the reduced UE category, to address overheating. The reduced UE DL category and reduced UE UL category should be indicated according to supported combinations for UE UL and DL Categories, see TS 36.306 [5], Table 4.1A-6.

reliabilityInfoSL Indicates the traffic reliability (i.e., PPPR) associated with the reported traffic pattern for V2X sidelink communication.

rlm-Event This field provides the RLM event ("early-out-of-sync" or "early-in-sync").

rlm-Report This field provides the RLM report for BL UEs and UEs in CE.

sps-AssistanceInformation Indicates the UE assistance information to assist E-UTRAN to configure SPS.

timingOffset This field indicates the estimated timing for a packet arrival in a SL/UL logical channel. Specifically, the value indicates the timing offset with respect to subframe#0 of SFN#0 in milliseconds.

trafficDestination Indicates the destination associated with the reported traffic pattern for V2X sidelink communication.

trafficPatternInfoListSL This field provides the traffic characteristics of sidelink logical channel(s) that are setup for V2X sidelink communication. If trafficPatternInfoListSL-v1530 is included, it includes the same number of entries, and listed in the same order, as in trafficPatternInfoListSL-r14.

trafficPatternInfoListUL This field provides the traffic characteristics of uplink logical channel(s).

trafficPeriodicity This field indicates the estimated data arrival periodicity in a SL/UL logical channel. Value sf20 corresponds to 20 ms, sf50 corresponds to 50 ms and so on.

type1 Indicates the preferred amount of increment/decrement to the connected mode DRX cycle length with respect to the current configuration. Value in number of milliseconds. Value ms40 corresponds to 40 milliseconds, msMinus40 corresponds to -40 milliseconds and so on.

type2 Indicates the preferred amount of increment/decrement to the coverage enhancement configuration with respect to the current configuration so that the Uu air interface delay changes by the indicated amount. Value in number of milliseconds. Value ms24 corresponds to 24 milliseconds, msMinus24 corresponds to -24 milliseconds and so on.

ul-Preference Indicates UE's preference on configuration of maximum PUSCH bandwidth. The value mhz1dot4 corresponds to CE mode usage in 1.4MHz bandwidth, and mhz5 corresponds to CE mode usage in 5MHz bandwidth.

< End of changes >