3GPP TSG-RAN WG2 Meeting #118-e	R2-2205230

# Electronic meeting, 9th May – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | - | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on HARQ RTT Timer extension in TS38.331 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2022-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the post email discussion of [POST117-e][101][NTN] RRC CR, all company agreed to specific DL/UL HARQ RTT Timer extension in RRC, but the implementation of HARQ RTT timer extension for RRC is needed to be further defined. There is a proposed text in post email: ##### 5.3.5.5.5 MAC entity configuration … 1> if the received mac-CellGroupConfig includes the drx-Config: 2> if this Serving cell is configured with bitmap downlinkHARQ-FeedbackDisabled: 3> extend value of drx-HARQ-RTT-TimerDL by UE-gNB RTT. 2> if this Serving Cell is configured with bitmap uplinkHARQ-Mode: 3> extend value of drx-HARQ-RTT-TimerUL by UE-gNB RTT.. However, the term of “extend value … by” has not descriptted how to extend the HARQ RTT timer detailly. |
| Summary of change: |  | Add implementation of HARQ RTT Timer extension in RRC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The HARQ RTT timer extension is missed in corresponding RRC procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.5.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

##### 5.3.5.5.5	MAC entity configuration

The UE shall:

1>	if SCG MAC is not part of the current UE configuration (i.e. SCG establishment):

2>	create an SCG MAC entity;

1>	if any DAPS bearer is configured:

2>	reconfigure the MAC main configuration for the target cell group in accordance with the received mac-CellGroupConfig excluding tag-ToReleaseList and tag-ToAddModList;

1>	else:

2>	reconfigure the MAC main configuration of the cell group in accordance with the received mac-CellGroupConfig excluding tag-ToReleaseList and tag-ToAddModList;

1>	if the received mac-CellGroupConfig includes the tag-ToReleaseList:

2>	for each TAG-Id value included in the tag-ToReleaseList that is part of the current UE configuration:

3>	release the TAG indicated by TAG-Id;

1>	if the received mac-CellGroupConfig includes the tag-ToAddModList:

2>	for each tag-Id value included in tag-ToAddModList that is not part of the current UE configuration (TAG addition):

3>	add the TAG, corresponding to the tag-Id, in accordance with the received timeAlignmentTimer;

2>	for each tag-Id value included in tag-ToAddModList that is part of the current UE configuration (TAG modification):

3>	reconfigure the TAG, corresponding to the tag-Id, in accordance with the received timeAlignmentTimer.

1>	if the received mac-CellGroupConfig includes the drx-Config:

2>	if this Serving cell is configured with bitmap downlinkHARQ-FeedbackDisabled:

3>	set drx-HARQ-RTT-TimerDL to the value of drx-HARQ-RTT-TimerDL plus UE-gNB RTT;

2>	if this Serving Cell is configured with bitmap uplinkHARQ-Mode:

3>	set drx-HARQ-RTT-TimerUL to the value of drx-HARQ-RTT-TimerUL plus UE-gNB RTT.

End of change