3GPP TSG-RAN WG2#118 electronic	R2-2205309

Online, May 9-20, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | DraftCR | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on pre-configured MG procedure in 38.321 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_POS_enh_Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In RAN2#117e, the following agreements about UL MAC CE activation/deactivation/cancellation procedure are made: - For triggering condition for the UL MAC CE, reuse current RRC condition for Rel-16 PRS gap request, taking into account preconfigured MG. If the preconfigured MG is there and can satisfy the UE’s requirement, the UE uses MAC CE, otherwise RRC message as in Rel-16. The selection is specified in RRC. Reuse the “not configured or not sufficient” language from Rel-16. - UL MAC CE for MG activation and deactivation is triggered by upper layers. the following options to cancel a triggered UL MAC CE for MG activation and deactivation should be captured in the spec; other options can be discussed in the running CR discussion. • When the MAC CE is transmitted • When a request from upper layers to transmit a new request to gNB for a new/modified gap configuration is received • When an indication from upper layers that the gaps are not needed any more or a gap with a new id needs to be activated is received • On MAC reset. The UL MAC CE activation/deactivation/cancellation procedure should be completely captured in MAC spec based on the agreement. However the current 38.321 section 5.25 only captures the first bullet of cancellation condition, which is incomplete. Also the activation/deactivation procedure is not clear. |
| Summary of change: |  | Complete the procedure of UL MAC CE activation/deactivation/cancellation procedure in 38.321, based on the R2 agreement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The R2 agreement of pre-configured MG and corresponding UL MAC CE will not be completely captured in 38.321. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Based on draft_38.321-h00_v4 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<omitted text>

5.25	Positioning Measurement Gap Activation/Deactivation Request

If the UE is configured with pre-configured measurement gap, the UE may request the network to activate or deactivate the Positioning measurement gap with UL MAC CE for Positioning Measurement Gap Activation/Deactivation Request in clause 6.1.3.40.

The MAC entity shall, when triggered by the upper layer to send Positioning Measurement Gap Activation/Deactivation Request, cancel the triggered Positioning Measurement Gap Activation/Deactivation Request, if any and trigger another Positioning Measurement Gap Activation/Deactivation Request according to the upper layer’s request.

The MAC entity shall,

- if Positioning Measurement Gap Activation/Deactivation Request MAC CE has been triggered by upper layer:

2>If a request from upper layers to transmit a new request to gNB for a new/modified gap configuration is received, or

2>an indication from upper layers that the gaps are not needed any more is received, or

2>an indication from upper layers that a gap with a new id needs to be activated is received:

- 3>cancel triggered Positioning Measurement Gap Activation/Deactivation Request MAC CE.

- 2>else:

3>if UL-SCH resources are available for a new transmission and these UL-SCH resources can accommodate the Positioning Measurement Gap Activation/Deactivation Request MAC CE plus its subheader as a result of logical channel prioritization:

4>	instruct the Multiplexing and Assembly procedure to generate the Positioning Measurement Gap Activation/Deactivation Request MAC CE according to the upper layer’s request;

4>	cancel triggered Positioning Measurement Gap Activation/Deactivation Request MAC CE.

3>	else:

4>	trigger a Scheduling Request for Positioning Measurement Gap Activation/Deactivation Request MAC CE.

<omitted text>