3GPP TSG-RAN WG2 Meeting #118-e  	               			 R2-2205337

Online, May 9 – May 20, 2022

Agenda item:	6.12.2.2.2

Source:	LG Electronics Inc.

Title:	Other CP aspects for DRX cycle

Document for:	Discussion and Decision

# 1. Introduction

In this contribution, we suggest corrections for the DRX cycle of the UE.

The following is the agreement reached in the RAN2 meeting about the calculation of DRX cycle of the UE.

For RRC_IDLE UE, when eDRX cycle is no longer than 10.24s, T is determined by IDLE eDRX cycle. When eDRX cycle is longer than 10.24s, during the CN PTW, T is determined by the shortest of UE specific DRX cycle, if configured by upper layer, and default paging cycle. For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and Inactive eDRX cycle is not configured, during CN PTW, T is determined by the shortest of UE specific DRX cycle, if configured by upper layer, RAN paging cycle and default paging cycle. For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is no longer than 10.24s, T is determined by the shortest of IDLE eDRX cycle and INACTIVE eDRX cycle. For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is no longer than 10.24s, during CN PTW, T is determined by the shortest of UE specific DRX cycle, if configured by upper layer, INACTIVE eDRX cycle and default paging cycle. A) For RRC_INACTIVE UE, when IDLE eDRX cycle is no longer than 10.24s and INACTIVE eDRX cycle is not configured, T is determined by the shortest of RAN paging cycle and IDLE eDRX cycle. B) For RRC_INACTIVE UE, when IDLE eDRX cycle is longer than 10.24s and INACTIVE eDRX cycle is not configured, outside CN PTW, T is determined by RAN paging cycle.

The following is the part of the spec released in the latest specification[1].

The following parameters are used for the calculation of PF and i_s above: T: DRX cycle of the UE. If eDRX is not configured as defined in clause 7.4: - T is determined by the shortest of the UE specific DRX value(s), if configured by RRC and/or upper layers, and a default DRX value broadcast in system information. In RRC_IDLE state, if UE specific DRX is not configured by upper layers, the default value is applied. In RRC_IDLE state, if eDRX is configured by upper layers, i.e., TeDRX, CN, according to clause 7.4: - If TeDRX, CN is no longer than 1024 radio frames: - T = TeDRX, CN; - else: - During CN configured PTW, T is determined by the shortest of UE specific DRX value, if configured by upper layers, and the default DRX value broadcast in system information. In RRC_INACTIVE state, if eDRX is configured by RRC, i.e., TeDRX, RAN , and/or upper layers, i.e., TeDRX, CN, as defined in clause 7.4: - If both TeDRX, CN and TeDRX, RAN are no longer than 1024 radio frames, T = min{TeDRX, RAN, TeDRX, CN. - If TeDRX, CN is no longer than 1024 radio frames and no TeDRX, RAN is configured, T = min{TeDRX, RAN, TeDRX, CN}. - If TeDRX, CN is longer than 1024 radio frames: - If TeDRX, RAN is not configured: - During CN configured PTW, T is determined by the shortest of the UE specific DRX value (s), TeDRX, RAN and/or TeDRX, CN if configured, and a default DRX value broadcast in system information. Outside the CN configured PTW, T is determined by the DRX value configured by RRC;- else if TeDRX, RAN is no longer than 1024 radio frames: - During CN configured PTW, T is determined by the shortest of the UE specific DRX value, TeDRX, CN and TeDRX, RAN if configured and a default DRX value broadcast in system information. Outside the CN configured PTW, T is determined by TeDRX, RAN. N: number of total paging frames in T Ns: number of paging occasions for a PF PF_offset: offset used for PF determination UE_ID: If an eDRX cycle is configured by RRC or upper layers and eDRX-Allowed is signalled in SIB1: - 5G-S-TMSI mod 4096 else: - 5G-S-TMSI mod 1024

# 2. Discussion

There are some ambiguity and inconsistency between agreement and specification [1] for the DRX cycle of the UE when TeDRX, RAN is not configured in RRC_INACTIVE.

- In the current spec, even if TeDRX, RAN is not configured, TeDRX, RAN is one of parameters for calculation.

- There is no mention of the RAN paging cycle in the DRX cycle calculation in RRC_INACTIVE. Instead, the default DRX value is mentioned where the RAN paging cycle is required for calculation. However, the default DRX value is ambiguous to distinguish between the RAN paging cycle and the default paging cycle.

Observation 1. There are some ambiguity and inconsistency between agreement and specification for the DRX cycle of the UE when TeDRX, RAN is not configured in RRC_INACTIVE.

Therefore, some of the current specifications need to be modified.

Firstly, for the RRC_INACTIVE UE, when TeDRX, CN is no longer than 10.24s and TeDRX, RAN is not configured, T should be determined by the shortest of RAN paging cycle and TeDRX, CN. However, in the current spec, T is determined by the shortest of TeDRX, RAN and TeDRX, CN. In this case, RAN paging cycle should be used instead of TeDRX, RAN.

Secondly, for RRC_INACTIVE UE, when TeDRX, CN is longer than 10.24s and TeDRX, RAN is not configured, during CN PTW, T should be determined by the shortest of TeDRX, CN, RAN paging cycle and default paging cycle. However, in the current spec, T is determined by the shortest of TeDRX, RAN and/or TeDRX, CN and a default DRX. In order to resolve the ambiguity, it is necessary to write a content that compares the length between the RAN paging cycle and the default paging cycle rather than expressing the default DRX value.

Lastly, for RRC_INACTIVE UE, when TeDRX, CN is longer than 10.24s and TeDRX, RAN is not configured, outside CN PTW, T should be determined by RAN paging cycle. However, in the spec, T is determined by the DRX value. In this case, it is clear to write RAN paging cycle instead of the DRX value.

As in Annex 5, we propose text proposal to follow the agreement and resolve ambiguity.

Proposal 1. RAN2 to adopt the TP in Annex to better reflect agreement.

# 3. Conclusion

Follows are our proposal:

Observation 1. There are some ambiguity and inconsistency between agreement and specification for the DRX cycle of the UE when TeDRX, RAN is not configured in RRC_INACTIVE.

Proposal 1. RAN2 to adopt the TP in Annex to better reflect agreement.

# 4. References

[1] 38304-h00.docx

# 5. Annex – Text proposals (based on 38.304-h00)

# 7	Paging

## 7.1	Discontinuous Reception for paging

The UE may use Discontinuous Reception (DRX) in RRC_IDLE and RRC_INACTIVE state in order to reduce power consumption. The UE monitors one paging occasion (PO) per DRX cycle. A PO is a set of PDCCH monitoring occasions and can consist of multiple time slots (e.g. subframe or OFDM symbol) where paging DCI can be sent (TS 38.213 [4]). One Paging Frame (PF) is one Radio Frame and may contain one or multiple PO(s) or starting point of a PO.

In multi-beam operations, the UE assumes that the same paging message and the same Short Message are repeated in all transmitted beams and thus the selection of the beam(s) for the reception of the paging message and Short Message is up to UE implementation. The paging message is same for both RAN initiated paging and CN initiated paging.

The UE initiates RRC Connection Resume procedure upon receiving RAN initiated paging. If the UE receives a CN initiated paging in RRC_INACTIVE state, the UE moves to RRC_IDLE and informs NAS. However, if a L2 U2N Relay UE in RRC_INACTIVE state receives a CN initiated paging for a L2 U2N Remote UE, the L2 U2N Relay UE does not move to RRC_IDLE state.

NOTE:	The L2 U2N Remote UE does not need to monitor the PO in order to receive the paging message.

The PF and PO for paging are determined by the following formulae:

SFN for the PF is determined by:

(SFN + PF_offset) mod T = (T div N)*(UE_ID mod N)

Index (i_s), indicating the index of the PO is determined by:

i_s = floor (UE_ID/N) mod Ns

The PDCCH monitoring occasions for paging are determined according to pagingSearchSpace as specified in TS 38.213 [4] and firstPDCCH-MonitoringOccasionOfPO and nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured as specified in TS 38.331 [3]. When SearchSpaceId = 0 is configured for pagingSearchSpace, the PDCCH monitoring occasions for paging are same as for RMSI as defined in clause 13 in TS 38.213 [4].

When SearchSpaceId = 0 is configured for pagingSearchSpace, Ns is either 1 or 2. For Ns = 1, there is only one PO which starts from the first PDCCH monitoring occasion for paging in the PF. For Ns = 2, PO is either in the first half frame (i_s = 0) or the second half frame (i_s = 1) of the PF.

When SearchSpaceId other than 0 is configured for pagingSearchSpace, the UE monitors the (i_s + 1)th PO. A PO is a set of 'S*X ' consecutive PDCCH monitoring occasions where 'S' is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is the nrofPDCCH-MonitoringOccasionPerSSB-InPO if configured or is equal to 1 otherwise. The [x*S+K]th PDCCH monitoring occasion for paging in the PO corresponds to the Kth transmitted SSB, where x=0,1,…,X-1, K=1,2,…,S. The PDCCH monitoring occasions for paging which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for paging in the PF. When firstPDCCH-MonitoringOccasionOfPO is present, the starting PDCCH monitoring occasion number of (i_s + 1)th PO is the (i_s + 1)th value of the firstPDCCH-MonitoringOccasionOfPO parameter; otherwise, it is equal to i_s * S*X. If X > 1, when the UE detects a PDCCH transmission addressed to P-RNTI within its PO, the UE is not required to monitor the subsequent PDCCH monitoring occasions for this PO.

NOTE 1:	A PO associated with a PF may start in the PF or after the PF.

NOTE 2:	The PDCCH monitoring occasions for a PO can span multiple radio frames. When SearchSpaceId other than 0 is configured for paging-SearchSpace the PDCCH monitoring occasions for a PO can span multiple periods of the paging search space.

The following parameters are used for the calculation of PF and i_s above:

T: DRX cycle of the UE.

If eDRX is not configured as defined in clause 7.4:

-	T is determined by the shortest of the UE specific DRX value(s), if configured by RRC and/or upper layers, and a default DRX value broadcast in system information. In RRC_IDLE state, if UE specific DRX is not configured by upper layers, the default value is applied.

In RRC_IDLE state, if eDRX is configured by upper layers, i.e., TeDRX, CN, according to clause 7.4:

-	If TeDRX, CN is no longer than 1024 radio frames:

-	T = TeDRX, CN;

-	else:

-	During CN configured PTW, T is determined by the shortest of UE specific DRX value, if configured by upper layers, and the default DRX value broadcast in system information.

In RRC_INACTIVE state, if eDRX is configured by RRC, i.e., TeDRX, RAN , and/or upper layers, i.e., TeDRX, CN, as defined in clause 7.4:

-	If both TeDRX, CN and TeDRX, RAN are no longer than 1024 radio frames, T = min{TeDRX, RAN, TeDRX, CN}.

-	If TeDRX, CN is no longer than 1024 radio frames and no TeDRX, RAN is configured, T is determined by the shortest of the UE specific DRX value, TeDRX, CN if configured, and the RAN paging cycle, if configured.

-	If TeDRX, CN is longer than 1024 radio frames:

-	If TeDRX, RAN is not configured:

-	During CN configured PTW, T is determined by the shortest of the UE specific DRX value, TeDRX, CN if configured, the RAN paging cycle, if configured, and the default paging cycle. Outside the CN configured PTW, T is determined by the RAN paging, if configured.

-	else if TeDRX, RAN is no longer than 1024 radio frames:

-	During CN configured PTW, T is determined by the shortest of the UE specific DRX value, TeDRX, CN and TeDRX, RAN if configured and a default DRX value broadcast in system information. Outside the CN configured PTW, T is determined by TeDRX, RAN.

N: number of total paging frames in T

Ns: number of paging occasions for a PF

PF_offset: offset used for PF determination

UE_ID:

If an eDRX cycle is configured by RRC or upper layers and eDRX-Allowed is signalled in SIB1:

-	5G-S-TMSI mod 4096

else:

-	5G-S-TMSI mod 1024

Parameters Ns, nAndPagingFrameOffset, nrofPDCCH-MonitoringOccasionPerSSB-InPO, and the length of default DRX Cycle are signaled in SIB1. The values of N and PF_offset are derived from the parameter nAndPagingFrameOffset as defined in TS 38.331 [3]. The parameter first-PDCCH-MonitoringOccasionOfPO is signalled in SIB1 for paging in initial DL BWP. For paging in a DL BWP other than the initial DL BWP, the parameter first-PDCCH-MonitoringOccasionOfPO is signaled in the corresponding BWP configuration.

If the UE has no 5G-S-TMSI, for instance when the UE has not yet registered onto the network, the UE shall use as default identity UE_ID = 0 in the PF and i_s formulas above.

5G-S-TMSI is a 48 bit long bit string as defined in TS 23.501 [10]. 5G-S-TMSI shall in the formulae above be interpreted as a binary number where the left most bit represents the most significant bit.

In RRC_INACTIVE state, if the UE supports inactiveStatePO-Determination and the network broadcasts ranPagingInIdlePO with value "true", the UE shall use the same i_s as for RRC_IDLE state. Otherwise, the UE determines the i_s based on the parameters and formula above.

In RRC_INACTIVE state, if eDRX value configured by upper layers is no longer than 1024 radio frames, the UE shall use the same i_s as for RRC_IDLE state.

In RRC_INACTIVE state, if eDRX value configured by upper layers is longer than 1024 radio frames, during CN PTW, the UE shall use the same i_s as for RRC_IDLE state.