3GPP TSG-RAN WG2 Meeting #118 Electronic	R2-2205344

Elbonia, 09 – 25 May 2022

Agenda item:	6.15.2.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Further Issues on Collision Avoidance of IUC messages

WID/SID:	NR_SL_enh-Core – Release 17

Document for:	Discussion and Decision

# 1	Introduction

In RAN1#108 meeting RAN1 has defined the content of the IUCRequest and IUCInformation. For both messages a new SCI format 2-C has been designed. In this contribution we discuss further issues on the topic and propose to reuse existing entries to avoid collision of IUC messages.

# 2	Issues with SCI format 2-C

## 2.1	Heading 2

In R1-2202256 RAN1 has discussed various options wrt the content of SCI format 2-C. Since the first stage SCI only allows 2-bit extension to indicate the format of the second stage SCI only 4 different 2nd stage SCI formats can be defined. Thus, RAN1 has only introduced a single new SCI format 2-C for Inter UE Coordination (IUC), i.e. the IUCRequest and IUCInformation (i.e. IUC response) use the same SCI format.

RAN1#108 made the following agreements on SCI format 2-C for IUCRequest and IUCInformation:

SCI format 2-C for IUCRequest:

| Field name | Field size (in bits) |
| --- | --- |
| Providing/requesting indicator | 1 |
| Resource combination(s) | Where is provided by the higher layer parameter sl-NumSubchannel, with that is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResource is configured; otherwise. |
| First resource location(s) | 8 |
| Reference slot location | Where is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| Resource set type | 1 |
| Lowest subchannel indices for the first resource location of each TRIV | where is provided by the higher layer parameter sl-NumSubchannel |
| (FFS) Actual number of resource combination | 1 Note: Support of this field is to be concluded by Feb 28. |

SCI format 2-C for IUCInformation:

| Field name | Field size (in bits) |
| --- | --- |
| Providing/requesting indicator | 1 |
| Priority | 3 |
| Number of subchannels | Where is provided by the higher layer parameter sl-NumSubchannel |
| Resource reservation period | Where with that is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResoure is configured; otherwise. |
| Resource selection window location | Where is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| Resource set type | 1 bit if determineResourceSetTypeScheme1 is set to ‘UE-B’s request’, otherwise, 0 bit |

- This agreement does not imply that new field requested by RAN2 cannot be further added.

One of the problems resulting from the design of SCI format 2-C is that the size of the 2nd-stage SCI needs to be fixed, SCI format 2-C always includes two resource combinations. Another problem is that SCI format 2-C only indicates with a single bit (providing/requesting indicator) to indicate whether it is a IUCRequest or IUCInformation, but there is no differentiation in the content (i.e. the field elements) in SCI format 2-C.

That results in a rather inefficient resource usage for the IUCRequest message if SCI format 2-C is used in unicast since half of the message size (~60bits) is wasted by padding bits, as the field resource combinations is not used.

As it was clear during RAN1#108 discussion, that the initial design of 2nd stage SCI for IUC scheme 1 may have some flaws and some level of incompleteness the rapporteur’s agreement captured “This agreement [on SCI format 2-C] does not imply that new field requested by RAN2 cannot be further added” as noted above for IUCRequest SCI format 2-C.

Observation 1: In case SCI format 2-C is used for IUC in unicast (instead of MAC CE) IUCRequest and IUCInformation use the same 2nd stage SCI message format 2-C.

Observation 2: Unused field entries in SCI format 2-C (e.g. resource combinations field in IUCRequest) are padded and hence lead to inefficient resource usage of 2nd stage SCI.

Observation 3: Unused field entries in SCI format 2-C (e.g. resource combinations field in IUCRequest) are padded and hence lead to inefficient resource usage of 2nd stage SCI.

Observation 4: Since SCI format 2-C has fixed size, approximately half of the message size is wasted for IUCRequest.

Instead of zero-padding the unused fields in SCI format 2-C, we rather propose to reuse the existing field entries in a more efficient manner. In order to avoid collisions among IUC transmissions the resource combinations field in the IUCRequest to be sent by UE-B can be used to indicate resources to be used by UE-A in its subsequent transmission of IUCInformation. That is, UE-B can provide (non)-preferred resources to UE-A so that UE-A can consider them in its resource selection for transmission of its response i.e. IUCInformation to UE-B. For example, UE-B can include a resource combination in its IUCRequest message where it expects low or no interference and is in Rx mode, and UE-A can use this indicated resource for its IUCInformation message transmission. This allows UE-B to successfully receive the IUCInformation from UE-A since potential collision at UE-B is avoided by already indicating UE-A the UE-B’s (non-)prefererred resources that have taken into account potential collision scenario at UE-B.

Proposal 1: The requesting UE (UE-B) shall use the so far unused (and zero-padded) field entry resource combinations in SCI format 2-C for the IUCRequest message to indicate the sidelink resources to be used by the responding UE (UE-A) in its IUCInformation message.

# 3	Conclusion

Observation 1: In case SCI format 2-C is used for IUC in unicast (instead of MAC CE) IUCRequest and IUCInformation use the same 2nd stage SCI message format 2-C.

Observation 2: Unused field entries in SCI format 2-C (e.g. resource combinations field in IUCRequest) are padded and hence lead to inefficient resource usage of 2nd stage SCI.

Observation 3: Unused field entries in SCI format 2-C (e.g. resource combinations field in IUCRequest) are padded and hence lead to inefficient resource usage of 2nd stage SCI.

Observation 4: Since SCI format 2-C has fixed size, approximately half of the message size is wasted for IUCRequest.

Proposal 1: The requesting UE (UE-B) shall use the so far unused (and zero-padded) field entry resource combinations in SCI format 2-C for the IUCRequest message to indicate the sidelink resources to be used by the responding UE (UE-A) in its IUCInformation message.