3GPP TSG-RAN WG2 Meeting #118 electronic          	          R2-2205352

Online, 9th May – 20th May, 2022

Agenda Item:	6.9.3.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on PDCCH skipping while SR is pending

Document for:	Discussion and decision

# Introduction

In previous RAN2 meetings, it was agreed that the UE ignores PDCCH skipping while SR is pending. Besides, companies further discussed the details of UE behaviours and it was tentatively clarified that the UE ignores PDCCH skipping on all serving cells of the corresponding cell group. A remaining issue is whether the PDCCH skipping is ignored on all serving cells of a CG or only on certain serving cells [1].

RAN2 clarifications to earlier RAN2 decisions: - P1: UE ignores PDCCH skipping on all serving cells of the corresponding CG while SR is pending (FFS if “all” can be further restricted).

In this contribution, we further discuss the leftover issue and propose solutions.

# Discussion

During the offline discussion in RAN2#117-e, the majority view was that the UE ignores PDCCH skipping on all serving cells on a cell group, without further optimization. Besides, some companies mentioned the impact of LCP restriction and DRX group.

In our view, the current clarification is sufficient and no further optimization is needed. Ignoring PDCCH skipping on all serving cells within a CG while SR is pending is a simple solution without any technical issues. Besides, according to RAN1’s reply LS [2], RAN1 is considering capturing the cases on UE ignoring PDCCH skipping in RAN1 spec. Thus, we think no additional MAC impact should be introduced for this feature.

Question 2: RAN2 would like to know if RAN1 prefers to capture the above agreements in RAN1 specification or prefer these to be captured in MAC specification.. Answer 2: RAN1 would discuss and conclude how to capture the above RAN2 agreements in RAN1 specification. - It is RAN1 understanding PDCCH skipping is not applied to perform PDCCH monitoring during RAR window/MsgB window/contention resolution timer or when SR is pending.

Proposal 1: Confirm the previous RAN2 conclusion that the UE ignores PDCCH skipping on all serving cells of the corresponding Cell Group while SR is pending.

# Conclusion

In this paper, we discuss the leftover issue on PDCCH skipping while SR is pending. Proposal is summarized as follows.

Proposal 1: Confirm the previous RAN2 conclusion that the UE ignores PDCCH skipping on all serving cells of the corresponding Cell Group while SR is pending.

# References

- Report of 3GPP TSG RAN WG2 meeting #117-e, Online.

- R2-2204121, LS reply on PDCCH skipping.