3GPP TSG-RAN WG2 Meeting #118 electronic                                            R2-2205426

Online, May 9th –20th, 2022

Source:	CATT

Title:	Discussion on the Remaining Issues of CPAC

Agenda Item:	6.2.3

Document for:	Discussion and Decision

1	Introduction

In this document, we will further discuss the remaining FFSs on CPAC. We also give the corresponding TP associated with the proposals of this contribution in the annex.

2	Discussion

For coexistence of CHO and CPAC, it is still FFS on how to implement the working assumption of CHO and CPAC coexistence:

## 10.1 General …… Configuration of CHO and CPC for simultaneous operation is not supported in this release. Editor's note: FFS how to reflect the RAN2 work assumption (and potential further agreements) on CHO and CPAC co-existence. ……

In RAN2#117e, the following enhancement is made on supporting the coexistence:

1-1 (revised via [200]). (For R17 CPAC completion) RAN2 agree to have MN-SN coordination for maximum number of conditional reconfigurations allowed for each node. MN can indicate (via CG-ConfigInfo) how many conditional reconfigurations SN is allowed to have. 12: (R16/R17 CHO/CPAC coex) If one conditional reconfiguration is executed, the other conditional reconfigurations should be released. Everything else is up to UE implementation. No other specification efforts in Rel-17 on CPAC/CHO coexistence.

Based on the above agreement, although there is not clear agreement, partial coexistence of CHO and CPAC can already be supported without issues foreseen, i.e., the coexistence of CHO and MN initiated CPA/CPC.

Observation 1: Partial coexistence of CHO and CPAC can be supported based on the agreement made in RAN2#117e.

However, due to the limited R17 time budget, it is better not to take any effort on the analysis of the coexistence issues, which means that RAN2 does not need to specify the detailed coexistence scenarios, and the coexistence can be supported as long as no further specification efforts is required.  Thus, we think it is enough to reflect the RAN2 work assumption of coexistence by deleting the restriction that “Configuration of CHO and CPC for simultaneous operation is not supported in this release.”

Proposal 1a: The restriction that “Configuration of CHO and CPC for simultaneous operation is not supported in this release.” should be deleted so as to reflect the RAN2 work assumption of coexistence.

Proposal 1b: RAN2 to agree to remove the following FSS in TS37.340:

- Editor's note: FFS how to reflect the RAN2 work assumption (and potential further agreements) on CHO and CPAC co-existence.

The corresponding TP on the above proposals can be found in the annex 1.

5	Conclusion

In the previous sections, we made the following observations and proposals:

Observation 1: Partial coexistence of CHO and CPAC can be supported based on the agreement made in RAN2#117e.

Proposal 1a: The restriction that “Configuration of CHO and CPC for simultaneous operation is not supported in this release.” should be deleted so as to reflect the RAN2 work assumption of coexistence.

Proposal 1b: RAN2 to agree to remove the following FSS in TS37.340:

- Editor's note: FFS how to reflect the RAN2 work assumption (and potential further agreements) on CHO and CPAC co-existence.

6	Reference

[1] 3GPP TS 37.340: "E-UTRA and NR; Multi-connectivity; Stage 2".

Annex 1: TP on TS 37.340

START OF CHANGE

# 10	Multi-Connectivity operation related aspects

## 10.1	General

Similar procedures as defined under clause 10.1.2.8 (Dual Connectivity operation) in TS 36.300 [2] apply for MR-DC.

Similar CHO principles as defined in TS 36.300 [2] and TS 38.300 [3] apply for the Conditional PSCell Change in MR-DC.

Conditional PSCell Change and conditional PSCell addition are not supported for the MR-DC options NE-DC and NGEN-DC.

In case MR-DC is configured, CHO is only supported in Master Node to eNB/gNB Change procedure in this release.

END OF CHANGE