3GPP TSG-RAN WG2 #118	R2-2205440

Electronical meeting, 9 May – 20 May 2022

Agenda Item:	6.14.3 Corrections

Source:	Ericsson

Title:	Discussion on naming of QoE measurements

Document for:	Discussion, Decision

# 1	Introduction

The work item for QoE measurements in NR is declared completed. However, some naming convention could still be discussed, as it is not fully clear what to call “regular” or “legacy” QoE.

# 2	Discussion

## 2.1	Naming of “regular” QoE

When RAN visible QoE was introduced in the QoE work item, it meant that two types of QoE measurements are now supported. In LTE, only one type of QoE measurements is supported and it is called QoE measurement collection or application layer measurement collection. In NR it is not clear whether QoE measurement collection means only the type used in LTE or whether it includes also RAN visible QoE, and in such case what the type used in LTE should be called in NR. In 38.300 the terms “regular QoE” and “non-RAN visible QoE” are used, but a specific name for it could be considered.

RAN visible QoE is named after the entity that initiated the measurements. If the same convention is used for “regular QoE” it could be called “OAM-QoE”. The general term QoE measurement collection or application layer measurement collection could then include both OAM-QoE and RAN visible QoE.

QoE measurements configured by OAM are called OAM-QoE measurements.

The QoE Measurement Collection function can support two types of QoE measurements: OAM-QoE measurements and RAN visible QoE measurements.

Update TS 38.300 and TS 38.331 with these definitions.

A TP on 38.300 is as follows:

## 3.2	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], in TS 36.300 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1] and TS 36.300 [2].

[..]

Numerology: corresponds to one subcarrier spacing in the frequency domain. By scaling a reference subcarrier spacing by an integer N, different numerologies can be defined.

OAM-QoE measurements: UE application layer measurements configured by the OAM for different service types.

OAM-QoE report: the result of OAM-QoE measurements.

Parent node: IAB-MT's next hop neighbour node; the parent node can be IAB-node or IAB-donor-DU

PC5 Relay RLC channel: an RLC channel between L2 U2N Remote UE and L2 U2N Relay UE, which is used to transport packets over PC5 for L2 UE-to-Network Relay.

PLMN Cell: a cell of the PLMN.

RAN visible QoE measurements: a subset of OAM-QoE measurements configured by the gNB and reported to the NG-RAN node.

RAN visible QoE report: the results of RAN Visible QoE measurements, reported from the UE the gNB in RRC format.

RedCap UE: A UE with reduced capabilities as specified in clause 4.2.21.1. in TS 38.306 [11].

[..]

## 21.1	Overview

The QoE Measurement Collection function enables collection of application layer measurements from the UE. The supported service types are:

-	QoE Measurement Collection for DASH streaming services;

-	QoE Measurement Collection for MTSI services;

-	QoE Measurement Collection for VR services.

Both signalling based and management based QoE measurement collection are supported.

NOTE:	The naming QoE Measurement is used in NG, Xn, and interfaces between the OAM and thegNB. In the Uu interface, the naming application layer measurement is used and it is equal to QoE Measurement.

The QoE Measurement Collection function can support two types of QoE measurements:

-	OAM-QoE measurements;

-	RAN Visible QoE measurements.

The QoE Measurement Collection function can support two types of QoE reports:

-	OAM-QoE reports, for reporting OAM-QoE measurements from the UE to the MCE;

-	RAN Visible QoE reports, for reporting RAN Visible QoE measurements from the UE to the gNB.

A TP on 38.331 is as follows:

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

[..]

NR sidelink communication:  AS functionality enabling at least V2X Communication as defined in TS 23.287 [55], between two or more nearby UEs, using NR technology but not traversing any network node.

OAM-QoE measurements: UE application layer measurements configured by the OAM for different service types.

OAM-QoE report: the result of OAM-QoE measurements.

PNI-NPN identity: an identifier of a PNI-NPN comprising of a PLMN ID and a CAG -ID combination.

Primary Cell: The MCG cell, operating on the primary frequency, in which the UE either performs the initial connection establishment procedure or initiates the connection re-establishment procedure.

PC5 Relay RLC channel: An RLC channel between L2 U2N Remote UE and L2 U2N Relay UE, which is used to transport packets over PC5 for L2 UE-to-Network relay.

Primary SCG Cell: For dual connectivity operation, the SCG cell in which the UE performs random access when performing the Reconfiguration with Sync procedure.

Primary Timing Advance Group: Timing Advance Group containing the SpCell.

PUCCH SCell: An SCell configured with PUCCH.

PUSCH-Less SCell: An SCell configured without PUSCH.

RAN visible QoE measurements: a subset of OAM-QoE measurements configured by the gNB and reported to the NG-RAN node.

RAN visible QoE report: the results of RAN Visible QoE measurements, reported from the UE the gNB in RRC format.

RedCap UE: A UE with reduced capabilities as specified in sub-clause 4.2.x.x. in TS 38.306 [26].

[..]

# Conclusion

RAN2 is kindly asked to discuss the following proposal:

Proposal 1	Application layer measurements configured by OAM are called OAM-QoE measurements.

Proposal 2	The QoE Measurement Collection function can support two types of QoE measurements: OAM-QoE measurements and RAN visible QoE measurements.

Proposal 3	Update 38.300 and 38.331 with these definitions.

# References

TR 38.890, Study on NR QoE management and optimizations for diverse services, Ericsson

RP-210913, NR QoE management and optimizations for diverse services, China Unicom