3GPP TSG-RAN WG2 #118	R2-2205441

Electronical meeting, 9 May – 20 May 2022

Agenda Item:	6.14.3 Corrections

Source:	Ericsson

Title:	Discussion on RIL issue E138 related to handover

Document for:	Discussion, Decision

# 1	Introduction

The work item for QoE measurements in NR is declared completed. However, some RIL issues have been added to the RRC CR during ASN.1 review and this contribution addresses RIL issues E138 related to handover for QoE.

# 2	Discussion

## 2.1	RIL issue E138

RAN2 has previously discussed the case when a handover occurs during the transmission of a QoE report. This is a relevant use case as the reports may be large and RRC segmentation may be used, which increases the probability that handover will occur during the transmission. There is no retransmission or forwarding mechanism defined for SRBs, but in some cases RRC messages are retransmitted after a handover. E.g. UEAssistanceInformation is retransmitted after a handover if it was transmitted in the source node less than one seconds before the handover occurred. This is to ensure that the target node gets the information also in case the source node had already started the handover preparation procedure when the message arrived in the source node.

When QoE measurements are performed, the UE may have recorded measurements for a very long time and it has consumed UE capacity. The reports may be large and contain a lot of information which would be very useful for the network to receive. It seems unnecessary that all the collected information is wasted in case a handover occurs when the report is transmitted, especially since it can be solved very easily. At handover, chapter “5.3.5.3	Reception of an RRCReconfiguration by the UE” is executed and reconfigurationWithSync is included in the message. At the end of this chapter UEAssistanceInformation is retransmitted in the target node, and retransmission of the QoE report could be added here as well, as indicated in yellow:

2>	if reconfigurationWithSync was included in masterCellGroup or secondaryCellGroup:

3>	if the UE initiated transmission of a UEAssistanceInformation message for the corresponding cell group during the last 1 second, and the UE is still configured to provide the concerned UE assistance information for the corresponding cell group; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution, and the UE is configured to provide UE assistance information for the corresponding cell group, and the UE has initiated transmission of a UEAssistanceInformation message for the corresponding cell group since it was configured to do so in accordance with 5.7.4.2:

4>	initiate transmission of a UEAssistanceInformation message for the corresponding cell group in accordance with clause 5.7.4.3 to provide the concerned UE assistance information;

4>	start or restart the prohibit timer (if exists) associated with the concerned UE assistance information with the timer value set to the value in corresponding configuration;

3>	if SIB12 is provided by the target PCell; and the UE initiated transmission of a SidelinkUEInformationNR message indicating a change of NR sidelink communication related parameters relevant in target PCell (i.e. change of sl-RxInterestedFreqList or sl-TxResourceReqList) during the last 1 second preceding reception of the RRCReconfiguration message including reconfigurationWithSync in spCellConfig of an MCG; or

3>	if the RRCReconfiguration message is applied due to a conditional reconfiguration execution and the UE is capable of NR sidelink communication and SIB12 is provided by the target PCell, and the UE has initiated transmission of a SidelinkUEInformationNR message since it was configured to do so in accordance with 5.8.3.2:

4>	initiate transmission of the SidelinkUEInformationNR message in accordance with 5.8.3.3;

3> if configured with application layer measurements and if application layer measurement report information has been received from upper layers for which the transmission was not completed:

4>	re-submit the MeasurementReportAppLayer message to lower layers for transmission via SRB4;

The UE retransmits application layer measurement reports in the target node if a handover occurs during the transmission of the report.

# Conclusion

RAN2 is kindly asked to discuss the following proposal:

Proposal 1	The UE retransmits application layer measurement reports in the target node if a handover occurs during the transmission of the report.

# References

TR 38.890, Study on NR QoE management and optimizations for diverse services, Ericsson

RP-210913, NR QoE management and optimizations for diverse services, China Unicom