3GPP TSG-RAN WG2 Meeting #118 electronic	R2-2205471

Online, May 9 – 20, 2022

Agenda Item:	6.10.3.2.1

Source:	Ericsson

Title:	RILs: TA:V313, V320, D1:L011, H801, X704

Document for:	Discussion, Decision

# Introduction

In this contribution we discuss RIL 313

## V313 Use of RAN area code with tracking area list

For NTN, tracking area list has been introduced in IE PLMN-IdentityInfoList as follows:

#### –	PLMN-IdentityInfoList

The IE PLMN-IdentityInfoList includes a list of PLMN identity information.

PLMN-IdentityInfoList information element

-- ASN1START

-- TAG-PLMN-IDENTITYINFOLIST-START

PLMN-IdentityInfoList ::=               SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-IdentityInfo

PLMN-IdentityInfo ::=                   SEQUENCE {

plmn-IdentityList                       SEQUENCE (SIZE (1..maxPLMN)) OF PLMN-Identity,

trackingAreaCode                        TrackingAreaCode                                            OPTIONAL,       -- Need R

ranac                                   RAN-AreaCode                                                OPTIONAL,       -- Need R

cellIdentity                            CellIdentity,

cellReservedForOperatorUse              ENUMERATED {reserved, notReserved},

...,

[[

iab-Support-r16                     ENUMERATED {true}                                               OPTIONAL       -- Need S

]],

[[

trackingAreaList-r17                SEQUENCE (SIZE (1..maxTAC-r17)) OF TrackingAreaCode             OPTIONAL       -- Need R

]]

}

-- TAG-PLMN-IDENTITYINFOLIST-STOP

-- ASN1STOP

PLMN-IdentityInfo field descriptions

cellReservedForOperatorUse Indicates whether the cell is reserved for operator use (per PLMN), as defined in TS 38.304 [20]. This field is ignored by IAB-MT.

iab-Support This field combines both the support of IAB and the cell status for IAB. If the field is present, the cell supports IAB and the cell is also considered as a candidate for cell (re)selection for IAB-node; if the field is absent, the cell does not support IAB and/or the cell is barred for IAB-node.

trackingAreaCode Indicates Tracking Area Code to which the cell indicated by cellIdentity field belongs. The absence of the field indicates that the cell only supports PSCell/SCell functionality (per PLMN).

trackingAreaList List of Tracking Areas to which the cell indicated by cellIdentity field belongs. If this field is present, the UE shall ignore trackingAreaCode, if present. Total number of TACs across different PLMN-IdentityInfoLists shall not exceed maxTAC.

The following RIL is added which needs discussion

[RIL]: V313 [Delegate]: vivo (Xiao) [WI]: NTN [Class]:2 [Status]: ToDo [TDoc]: None [Proposed Conclusion]: v02

[Description]: Introdction of per-TAC RAN area code configuration when trackingAreaList is configured

[Proposed Change]: In NTN, when the trackingAreaList is configured, it is possible that different TACs included in the list correspond to different RAN area codes. Therefore, the existing ranac included in PLMN-IdentityInfo may not be sufficient in this case.

It is proposed to introduce a per-TAC RAN area code configuration when trackingAreaList is configured, and it can be further determined whether for a given TAC more than one RAN area code can be configured.

It seems that the issue is about that a RANAC is associated with a tracking area (represented by PLMN ID TAC) and the RANAC is unique only within that tracking area, and if only one RANAC is broadcast in the cell while multiple TACs are broadcast in the same cell, then it is ambiguous which TAC the RANAC “belongs” to. And to remove this ambiguity it is proposed to configure a RANAC per broadcast TAC.

However, it is unclear why the broadcast RANAC could not be valid in all the tracking areas whose TACs are broadcast. After all, the same RANAC can be reused in every tracking area.

The supported TA list is signaled from the RAN to the AMF at NG setup and RAN configuration update. The NGAP signaling is as follows:

#### 9.2.6.1         NG SETUP REQUEST

This message is sent by the NG-RAN node to transfer application layer information for an NG-C interface instance.

Direction: NG-RAN node ® AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | M |  | Paging DRX 9.3.1.90 |  | YES | ignore |
| UE Retention Information | O |  | 9.3.1.117 |  | YES | ignore |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |

So, up to 256 TACs can be supported by a gNB, and up to 12 PLMNs can be broadcast per TAC. NGAP does not preclude any relation between the different TACs and different RAN area codes, i.e. no cross-check is specified.

Seems that the proposed per-TAC RAN area code would restrict this possible RAN area code space to avoid duplicates. It should be clarified why would be needed.

When the RANAC alternative is used for the RNA configuration, there is a list TACs and for each of these TACs there is one or more RANAC(s). If the UE can find a TAC broadcast in the cell that matches one of the TACs in its RNA configuration and one of the RANACs associated with that matching TAC in the RNA configuration is equal to the RANAC broadcast in the cell, then the UE would consider the cell as being part of the UE’s RNA.

RAN2 to discuss whether there is any issue to be solved with respect to ranac and TAC list.

## V320 Use of RAN area code with tracking area list

[RIL]: V320 [Delegate]: vivo (Xiao) [WI]: NR_NTN_enh-Core [Class]:1 [Status]: ToDo [TDoc]: R2-22xxxxx [Proposed Conclusion]: v66

[Description]: Erroneous CGI reporting in case tackingAreaList is confiugred.

[Proposed Change]: If the concerned cell configured for CGI reporting includes trackingAreaList (i.e an NTN cell), the procedure here still requires the UE to report the legacy trackingAreaCode. However, in case trackingAreaList is configured, the field description requires the legacy trackingAreaCode to be ignored by the UE, which means that the trackingAreaCode included may be an invalid/useless one. As a result, the existing procedure would lead to incorrect CGI information reported to the network, with the serving cell unable to tell whether the trackingAreaList is also configured, or tell the NW type of the concenred cell. As whether ANR is invovled in NTN or between TN and NTN was not really discussed in earlier meetings, we will bring a separate Tdoc to discuss this issue.

[Comments]: vivo (Xiao) v66: Note that RAN4 agreed the NTN operating bands n256 and 255 (as now captured in 38.101-5) which are respectively overlapped with the the legacy TN operating bands n65 (partially) and n24. This makes it possible that the concerned cell for which CGI reporting is configured by the serving cell is either a TN or an NTN cell (at least on the above frequencies).

When reporting CGI for measurement reports we have the following procedure:

3>  if the cell indicated by cellForWhichToReportCGI is an NR cell:

4>  if plmn-IdentityInfoList of the cgi-Info for the concerned cell has been obtained:

5>  include the plmn-IdentityInfoList including plmn-IdentityList, trackingAreaCode (if available), ranac (if available), cellIdentity and cellReservedForOperatorUse for each entry of the plmn-IdentityInfoList;

5>  include frequencyBandList if available;

4>  if nr-CGI-Reporting-NPN is supported by the UE and npn-IdentityInfoList of the cgi-Info for the concerned cell has been obtained:

5>  include the npn-IdentityInfoList including npn-IdentityList, trackingAreaCode, ranac (if available), cellIdentity and cellReservedForOperatorUse for each entry of the npn-IdentityInfoList;

5>  include cellReservedForOtherUse if available;

4>  else if MIB indicates the SIB1 is not broadcast:

5>  include the noSIB1 including the ssb-SubcarrierOffset and pdcch-ConfigSIB1 obtained from MIB of the concerned cell;

Thus, UE would anyway include trackingarea code only if it is available. For NTN cell it would not be as UE is ignoring it but it is not clear what is the actual issue. RAN2 has not agreed to support CGI reporting in NTN and it is not part of the WID either.

As the work item is closed RAN2 should not add more features, hence we propose to reject the RIL.

RAN2 to reject RIL V320

.

## Location reporting event D1:L011, H801, X704

Couple of RILs were raised in cotext of D1 report

[RIL]: L011 [Delegate]: LGE(SungHoon)  [WI]: NTN [Class]: 2 [Status]: ToDo [TDoc]: None [Proposed Conclusion]:

[Description]: A cell triggering event D1 is not included in the measurement report

[Proposed Change]: In the current formulation, MeasurementReport triggered by event D1 does not include the cell meeting event D1 and its cell. So we propose to add the procedure text to include the cell meeting event D1. There are a couple of ways to enable this, and we think it is most straightforward to include the cell in cellsTriggeredList, as similar to other event cases.

[Comments]:

In the event D1, there is no cell that triggers the event so it is unclear how cells could be added based on the triggering. A related RIL, acknowledges this and proposes to add PCI in the D1:

[RIL]: H801 [Delegate]: Huawei (Lili) [WI]: NTN [Class]: 1 [Status]: ToDo [TDoc]: Yes [Proposed Conclusion]: v167

[Description]: For event D1, there is a reference location of neighbour cell, but the UE does not know which neighbour cell it corresponds to.

In fixed cell scenarios, there is no problem.

However in moving cell scenarios, the UE needs to predict the trajectory of the reference location based on the ephemeris of the neighbour cell. So UE should know which cell the reference location belons to.

[Proposed Change]: Add a PCI in the configuration of event D1 and modify the field description accordingly.

We will submit a Tdoc addressing this issue.

However, it is unclear what is the use of the PCI here. Network knows which location it has configured as ”target cell location” and the event has measID associated. Thus, when report is sent, network knows which event triggered it. Note that it is not actually mandated that the reference location2 is associetd to any actual cell. It is just a location coordinate. Secondly, it should be further elaborated what does the UE do with the information of the PCI.

RAN2 to discuss what is the added benefit of adding PCI to D1.

Yet another RIL is as follows:

[RIL]:X704 [Delegate]: Xiaomi(Yi)  [WI]:NTN [Class]:2 [Status]: ToDo [TDoc]: None [Proposed Conclusion]:

[Description]: Addition of a parameter reportOnLeave-r17

[Proposed Change]: The parameter reportOnLeave-r17 which has been used for many report events, e.g. Event A3, Event A5. But it is not included in Event D1. Event D1 is used for NTN. When UE location satisfies the leaving condition of Event D1, triggering UE to report location and measurement results may be better for reliability of mobility. And the addition of reportOnLeave-r17 enables NW has more control for UE report. Hence, we suggested to add reportOnLeave-r17.

[Comments]:

We would support adding report on leave to event D1

RAN2 to agree to add report on leave for D1

# Conclusion

In the previous sections we made the following observations:

No table of figures entries found.Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to discuss whether there is any issue to be solved with respect to ranac and TAC list.

Proposal 2	RAN2 to reject RIL V320

Proposal 3	RAN2 to discuss what is the added benefit of adding PCI to D1.

Proposal 4	RAN2 to agree to add report on leave for D1

# References

RP-201256, “Solutions for NR to support non-terrestrial networks (NTN),” 3GPP TSG RAN #88e, June 2020.