3GPP TSG-RAN WG2 Meeting #118 electronic	R2-2205580

e-Meeting, 9th May – 20th May 2022

Source:	vivo

Title:          Discussion on the remaining issue about positioning in RRC_INACTIVE

Agenda Item:	6.11.2.1

Document for:	Discussion and Decision

- Introduction

In RAN2 #117e meeting, the following agreements were reached regarding the validation of SRS configuration in RRC_INACTIVE, and the agreements were captured in the RRC CR [1].

Agreements:

Proposal 6: TA timer configuration of SRS for positioning (SRSp) is invalidated upon any cell reselection (i.e. even if the UE does not initiate the RRC resume procedure) (11/12)

Proposal 7: Follow CG-SDT solution for (a) RSRP derivation for positioning SRS TA validation, and (b) definition of stored downlink pathloss reference RSRP value at the very first positioning SRS transmission (12/12)

Besides, RAN2 sent the LS [2] to SA2 to decide the spec impacts on Low Power Periodic and Triggered 5GC-MT-LR Procedures with SDT and SA2 send the reply LS in [3]

In this contribution, we further discuss the remaining issues to support positioning in RRC_INACTIVE.

- Handling of SRS configuration upon cell re-selection

- SRS configuration request from UE to LMF

- Discussion

  - Handling of SRS configuration upon cell re-selection

The procedure of the SRS configuration for RRC_INACTIVE during RRC resume is captured as follows:

1> if the UE performs connection resumption in a different cell than the cell where srs-PosRRC-InactiveConfig was configured; 2> release srs-PosRRC-InactiveConfig, if configured; Editor's Note: This agreement/clause may not be needed because we have this agreement TA timer configuration is invalidated upon any cell reselection; see 5.7.15.

#### 5.7.15.2 Actions Related to SRS for Positioning at Cell Re-selection in RRC_INACTIVE The UE shall: 1> if cell reselection occurs when srs-PosRRC-InactiveConfig is configured: 2> consider the Timing Advance value for SRS for Positioning transmission to be invalid; 2> release the srs-PosRRC-InactiveConfig.

From the above description, in RRC_INACTIVE Mode, the UE should release the SRS configuration for positioning upon cell re-selection. That is, the TA value should be discarded and the corresponding SRS configuration for positioning should be released once the cell re-selection occurs. As a consequence, the procedure to release the SRS configuration when performing RRC resumption in a different cell will never happen. We hence agree with the EN that the occasion in Section 5.3.13.2 is just covered by Section 5.7.15.2.

Proposal 1: Remove the description of the UE behavior when performing connection resumption in a different cell than the cell where srs-PosRRC-InactiveConfig was configured.

However, in the current CRs, the procedures between RRC and MAC are not aligned to stop the srs-TimeAlignmentTimer, which should be clarified as follows:

#### 5.7.15.2 Actions Related to SRS for Positioning at Cell Re-selection in RRC_INACTIVE The UE shall: 1> if cell reselection occurs when srs-PosRRC-InactiveConfig is configured: 2> consider the Timing Advance value for SRS for Positioning transmission to be invalid; 2> instruct MAC to stop the srs-TimeAlignmentTimer; 2> release the srs-PosRRC-InactiveConfig.

Proposal 2: Add the description of the UE behavior upon cell reselection, i.e., to instruct MAC to stop the srs-TimeAlignmentTimer.

  - SRS configuration request from UE

SA2 discussed the positioning in RRC_INACTIVE and captured three separate procedures in TS 23.273 to support each of UL positioning, DL positioning and UL+DL positioning [4].

Figure1: Low Power Periodic and Triggered 5GC-MT-LR Procedure in RRC INACTIVE state with UL+DL Positioning

For the UL+DL positioning, in step 4, the UE includes an LPP positioning message in the supplementary services event report message that includes a request for a UL Configuration to support the UL+DL positioning method indicated to the UE in step 1. Based on the request, in step 7, the LMF uses the Network Assisted Positioning Procedure to request the receiving gNB node to provide a UL configuration to the UE in step 11.

The specific UL configuration request is as follws:

NR-Multi-RTT-RequestAssistanceData-r16 ::= SEQUENCE { nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, nr-AdType-r16 BIT STRING { dl-prs (0), ul-srs (1) } (SIZE (1..8)), ..., [[ nr-on-demand-DL-PRS-Request-r17 NR-On-Demand-DL-PRS-Request-r17 OPTIONAL ]] }

Figure 2: Low Power Periodic and Triggered 5GC-MT-LR Procedure in RRC INACTIVE state with UL Positioning

For the UL positioning, step 4, the UE does not include an LPP positioning message in the supplementary services event report message. However, the LMF still needs this request information as the LMF needs to request the receiving gNB node to provide a UL configuration to the UE in step 7.

Proposal 3: The UE should be able to request UL configuration for positioning in RRC_INACTIVE when the previous configuration turns invalid.

One simple way is to add a new nr-UL-RequestAssistanceData IE in the RequestAssistanceData.

RequestAssistanceData-r9-IEs ::= SEQUENCE { commonIEsRequestAssistanceData CommonIEsRequestAssistanceData OPTIONAL, a-gnss-RequestAssistanceData A-GNSS-RequestAssistanceData OPTIONAL, otdoa-RequestAssistanceData OTDOA-RequestAssistanceData OPTIONAL, epdu-RequestAssistanceData EPDU-Sequence OPTIONAL, ..., [[ sensor-RequestAssistanceData-r14 Sensor-RequestAssistanceData-r14 OPTIONAL, tbs-RequestAssistanceData-r14 TBS-RequestAssistanceData-r14 OPTIONAL, wlan-RequestAssistanceData-r14 WLAN-RequestAssistanceData-r14 OPTIONAL ]], [[ nr-Multi-RTT-RequestAssistanceData-r16 NR-Multi-RTT-RequestAssistanceData-r16 OPTIONAL, nr-DL-AoD-RequestAssistanceData-r16 NR-DL-AoD-RequestAssistanceData-r16 OPTIONAL, nr-DL-TDOA-RequestAssistanceData-r16 NR-DL-TDOA-RequestAssistanceData-r16 OPTIONAL ]] [[ nr-UL-RequestAssistanceData-r17 NR-UL-RequestAssistanceData-r17 OPTIONAL, ]] } NR-UL-RequestAssistanceData-r17 ::= SEQUENCE { nr-PhysCellID-r16 NR-PhysCellID-r16 OPTIONAL, nr-AdType-r16 BIT STRING { ul-srs (0) } (SIZE (1..8)), ..., }

Proposal 4: Add a new nr-UL-RequestAssistanceData IE in the RequestAssistanceData.

- Conclusion

Proposal 1: Remove the description of the UE behavior when performing connection resumption in a different cell than the cell where srs-PosRRC-InactiveConfig was configured.

Proposal 2: Add the description of the UE behavior upon cell reselection, i.e., to instruct MAC to stop the srs-TimeAlignmentTimer.

Proposal 3: The UE should be able to request UL configuration for positioning in RRC_INACTIVE when the previous configuration turns invalid.

Proposal 4: Add a new nr-UL-RequestAssistanceData IE in the RequestAssistanceData.

- Reference

- R2-2204242	Introduction of Enhanced Positioning feature

- R2-2203949	LS on Positioning in RRC_INACTIVE State

- R2-2204521	Reply LS on Positioning in RRC_INACTIVE State

- S2-2203251	Periodic and Triggered 5GC-MT-LR Procedure in RRC INACTIVE state