3GPP TSG-RAN WG2 Meeting # 118 Electronic	R2-2205652

Online, May 09 – May 20, 2022

Agenda Item:	[6.3.3] [NW switching for multi-SIM without leaving RRC_CONNECTED]

Source: 	Apple Inc

Title:  	Additional issues related to MUSIM - Aspects of RRC Processing Delay and Caller ID retrieval requirements

Document for:	Discussion and Decision

# Introduction

Multi-SIM devices have proven to be very popular in certain cellular markets owing to the flexibility it provides to the user to manage multiple SIM subscriptions in the same physical device. Different flavours of multi-SIM implementation are commercially available, based on the Radio Access Technology combinations that each SIM instance can support, or based on the number of concurrent Transmit and/or Receive chains that are possible to be handled.

Until 3GPP R17, commercially available multi-SIM solutions are primarily based of UE proprietary implementations, leading to varied UE side behaviour and inconsistent user experience. A new work item on standardizing multi-SIM behaviour was approved [1] and this work item covers one of the important aspects of multi-SIM which deals with network switching of multi-SIM devices.

As indicated before there can be multiple flavours of multi-SIM implementation. Specifically based on the number of concurrent Transmit and/or Receive chains that are possible, the following combinations are most prevalent UE architecture for multi-SIM. Note for the case of generality the paper refers to multi-SIM, but commercial UE implementations typically have two SIM instances (Dual-SIM).

- Single-Rx / Single-Tx

  - One single Rx/Tx that is shared across the two SIM instances, allowing the MUSIM UE to receive traffic from only one network at a time and transmit traffic to only one network at a time.

- Dual-Rx / Single-Tx

  - Each SIM instance has its own dedicated Rx, and One single Tx chain that is shared across the two SIM instances, thereby allowing the MUSIM UE to receive traffic from both networks at a time and transmit traffic to only one network at a time.

- Dual-Rx / Dual-Tx

  - Each SIM instance has its own dedicated Rx and Tx, thereby allowing the MUSIM UE to transmit and receive traffic to their respective networks simultaneously.

In this contribution, we would like to highlight certain issues related to

- MUSIM RRC Processing Delay Requirements

- MUSIM Caller ID Retrieval Requirements

# Discussion

## MUSIM RRC Processing Delay Requirements

Current LTE and NR RRC specifications [1][2], provide an upper limit on the RRC Processing Delay to specify UE performance requirements for RRC procedures. These processing delay requirements are intended for the UE doing only one RRC procedure at a time.

Observation 1: Current LTE and NR RRC Processing Delay Requirements are meant for single SIM cases, wherein only one RRC procedure is expected to run at any given instance of time.

In a MUSIM UE scenario, there can be multiple UE RRC and each RRC can independently be in RRC CONNECTED or RRC IDLE or RRC INACTIVE modes, depending on the current use case. In such cases, it is possible that a given RRC procedure on a given SIM instance can be pre-empted due to a higher priority RRC procedure on the other SIM instance (e.g.) Network Switching for instance.

In these circumstances it is not clear whether the MUSIM UE should consider that,

- The current RRC Processing Delay Requirement mentioned in [1] [2] is applicable for MUSIM cases as well or

- The RRC Processing Delay Requirement for MUSIM UEs to be adapted accordingly based on the number of concurrent SIM instances, and if so by how much.

Consider a use case wherein a MUSIM UE is in RRC CONNECTED state on NW A, and is on RRC IDLE/INACTIVE on NW B. As part of the Network Switching work item for R17 MUSIM, it is currently being discussed to use Scheduling Gap for Periodic and Aperiodic switching from NW A to NW B, while maintaining the RRC CONNECTED state on NW A. So, during the time the MUSIM UE switches to the scheduling gap to carry out any protocol activity on NW B, its current CONNECTED mode activity on NW A would be paused. This might include RRC Signalling handling of any ongoing activity on NW A. As seen in the figure below, the RRCReconfiguration handling in RRC CONNECTED state on NW A gets interrupted for Gap duration G1 and G2, when the MUSIM UE switches over to NW B. This would reduce the total RRC processing duration available on NW A by at least G1+G2 units of time.

Observation 2: MUSIM UEs can have an ongoing RRC procedure pre-empted due to a concurrent MUSIM use case on the other SIM instance, and this might result in very tight RRC processing deadline on the SIM instance where it got pre-empted out.

Proposal 1: Clarify in LTE and RRC specifications for Release-17 that the existing RRC Processing Delay requirements is applicable only for UE operating in Single-SIM mode and is NOT applicable for RRC procedures for UE’s operating in MUSIM mode of operation.

Proposal 2: RAN2 to further study the RRC Processing Delay Requirements for MUSIM UEs based on the solutions agreed for the other MUSIM WI objective (Paging Collision, Network Switching, Busy Indication etc.)

## MUSIM Caller ID Retrieval Requirements

Consider a Dual Rx/Single Tx MUSIM UE, wherein the first MUSIM instance (A) is in RRC CONNECTED state, and at the same time, the second MUSIM instance (B) receives an incoming page for a MT voice call. Based on the recent agreements in SA2 and RAN2, it is possible for the MUSIM UE to understand if the incoming page is for a MT voice call, thanks to the newly introduced paging cause for voice (R17 onwards). With the understanding that the incoming page is for a MT voice call, the MUSIM UE on SIM instance B, has to setup the RRC Connection, after indicating a NW switching via SIM instance A, do the SIP signalling (atleast upto the INVITE) to acquire the incoming caller ID on SIM instance B.

In the process of setting up the RRC Connection on SIM instance B, if the NW configures a higher number of Rx/Tx chains (including higher number of Component Carriers, or configures Dual Connectivity), it is going to severely impact the ongoing RRC Connection on SIM instance A, given that the total number of Rx/Tx chains are limited to a finite number, and SIM instance A is already using the majority of such chains for its ongoing CONNECTED state activity.

Observation 3: In Dual Rx/Single Tx MUSIM UE, initial Connected state configuration on the second SIM instance to retrieve the caller ID can impact the ongoing Connected state configuration on the first SIM instance.

Additionally, if the user agrees to accept the incoming call on MUSIM instance (B), there needs to be a signalling flow defined to indicate a quick RRC Connection Release on the MUSIM instance (A), as otherwise it will lead to resource wastage given that the MUSIM UE is not going to continue with the original Connected mode activity on MUSIM instance (A). If the MUSIM UE does not gracefully release the pre-existing RRC Connection on MUSIM instance (A), the NW would potentially release the RRC Connection after a pre-defined RRC tail time, leading to wastage of radio resources.

Observation 4: In Dual Rx/Single Tx MUSIM UE, it is important to signal to the first MUSIM instance about graceful RRC Connection Release while the user accepts the incoming MT call on the second MUSIM instance, to avoid radio resource wastage.

Proposal 3: RAN2 to consider the problem statements for MUSIM UEs related to caller ID identification and optimal signalling to ensure faster RRC Connection Release with the intent to avoid radio resource wastage.

# Conclusion

Observation 1: Current LTE and NR RRC Processing Delay Requirements are meant for single SIM cases, wherein only one RRC procedure is expected to run at any given instance of time.

Observation 2: MUSIM UEs can have an ongoing RRC procedure pre-empted due to a concurrent MUSIM use case, and this might result in very tight RRC processing deadline.

Proposal 1: Clarify in LTE and RRC specifications for Release-17 that the existing RRC Processing Delay requirements is applicable only for UE operating in Single-SIM mode and is NOT applicable for RRC procedures for UE’s operating in MUSIM mode of operation.

Proposal 2: RAN2 to further study the RRC Processing Delay Requirements for MUSIM UEs based on the solutions agreed for the other MUSIM WI objective (Paging Collision, Network Switching, Busy Indication etc.)

Observation 3: In Dual Rx/Single Tx MUSIM UE, initial Connected state configuration on the second SIM instance to retrieve the caller ID can impact the ongoing Connected state configuration on the first SIM instance.

Observation 4: In Dual Rx/Single Tx MUSIM UE, it is important to signal to the first MUSIM instance about graceful RRC Connection Release while the user accepts the incoming MT call on the second MUSIM instance, to avoid radio resource wastage.

Proposal 3: RAN2 to consider the problem statements for MUSIM UEs related to caller ID identification and optimal signalling to ensure faster RRC Connection Release with the intent to avoid radio resource wastage.

# Reference

[1] 3GPP TS 36.331 V16.3.0 (2021-01) – EUTRA: RRC Protocol Specification Release 16

[2] 3GPP TS 38.331 V16.3.1 (2021-01) – NR: RRC Protocol Specification Release 16