3GPP TSG-RAN2 Meeting #118-e	R2-2205659

# Online, 9th May – 20th May, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1279 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on FR2 UL gap |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh2 |  |  |  |  |  | Date: |  |  | 2022-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | R17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | RAN4 agreed with the following UE behavior in FR2 UL gap in R4-2206511. During UL gaps, except for the signals used for random access procedure according to TS 38.321, CG-PUSCH (type 1 and 2) and PUCCH allocations for SR and LRR [and for the signals used for other RAN4 agreed procedures], UE is not required to conduct transmission to the corresponding NR serving cells in FR2 single CC, intra-band CA. For inter-band FR2-FR2 CA/DC, UE may or may not be required to conduct transmission to the corresponding NR serving cells based on UE capability whether UL transmission within a gap is feasible. Note that CR 1191 (R2-2204231) already captured that RACH procedure is prioritized during FR2 UL gap. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture that during FR2 UL gap, if UE does not support UL Tx transmission, UE should not perform certain actions. CG-PUSCH, PUCCH for SR and LRR are the exceptions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It’s absent how MAC handles the UL transmission during FR2 UL gap. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.xx (new section) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Changes

## 5.xx	Handling of FR2 UL gap

During the FR2 UL gap configured by ul-GapFR2-Config as specified in TS 38.331 [5], the MAC entity shall, on the Serving Cell(s) of FR2 single CC and intra-band CA, or on the Serving Cell(s) in FR2 band(s) where UE does not support UL Tx transmission within FR2 UL gap:

1>	not perform the transmission of HARQ feedback, and CSI;

1>	not report SRS;

1>	not transmit on UL-SCH except for configured grant and Msg3 or the MSGA payload as specified in clause 5.4.2.2;