3GPP TSG-RAN WG2 #118                                                                                         R2-2205666

eMeeting, 09th May – 20th May, 2022

Agenda Item:	    6.24.1

Source: 	    Apple

Title:		    Summary of [Pre118-e][004][NR17 FR2 UL Gap] 38331 CR and rapporteur resolutions (Apple)

Document for:	    Discussion and decision

# 1 Introduction

The paper discusses the FR2 UL gap related RIL issues (based on v149) and Class0 editorial issues (based on v57).

1) RIL(s): Z151, Z152, A803, A804, A807, A808, N129

2) Class 0 issues: 155, 156, 193, 425

# 2 Discussion

## 2.1 RIL(s)

[RIL]: Z151 [Delegate]: ZTE (LiuJing) [WI]: ULGAP [Class]: 2 [Status]: ToDo [TDoc]: xxx [Proposed Conclusion]:

[Description]: Based on the gap patterns defined by RAN4, we think it is not necessary to report multiple gap patterns. In addition, from network perspective, it is unclear about UE’s expectation if multiple patterns are reported. If the UE can accept gap pattern #0 and #1, the UE can indicate preference for pattern #1 only. It is up to network to decide whether to configure pattern #0 instead. And if the UE can only accept #0(with smaller periodicity), the UE can indicate preference for pattern#0 only, there is no need to include other patterns. So instead of a bitmap, we prefer UE to indicate its preference for a given gap pattern.

|  | UGL [ms] | UGRP [ms] |
| --- | --- | --- |
| UL MGP #0 | 1.0 | 20 |
| UL MGP #1 | 1.0 | 40 |
| UL MGP #2 | 0.5 | 160 |
| UL MGP #3 | 0.125 when SCS of active UL BWP =120kHz 0.25 when SCS of active UL BWP =60kHz | 5 |

[Proposed Change]: Change “BIT STRING (SIZE (4))” into “INTEGER (0..3)”.

[Comments]: [Apple (Yuqin Chen)]: As FR2 UL gap rapporteur, mark this RIL as propAgree.

[RIL]: Z152 [Delegate]: ZTE (LiuJing) [WI]: ULGAP [Class]: 1 [Status]: ToDo [TDoc]: xxx [Proposed Conclusion]:

[Description]: “configures FR2 bands to UE” is a bit misleading, it can be FR2 serving cells or RRM measured FR2 target bands.

[Proposed Change]: Suggest to change the sentence into “the network (MN or SN) which configured with FR2 serving cell(s) decides and configures the FR2 UL gap pattern.”.

[Comments]: [Apple (Yuqin Chen)]: As FR2 UL gap rapporteur, mark this RIL as propAgree.

[RIL]:  A803 [Delegate]: Apple (Yuqin)  [Class]: Class 1 [WI]: ULGAP

[Description]: Missing the release of ul-GapFR2-PreferenceConfig.

[Proposed Change]: Add one sentence above:

2> release ul-GapFR2-PreferenceConfig, if configured;

[Comments]: [Apple (Yuqin Chen)]: As FR2 UL gap rapporteur, mark this RIL as propAgree.

[RIL]:  A804 [Delegate]: Apple (Yuqin)  [WI]: ULGAP, MUSIM

[Description]: Missing the release of ul-GapFR2-PreferenceConfig, musim-GapAssistanceConfig and musim-LeaveAssistanceConfig

[Proposed Change]: Add sentences above:

3>	release musim-GapAssistanceConfig, if configured and stop timer T3yy, if running;

3>	release musim-LeaveAssistanceConfig, if configured;

3> release ul-GapFR2-PreferenceConfig, if configured;

[Proposed Conclusion]: [Apple (Yuqin Chen)]: As FR2 UL gap rapporteur, mark this RIL as propAgree. MUSIM change is left to MUSIM WI.

[RIL]:  A807 [Delegate]: Apple (Yuqin) [WI]: ULGap

[Description]: Missing the release of FR2 gap configuration.

[Proposed Change]: Add one sentence below:

1> release the FR2 UL gap configuration indicated by the ul-GapFR2-Config, if configured;

[Proposed Conclusion]: [Apple (Yuqin Chen)]: As FR2 UL gap rapporteur, mark this RIL as propAgree.

[RIL]:  A808 [Delegate]: Apple (Yuqin)  [WI]: ULGAP

[Description]: Missing the release of ul-GapFR2-PreferenceConfig.

[Proposed Change]: Add one sentence above:

2> release ul-GapFR2-PreferenceConfig, if configured;

[Proposed Conclusion]: [Apple (Yuqin Chen)]: As FR2 UL gap rapporteur, mark this RIL as propAgree.

[RIL]: N129 [Delegate]: Nokia(Tero)  [WI]: ULGap [Class]: 2 [Status]: ToDo [TDoc]: Yes [Proposed Conclusion]:

[Description]: How does UE release the preference, i.e. indicate that it no longer needs the gap?

[Proposed Change]: We will provide RAN2#118e Tdoc on this.

[Comments]:

[Proposed Conclusion]: [Apple (Yuqin Chen)]: As FR2 UL gap rapporteur, mark this RIL as Discuss (At meeting). For information, this was discussed in post #117 meeting email [Post117-e][058][NR17] FR2 UL Gap (Apple). Companies preferred to follow the logic in UAI that absence of a field indicates “no preference”. But there is no harm to discuss it further.

## 2.2 Class0 issues

| 155 | Y | RRCReconfiguration-v1700-IEs ::= SEQUENCE { otherConfig-v1700 OtherConfig-v1700 OPTIONAL, -- Need M ul-GapFR2-Config-r17 SetupRelease { UL-GapFR2-Config-r17 } OPTIONAL, -- Need M sl-L2RelayUEConfig-r17 SetupRelease { SL-L2RelayUEConfig-r17 } OPTIONAL, -- Cond L2RelayUE sl-L2RemoteUEConfig-r17 SetupRelease { SL-L2RemoteUEConfig-r17 } OPTIONAL, -- Cond L2RemoteUE dedicatedPagingDelivery-r17 OCTET STRING (CONTAINING Paging) OPTIONAL, -- L2U2NRelay needForNCSG-ConfigNR-r17 SetupRelease {NeedForNCSG-ConfigNR-r17} OPTIONAL, -- Need M needForNCSG-ConfigEUTRA-r17 SetupRelease {NeedForNCSG-ConfigEUTRA-r17} OPTIONAL, -- Need M musim-GapConfig-r17 SetupRelease {MUSIM-GapConfig-r17} OPTIONAL, -- Need M scg-State-r17 ENUMERATED { deactivated } OPTIONAL, -- Need S appLayerMeasConfig-r17 AppLayerMeasConfig-r17 OPTIONAL, -- Need M nonCriticalExtension SEQUENCE {} OPTIONAL } | NCSG, UL FR2 gap, MUSIM gap are all “gap” related features, so suggest to put those IEs together. RRCReconfiguration-v1700-IEs ::= SEQUENCE { otherConfig-v1700 OtherConfig-v1700 OPTIONAL, -- Need M ul-GapFR2-Config-r17 SetupRelease { UL-GapFR2-Config-r17 } OPTIONAL, -- Need M sl-L2RelayUEConfig-r17 SetupRelease { SL-L2RelayUEConfig-r17 } OPTIONAL, -- Cond L2RelayUE sl-L2RemoteUEConfig-r17 SetupRelease { SL-L2RemoteUEConfig-r17 } OPTIONAL, -- Cond L2RemoteUE dedicatedPagingDelivery-r17 OCTET STRING (CONTAINING Paging) OPTIONAL, -- L2U2NRelay needForNCSG-ConfigNR-r17 SetupRelease {NeedForNCSG-ConfigNR-r17} OPTIONAL, -- Need M needForNCSG-ConfigEUTRA-r17 SetupRelease {NeedForNCSG-ConfigEUTRA-r17} OPTIONAL, -- Need M musim-GapConfig-r17 SetupRelease {MUSIM-GapConfig-r17} OPTIONAL, -- Need M ul-GapFR2-Config-r17 SetupRelease { UL-GapFR2-Config-r17 } OPTIONAL, -- Need M scg-State-r17 ENUMERATED { deactivated } OPTIONAL, -- Need S appLayerMeasConfig-r17 AppLayerMeasConfig-r17 OPTIONAL, -- Need M nonCriticalExtension SEQUENCE {} OPTIONAL } | liu.jing30@zte.com.cn |
| --- | --- | --- | --- | --- |
| 156 | N | #### 5.3.5.15 FR2 UL gap configuration The UE shall: 1> if ul-GapFR2-Config is set to setup: 2> if an FR2 UL gap configuration is already setup, release the FR2 UL gap configuration; 2> setup the FR2 UL gap configuration indicated by the ul-GapFR2-Config in accordance with the received gapOffset, i.e., the first subframe of each gap occurs at an SFN and subframe meeting the following condition: SFN mod T = FLOOR (gapOffse/10); if the UGRP is larger than 5ms: subframe = gapOffset mod 10; else: subframe = gapOffset or (gapOffset +5); with T = CEIL(UGRP/10). 1> else if ul-GapFR2-Config is set to release: 2> release the FR2 UL gap configuration. | gapOffse => gapOffset ul-GapFR2-Config and gapOffset in section 5.3.5.15 should be italic. | liu.jing30@zte.com.cn |
| 193 | Y | UL-GapFR2-Config-r17 ::= SEQUENCE { gapOffset-r17 INTEGER (0..159), ugl-r17 ENUMERATED {ms0dot125, ms0dot25, ms0dot5, ms1}, ugrp-r17 ENUMERATED {ms5, ms20, ms40, ms160}, refFR2ServCellAsyncCA-r17 ServCellIndex OPTIONAL –- Cond AsyncCA } | Missing hyphen, should be refFR2-ServCellAsyncCA-r17 | nathan.tenny@mediatek.com |
| 425 | N | RRCReconfiguration-IEs field descriptions: There is a redundant suffix in the name ul-GapFR2-Config-r17. ul-GapFR2-Config-r17 Indicates the FR2 UL gap configuration to UE. In EN-DC and NGEN-DC, the SN decides and configures the FR2 UL gap pattern. | Remove suffix “-r17” of name ul-GapFR2-Config-r17. | hchoi5@lenovo.com |

Rapporteur resolution:

- 155, 156, 425 are marked propAgree.

- 193: The field name was used in Rel-16 for legacy FR2 gap configuration in async CA. It’s not very clear if we introduce a new field name, whether any unnecessary risks would be caused due to the similarity between two fields (one for Rel-17 and one for Rel-16). Would like to hear views from companies.

## 2.3 Discussions

Question 1: Companies are invited to provide comment on Class0 issue 193, whether to change refFR2ServCellAsyncCA-r17 to refFR2-ServCellAsyncCA-r17?

| Company | Yes / No | Comments |
| --- | --- | --- |

Question 2: Please companies indicate if they have comments on how to handle the RIL(s) and Class0 issues.

| Company | RIL No. / Class 0 No. | Comments |
| --- | --- | --- |

# 3 Conclusions

Proposal 1: Agree the following RIL(s) and Class 0 issues and reflect them in rapporteur CR.

- RIL: Z151, Z152, A803, A804, A807, A808

- Class 0 issues: 155, 156, 193, 425

Proposal 2: Discuss RIL N129 in RAN2#118 meeting.

# 4 References

[1]