3GPP TSG-RAN WG2 Meeting #118-e	R2-2205684

e-Meeting, 09 May – 20 May 2022

Agenda item:	6.0.1

Source:	Apple

Title:	Discussion on ul-AccessConfigListDCI (RIL A402, A405)

WID/SID:	NR_IIOT_URLLC_enh-Core, NR_ext_to_71GHz-Core

Document for:	Discussion / Agreement

# Introduction

A list of combinations of cyclic prefix extension and UL channel access types is sent in ul-AccessConfigListDCI for NR, as defined in TS 38.331 [4].

PUCCH-Config field descriptions

ul-AccessConfigListDCI-1-1, ul-AccessConfigListDCI-1-2 List of the combinations of cyclic prefix extension and UL channel access type (See TS 38.212 [17], Clause 7.3.1) applicable, respectively, to DCI format 1_1 and DCI format 1_2. The field ul-AccessConfigListDCI-1-1-r17 indicates a list which only contains UL channel access types and is only applicable for FR2-2 (see TS 38.212 [17], Table 7.3.1.2.2-6A).

The following ASN.1 review comment was raised for this:

[RIL]: A402 [Delegate]: Apple (Ralf)  [WI]: MULTI [Class]: 1 [Status]: ToDo [TDoc]: None [Proposed Conclusion]:

[Description]: The field description is not easily accessible.

[Proposed Change]: If an additional description is needed for R17 the meaning of the ul-AccessConfigListDCI-1-1 field in R16 should be explained as well.

[Comments]:

Later during the review process, in v175 of the ASN.1 review file, a similar RIL was added in A405 also for the field description of ul-AccessConfigListDCI-0-1, ul-AccessConfigListDCI-0-2, for consistency with DCI formats 0_1 and 0_2. We will treat this case here as well.

# Discussion

The field description of ul-AccessConfigListDCI-1-1 and ul-AccessConfigListDCI-1-2 got updated by multiple Rel-17 work items for slightly different but related aspects

- ul-AccessConfigListDCI-1-2-r17 was introduced for URLLC/IIoT

- ul-AccessConfigListDCI-1-1-r17 was introduced for NR_ext_to_71GHz

The different parts of changes added in Rel-17 are shown below.

The description of functionality added in Rel-17 vs the description of existing functionality added in Rel-16 has become ambiguous, and different interpretations are possible.

- Interpretation 1: The straightforward interpretation is that the first sentence applies to both R16 and R17 except for ul-AccessConfigListDCI-1-1-r17, which is explained by the second sentence.

- Interpretation 2: The field ul-AccessConfigListDCI-1-2-r17 indicates a list which only contains UL channel access types and is only applicable for FR1 (see TS 38.212, Table 7.3.1.2.2-6).

- Interpretation 3: The fields ul-AccessConfigListDCI-1-1-r16 and ul-AccessConfigListDCI-1-2-r17 is only applicable for FR1 (see TS 38.212, Table 7.3.1.2.2-6).

In fact the field description is incomplete as Rel-17 not only added additional fields but also repurposed the field ul-AccessConfigListDCI-1-1-r16 to be used in FR1 only. The differences as to which fields can be used in FR1 and which field can be used in FR2-2 are not currently visible from the field description.

In order to understand the changes, the RAN1 CRs in [1] and [2] can be used. It can be seen that the field ul-AccessConfigListDCI-1-1-r16 got redefined to be used in FR1 (previously there was no limitation on frequency range). In addition, ul-AccessConfigListDCI-1-2-r17 was added for exclusive usage in FR1 as well [3]. The field ul-AccessConfigListDCI-1-1-r17 is defined to be used in FR2-2 only.

It is therefore proposed to extend the field description. In our understanding, the contents should identify relevant differences between Rel-16 and Rel-17 or indicate which fields are intended for FR1 and which fields are intended for FR2-2.

Proposal 1: Update the field description of ul-AccessConfigListDCI-1-1, ul-AccessConfigListDCI-1-2 to indicate fields that are intended for FR1 and fields that are intended for FR2-2.

A text proposal is provided in the annex in section 5.1.

Moreover, an inspection of the field descriptions in ul-AccessConfigListDCI-0-1, ul-AccessConfigListDCI-0-2 for the corresponding DCI formats for scheduling of PUSCH in DCI format 0_1 and 0_2 shows that the same problem exists for these descriptions as well. It is therefore proposed to align the field description of these fields in a similar manner.

Proposal 2: Update the field description of ul-AccessConfigListDCI-0-1, ul-AccessConfigListDCI-0-2 to indicate fields that are intended for FR1 and fields that are intended for FR2-2.

A text proposal is provided in the annex in section 5.2.

Proposal 3: RAN2 adopts the text proposals in R2-2205684 as a baseline.

# Conclusions

This contribution discusses the consistency of field description for the ul-AccessConfigListDCI parameters. It was found that a few updates are missing. We have the following proposals:

Proposal 1: Update the field description of ul-AccessConfigListDCI-1-1, ul-AccessConfigListDCI-1-2 to indicate fields that are intended for FR1 and fields that are intended for FR2-2.

Proposal 2: Update the field description of ul-AccessConfigListDCI-0-1, ul-AccessConfigListDCI-0-2 to indicate fields that are intended for FR1 and fields that are intended for FR2-2.

Proposal 3: RAN2 adopts the text proposals in R2-2205684 as a baseline.

# References

[1] R1-2112944, Introduction of Rel-17 enhanced IIoT and URLLC, TS 38.212, CR0092

[2] R1-2112937, Introduction of features to extend current NR operation to 71 GHz, TS 38.212, CR0085

[3] TS 38.212, v17.1.0

[4] TS 38.331, v17.0.0

# Annex with TPs

This section provides two text proposals for uplink and downlink, respectively. The TPs in this section are based on the Rel-17 version of TS 38.331 in v17.0.0 [4].

## Text Proposal 1

START OF CHANGES

### 6.3.2	Radio resource control information elements

< Unchanged parts omitted >

PUCCH-Config field descriptions

ul-AccessConfigListDCI-1-1, ul-AccessConfigListDCI-1-2 List of the combinations of cyclic prefix extension and UL channel access type (see TS 38.212 [17], clause 7.3.1) applicable, respectively, to DCI format 1_1 and DCI format 1_2. The fields ul-AccessConfigListDCI-1-1-r16 and ul-AccessConfigListDCI-1-2-r17 are only applicable for FR1 (see TS 38.212 [17], Table 7.3.1.2.2-6). The field ul-AccessConfigListDCI-1-1-r17 indicates a list which only contains UL channel access types and is only applicable for FR2-2 (see TS 38.212 [17], Table 7.3.1.2.2-6A).

< Unchanged parts omitted >

END OF CHANGES

## Text Proposal 2

START OF CHANGES

### 6.3.2	Radio resource control information elements

< Unchanged parts omitted >

PUSCH-Config field descriptions

ul-AccessConfigListDCI-0-1, ul-AccessConfigListDCI-0-2 List of the combinations of cyclic prefix extension, channel access priority class (CAPC), and UL channel access type (see TS 38.212 [17], clause 7.3.1) applicable for DCI format 0_1 and DCI format 0_2, respectively. The fields ul-AccessConfigListDCI-0-1-r16 and ul-AccessConfigListDCI-0-2-r17 are only applicable for FR1 (see TS 38.212 [17], Table 7.3.1.1.2-35). The field ul-AccessConfigListDCI-0-1-r17 only contains a list of UL channel access types and is only applicable for FR2-2 (see TS 38.212 [17], Table 7.3.1.1.2-35A).

< Unchanged parts omitted >

END OF CHANGES