3GPP TSG-RAN WG2 Meeting #118 Electronic	R2-2205698

Online, May 09 - May 20, 2022

Agenda item:	6.10.3.2.1

Source:	Samsung

Title:	Discussion on SMTC open issues

WID/SID:	NR_NTN_solutions-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

In RAN2-117e meeting, agreements on SMTC were made as follows.

Agreement:

- RAN2 assumes that in addition to the ephemeris information, assistance information is needed for UE-based SMTC adjustment in idle and inactive mode. (FFS on the option to enable this).

- SMTC offset and change rate is needed to assist UE-based SMTC adjustment in idle and inactive mode (FFS on the signalling details, e.g. whether to broadcast feeder link delay difference or something different).

- It’s up to network implementation to guarantee that the measurement gaps can cover all SMTCs configured for one frequency layer in gap-assisted scenarios.

In this document, we discuss the details of assistance information signaling, including SMTC offset and change rate, for UE-based SMTC adjustment in idle and inactive mode.

# 2	Discussion

In NTN, the varying propagation delay differences from serving cell and neighbor cells results in varying offsets for SSB measurements. In order to enable UE in idle/inactive state to measure SSBs properly, the varying propagation delay difference has to be provided in SMTC configuration in system information.

The propagation delay consists of the service link delay and the feeder link delay. Note that the service link delay and the feeder link delay does not necessarily refer to the UE specific TA and common TA, respectively. It depends on where the reference location is set by the network to determine the common TA. The service link delay of the serving/neighbor cell can be determined by the UE based on its location information and the ephemeris information broadcasted in the serving/neighbor cell system information. Then, the service link delay difference between the serving cell and a neighbor cell can be determined at a certain time instance t, which is denoted ds(t).

To determine the propagation delay difference, the UE needs the assistance information of feeder link delay difference. That means the feeder link delay difference between the serving cell and each neighbor cell to be measured need to be provided in the serving cell system information, which is denoted df. Since the propagation delay difference is changing over time, the timestamp of the feeder link delay difference, i.e. the epoch time tepoch, and the change rate of the feeder link delay difference, denoted rdf, need to be broadcasted associating with the feeder link delay difference. Given the assistance information, the UE can calculate the propagation delay difference at time instance t between the serving cell and a neighbor cell by Ot=dst+dftepoch+rdf(t-tepoch), which can be used to determine the timing difference between serving cell SSB and neighbor cell SSB. The epoch time of feeder link delay difference may or may not be the same as the epoch time of serving cell ephemeris, which can be up to the network implementation.

Proposal 1: In the serving cell system information, the feeder link delay difference at an epoch time between the serving cell and each neighbour cell needs to be provided.

Proposal 2: The change rate of feeder link delay difference between the serving cell and each neighbour cell needs to be provided.

# 3	Conclusion

Based on the discussion, we have the following proposal.

Proposal 1: In the serving cell system information, the feeder link delay difference at an epoch time between the serving cell and each neighbour cell needs to be provided.

Proposal 2: The change rate of feeder link delay difference between the serving cell and each neighbour cell needs to be provided.

# Reference