# 3GPP TSG RAN WG2 Meeting #118-e                     	    R2-2205707 E-meeting, 9th – 20th May 2022

Agenda item:	6.15.2.2

Source:	Qualcomm Incorporated

Title:	Discussion on Configuration Aspects

WID/SID:	NR_SL_enh – Release 17

Document for:	Discussion and Decision

# Introduction

RAN2 made the following agreements at RAN2 #117-e meeting [1].

Agreements on Tx Profile

12: For GC, we will check with SA2 whether the mapping from L2 id to TX profile is feasible in the gNB (like what we did in LTE). Working assumption: no additional RAN2 work if SA2 confirms it’s feasible.

20:	Check with SA2 whether a same L2 ID may associate with multiple Tx profiles, and thus may associate with both DRX-based Tx profile and non-DRX based Tx profile in Rel-16. Then also check with SA2 if feasible for Rel-17 SL DRX operation, L2 id is only associated with either DRX-based TX profile(s) or non-DRX based TX profile(s). DCR issue raised by ZTE can be discussed as part of LS preparation. If the question is valid to companies, we’re also adding that question otherwise we’re not adding it. Working assumption: no additional RAN2 work if SA2 confirms it’s feasible for Rel-17 SL DRX operation, L2 id is only associated with either DRX-based TX profile(s) or non-DRX based TX profile(s).

33:	The Tx profile should include at least the information of DRX support or not. Include this agreement into the LS to SA2.

RAN2 made the following agreements at RAN2 #116-e meeting [5].

Agreements on SL-DRX for ProSe:

1:	RAN2 confirm R17 SL-DRX design can support non-relay-related ProSe communication directly without additional specific solution discussion / specification effort.

2:	RAN2 confirm the R17 SL-DRX design can support non-relay-related ProSe discovery by reusing SL default-DRX configuration used for communication without further additional specific solution discussion / specification effort.

Agreements on SL-DRX for ProSe:

1:	RAN2 confirms Rel-17 SL-DRX design can be reused for relay-related ProSe communication in layer-3 relay without additional specific solution discussion/specification effort.

2:	Keep RAN2 previous agreement (prioritize the non-relay case without consideration of relay specific optimization in Rel-17) but we’re not going to make any conclusion if L2 relay-related ProSe communication is supported or not in Rel-17 now.

3:	RAN2 confirms Rel-17 SL-DRX design can be reused for L3 relay-related ProSe discovery without additional specific solution discussion/specification effort (by applying SL default-DRX configuration). No conclusion if L2 relay-related ProSe discovery is supported or not in Rel-17 now. RAN2 does not specify any restriction now.

In this contribution, we further discuss configuration aspects for TX profile, , time unit for SlotOffset, and SL DRX support to L3 ProSe discovery and communication.

# Discussion

## Tx profile configuration for SL DRX

At AS layer, SL DRX for groupcast is configured per each L2 destination ID, therefore whether an SL DRX is enabled or disabled for backward compatibility should logically be per each L2 destination ID. Based on this assumption, the configuration for Tx profile should be per each L2 destination ID.

Additionally, SA2 confirmed in “Reply LS on Tx Profile” [2] with the follows.

When the mapping from L2 destination ID to NR Tx Profile is available at the NG-RAN, the Tx Profile may be configured per each L2 destination ID by gNB via dedicated RRC configuration message to a UE in RRC CONNECTED, as exemplified below.

sl-TxProfile-v17 ::=              SEQUENCE {

sl-DestinationIdentity-r16       SL-DestinationIdentity-r16,					OPTIONAL

sl-TxProfile                  ENUMERATED {sldrx, nonsldrx, spare1}, 	OPTIONAL}

Alternatively, it’s also possible to configure Tx Profiles with a list of L2 destination IDs in SIB for UEs in RRC IDLE or RRC INACTIVITY.

Proposal 1. NG-RAN may optionally provide mapping between L2 destination ID and Tx Profile to a UE via dedicated RRC configuration message or in SIB.

However, there is other case that the mapping from L2 destination ID to NR Tx Profile is not available, as stated by SA2 in the follow.

In this case where the L2 destination ID is generated with group ID information provided by the application layer, AS layer needs the upper layer (e.g., V2X layer) to pass the Tx Profile mapping with L2 destination ID.

Additionally, SA2 also confirmed the follow.

In this case where the upper layer does not provide NR Tx Profile (e.g., Release 16 service), AS layer does not know if any service of the services associated to an L2 destination ID does not provide NR Tx Profile and thus cannot decide if the SL DRX associated to the L2 destination ID should be disabled or not.

To solve this issue, Upper layer (e.g., V2X layer) may pass to AS layer a default NR Tx Profile which is for “no SL DRX” for the service which does not provide mapping to NR Tx Profile to the associated L2 destination ID. The AS layer may disable SL DRX if the Tx Profile list (associated to the L2 destination) from Upper layer contains a default NR Tx Profile.

Alternatively, Upper layer (e.g., V2X layer) may not pass any Tx Profile if any service does not provide NR Tx Profile to the associated L2 destination ID. The AS layer may decide to disable SL DRX if no Tx Profile is passed down from Upper layer.

The second solution is simple, since it does not require any extra AS layer work, but the default Tx Profile solution may be more future compatible in case more features are included in Tx Profile in future releases.

Proposal 2. RAN2 needs to discuss the following options and then send LS to SA2 based on the decision made.

Upper layer (V2X layer) passes a list of NR Tx Profiles associated with an L2 destination ID to AS layer. Each Tx Profile of the list of NR Tx Profiles is associated with an active Service Type using the L2 destination ID

- Option1: If a Service Type does not have mapping to NR Tx Profile, it should use a “default NR Tx Profile” which indicates no SL DRX.

- Option2: If a Service Type does not have mapping to NR Tx Profile (i.e., SL DRX should not be enabled) for the associated L2 destination ID, Upper layer (V2X layer) should not pass ANY NR Tx Profile (i.e., the NR Tx Profile list is empty) down to AS layer so that SL DRX cannot be enabled.

## Time granularity of sl-drx-SlotOffset

As specified in “5.28.2 Behaviour of UE receiving SL-SCH Data” of TS 38.321[3] (as shown below). The sl-drx-SlotOffset is derived from modulo operation with sl-drx-onDurationTimer.

However, the sl-drx-SlotOffset is in time unit of 1/32 ms, and sl-drx-onDurationTimer is in time unit of ms or 1/32 ms as configured below in TS 38.331[4].

To ensure that sl-drx-SlotOffset is in time unit of 1/32 ms, the sl-drx-onDurationTimer should be in time unit of 1/32 ms for deriving sl-drx-SlotOffset.

Proposal 3. The sl-drx-onDurationTimer should be in time unit of 1/32 ms for deriving the sl-drx-SlotOffset of the SL DRX for groupcast or broadcast.

## SL DRX for ProSe

RAN2 agreed that R17 SL-DRX design can support non-relay-related ProSe discovery and communication and L3 relay-related ProSe discovery and communication directly without additional specific solution discussion [5]. However, the content for R17 SL DRX and content for R17 L3 Relay are organized separately in TS 38.331 [4], R17 SL DRX related procedures are not linked to R17 L3 Relay. To make the specification clearer, A note may be added stating that R17 SL DRX may be applied to R17 L3 Relay.

Proposal 4. Add a note in TS 38.331 that R17 SL DRX may be applied to R17 L3 Relay.

# Conclusion

In this contribution, we further discuss configuration aspects for TX profile, , time unit for Slotoffset, and support to L3 ProSe discovery and communication and concluded the following proposals.

Proposal 1. Tx Profile may be configured per each L2 destination ID by gNB via dedicated RRC configuration message when the mapping from L2 destination ID to NR Tx Profile is available at the NG-RAN.

Proposal 2. RAN2 needs to discuss the following options and then send LS to SA2 based on the decision made.

Upper layer (V2X layer) passes a list of NR Tx Profiles associated with an L2 destination ID to AS layer. Each Tx Profile of the list of NR Tx Profiles is associated with an active Service Type using the L2 destination ID

- Option1: If a Service Type does not have mapping to NR Tx Profile, it should use a “default NR Tx Profile” which indicates no SL DRX.

- Option2: If a Service Type does not have mapping to NR Tx Profile (i.e., SL DRX should not be enabled) for the associated L2 destination ID, Upper layer (V2X layer) should not pass ANY NR Tx Profile (i.e., the NR Tx Profile list is empty) down to AS layer so that SL DRX cannot be enabled.

Proposal 3. The sl-drx-onDurationTimer should be in time unit of 1/32 ms for deriving the sl-drx-SlotOffset of the SL DRX for groupcast or broadcast.

Proposal 4. Add a note in TS 38.331 that R17 SL DRX may be applied to R17 L3 Relay.

# References

[1] “RAN2-117-e_Rel16_NR SL_Rel17_NR SL enh_EoM”, RAN2 #117-e, 21st February – 3rd March 2022

[2] R2-2204525 “Reply LS on Tx Profile” (SA2) RAN2 118-e, 9th - 20th May 2022.

[3] TS 38.321 “Medium Access Control (MAC) protocol specification (Release 17)”, Version 17.0.0, 14th April 2022.

[4] TS 38.331 “Radio Resource Control (RRC) protocol specification (Release 17)”, Version 17.0.0, 19th April 2022.

[5] “RAN2-116-e_Rel16_NR SL_Rel17_NR SL enh_EoM”, RAN2 #116-e, 1st - 12th November 2021.