3GPP TSG-RAN2 Meeting #118 electronic	R2-2205729

Online, May 9th – 20th, 2022

Source: 			ZTE Corporation, Sanechips

Title: 	Further clarification on the waiting timer for leaving connected state [Z294][O802]

Agenda item:	6.3.4

Document for: 	Discussion and Decision

- Introduction

In the last meeting, there are some clarifications on the waiting timer T3xx, in this paper we give our further clarifications on the stop conditions of the T3xx.

- Discussion

According to the current spec, the T3xx would stopped as below

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T3xx | Upon transmitting UEAssistanceInformation message with musim-PreferredRRC-State. | Upon receiving RRCRelease, or upon entering Idle state, or upon receiving musim-LeaveAssistanceConfig set to release. | Perform the actions as specified in 5.3.8.6. |

About the stop condition, there are 2 RILs as below:

[RIL]: O802 [Description]: T3xx is stopped upon receiving RRCRelease, not upon entering idle state. [Proposed Change]: Upon receiving RRCRelease, or upon receiving musim-LeaveAssistanceConfig set to release. [RIL]: Z294 [Description]: The corresponding timer shall also be stopped when the musim-LeaveAssistanceConfig was released. [Proposed Change]: 2>release musim-LeaveAssistanceConfig, if configured and stop timer T3xx, if running;

For the Z294, in the last meeting, it has been agreed that when the NW release the musim-LeaveAssistanceConfig, UE stops the timer (even if running).

- If NW releases musim-LeaveAssistanceConfig, UE stops the timer (even if running) (i.e. if UE leaves NW A, it is as per UE implementation-specific operation that is not specified in 3GPP).

We think for the UE side, the similar principle can be applied, when the UE release the musim-LeaveAssistanceConfig, the corresponding timer shall also be stopped.

Proposal 1: When the UE release the musim-LeaveAssistanceConfig, the corresponding timer shall also be stopped.

If the proposal 1 was not agreed, then it means that the waiting timer is still running when the reestablish procedure was triggered, then the UE may enter into the Idle state for some abnormal cases e.g. the T311 expiry. For this case, the UE would enter into the IDLE state locally and the waiting timer shall be stopped.

Proposal 2: If the proposal 1 was not agreed, the UE shall stop the timer when the UE enter into the IDLE state for some abnormal cases e.g. the T311 expiry.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: When the UE release the musim-LeaveAssistanceConfig, the corresponding timer shall also be stopped.

Proposal 2: If the proposal 1 was not agreed, the UE shall stop the timer when the UE enter into the IDLE state for some abnormal cases e.g. the T311 expiry.