3GPP TSG-RAN WG2 #118-e	R2-2205754

Electronic meeting, 9 - 20 May 2022

Agenda item:		6.10.3.1.1	Known Corrections

Title:		RIL# H803/TS38.300: Clarification on SIB19 Provisioning

Source:		NEC

Document for:		Discussion and Decision

# 1. Discussion

In this contribution, we would like to confirm that SIB19 should be always broadcasting.

SIB19 is categorized as other SI. however, as indicated in RIL#H803, UE must have valid SIB19 information first before triggering RACH procedure and or initial access, otherwise UE cannot do TA compensation for preamble transmission. Consequently, on-demand SI provision is not possible for SIB19. Text proposal to TS 38.300 has been shown in section 2, we expect Huawei will provide TP to TS 38.331, but an example way of changing TS38.331 is also included

# 2. Text proposals

4.1 TP to TS38.300

## 7.3	System Information Handling

### 7.3.1	Overview

System Information (SI) consists of a MIB and a number of SIBs, which are divided into Minimum SI and Other SI:

-	Minimum SI comprises basic information required for initial access and information for acquiring any other SI. Minimum SI consists of:

-	MIB contains cell barred status information and essential physical layer information of the cell required to receive further system information, e.g. CORESET#0 configuration. MIB is periodically broadcast on BCH.

-	SIB1 defines the scheduling of other system information blocks and contains information required for initial access. SIB1 is also referred to as Remaining Minimum SI (RMSI) and is periodically broadcast on DL-SCH or sent in a dedicated manner on DL-SCH to UEs in RRC_CONNECTED.

-	Other SI encompasses all SIBs not broadcast in the Minimum SI. Those SIBs, except SIB19 can either be periodically broadcast on DL-SCH, broadcast on-demand on DL-SCH (i.e. upon request from UEs in RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED), or sent in a dedicated manner on DL-SCH to UEs in RRC_CONNECTED (i.e., upon request, if configured by the network, from UEs in RRC_CONNECTED or when the UE has an active BWP with no common search space configured or when the UE configured with inter cell beam management is receiving DL-SCH from a TRP with PCI different from serving cell’s PCI). SIB19 is always periodically broadcast on DL-SCH and may be sent in a dedicated manner on DL-SCH to UEs in RRC_CONNECTED. Other SI consists of:

-	SIB2 contains cell re-selection information, mainly related to the serving cell;

-	SIB3 contains information about the serving frequency and intra-frequency neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SIB4 contains information about other NR frequencies and inter-frequency neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters), which can also be used for NR idle/inactive measurements;

-	SIB5 contains information about E-UTRA frequencies and E-UTRA neighbouring cells relevant for cell re-selection (including cell re-selection parameters common for a frequency as well as cell specific re-selection parameters);

-	SIB6 contains an ETWS primary notification;

-	SIB7 contains an ETWS secondary notification;

-	SIB8 contains a CMAS warning notification;

-	SIB9 contains information related to GPS time and Coordinated Universal Time (UTC);

-	SIB10 contains the Human-Readable Network Names (HRNN) of the NPNs listed in SIB1;

-	SIB11 contains information related to idle/inactive measurements;

-	SIB15 contains information related to disaster roaming;

-	SIB17 contains information related to TRS configuration for UEs in RRC_IDLE/RRC_INACTIVE;

-	SIBpos contains positioning assistance data as defined in TS 37.355 [43] and TS 38.331 [12];

-	SIB18 contains information related to the Group IDs for Network selection (GINs) associated with SNPNs listed in SIB1.

For sidelink, Other SI also includes:

-	SIB12 contains information related to NR sidelink communication;

-	SIB13 contains information related to SystemInformationBlockType21 for V2X sidelink communication as specified in TS 36.331 clause 5.2.2.28 [29];

-	SIB14 contains information related to SystemInformationBlockType26 for V2X sidelink communication as specified in TS 36.331 clause 5.2.2.33 [29].

For non-terrestrial network, Other SI also includes:

-	SIB19 contains NTN-specific parameters for serving cell and/or neighbour cells as defined in TS 38.331 [12].

For MBS broadcast, Other SI also includes:

-	SIB20 contains of MCCH configuration;

-	SIB21 contains information related to service continuity for MBS broadcast reception.

Figure 7.3.1-1 below summarises System Information provisioning.

Figure 7.3.1-1: System Information Provisioning

For a cell/frequency that is considered for camping by the UE, the UE is not required to acquire the contents of the minimum SI of that cell/frequency from another cell/frequency layer. This does not preclude the case that the UE applies stored SI from previously visited cell(s).

If the UE cannot determine the full contents of the minimum SI of a cell by receiving from that cell, the UE shall consider that cell as barred.

In case of BA, the UE only acquires SI on the active BWP.

If the UE is configured with inter cell beam management:

-	the UE is not required to acquire the SI from the serving cell while it is receiving DL-SCH from a TRP with PCI different from serving cell’s PCI.

4.2 TP to TS38.331

#### –	SI-SchedulingInfo

The IE SI-SchedulingInfo contains information needed for acquisition of SI messages.

SI-SchedulingInfo information element

-- ASN1START

-- TAG–SI-SCHEDULINGINFO-START

SI-SchedulingInfo ::=               SEQUENCE {

schedulingInfoList                  SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo,

si-WindowLength                     ENUMERATED {s5, s10, s20, s40, s80, s160, s320, s640, s1280},

si-RequestConfig                    SI-RequestConfig                                                OPTIONAL,  -- Cond MSG-1

si-RequestConfigSUL                 SI-RequestConfig                                                OPTIONAL,  -- Cond SUL-MSG-1

systemInformationAreaID             BIT STRING (SIZE (24))                                          OPTIONAL,   -- Need R

...

}

SchedulingInfo ::=                  SEQUENCE {

si-BroadcastStatus                  ENUMERATED {broadcasting, notBroadcasting},

si-Periodicity                      ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512},

sib-MappingInfo                     SIB-Mapping

}

SI-SchedulingInfo-v1700 ::=         SEQUENCE {

schedulingInfoList2-r17             SEQUENCE (SIZE (1..maxSI-Message)) OF SchedulingInfo2-r17

}

SchedulingInfo2-r17 ::=             SEQUENCE {

si-BroadcastStatus-r17              ENUMERATED {broadcasting, notBroadcasting},

si-WindowPosition-r17               INTEGER (1..256),

si-Periodicity-r17                  ENUMERATED {rf8, rf16, rf32, rf64, rf128, rf256, rf512},

sib-MappingInfo-r17                 SIB-Mapping-v1700

}

SIB-Mapping ::=                     SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo

SIB-Mapping-v1700  ::=              SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo-v1700

SIB-TypeInfo ::=                    SEQUENCE {

type                                ENUMERATED {sibType2, sibType3, sibType4, sibType5, sibType6, sibType7, sibType8, sibType9,

sibType10-v1610, sibType11-v1610, sibType12-v1610, sibType13-v1610,

sibType14-v1610, spare3, spare2, spare1,... },

valueTag                            INTEGER (0..31)                                                 OPTIONAL, -- Cond SIB-TYPE

areaScope                           ENUMERATED {true}                                               OPTIONAL -- Need S

}

SIB-TypeInfo-v1700 ::=              SEQUENCE {

sibType-r17                         CHOICE {

type1-r17                           ENUMERATED {sibType15, sibType16, sibType17, sibType18, sibType19, sibType20, sibType21,...},

type2-r17                           SEQUENCE {

posSibType-r17                      ENUMERATED {posSibType1-9, posSibType1-10, posSibType2-24, posSibType2-25, posSibType6-4, posSibType6-5, posSibType6-6,...},

encrypted-r17                       ENUMERATED { true }                                     OPTIONAL, -- Need R

gnss-id-r17                         GNSS-ID-r16                                             OPTIONAL, -- Need R

sbas-id-r17                         SBAS-ID-r16                                             OPTIONAL  -- Need R

}

},

valueTag-r17                            INTEGER (0..31)                                             OPTIONAL, -- Cond SIB-TYPE-POS

areaScope-r17                           ENUMERATED {true}                                           OPTIONAL  -- Need S

}

-- TAG-SI-SCHEDULINGINFO-STOP

-- ASN1STOP

SchedulingInfo field descriptions

areaScope Indicates that a SIB is area specific. If the field is absent, the SIB is cell specific.

si-BroadcastStatus Indicates if the SI message is being broadcasted or not. Change of si-BroadcastStatus should not result in system information change notifications in Short Message transmitted with P-RNTI over DCI (see clause 6.5). The value of the indication is valid until the end of the BCCH modification period when set to broadcasting. si-BroadcastStatus of the SI where SIB19 mapped to has to be broadcasting.

si-Periodicity Periodicity of the SI-message in radio frames. Value rf8 corresponds to 8 radio frames, value rf16 corresponds to 16 radio frames, and so on.