3GPP TSG-RAN WG2 #118e	Tdoc R2-2205757

Electronic meeting, May 9-20, 2022

Agenda Item:	6.3.4

Source:	Ericsson

Title:	Behaviour of wait timer

Document for:	Discussion, Decision

# 1	Introduction

The behaviour of wait timer in the field description is currently unclear in 38.331. This contribution discusses this issue and way forward.

# 2	Discussion

The field description of musim-LeaveWithoutResponseTimer is captured in [1] as follows:

musim-LeaveWithoutResponseTimer

Indicates the timer for to leave RRC_CONNECTED without network response. When T3xx expires, UE autonomously leaves RRC_CONNECTED state and enters RRC_IDLE for MUSIM purpose.

In [2], a simplification to this field was proposed:

musim-LeaveWithoutResponseTimer

Indicates the timer for the UE to enter RRC_IDLE for MUSIM purpose as defined in clause 5.3.8.x.

This was initially captured in the [Post117-e][237][MUSIM] NR RRC CR for MUSIM (vivo) but a comment was received that the current sentence was discussed and agreed and should be kept.

However, the proper behavior for the UE upon the wait timer expiration is captured in clause 5.3.8.X:

#### 5.3.8.X	T3xx expiry

The UE shall:

1>	if T3xx expires:

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'other'.

The wording “autonomously leaves” may cause confusion, and it is not consistent with what is already specified above. There is no reason to have duplicate text and ambiguity between the behaviour specified in the section above and the field description of musim-LeaveWithoutResponseTimer.

The field description of musim-LeaveWithoutResponseTimer is not consistent with the related procedural section.

Hence, it is proposed to update the field description to simplify it and refer to clause 5.3.8.X.

The field description of musim-LeaveWithoutResponseTimer is simplified to refer to clause 5.3.8.X.

As also proposed in [2] and shown above, this can be captured as follows:

musim-LeaveWithoutResponseTimer

Indicates the timer for the UE to enter RRC_IDLE for MUSIM purpose as defined in clause 5.3.8.x.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The field description of musim-LeaveWithoutResponseTimer is not consistent with the related procedural section.

Based on the discussion in the previous sections we propose the following:

Proposal 1	The field description of musim-LeaveWithoutResponseTimer is simplified to refer to clause 5.3.8.X.

# References

R2-2204207, Introduction of NR RRC support for MUSIM, Vivo, RAN2#117-e

R2-2203440, Corrections to the NR RRC CR for MUSIM (38.331), Ericsson, RAN2#117e