3GPP TSG-RAN2 Meeting #118	R2-22005781

# E-meeting, May 9th - May 20th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | - | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Misc correction on 38.300 for SL relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_Relay-Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Added text to specify resource allocation for L3 U2N relays, editorial changes and rearranged text for better readability. - Editorial changes/corrections and cause value setting procedure in the case when the L2 U2N relay starts from IDLE/INACTIVE - Missing U2N description for relay UE - Editorial changes/corrections and note added to cover the case of RAN sharing |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added text for missing L3 U2N relays behaviour, editorial changes and sentences re-organized to make the text clear and included the missing details from agreements made in the previous meetings. - Editorial changes/corrections and added a note to clarify U2N relay UE cause value setting for when initiating connection establishment from IDLE/INACTIVE state. - Added U2N description for a relay UE - Editorial changes/corrections and added a note to clarify U2N Remote UE behaviour in the case of RAN sharing - Clarified that path switch between indirect path and group mobility are not supported in Rel-17. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Resource allocation procedure for L3 U2N relays not specified and misintrepretation of specification - Cause value setting during L2 U2N relay connection establishment is essential - Better readability and might create confusion when U2U relays are specified in Rel-18 - Might lead to incorrect reporting in the case of RAN sharing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

## 16.12	Sidelink Relay

### 16.12.1	General

Sidelink relay is introduced to support 5G ProSe UE-to-Network Relay (U2N Relay) function (specified in TS 23.304 [xx]) to provide connectivity to the network for U2N Remote UE(s). Both L2 and L3 U2N Relay architectures are supported.  The L3 U2N Relay architecture is transparent to the serving RAN of the U2N Relay UE, except for controlling sidelink resources. The detailed architecture and procedures for L3 U2N Relay can be found in TS 23.304 [xx].

A U2N Relay UE shall be in RRC_CONNECTED to perform relaying of unicast data.

For L2 U2N Relay operation, the following RRC state combinations are supported:

-	Both U2N Relay UE and U2N Remote UE shall be in RRC CONNECTED to perform transmission/reception of relayed unicast data.

-	The U2N Relay UE can be in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED as long as all the U2N Remote UE(s) that are connected to the U2N Relay UE are either in RRC_INACTIVE or in RRC_IDLE.

A single unicast link is established between one L2 U2N Relay UE and one L2 U2N Remote UE. The traffic of U2N Remote UE via a given U2N Relay UE and the traffic of the U2N Relay UE shall be separated in different Uu RLC channels over Uu.

For both L2 and L3 U2N Relay, the U2N Remote UE can only be configured to use resource allocation mode 2 (as specified in 5.7.2 and 16.9.3.1) for data to be relayed.

### 16.x.2	Protocol Architecture

#### 16.x.2.1	L2 UE-to-Network Relay

The protocol stacks for the user plane and control plane of L2 U2N Relay architecture are illustrated in Figure 16.x.2.1-1 and Figure 16.x.2.1-2. The SRAP sublayer is placed above the RLC sublayer for both CP and UP at both PC5 interface and Uu interface. The Uu SDAP, PDCP and RRC are terminated between L2 U2N Remote UE and gNB, while SRAP, RLC, MAC and PHY are terminated in each hop (i.e. the link between L2 U2N Remote UE and L2 U2N Relay UE and the link between L2 U2N Relay UE and the gNB).

For L2 U2N Relay, the SRAP sublayer over PC5 hop is only for the purpose of bearer mapping. The SRAP sublayer is not present over PC5 hop for relaying the L2 U2N Remote UE’s message on BCCH and PCCH. For L2 U2N Remote UE’s message on SRB0, the SRAP sublayer is not present over PC5 hop, but the SRAP sublayer is present over Uu hop for both DL and UL.

Figure 16.x.2.1-1: User plane protocol stack for L2 UE-to-Network Relay

Figure 16.x.2.1-2: Control plane protocol stack for L2 UE-to-Network Relay

For L2 U2N Relay, for uplink:

-	The Uu SRAP sublayer performsUL bearer mapping between ingress PC5 Relay RLC channels for relaying and egress Uu Relay RLC channels over the L2 U2N Relay UE Uu interface. For uplink relaying traffic, the different end-to-end Uu RBs (SRBs or DRBs) of the same Remote UE and/or different Remote UEs can be multiplexed over the same egress Uu Relay RLC channel.

-	The Uu SRAP sublayer supports L2 U2N Remote UE identification for the UL traffic. The identity information of L2 U2N Remote UE end-to-end Uu Radio Bearer and a local Remote UE ID are included in the Uu SRAP header at UL in order for gNB to correlate the received packets for the specific PDCP entity associated with the right Uu Radio Bearer of a Remote UE.

-	The PC5 SRAP sublayer at the L2 U2N Remote UE supports UL bearer mapping between Remote UE Uu Radio Bearers and egress PC5 Relay RLC channels.

For L2 U2N Relay, for downlink:

-	The Uu SRAP sublayer performs DL bearer mapping at gNB to map end-to-end Uu Radio Bearer (SRB, DRB) of Remote UE into Uu Relay RLC channel over Relay UE Uu interface. The Uu SRAP sublayer performs DL bearer mapping and data multiplexing between multiple end-to-end Radio Bearers (SRBs or DRBs) of a L2 U2N Remote UE and/or different L2 U2N Remote UEs and one Uu Relay RLC channel over the Relay UE Uu interface.

-	The Uu SRAP sublayer supports U2N Remote UE identification for DL traffic. The identity information of the U2N Remote UE end-to-end Uu Radio Bearer and a local Remote UE ID are included into the Uu SRAP header by the gNB at DL for the U2N Relay UE to map the received packets from U2N Remote UE end-to-end Uu Radio Bearer to its associated PC5 Relay RLC channel.

-	The PC5 SRAP sublayer at the U2N Relay UE performs DL bearer mapping between ingress Uu Relay RLC channels and egress PC5 Relay RLC channels.

-	The PC5 SRAP sublayer at the U2N Remote UE correlates the received packets for the specific PDCP entity associated with the right Uu Radio Bearer of a Remote UE based on the identity information included in the Uu SRAP header.

A local Remote UE ID is included in both PC5 SRAP header and Uu SRAP header. L2 U2N Relay UE is configured by the gNB with the local Remote UE ID to be used in SRAP header. Remote UE obtains the local Remote ID from the gNB via Uu RRC messages including RRCSetup, RRCReconfiguration, RRCResume and RRCReestablishment. Uu DRB(s) and Uu SRB(s) are mapped to different PC5 Relay RLC channels and Uu Relay RLC channels in both PC5 hop and Uu hop.

It is the gNB responsibility to avoid collision on the usage of local Remote UE ID. The gNB can update the local Remote UE ID by sending the updated local Remote ID via RRCReconfiguration message to the Relay UE. The serving gNB can perform local Remote UE ID update independent of the PC5 unicast link L2 ID update procedure.

### 16.x.3	Relay Discovery

Model A and Model B discovery models as defined in TS 23.304 [xx] are supported for U2N Relay discovery. The protocol stack used for discovery is illustrated in Figure 16.x.3-1.

Figure 16.x.3.1: Protocol Stack of Discovery Message for UE-to-Network Relay

The U2N Remote UE can perform Relay discovery message (i.e. as specified in TS 23.304 [xx]) transmission and may monitor the sidelink for Relay discovery message while in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED. The network may broadcast a threshold, which is used by the U2N Remote UE to determine if it can transmit Relay discovery messages to U2N Relay UE(s).

The U2N Relay UE can perform Relay discovery message (i.e. as specified in TS 23.304 [xx]) transmission and may monitor the sidelink for Relay discovery message while in RRC_IDLE, RRC_INACTIVE or  RRC_CONNECTED. The network may broadcast a maximum Uu RSRP threshold, a minimum Uu RSRP threshold, or both, which are used by the U2N Relay UE to determine if it can transmit Relay discovery messages to U2N Remote UE(s).

The network may provide the Relay discovery configuration using broadcast or dedicated signalling for Relay discovery. In addition, the U2N Remote UE and U2N Relay UE may use pre-configuration for Relay discovery.

The resource pool(s) used for NR sidelink communication can be used for Relay discovery or the network may configure a resource pool(s) dedicated for Relay discovery. Resource pool(s) dedicated for Relay discovery can be configured simultaneously with resource pool(s) for NR sidelink communication in system information, dedicated signalling and/or pre-configuration. Whether a dedicated resource pool(s) for Relay discovery is configured is based on network implementation. If resource pool(s) dedicated for Relay discovery are configured, only those resource pool(s) dedicated for Relay discovery shall be used for Relay discovery. If only resource pool(s) for NR sidelink communication are configured, all the configured transmission resource pool(s) can be used for Relay discovery and sidelink communication.

For U2N Remote UE (including both in-coverage and out of coverage cases) that has been connected to the network via a U2N Relay UE, only resource allocation mode 2 is used for discovery message transmission.

The Relay discovery reuses NR sidelink resource allocation principles for in-coverage U2N Relay UE, and for both in-coverage and out of coverage U2N Remote UEs.

The sidelink power control for the transmission of Relay discovery messages is same as for NR sidelink communication.

No ciphering or integrity protection in PDCP layer is applied for the Relay discovery messages.

The U2N Remote UE and U2N Relay UE can determine from SIB12 whether the gNB supports Relay discovery, Non-Relay discovery, or both.

Next Modified Subclause

### 16.x.5	Control plane procedures for L2 U2N Relay

#### 16.x.5.1	RRC Connection Management

The U2N Remote UE needs to establish its own PDU sessions/DRBs with the network before user plane data transmission.

The NR V2X PC5 unicast link establishment procedures can be reused to setup a secure unicast link between U2N Remote UE and U2N Relay UE before U2N Remote UE establishes a Uu RRC connection with the network via U2N Relay UE.

The establishment of Uu SRB1/SRB2 and DRB of the U2N Remote UE is subject to Uu configuration procedures for L2 UE-to-Network Relay.

The following high level connection establishment procedure in Figure 16.x.5.1-1 applies to L2 U2N Relay:

Figure 16.x.5.1-1: Procedure for L2 U2N Remote UE connection establishment

1.	The U2N Remote and U2N Relay UE perform discovery procedure, and establish PC5-RRC connection using NR V2X procedure.

2.	The U2N Remote UE sends the first RRC message (i.e., RRCSetupRequest) for its connection establishment with gNB via the U2N Relay UE, using a specified PC5 Relay RLC channel configuration. If the U2N Relay UE is not in RRC_CONNECTED, it needs to do its own connection establishment upon reception of a message on the specified PC5 Relay RLC channel. During U2N Relay UE’s RRC connection establishment procedure, gNB may configure SRB0 relaying Uu Relay RLC channel to the U2N Relay UE. The gNB responds with an RRCSetup message to U2N Remote UE. The RRCSetup message is sent to the U2N Remote UE using SRB0 relaying channel over Uu and a specified PC5 Relay RLC channel over PC5.

NOTE x:	During the U2N relay UE’s RRC connection establishment procedure, it is left upto the U2N relay UE’s implementation on how the establishment value is set in in the RRCSetup message.

3.	The gNB and U2N Relay UE perform relaying channel setup procedure over Uu. According to the configuration from gNB, the U2N Relay/Remote UE establishes an PC5 Relay RLC channel for relaying of SRB1 towards the U2N Remote/Relay UE over PC5.

4.	The RRCSetupComplete message is sent by the U2N Remote UE to the gNB via the U2N Relay UE using SRB1 relaying channel over PC5 and SRB1 relaying channel configured to the U2N Relay UE over Uu. Then the U2N Remote UE is RRC connected over Uu.

5.	The U2N Remote UE and gNB establish security following Uu procedure and the security messages are forwarded through the U2N Relay UE.

6.	The gNB sends an RRCReconfiguration message to the U2N Remote UE via the U2N Relay UE, to setup the SRB2/DRBs for relaying purpose. The U2N Remote UE sends an RRCReconfigurationComplete message to the gNB via the U2N Relay UE as a response. In addition, the gNB may configure additional Uu Relay RLC channels between the gNB and U2N Relay UE, and PC5 Relay RLC channels between U2N Relay UE and U2N Remote UE for the relay traffic.

Next Modified Subclause

#### 16.x.5.5	System Information

The in-coverage U2N Remote UE is allowed to acquire any necessary SIB(s) over Uu interface irrespective of its PC5 connection to Relay UE. The U2N Remote UE can also receive the system information from the U2N Relay UE after PC5 connection establishment with U2N Relay UE.

The U2N Remote UE in RRC_CONNECTED can use the on-demand SIB framework as specified in TS 38.331 [12] to request the SIB(s) via U2N Relay UE. The U2N Remote UE in RRC_IDLE or RRC_INACTIVE can inform U2N Relay UE of its requested SIB type(s) via PC5-RRC message. Then, U2N Relay UE triggers on-demand SI/SIB acquisition procedure as specified in TS38.331 [12] according to its own RRC state (if needed) and sends the acquired SI(s)/SIB(s) to U2N Remote UE via PC5-RRC.

Any SIB that the RRC_IDLE or RRC_INACTIVE U2N Remote UE has a requirement to use (e.g., for relay purpose) can be requested by the U2N Remote UE (from the U2N Relay UE or the network). For SIBs that have been requested by the U2N Remote UE from the U2N Relay UE, the U2N Relay UE forwards them again in case of any update for requested SIB(s). In case of RRC_CONNECTED U2N Remote UE(s), it is the responsibility of the network to send updated SIB(s) to U2N Remote UE(s) when they are updated. The U2N Remote UE de-configures SI request with U2N Relay UE when entering into RRC_CONNECTED state.

For SIB1 forwarding, for U2N Remote UE, both request-based delivery (i.e., SIB1 request by the U2N Remote UE) and unsolicited forwarding are supported by U2N Relay UE, of which the usage is left to U2N Relay UE implementation. If SIB1 changes, for U2N Remote UE in RRC_IDLE or RRC_INACTIVE, the U2N Relay UE always forwards SIB1.

For the L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE, the short message over Uu interface is not forwarded by the L2 U2N Relay UE to the L2 U2N Remote UE. The L2 U2N Relay UE can forward PWS SIBs to its connected L2 U2N Remote UE(s).

RAN sharing is supported for L2 U2N Relay UE. In particular, the L2 U2N Relay UE may forward, via discovery message, cell access related information before the establishment of a PC5-RRC connection.

Next Modified Subclause

#### 16.x.6	Service Continuity for L2 U2N relay

#### 16.x.6.1	General

Service continuity requirement is applicable only for the mobility cases of path switch from indirect to direct path and from direct to indirect path only when the L2 U2N Remote UE and L2 U2N Relay UE belong to the same gNB.

Mobility for path switch from a indirect path to another indirect path as well as group mobility are not supported in this version of the specification.

#### 16.x.6.2 Switching from indirect to direct path

For service continuity of L2 U2N Relay, the following procedure is used, in case of U2N Remote UE switching to direct path:

Figure 16.x.6.1-1: Procedure for U2N Remote UE switching to direct Uu cell

- The Uu measurement configuration and measurement report signalling procedures are performed to evaluate both relay link measurement and Uu link measurement. The measurement results from U2N Remote UE are reported when configured measurement reporting criteria are met. The sidelink relay measurement report shall include at least U2N Relay UE’s source L2 ID, serving cell ID (i.e., NCGI/NCI), and sidelink measurement quantity information. The sidelink measurement quantity can be SL-RSRP of the serving U2N Relay UE, and if SL-RSRP is not available, SD-RSRP is used.

NOTE x:	In the case of RAN sharing, the serving cell ID included in the measurement report is NCGI, otherwise, NCI is included.

2.	The gNB decides to switch the U2N Remote UE onto direct Uu path.

3.	The gNB sends RRCReconfiguration message to the U2N Remote UE. The U2N Remote UE stops UP and CP transmission via U2N Relay UE after reception of RRCReconfiguration message with the path switch configuration from the gNB.

4.	The U2N Remote UE synchronizes with the gNB and performs Random Access.

5.	The UE (i.e., U2N Remote UE in previous steps) sends the RRCReconfigurationComplete to the gNB via direct path, using the configuration provided in the RRCReconfiguration message. From this step, the UE (i.e., U2N Remote UE in previous steps) uses the RRC connection via the direct path to the gNB.

6.	The gNB sends RRCReconfiguration message to the U2N Relay UE to reconfigure the connection between the U2N Relay UE and the gNB. The RRCReconfiguration message to the U2N Relay UE can be sent any time after step 3 based on gNB implementation (e.g., to release Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration between PC5 RLC and Uu RLC).

7.	Either U2N Relay UE or U2N Remote UE can initiate the PC5 unicast link release (PC5-S). The timing to execute link release is up to UE implementation. The U2N Relay UE can execute PC5 connection reconfiguration to release PC5 Relay RLC channel for relaying upon reception of RRC Reconfiguration by gNB in Step 6, or the UE (i.e., previous U2N Remote UE) can execute PC5 connection reconfiguration to release PC5 Relay RLC channel for relaying upon reception of RRCReconfiguration by gNB in Step 3.

8.	The data path is switched from indirect path to direct path between the UE (i.e., previous U2N Remote UE) and the gNB. The DL/UL lossless delivery during the path switch is done according to PDCP data recovery procedure.

NOTE:	Step 8 can be executed any time after step 4. Step 8 is independent of step 6 and step 7.

#### 16.x.6.3 Switching from direct to indirect path

The gNB can select a U2N Relay UE in any RRC state i.e., RRC_IDLE, RRC_INACTIVE, or RRC_CONNECTED, as a target U2N Relay UE for direct to indirect path switch.

For service continuity of L2 U2N Remote UE, the following procedure is used, in case of the L2 U2N Remote UE switching to indirect path via a U2N Relay UE in RRC_CONNECTED:

Figure 16.x.6.2-1: Procedure for U2N Remote UE switching to indirect path

1.	The U2N Remote UE reports one or multiple candidate U2N Relay UE(s) and Uu measurements, after it measures/discovers the candidate U2N Relay UE(s).

- The U2N Remote UE reports the appropriate U2N Relay UE(s) according to Relay selection criteria. The U2N Remote UE shall report only the U2N Relay UE candidate(s) that fulfil the higher layer criteria.

- The reporting can include at least U2N Relay UE ID, U2N Relay UE’ s serving cell ID, and sidelink measurement quantity information. The sidelink measurement quantity can be SL-RSRP of the candidate U2N Relay UE, and if SL-RSRP is not available, SD-RSRP is used.

2.	The gNB decides to switch the U2N Remote UE to a target U2N Relay UE. Then the gNB sends an RRCReconfiguration message to the target U2N Relay UE, which can include at least Remote UE’s local ID and L2 ID, Uu and PC5 Relay RLC channel configuration for relaying, and bearer mapping configuration.

3.	The gNB sends the RRCReconfiguration message to the U2N Remote UE. The contents in the RRCReconfiguration message can include at least U2N Relay UE ID, PC5 Relay RLC channel configuration for relay traffic and the associated end-to-end radio bearer(s). The U2N Remote UE stops UP and CP transmission over Uu after reception of RRCReconfiguration message from the gNB.

4.	The U2N Remote UE establishes PC5 connection with target U2N Relay UE, if not already established.

5.	The U2N Remote UE completes the path switch procedure by sending the RRCReconfigurationComplete message to the gNB via the Relay UE.

6.	The data path is switched from direct path to indirect path between the U2N Remote UE and the gNB.

In case the selected U2N Relay UE for direct to indirect path switch is in RRC_IDLE or RRC_INACTIVE, after receiving the path switch command, the U2N Remote UE establishes a PC5 link with the U2N Relay UE and sends the RRCReconfigurationComplete message via the U2N Relay UE, which will trigger the U2N Relay UE to enter RRC_CONNECTED state. The procedure for U2N Remote UE switching to indirect path in Figure 16.x.6.2-1 can be also applied for the case that the selected U2N Relay UE for direct to indirect path switch is in RRC_IDLE or RRC_INACTIVE with the exception that step 4 is performed before step 2.