3GPP TSG-RAN WG2 Meeting #118 Electronic	R2-2205787

Online, 09-20 May 2022

Agenda item:	6.12.4.1

Source:	Nokia, Nokia Shanghai Bell

Title:	On RedCap UE capabilities

WID/SID:	NR_redcap-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

This contribution discusses open issues on RedCap UE capabilities on eDRX in IDLE and INACTIVE.

# 2	Discussion

The following is captured in the RAN2#117 chairman notes:

R2-2203563	[offline-107] UE caps open issues - second round	Intel	discussion	Rel-17	NR_redcap-Core

For online discussion:

At117-Proposal 3.2.2-1: [online discussion] [9 vs 7] a UE supports eDRX, must support Edrx in RRC_IDLE and RRC_INACTIVE simultaneously;

- Come back in the next meeting

At117-Proposal 3.2.2-2: [online discussion] [10] Assuming a UE supports eDRX, must support Edrx in RRC_IDLE and RRC_INACTIVE simultaneously, the eDRX in RRC_INACTIVE is introduced together with eDRX in RRC_IDLE as

Definitions for feature

Rel-17 extended DRX in RRC_IDLE and RRC_INACTIVE It is optional for UE to support Rel-17 extended DRX cycle values up to 10485.76 seconds for RRC_IDLE and up to 10.24 seconds for RRC_INACTIVE, and paging in extended DRX in RRC_IDLE and RRC_INACTIVE as specified in TS 38.331 [9] and TS 38.304 [21].

- Come back in the next meeting

IOT testing may not be available for IDLE and INACTIVE eDRX at the same time and therefore separate capabilties are needed. In addition IDLE and INACTIVE eDRX includes different functionality and therefore it would be natural to have separate capabilities for them.

Proposal 1: Separate UE capabilities are introduced for eDRX in RRC_IDLE and RRC_INACTIVE.

eDRX for RRC_INACTIVE UE is configured with dedicated signaling using RRCRelease message and therefore NW/RAN needs to know whether the UE supports eDRX for RRC_INACTIVE.

Proposal 2: Support for eDRX in RRC_INACTIVE is optional with capability signaling.

eDRX for RRC_IDLE UE is configured with NAS signaling and therefore there is no need for NW/RAN to know whether the UE supports eDRX for RRC_IDLE.

Proposal 3: Support for eDRX in RRC_IDLE is optional without capability signaling.

# 3	Conclusion

The following is proposed:

Proposal 1: Separate UE capabilities are introduced for eDRX in RRC_IDLE and RRC_INACTIVE.

Proposal 2: Support for eDRX in RRC_INACTIVE is optional with capability signaling.

Proposal 3: Support for eDRX in RRC_IDLE is optional without capability signaling.