3GPP TSG-RAN WG2 Meeting #118 Electronic	R2-2205792

Elbonia, 09 – 20 April 2022

Agenda item:	8.20.2

Source:	Intel Corporation

Title:	Remaining UE capabilities on NR operation for upto 71GHz

WID/SID:	NR_ext_to_71GHz-Core - Release 17

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, the following remaining issues on UE capability are discussed:

- Updates to the supported bandwidths for FR2-2

- Introduce the UE capability for the introduction of new values for drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL

- Other miscellaneous corrections

# 2	Discussion

## 2.1 Updates to the supported bandwidths for FR2-2

With the agreement from RAN4 below, UE capabilities related to the supported bandwidth needs to be updated.

At RAN4#100, the following was agreed:

Agreement: For intermediate CBWs between min and max CBWs,

- Integer multiples of the min CBW for each SCS

  - 120 kHz: 100 MHz (min), 400 MHz (max)

  - 480 kHz: 400 MHz (min), 800 MHz, 1600 MHz (max)

  - 960 kHz: 400 MHz (min), 800 MHz, 1600 MHz, 2000 MHz (max)

- FFS whether 1200Mhz CBW is needed for 480KHz SCS and 960Khz SCS

- FFS whether 200MHz CBW is needed for 120KHz SCS

The followings are the UE capabilities that are impacted by the above agreement and required updating to support FR2-2:

- Per FSPC

  - supportedBandwidthDL

  - supportedBandwidthUL

- Per band

  - channelBWs-DL

  - channelBWs-UL

### 2.1.1 Updates to supportedBandwidth (UL/DL)

The existing field descriptions for the supportedBandwidthDL/UL are as follow:

| supportedBandwidthDL Indicates maximum DL channel bandwidth supported for a given SCS that UE supports within a single CC (and in case of intra-frequency DAPS handover for the source and target cells), which is defined in Table 5.3.5-1 in TS 38.101-1 [2] for FR1 and Table 5.3.5-1 in TS 38.101-2 [3] for FR2. For FR1, all the bandwidths listed in TS38.101-1 Table 5.3.5-1 for each band shall be mandatory with a single CC unless indicated optional. For FR2, the set of mandatory CBW is 50, 100, 200 MHz. When this field is included in a band combination with a single band entry and a single CC entry (i.e. non-CA band combination), the UE shall indicate the maximum channel bandwidth for the band according to TS 38.101-1 [2] and TS 38.101-2 [3]. The UE may report a supportedBandwidthDL wider than the channelBWs-DL; this supportedBandwidthDL may not be included in the Table 5.3.5-1 of TS 38.101-1[2]/TS 38.101-2[3] for the case that the UE is unable to report the actual supported bandwidth according to the Table 5.3.5-1 of TS 38.101-1[2]/TS 38.101-2[3]. For each band, RedCap UEs shall indicate its maximum channel bandwidth, which is the maximum of those channel bandwidths that are less than or equal to 20 MHz for FR1 and less than or equal to 100 Mhz for FR2, taking restrictions in TS 38.101-1 [2] and TS 38.101-2 [3] into consideration. NOTE: To determine whether the UE supports a channel bandwidth of 90 MHz, the network may ignore this capability and validate instead the channelBW-90mhz, the supportedBandwidthCombinationSet and the supportedBandwidthCombinationSetIntraENDC. For serving cell(s) with other channel bandwidths the network validates the channelBWs-DL, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, the asymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]), supportedBandwidthDL and supportedMinBandwidthDL. | FSPC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |
| supportedBandwidthUL Indicates maximum UL channel bandwidth supported for a given SCS that UE supports within a single CC (and in case of intra-frequency DAPS handover for the source and target cells), which is defined in Table 5.3.5-1 in TS38.101-1 [2] for FR1 and Table 5.3.5-1 in TS 38.101-2 [3] for FR2. For FR1, all the bandwidths listed in TS38.101-1 Table 5.3.5-1 for each band shall be mandatory with a single CC unless indicated optional. For FR2, the set of mandatory CBW is 50, 100, 200 MHz. When this field is included in a band combination with a single band entry and a single CC entry (i.e. non-CA band combination), the UE shall indicate the maximum channel bandwidth for the band according to TS 38.101-1 [2] and TS 38.101-2 [3]. The UE may report a supportedBandwidthUL wider than the channelBWs-UL; this supportedBandwidthUL may not be included in the Table 5.3.5-1 of TS 38.101-1[2]/TS 38.101-2[3] for the case that the UE is unable to report the actual supported bandwidth according to the Table 5.3.5-1 of TS 38.101-1[2]/TS 38.101-2[3]. For each band, RedCap UEs shall indicate its maximum channel bandwidth, which is the maximum of those channel bandwidths that are less than or equal to 20 MHz for FR1 and less than or equal to 100 Mhz for FR2, taking restrictions in TS 38.101-1 [2] and TS 38.101-2 [3] into consideration. NOTE: To determine whether the UE supports a channel bandwidth of 90 MHz the network may ignore this capability and validate instead the channelBW-90mhz, the supportedBandwidthCombinationSet and the supportedBandwidthCombinationSetIntraENDC. For serving cell(s) with other channel bandwidths the network validates the channelBWs-UL, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, the asymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]), supportedBandwidthUL and supportedMinBandwidthUL. | FSPC | CY | N/A | N/A |

For FR2-1, the set of mandatory CBW is 50,100,200MHz. With the same logic, for FR2-2 band, a set of CBW may also be defined as mandatory by RAN4.  However, this addition can wait for RAN4 progress. From RAN2 point of view, RAN2 just need to update that the mandatory CBW for FR2 is just for FR2-1 bands.

Observation#1: In existing supportedBandwidthDL/supportedBandwidthUL UE capabilities for FR2-1, the set of mandatory CBW is 50,100,200MHz.  It is assumed that similar mandatory CBW will be provided by RAN4 for FR2-2

Proposal#1_1: Wait for RAN4 to indicate which CBW (channel bandwidth) is mandatory for FR2-2.

Proposal#1_2: Update “For FR2, the set of mandatory CBW is 50, 100, 200 MHz.” in supportedBandwidthUL/DL is just for FR2-1.

As for the ASN.1, take example for the DL (similar structure for the UL)

FeatureSetDownlinkPerCC ::=         SEQUENCE {

supportedSubcarrierSpacingDL        SubcarrierSpacing,

supportedBandwidthDL                SupportedBandwidth,

channelBW-90mhz                     ENUMERATED {supported}                     OPTIONAL,

maxNumberMIMO-LayersPDSCH           MIMO-LayersDL                              OPTIONAL,

supportedModulationOrderDL          ModulationOrder                            OPTIONAL

}

FeatureSetDownlinkPerCC-v1620 ::=   SEQUENCE {

-- R1 16-2a: Mulit-DCI based multi-TRP

multiDCI-MultiTRP-r16               MultiDCI-MultiTRP-r16                     OPTIONAL,

-- R1 16-2b-3: Support of single-DCI based FDMSchemeB

supportFDM-SchemeB-r16              ENUMERATED {supported}                    OPTIONAL

}

FeatureSetDownlinkPerCC-v17xy ::=   SEQUENCE {

supportedMinBandwidthDL-r17         SupportedBandwidth-v17xy                    OPTIONAL,

broadcast-SCell-r17          		ENUMERATED {supported}                      OPTIONAL,

}

The IE supportedBandwidthDL/UL is defined by SupportedBandwidth and the SupportedBandwidth definition is as follow:

–                  SupportedBandwidth

The IE SupportedBandwidth is used to indicate the channel bandwidth supported by the UE on one carrier of a band of a band combination.

SupportedBandwidth information element

-- ASN1START

-- TAG-SUPPORTEDBANDWIDTH-START

SupportedBandwidth::=      CHOICE {

fr1    ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100},

fr2    ENUMERATED {mhz50, mhz100, mhz200, mhz400}

}

SupportedBandwidth-v1700 ::=      CHOICE {

fr1-r17         ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz35, mhz40, mhz45, mhz50, mhz60, mhz70, mhz80, mhz90, mhz100},

fr2-r17         ENUMERATED {mhz50, mhz100, mhz200, mhz400}

}

-- TAG-SUPPORTEDBANDWIDTH-STOP

-- ASN1STOP

Since the FeatureSetDownlinkPerCC and FeatureSetUplinkPerCC indicate its corresponding SCS, it just needs to further include the new CBW for SCS 480kHz and SCS 960kHz to the new Rel-17 range for FR2 as shown below in red ink.

–                  SupportedBandwidth

The IE SupportedBandwidth is used to indicate the channel bandwidth supported by the UE on one carrier of a band of a band combination.

SupportedBandwidth information element

-- ASN1START

-- TAG-SUPPORTEDBANDWIDTH-START

SupportedBandwidth::=      CHOICE {

fr1    ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100},

fr2    ENUMERATED {mhz50, mhz100, mhz200, mhz400}

}

SupportedBandwidth-v1700 ::=      CHOICE {

fr1-r17         ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz35, mhz40, mhz45, mhz50, mhz60, mhz70, mhz80, mhz90, mhz100},

fr2-r17         ENUMERATED {mhz50, mhz100, mhz200, mhz400, mhz800, mhz1600, mhz2000}

}

-- TAG-SUPPORTEDBANDWIDTH-STOP

-- ASN1STOP

Proposal#2: Introduce the new CBWs (800MHz, 1600MHz and 2000MHz) for SCS 480kHz and 960kHz to the SupportedBandwidth-v1700.

To be able to use the new values for the maximum bandwidth, the FeatureSetDownlinkPerCC needs to be updated as follow:

FeatureSetDownlinkPerCC ::=         SEQUENCE {

supportedSubcarrierSpacingDL        SubcarrierSpacing,

supportedBandwidthDL                SupportedBandwidth,

channelBW-90mhz                     ENUMERATED {supported}                     OPTIONAL,

maxNumberMIMO-LayersPDSCH           MIMO-LayersDL                              OPTIONAL,

supportedModulationOrderDL          ModulationOrder                            OPTIONAL

}

FeatureSetDownlinkPerCC-v1620 ::=   SEQUENCE {

-- R1 16-2a: Mulit-DCI based multi-TRP

multiDCI-MultiTRP-r16               MultiDCI-MultiTRP-r16                     OPTIONAL,

-- R1 16-2b-3: Support of single-DCI based FDMSchemeB

supportFDM-SchemeB-r16              ENUMERATED {supported}                    OPTIONAL

}

FeatureSetDownlinkPerCC-v17xy ::=   SEQUENCE {

supportedMinBandwidthDL-r17         SupportedBandwidth-v17xy                    OPTIONAL,

broadcast-SCell-r17          		ENUMERATED {supported}                      OPTIONAL,

supportedBandwidthDL-v17xy          SupportedBandwidth-v17xy					OPTIONAL

}

With the above addition, there is a need to update the field description to provide the relation between the supportedBandwidthDL and supportedBandwidthDL-v17xy. In the case of FR2-2, for SCS480 and 960 kHz, UE can also set the supportedBandwidthDL-v17xy if the maximum DL channel bandwidth supported is greater than 400MHz or otherwise it is absent as follow:

| supportedBandwidthDL, supportedBandwidthDL-v17xy Indicates maximum DL channel bandwidth supported for a given SCS that UE supports within a single CC (and in case of intra-frequency DAPS handover for the source and target cells), which is defined in Table 5.3.5-1 in TS 38.101-1 [2] for FR1 and Table 5.3.5-1 in TS 38.101-2 [3] for FR2. For FR1, all the bandwidths listed in TS38.101-1 Table 5.3.5-1 for each band shall be mandatory with a single CC unless indicated optional. For FR2-1, the set of mandatory CBW is 50, 100, 200 MHz. When this field is included in a band combination with a single band entry and a single CC entry (i.e. non-CA band combination), the UE shall indicate the maximum channel bandwidth for the band according to TS 38.101-1 [2] and TS 38.101-2 [3]. For SCS 480kHz and 960kHz in FR2-2, supportedBandwidthDL (with suffix) can be set if the maximum DL channel bandwidth supported is greater than 400MHz, otherwise it is absent. The UE may report a supportedBandwidthDL wider than the channelBWs-DL; this supportedBandwidthDL may not be included in the Table 5.3.5-1 of TS 38.101-1[2]/TS 38.101-2[3] for the case that the UE is unable to report the actual supported bandwidth according to the Table 5.3.5-1 of TS 38.101-1[2]/TS 38.101-2[3]. For each band, RedCap UEs shall indicate its maximum channel bandwidth, which is the maximum of those channel bandwidths that are less than or equal to 20 MHz for FR1 and less than or equal to 100 Mhz for FR2, taking restrictions in TS 38.101-1 [2] and TS 38.101-2 [3] into consideration. NOTE: To determine whether the UE supports a channel bandwidth of 90 MHz, the network may ignore this capability and validate instead the channelBW-90mhz, the supportedBandwidthCombinationSet and the supportedBandwidthCombinationSetIntraENDC. For serving cell(s) with other channel bandwidths the network validates the channelBWs-DL, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, the asymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]), supportedBandwidthDL/ supportedBandwidthDL (with suffix) and supportedMinBandwidthDL. | FSPC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |

Proposal#3: The field description for supportedBandwidthDL is updated as follow (similarly also to supportedBandwidthUL):

| supportedBandwidthDL, supportedBandwidthDL-v17xy Indicates maximum DL channel bandwidth supported for a given SCS that UE supports within a single CC (and in case of intra-frequency DAPS handover for the source and target cells), which is defined in Table 5.3.5-1 in TS 38.101-1 [2] for FR1 and Table 5.3.5-1 in TS 38.101-2 [3] for FR2. For FR1, all the bandwidths listed in TS38.101-1 Table 5.3.5-1 for each band shall be mandatory with a single CC unless indicated optional. For FR2-1, the set of mandatory CBW is 50, 100, 200 MHz. When this field is included in a band combination with a single band entry and a single CC entry (i.e. non-CA band combination), the UE shall indicate the maximum channel bandwidth for the band according to TS 38.101-1 [2] and TS 38.101-2 [3]. For SCS 480kHz and 960kHz in FR2-2, supportedBandwidthDL (with suffix) can be set if the maximum DL channel bandwidth supported is greater than 400MHz, otherwise it is absent. The UE may report a supportedBandwidthDL wider than the channelBWs-DL; this supportedBandwidthDL may not be included in the Table 5.3.5-1 of TS 38.101-1[2]/TS 38.101-2[3] for the case that the UE is unable to report the actual supported bandwidth according to the Table 5.3.5-1 of TS 38.101-1[2]/TS 38.101-2[3]. For each band, RedCap UEs shall indicate its maximum channel bandwidth, which is the maximum of those channel bandwidths that are less than or equal to 20 MHz for FR1 and less than or equal to 100 Mhz for FR2, taking restrictions in TS 38.101-1 [2] and TS 38.101-2 [3] into consideration. NOTE: To determine whether the UE supports a channel bandwidth of 90 MHz, the network may ignore this capability and validate instead the channelBW-90mhz, the supportedBandwidthCombinationSet and the supportedBandwidthCombinationSetIntraENDC. For serving cell(s) with other channel bandwidths the network validates the channelBWs-DL, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, the asymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]), supportedBandwidthDL/ supportedBandwidthDL (with suffix) and supportedMinBandwidthDL. | FSPC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |

Proposal#3-1: The corresponding ASN.1 update for FeatureSetDownlinkPerCC (also FeatureSetUplinkPerCC) is as follow:

FeatureSetDownlinkPerCC ::=         SEQUENCE {

supportedSubcarrierSpacingDL        SubcarrierSpacing,

supportedBandwidthDL                SupportedBandwidth,

channelBW-90mhz                     ENUMERATED {supported}                     OPTIONAL,

maxNumberMIMO-LayersPDSCH           MIMO-LayersDL                              OPTIONAL,

supportedModulationOrderDL          ModulationOrder                            OPTIONAL

}

FeatureSetDownlinkPerCC-v1620 ::=   SEQUENCE {

-- R1 16-2a: Mulit-DCI based multi-TRP

multiDCI-MultiTRP-r16               MultiDCI-MultiTRP-r16                     OPTIONAL,

-- R1 16-2b-3: Support of single-DCI based FDMSchemeB

supportFDM-SchemeB-r16              ENUMERATED {supported}                    OPTIONAL

}

FeatureSetDownlinkPerCC-v17xy ::=   SEQUENCE {

supportedMinBandwidthDL-r17         SupportedBandwidth-v1700                    OPTIONAL,

broadcast-SCell-r17          		ENUMERATED {supported}                      OPTIONAL,

supportedBandwidthDL-v17xy          SupportedBandwidth-v1700					OPTIONAL

}

### 2.1.2 Updates to channelBWs (UL/DL)

The existing field descriptions for the channelBWs-DL and channelBWs-UL are as follow:

| channelBWs-DL Indicates for each subcarrier spacing the UE supported channel bandwidths. Absence of the channelBWs-DL (without suffix) for a band or absence of specific scs-XXkHz entry for a supported subcarrier spacing means that the UE supports the channel bandwidths among [5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100] and [50, 100, 200] that were defined in clause 5.3.5 of TS 38.101-1 version 15.7.0 [2] and TS 38.101-2 version 15.7.0 [3] for the given band or the specific SCS entry. For IAB-MT, to determine whether the IAB-MT supports a channel bandwidth of 100 MHz, the network checks channelBW-DL-IAB-r16. For FR1, the bits in channelBWs-DL (without suffix) starting from the leading / leftmost bit indicate 5, 10, 15, 20, 25, 30, 40, 50, 60 and 80MHz. For FR2, the bits in channelBWs-DL (without suffix) starting from the leading / leftmost bit indicate 50, 100 and 200MHz. The third / rightmost bit (for 200MHz) shall be set to 1. For IAB-MT the third / rightmost bit (for 200MHz) is ignored. To determine whether the IAB-MT supports a channel bandwidth of 200 MHz, the network checks channelBW-DL-IAB-r16. For FR1, the leading/leftmost bit in channelBWs-DL-v1590 indicates 70MHz, the second leftmost bit indicates 45MHz, the third leftmost bit indicates 35MHz, the fourth leftmost bit indicates 100MHz and all the remaining bits in channelBWs-DL-v1590 shall be set to 0. The fourth leftmost bit (for 100MHz) is not applicable for bands n41, n48, n77, n78, n79 and n90 as defined in TS 38.101-1 [2]. NOTE: To determine whether the UE supports a specific SCS for a given band, the network validates the supportedSubCarrierSpacingDL and the scs-60kHz. To determine whether the UE supports a channel bandwidth of 90 MHz, the network may ignore this capability and validate instead the channelBW-90mhz, the supportedBandwidthCombinationSet and the supportedBandwidthCombinationSetIntraENDC. For serving cell(s) with other channel bandwidths the network validates the channelBWs-DL, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, the asymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]) and supportedBandwidthDL. | Band | Yes | N/A | N/A |
| --- | --- | --- | --- | --- |
| channelBWs-UL Indicates for each subcarrier spacing the UE supported channel bandwidths. Absence of the channelBWs-UL (without suffix) for a band or absence of specific scs-XXkHz entry for a supported subcarrier spacing means that the UE supports the channel bandwidths among [5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100] and [50, 100, 200] that were defined in clause 5.3.5 of TS 38.101-1 version 15.7.0 [2] and TS 38.101-2 version 15.7.0 [3] for the given band or the specific SCS entry. For IAB-MT, to determine whether the IAB-MT supports a channel bandwidth of 100 MHz, the network checks channelBW-UL-IAB-r16. For FR1, the bits in channelBWs-UL (without suffix) starting from the leading / leftmost bit indicate 5, 10, 15, 20, 25, 30, 40, 50, 60 and 80MHz. For FR2, the bits in channelBWs-UL (without suffix) starting from the leading / leftmost bit indicate 50, 100 and 200MHz. The third / rightmost bit (for 200MHz) shall be set to 1. For IAB-MT the third / rightmost bit (for 200MHz) is ignored. To determine whether the IAB-MT supports a channel bandwidth of 200 MHz, the network checks channelBW-UL-IAB-r16. For FR1, the leading/leftmost bit in channelBWs-UL-v1590 indicates 70 MHz, the second leftmost bit indicates 45MHz, the third leftmost bit indicates 35MHz, the fourth leftmost bit indicates 100MHz and all the remaining bits in channelBWs-UL-v1590 shall be set to 0. The fourth leftmost bit (for 100MHz) is not applicable for bands n41, n48, n77, n78, n79 and n90 as defined in TS 38.101-1 [2]. NOTE: To determine whether the UE supports a specific SCS for a given band, the network validates the supportedSubCarrierSpacingUL and the scs-60kHz. To determine whether the UE supports a channel bandwidth of 90 MHz the network may ignore this capability and validate instead the channelBW-90mhz, the supportedBandwidthCombinationSet and the supportedBandwidthCombinationSetIntraENDC. For serving cell(s) with other channel bandwidths the network validates the channelBWs-UL, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, the asymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]) and supportedBandwidthUL. | Band | Yes | N/A | N/A |

As the existing channelBWs-DL/UL are mandatory with capability signalling only for FR1 and FR2-1 as follow,

a new channelBWs-DL/UL may need to be introduced to include SCS 120kHz, 480kHz and 960kHz channel bandwidth if based on the legacy Rel-15 signalling.

The field descriptions for the new channelBW-DL/UL for FR2-2 which is mandatory with capability signalling as like existing Rel-15/16 capability will be as follow:

| channelBWs-DL-FR2-2-r17 Indicates for each subcarrier spacing the UE supported channel bandwidths in DL. Absence of the channelBWs-DL-FR2-2 for a FR2-2 band or absence of specific scs-XXkHz entry for a supported subcarrier spacing means that the UE supports the channel bandwidth that were defined in clause 5.3.5 of TS 38.101-1 version 15.7.0 [2] and TS 38.101-2 version 15.7.0 [3] for the given band or the specific SCS entry. For SCS 120kHz, the bits in channelBWs-DL-FR2-2 starting from the leading / leftmost bit indicate 100 and 400MHz. For SCS 480kHz, the bits in channelBWs-DL-FR2-2 starting from the leading / leftmost bit indicate 400, 800 and 1600MHz.. For SCS 960kHz, the bits in channelBWs-DL-FR2-2 starting from the leading / leftmost bit indicate 400, 800, 1600 and 2000MHz.. This feature is mandatory for FR2-2 band, otherwise it is absent NOTE: To determine whether the UE supports a specific SCS for a given band, the network validates the supportedSubCarrierSpacingDL. For serving cell(s) with other channel bandwidths the network validates the channelBWs-DL-FR2-2-r17, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, the asymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]) and supportedBandwidthDL (with suffix). | Band | CY | N/A | N/A |
| --- | --- | --- | --- | --- |
| channelBWs-UL-FR2-2-r17 Indicates for each subcarrier spacing the UE supported channel bandwidths in the UL. Absence of the channelBWs-UL-FR2-2 for a FR2-2 band or absence of specific scs-XXkHz entry for a supported subcarrier spacing means that the UE supports the channel bandwidth that were defined in clause 5.3.5 of TS 38.101-1 version 15.7.0 [2] and TS 38.101-2 version 15.7.0 [3] for the given band or the specific SCS entry. For SCS 120kHz, the bits in channelBWs-UL-FR2-2 starting from the leading / leftmost bit indicate 100 and 400MHz. For SCS 480kHz, the bits in channelBWs-UL-FR2-2 starting from the leading / leftmost bit indicate 400, 800 and 1600MHz.. For SCS 960kHz, the bits in channelBWs-UL-FR2-2 starting from the leading / leftmost bit indicate 400, 800, 1600 and 2000MHz.. This feature is mandatory for FR2-2 band, otherwise it is absent NOTE: To determine whether the UE supports a specific SCS for a given band, the network validates the supportedSubCarrierSpacingUL. For serving cell(s) with other channel bandwidths the network validates the channelBWs-UL-FR2-2-r17, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, the asymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]) and supportedBandwidthUL (with suffix). | Band | CY | N/A | N/A |

Based on the existing channelBWs-UL/DL, the number of bits for the supported BW for each SCS are in accordance with the BW supported for each SCS for FR2-2.

Agreement: For intermediate CBWs between min and max CBWs,

- Integer multiples of the min CBW for each SCS

  - 120 kHz: 100 MHz (min), 400 MHz (max)

  - 480 kHz: 400 MHz (min), 800 MHz, 1600 MHz (max)

  - 960 kHz: 400 MHz (min), 800 MHz, 1600 MHz, 2000 MHz (max)

channelBWs-DL-FR2-2-r17                       SEQUENCE {

scs-120kHz                           BIT STRING (SIZE (2))                      OPTIONAL,

scs-480kHz                           BIT STRING (SIZE (3))                      OPTIONAL,

scs-960kHz                           BIT STRING (SIZE (4))                      OPTIONAL

}                                                                                  OPTIONAL,

channelBWs-UL-FR2-2-r17                       SEQUENCE {

scs-120kHz                           BIT STRING (SIZE (2))                      OPTIONAL,

scs-480kHz                           BIT STRING (SIZE (3))                      OPTIONAL,

scs-960kHz                           BIT STRING (SIZE (4))                      OPTIONAL

}                                                                                   OPTIONAL,

However, our understanding is that these per band channel bandwidth capabilities for UL and DL are also being discussed in RAN4 and there is also the option that UE just needs to optionally indicate the SCS supported for FR2-2 (as per RAN1 features 24-1/1a (for 120kHz) SCS) 24-4/4a (for 480kHz SCS) and 24-5/5a (for 960kHz SCS)) and if UE indicates a SCS is supported per R1 feature, UE mandatory supports all the bandwidths corresponding to the SCS without capability signalling.  Hence RAN2 should wait for RAN4 conclusion on this.

Proposal#4: For the per band channelBWs-DL/UL UE capabilities for FR2-2, RAN2 should wait for RAN4 progress on the R4 feature list.

For the existing channelBWs-UL/DL, it is currently mandatory set. It should be changed to conditional mandatory for FR1 and FR2-1 band.

Proposal#5: The existing channelBWs-UL/DL should be changed to conditional mandatory for FR1 and FR2-1 band as follow:

| channelBWs-DL Indicates for each subcarrier spacing the UE supported channel bandwidths. Absence of the channelBWs-DL (without suffix) for a band or absence of specific scs-XXkHz entry for a supported subcarrier spacing means that the UE supports the channel bandwidths among [5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100] and [50, 100, 200] that were defined in clause 5.3.5 of TS 38.101-1 version 15.7.0 [2] and TS 38.101-2 version 15.7.0 [3] for the given band or the specific SCS entry. For IAB-MT, to determine whether the IAB-MT supports a channel bandwidth of 100 MHz, the network checks channelBW-DL-IAB-r16. For FR1, the bits in channelBWs-DL (without suffix) starting from the leading / leftmost bit indicate 5, 10, 15, 20, 25, 30, 40, 50, 60 and 80MHz. For FR2, the bits in channelBWs-DL (without suffix) starting from the leading / leftmost bit indicate 50, 100 and 200MHz. The third / rightmost bit (for 200MHz) shall be set to 1. For IAB-MT the third / rightmost bit (for 200MHz) is ignored. To determine whether the IAB-MT supports a channel bandwidth of 200 MHz, the network checks channelBW-DL-IAB-r16. For FR1, the leading/leftmost bit in channelBWs-DL-v1590 indicates 70MHz, the second leftmost bit indicates 45MHz, the third leftmost bit indicates 35MHz, the fourth leftmost bit indicates 100MHz and all the remaining bits in channelBWs-DL-v1590 shall be set to 0. The fourth leftmost bit (for 100MHz) is not applicable for bands n41, n48, n77, n78, n79 and n90 as defined in TS 38.101-1 [2]. This feature is mandatory for FR1 and FR2-1 band, otherwise it is absent. NOTE: To determine whether the UE supports a specific SCS for a given band, the network validates the supportedSubCarrierSpacingDL and the scs-60kHz. To determine whether the UE supports a channel bandwidth of 90 MHz, the network may ignore this capability and validate instead the channelBW-90mhz, the supportedBandwidthCombinationSet and the supportedBandwidthCombinationSetIntraENDC. For serving cell(s) with other channel bandwidths the network validates the channelBWs-DL, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, the asymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]) and supportedBandwidthDL. | Band | YesCY | N/A | N/A |
| --- | --- | --- | --- | --- |
| channelBWs-UL Indicates for each subcarrier spacing the UE supported channel bandwidths. Absence of the channelBWs-UL (without suffix) for a band or absence of specific scs-XXkHz entry for a supported subcarrier spacing means that the UE supports the channel bandwidths among [5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100] and [50, 100, 200] that were defined in clause 5.3.5 of TS 38.101-1 version 15.7.0 [2] and TS 38.101-2 version 15.7.0 [3] for the given band or the specific SCS entry. For IAB-MT, to determine whether the IAB-MT supports a channel bandwidth of 100 MHz, the network checks channelBW-UL-IAB-r16. For FR1, the bits in channelBWs-UL (without suffix) starting from the leading / leftmost bit indicate 5, 10, 15, 20, 25, 30, 40, 50, 60 and 80MHz. For FR2, the bits in channelBWs-UL (without suffix) starting from the leading / leftmost bit indicate 50, 100 and 200MHz. The third / rightmost bit (for 200MHz) shall be set to 1. For IAB-MT the third / rightmost bit (for 200MHz) is ignored. To determine whether the IAB-MT supports a channel bandwidth of 200 MHz, the network checks channelBW-UL-IAB-r16. For FR1, the leading/leftmost bit in channelBWs-UL-v1590 indicates 70 MHz, the second leftmost bit indicates 45MHz, the third leftmost bit indicates 35MHz, the fourth leftmost bit indicates 100MHz and all the remaining bits in channelBWs-UL-v1590 shall be set to 0. The fourth leftmost bit (for 100MHz) is not applicable for bands n41, n48, n77, n78, n79 and n90 as defined in TS 38.101-1 [2]. This feature is mandatory for FR1 and FR2-1 band, otherwise it is absent. NOTE: To determine whether the UE supports a specific SCS for a given band, the network validates the supportedSubCarrierSpacingUL and the scs-60kHz. To determine whether the UE supports a channel bandwidth of 90 MHz the network may ignore this capability and validate instead the channelBW-90mhz, the supportedBandwidthCombinationSet and the supportedBandwidthCombinationSetIntraENDC. For serving cell(s) with other channel bandwidths the network validates the channelBWs-UL, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, the asymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]) and supportedBandwidthUL. | Band | YesCY | N/A | N/A |

## 2.2	UE capability for new values for drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL

RAN2 agreed to the introduce conditional mandatory UE capability for

- RAN2 to confirm to introduce values up to 448 (integer 0..448) for DRX parameters for SCS of 480 and 960 kHz, for drx-HARQ-RTT-TimerDL and drx-HARQ-RTT-TimerUL. These are conditionally mandatory for FR2-2 UEs.

- The capability clarification can be done in the next meeting.

From the RRC implementation, 2 new IEs are added {drx-HARQ-RTT-TimerDL-r17 and drx-HARQ-RTT-TimerUL-r17} with the following field description:

drx-HARQ-RTT-TimerDL Value in number of symbols of the BWP where the transport block was received. drx-HARQ-RTT-TimerDL-r17 is only applicable for SCS 480 kHz and 960 kHz. If configured, the UE shall ignore drx-HARQ-RTT-TimerDL (without suffix).

drx-HARQ-RTT-TimerUL Value in number of symbols of the BWP where the transport block was transmitted. drx-HARQ-RTT-TimerUL-r17 is only applicable for SCS 480 kHz and 960 kHz. If configured, the UE shall ignore drx-HARQ-RTT-TimerUL (without suffix).

Conditionally mandatory can be introduced with or without signalling.  Our understanding is that these new configurations are only applicable to UE supporting FR2-2 bands using SCS 480kHz and 960kHz. Hence, we assume that it is for without signalling. A new conditionally mandatory UE cap can be added to Section 6 of TS38.306 as follow:

| Features | Condition |
| --- | --- |
| Extended values for drx-HARQ-RTT-TimerDL/UL | It is mandatory for UEs which support FR2-2 bands with SCS 480kHz and/or 960kHz. |

Proposal#6: Introduce new conditionally mandatory without capability signalling for UE supporting the extended values of DRX HARQ RTT Timer {drx-HARQ-RTT-TimerDL-r17 and drx-HARQ-RTT-TimerUL-r17} as follow in Section 6:

| Features | Condition |
| --- | --- |
| Extended values for drx-HARQ-RTT-TimerDL/UL | It is mandatory for UEs which support FR2-2 bands with SCS 480kHz and/or 960kHz. |

## 2.3	Other miscellaneous correction

While doing some check, we noticed that the following UE capability in MAC with FRx differentiation has not been implemented for further differentiation between FR2-1 and FR2-2

| drx-Adaptation-r16 Indicates whether the UE supports DRX adaptation comprised of the following functional components: - Configured ps-Offset for the detection of DCI format 2_6 with CRC scrambling by ps-RNTI and reported MinTimeGap before the start of drx-onDurationTimer of Long DRX - Indication of UE whether or not to start drx-onDurationTimer for the next Long DRX cycle by detection of DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active Time - Configured periodic CSI report apart from L1-RSRP (ps-TransmitOtherPeriodicCSI) when impacted by DCI format 2_6 that drx-onDurationTimer does not start for the next Long DRX cycle - Configured periodic L1-RSRP report (ps-TransmitPeriodicL1-RSRP) when impacted by DCI format 2_6 that drx-onDurationTimer does not start for the next Long DRX cycle The capability signalling includes the minimum time gap between the end of the slot of last DCI format 2_6 monitoring occasion and the beginning of the slot where the UE would start the drx-onDurationTimer of Long DRX for each SCS. The value sl1 indicates 1 slot. The value sl2 indicates 2 slots, and so on. Support of this feature is reported for licensed and unlicensed bands, respectively. When this field is reported, either of sharedSpectrumChAccess-r16 or non-SharedSpectrumChAccess-r16 shall be reported, at least. | UE | No | No | Yes |
| --- | --- | --- | --- | --- |

Hence, it needs to be aligned with the other in MAC.

Proposal#7: Introduce further differentiation between FR2-1 and FR2-2 for drx-Adaptation-r16.

# 3	Conclusion

It is requested that RAN2 agree to the proposals below:

Proposal#1_1: Wait for RAN4 to indicate which CBW (channel bandwidth) is mandatory for FR2-2.

Proposal#1_2: Update “For FR2, the set of mandatory CBW is 50, 100, 200 MHz.” in supportedBandwidthUL/DL is just for FR2-1.

Proposal#2: Introduce the new CBWs (800MHz, 1600MHz and 2000MHz) for SCS 480kHz and 960kHz to the SupportedBandwidth-v1700 as follow.

–                  SupportedBandwidth

The IE SupportedBandwidth is used to indicate the channel bandwidth supported by the UE on one carrier of a band of a band combination.

SupportedBandwidth information element

-- ASN1START

-- TAG-SUPPORTEDBANDWIDTH-START

SupportedBandwidth::=      CHOICE {

fr1    ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz40, mhz50, mhz60, mhz80, mhz100},

fr2    ENUMERATED {mhz50, mhz100, mhz200, mhz400}

}

SupportedBandwidth-v1700 ::=      CHOICE {

fr1-r17         ENUMERATED {mhz5, mhz10, mhz15, mhz20, mhz25, mhz30, mhz35, mhz40, mhz45, mhz50, mhz60, mhz70, mhz80, mhz90, mhz100},

fr2-r17         ENUMERATED {mhz50, mhz100, mhz200, mhz400, mhz800, mhz1600, mhz2000}

}

-- TAG-SUPPORTEDBANDWIDTH-STOP

-- ASN1STOP

Proposal#3: The field description for supportedBandwidthDL is updated as follow (similarly also to supportedBandwidthUL):

| supportedBandwidthDL, supportedBandwidthDL-v17xy Indicates maximum DL channel bandwidth supported for a given SCS that UE supports within a single CC (and in case of intra-frequency DAPS handover for the source and target cells), which is defined in Table 5.3.5-1 in TS 38.101-1 [2] for FR1 and Table 5.3.5-1 in TS 38.101-2 [3] for FR2. For FR1, all the bandwidths listed in TS38.101-1 Table 5.3.5-1 for each band shall be mandatory with a single CC unless indicated optional. For FR2-1, the set of mandatory CBW is 50, 100, 200 MHz. When this field is included in a band combination with a single band entry and a single CC entry (i.e. non-CA band combination), the UE shall indicate the maximum channel bandwidth for the band according to TS 38.101-1 [2] and TS 38.101-2 [3]. For SCS 480kHz and 960kHz in FR2-2, supportedBandwidthDL (with suffix) can be set if the maximum DL channel bandwidth supported is greater than 400MHz, otherwise it is absent. The UE may report a supportedBandwidthDL wider than the channelBWs-DL; this supportedBandwidthDL may not be included in the Table 5.3.5-1 of TS 38.101-1[2]/TS 38.101-2[3] for the case that the UE is unable to report the actual supported bandwidth according to the Table 5.3.5-1 of TS 38.101-1[2]/TS 38.101-2[3]. For each band, RedCap UEs shall indicate its maximum channel bandwidth, which is the maximum of those channel bandwidths that are less than or equal to 20 MHz for FR1 and less than or equal to 100 Mhz for FR2, taking restrictions in TS 38.101-1 [2] and TS 38.101-2 [3] into consideration. NOTE: To determine whether the UE supports a channel bandwidth of 90 MHz, the network may ignore this capability and validate instead the channelBW-90mhz, the supportedBandwidthCombinationSet and the supportedBandwidthCombinationSetIntraENDC. For serving cell(s) with other channel bandwidths the network validates the channelBWs-DL, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, the asymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]), supportedBandwidthDL(supportedBandwidthDL (with suffix)) and supportedMinBandwidthDL. | FSPC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |

Proposal#3-1: The corresponding ASN.1 update for FeatureSetDownlinkPerCC (also FeatureSetUplinkPerCC) is as follow:

FeatureSetDownlinkPerCC ::=         SEQUENCE {

supportedSubcarrierSpacingDL        SubcarrierSpacing,

supportedBandwidthDL                SupportedBandwidth,

channelBW-90mhz                     ENUMERATED {supported}                     OPTIONAL,

maxNumberMIMO-LayersPDSCH           MIMO-LayersDL                              OPTIONAL,

supportedModulationOrderDL          ModulationOrder                            OPTIONAL

}

FeatureSetDownlinkPerCC-v1620 ::=   SEQUENCE {

-- R1 16-2a: Mulit-DCI based multi-TRP

multiDCI-MultiTRP-r16               MultiDCI-MultiTRP-r16                     OPTIONAL,

-- R1 16-2b-3: Support of single-DCI based FDMSchemeB

supportFDM-SchemeB-r16              ENUMERATED {supported}                    OPTIONAL

}

FeatureSetDownlinkPerCC-v17xy ::=   SEQUENCE {

supportedMinBandwidthDL-r17         SupportedBandwidth-v17xy                    OPTIONAL,

broadcast-SCell-r17          		ENUMERATED {supported}                      OPTIONAL,

supportedBandwidthDL-v17xy          SupportedBandwidth-v1700					OPTIONAL

}

Proposal#4: For the per band channelBWs-DL/UL UE capabilities for FR2-2, RAN2 should wait for RAN4 progress on the feature list

Proposal#5: The existing channelBWs-UL/DL should be changed to conditional mandatory for FR1 and FR2-1 band as follow:

| channelBWs-DL Indicates for each subcarrier spacing the UE supported channel bandwidths. Absence of the channelBWs-DL (without suffix) for a band or absence of specific scs-XXkHz entry for a supported subcarrier spacing means that the UE supports the channel bandwidths among [5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100] and [50, 100, 200] that were defined in clause 5.3.5 of TS 38.101-1 version 15.7.0 [2] and TS 38.101-2 version 15.7.0 [3] for the given band or the specific SCS entry. For IAB-MT, to determine whether the IAB-MT supports a channel bandwidth of 100 MHz, the network checks channelBW-DL-IAB-r16. For FR1, the bits in channelBWs-DL (without suffix) starting from the leading / leftmost bit indicate 5, 10, 15, 20, 25, 30, 40, 50, 60 and 80MHz. For FR2, the bits in channelBWs-DL (without suffix) starting from the leading / leftmost bit indicate 50, 100 and 200MHz. The third / rightmost bit (for 200MHz) shall be set to 1. For IAB-MT the third / rightmost bit (for 200MHz) is ignored. To determine whether the IAB-MT supports a channel bandwidth of 200 MHz, the network checks channelBW-DL-IAB-r16. For FR1, the leading/leftmost bit in channelBWs-DL-v1590 indicates 70MHz, the second leftmost bit indicates 45MHz, the third leftmost bit indicates 35MHz, the fourth leftmost bit indicates 100MHz and all the remaining bits in channelBWs-DL-v1590 shall be set to 0. The fourth leftmost bit (for 100MHz) is not applicable for bands n41, n48, n77, n78, n79 and n90 as defined in TS 38.101-1 [2]. This feature is mandatory for FR1 or FR2-1 band, otherwise it is absent. NOTE: To determine whether the UE supports a specific SCS for a given band, the network validates the supportedSubCarrierSpacingDL and the scs-60kHz. To determine whether the UE supports a channel bandwidth of 90 MHz, the network may ignore this capability and validate instead the channelBW-90mhz, the supportedBandwidthCombinationSet and the supportedBandwidthCombinationSetIntraENDC. For serving cell(s) with other channel bandwidths the network validates the channelBWs-DL, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, the asymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]) and supportedBandwidthDL. | Band | YesCY | N/A | N/A |
| --- | --- | --- | --- | --- |
| channelBWs-UL Indicates for each subcarrier spacing the UE supported channel bandwidths. Absence of the channelBWs-UL (without suffix) for a band or absence of specific scs-XXkHz entry for a supported subcarrier spacing means that the UE supports the channel bandwidths among [5, 10, 15, 20, 25, 30, 40, 50, 60, 80, 100] and [50, 100, 200] that were defined in clause 5.3.5 of TS 38.101-1 version 15.7.0 [2] and TS 38.101-2 version 15.7.0 [3] for the given band or the specific SCS entry. For IAB-MT, to determine whether the IAB-MT supports a channel bandwidth of 100 MHz, the network checks channelBW-UL-IAB-r16. For FR1, the bits in channelBWs-UL (without suffix) starting from the leading / leftmost bit indicate 5, 10, 15, 20, 25, 30, 40, 50, 60 and 80MHz. For FR2, the bits in channelBWs-UL (without suffix) starting from the leading / leftmost bit indicate 50, 100 and 200MHz. The third / rightmost bit (for 200MHz) shall be set to 1. For IAB-MT the third / rightmost bit (for 200MHz) is ignored. To determine whether the IAB-MT supports a channel bandwidth of 200 MHz, the network checks channelBW-UL-IAB-r16. For FR1, the leading/leftmost bit in channelBWs-UL-v1590 indicates 70 MHz, the second leftmost bit indicates 45MHz, the third leftmost bit indicates 35MHz, the fourth leftmost bit indicates 100MHz and all the remaining bits in channelBWs-UL-v1590 shall be set to 0. The fourth leftmost bit (for 100MHz) is not applicable for bands n41, n48, n77, n78, n79 and n90 as defined in TS 38.101-1 [2]. This feature is mandatory for FR1 or FR2-1 band, otherwise it is absent NOTE: To determine whether the UE supports a specific SCS for a given band, the network validates the supportedSubCarrierSpacingUL and the scs-60kHz. To determine whether the UE supports a channel bandwidth of 90 MHz the network may ignore this capability and validate instead the channelBW-90mhz, the supportedBandwidthCombinationSet and the supportedBandwidthCombinationSetIntraENDC. For serving cell(s) with other channel bandwidths the network validates the channelBWs-UL, the supportedBandwidthCombinationSet, the supportedBandwidthCombinationSetIntraENDC, the asymmetricBandwidthCombinationSet (for a band supporting asymmetric channel bandwidth as defined in clause 5.3.6 of TS 38.101-1 [2]) and supportedBandwidthUL. | Band | YesCY | N/A | N/A |

Proposal#6: Introduce new conditionally mandatory without capability signalling for UE supporting the extended values of DRX HARQ RTT Timer {drx-HARQ-RTT-TimerDL-r17 and drx-HARQ-RTT-TimerUL-r17} as follow in Section 6:

| Features | Condition |
| --- | --- |
| Extended values for drx-HARQ-RTT-TimerDL/UL | It is mandatory for UEs which support FR2-2 bands with SCS 480kHz and/or 960kHz. |

Proposal#7: Introduce further differentiation between FR2-1 and FR2-2 for drx-Adaptation-r16

The corresponding CRs can be found in [1] and [2].

# 5	References

[1] R2-2205793 Further updates for 71GHz UE capabilities (TS38.306), Intel Corporation

[1] R2-2205794 Further updates for 71GHz UE capabilities (TS38.331), Intel Corporation