3GPP TSG-RAN WG2 Meeting #118-e					          R2-2205817

e-Meeting, May 9th – 20th, 2022

Agenda Item:	6.11.2.9

Source:	Ericsson

Title:	[RIL E061] Discussion on RRC Procedure Structure on section

Document for:	Discussion, Decision

# 1	Introduction

In this paper, we discuss the RRC structure on how to capture the NR positioning.

# 2	Discussion

## 2.1		Current Structure

The current structure is shown below; it shows the new positioning procedure is mainly part of other config procedure.

Then the legacy positioning measurement gap request is part of Measurement procedure.

## 2.2		Proposed Structure

Considering Rel-18 would further require RRC changes; it may benefit if a dedicated Positioning section is defined similar to MBS broadcast, Sidelink.

This section can basically include the current 5.7.14 and 5.7.15 and the introduction can point to the legacy existing procedure.

## 5.10	Positioning

### 5.10.1	Introduction

#### 5.10.1.1	General

The RRC protocol includes the following positioning functions:

-	Location measurement gap request procedures as described in section 5.5.6:

-	Broadcast of positioning system information:

-	SRS for Positioning Configuration:

-	Preconfiguration of measurement gaps and PPW:

### 5.10.2	UE Positioning Assistance Information

#### 5.10.2.1	General

Figure 5.7.14.1-1: UE Positioning Assistance Information procedure

The UE Positioning Assistance Information procedure is used by UE to report the UE Positioning Assistance Information. The UE reports the association between UL-SRS resources for positioning and the UE Tx TEG ID.

#### 5.10.2.2	Initiation

A UE capable of providing the association between UL SRS Resource for positioning and UE Tx TEG ID in RRC_CONNECTED may initiate the procedure upon being configured to provide this association information.

Upon initiation of the procedure, the UE shall:

1> initiate transmission of the UEPositioningAssistanceInfo message in accordance with 5.7.14.3 to provide the association.

#### 5.10.2.3	Actions related to transmission of UEPositioningAssistanceInfo message

The UE shall set the contents of the UEPositioningAssistanceInfo message as follows:

1>	if transmission of the UEPositioningAssistanceInfo message is initiated to provide the association between UL SRS Resource for positioning and Tx TEG according to 5.7.14.2;

2> if ue-TxTEG-RequestUL-TDOA-Config in SRS-Config IE is configured with periodicReporting;

3> include ue-TxTEG-AssociationList in the UEPositioningAssistanceInfo message on expiry of each configured period;

2> else if ue-TxTEG-RequestUL-TDOA-Config in SRS-Config IE is configured with oneShot:

3>	include ue-TxTEG-AssociationList in the UEPositioningAssistanceInfo message.

### 5.10.3	SRS for Positioning in RRC_INACTIVE

#### 5.10.3.1	General

SRS for Positioning can be configured to be transmitted in RRC_INACTIVE mode.

Figure 5.7.15.1-1: SRS For Positioning Configuration in RRC INACTIVE Mode

#### 5.10.3.2	Actions Related to SRS for Positioning at Cell Re-selection in RRC_INACTIVE

The UE shall:

1> if cell reselection occurs when srs-PosRRC-InactiveConfig is configured:

2>	consider the Timing Advance value for SRS for Positioning transmission to be invalid;

2>	release the srs-PosRRC-InactiveConfig.

Editor's Note: To check further if clause 5.7.14 and 5.7.15 can be consolidated under new clause 5.X NR Positioning similar to clause 5.8 Sidelink.

Instead of Other, a sepertae positioning section can be easy to maintain and follow

A new Positioning section is defined.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Instead of Other, a sepertae positioning section can be easy to maintain and follow

Based on the discussion in the previous sections we propose the following:

Proposal 1	A new Positioning section is defined.

# References

3GPP TS 38.331 v17.0.0