3GPP TSG RAN WG2 Meeting #118-e                                                    R2-2205823

# Electronic meeting, 9th-20th May 2022

Agenda item:	6.6.3

Source:	Intel Corporation

Title:	[I507] Clarification of “part of the UE configuration” for SDT

Document for:	Discussion and decision

# Introduction

This document discusses the RIL [I507] that propose clarifying the reference to “part of the UE configuration” in the release procedural text introduced for SDT.

# Discussion

The procedure that describes UE’s operation upon reception of the RRCRelease captures the following handling for the RLC bearers:

3>	for each of the RLC bearer that is part of the UE configuration:

4>	re-establish the RLC entity as specified in TS 38.322 [4];

In our understanding, “part of the UE configuration” is an unclear term e.g. whether this refers to UE’s configuration that is stored and/or suspended and/or resumed. In our understanding this statement needs to be performed under the following scenarios:

- If UE is in RRC_CONNECTED and receives RRCRelease including sdt-Config.

- If UE is in RRC_INACTIVE with an SDT session ongoing and receives RRCRelease including sdt-Config.

For scenario 1, “part of the UE configuration” refers to the configuration in used by UE in RRC_CONNECTED (as it is the only configuration available). However for scenario 2), “part of the UE configuration” is ambiguous, i.e whether it refers to the configuration resumed and in used, vs the one suspended vs the one stored (including the non-SDT RBs). In our understanding, re-establishment is performed only on the RLC entities for the RBs bearers that are resumed for SDT.  Therefore, to avoid this and address both scenarios, we suggest referring to this as RLC bearers associated with RBs that are not suspended.

TS 38.331 captures RLC bearers for any RB part of UE configuration is resumed. For a UE in RRC_CONNECTED, this refers to RB of the configuration in used. For SDT, this is ambiguous as UE has RBs resumed and in used, as well as, others suspended and stored in UE AS Context (i.e. non-SDT RBs); however for SDT,  only resumed RBs should perform re-establishment of the RLC entities.

- Avoid the term “part of the UE configuration” when referring to UE’s configuration in used by a UE in RRC_CONNECTED or by a UE in RRC_INACTIVE with an SDT session ongoing (which should refer to SDT RBs and not to non-SDT RBs). Agreed on related TP available in section 4.1 of Annex (which suggest the use of “each RLC bearer associated with a RB that is not suspended”).

# Conclusion

The observations captured are the following:

Observation 1.	TS 38.331 captures RLC bearers for any RB part of UE configuration is resumed. For a UE in RRC_CONNECTED, this refers to RB of the configuration in used. For SDT, this is ambiguous as UE has RBs resumed and in used, as well as, others suspended and stored in UE AS Context (i.e. non-SDT RBs); however for SDT,  only resumed RBs should perform re-establishment of the RLC entities.

The proposals captured are the following:

Proposal 1.	Avoid the term “part of the UE configuration” when referring to UE’s configuration in used by a UE in RRC_CONNECTED or by a UE in RRC_INACTIVE with an SDT session ongoing (which should refer to SDT RBs and not to non-SDT RBs). Agreed on related TP available in section 4.1 of Annex (which suggest the use of “each RLC bearer associated with a RB that is not suspended”).

# Annex: Text Proposals (TPs)

Latest draft TS are used from https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/Specifications/202203_draft_specs_after_RAN_95/

- TP to 38.331 v17.0.0 (2022-03) i.e. version -h00-v6

## [I507] Clarify the reference to “part of the UE configuration” in the procedural text

- Modified Section

#### 5.3.8.3	Reception of the RRCRelease by the UE

The UE shall:

1>	delay the following actions defined in this sub-clause 60 ms from the moment the RRCRelease message was received or optionally when lower layers indicate that the receipt of the RRCRelease message has been successfully acknowledged, whichever is earlier;

1>	stop timer T380, if running;

1>	stop timer T320, if running;

1>	if timer T316 is running;

2>	stop timer T316;

2>	clear the information included in VarRLF-Report, if any;

1>	stop timer T350, if running;

1>	stop timer T3xx, if running;

1>	if the AS security is not activated:

2>	ignore any field included in RRCRelease message except waitTime;

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11 with the release cause 'other' upon which the procedure ends;

1>	if the RRCRelease message includes redirectedCarrierInfo indicating redirection to eutra:

2>	if cnType is included:

3>	after the cell selection, indicate the available CN Type(s) and the received cnType to upper layers;

NOTE 1:	Handling the case if the E-UTRA cell selected after the redirection does not support the core network type specified by the cnType, is up to UE implementation.

2>	if voiceFallbackIndication is included:

3>	consider the RRC connection release was for EPS fallback for IMS voice (see TS 23.502 [43]);

1>	if the RRCRelease message includes the cellReselectionPriorities or freqPriorityListNRSlicing:

2>	store the cell reselection priority information provided by the cellReselectionPriorities or freqPriorityListNRSlicing;

2>	if the t320 is included:

3>	start timer T320, with the timer value set according to the value of t320;

1>	else:

2>	apply the cell reselection priority information broadcast in the system information;

1>	if deprioritisationReq is included and the UE supports RRC connection release with deprioritisation:

2>	start or restart timer T325 with the timer value set to the deprioritisationTimer signalled;

2>	store the deprioritisationReq until T325 expiry;

NOTE 1a:	The UE stores the deprioritisation request irrespective of any cell reselection absolute priority assignments (by dedicated or common signalling) and regardless of RRC connections in NR or other RATs unless specified otherwise.

1>	if the RRCRelease includes the measIdleConfig:

2>	if T331 is running:

3> stop timer T331;

3>	perform the actions as specified in 5.7.8.3;

2>	if the measIdleConfig is set to setup:

3>	store the received measIdleDuration in VarMeasIdleConfig;

3>	start timer T331 with the value set to measIdleDuration;

3>	if the measIdleConfig contains measIdleCarrierListNR:

4>	store the received measIdleCarrierListNR in VarMeasIdleConfig;

3>	if the measIdleConfig contains measIdleCarrierListEUTRA:

4>	store the received measIdleCarrierListEUTRA in VarMeasIdleConfig;

3>	if the measIdleConfig contains validityAreaList:

4>	store the received validityAreaList in VarMeasIdleConfig;

1>	if the RRCRelease includes suspendConfig:

2>	reset MAC and release the default MAC Cell Group configuration, if any;

2>	apply the received suspendConfig except the received nextHopChainingCount;

2>	if the sdt-Config is configured:

3>	for each of the DRB in the sdt-DRB-List:

4>	consider the DRB to be configured for SDT;

3>	if sdt-SRB2-Indication is configured:

4>	consider the SRB2 to be configured for SDT;

3>	for each RLC bearer associated with a RB that is not suspended:

4>	re-establish the RLC entity as specified in TS 38.322 [4];

3>	for SRB2, if it is resumed and for SRB1:

4>	trigger the PDCP entity to perform SDU discard as specified in TS 38.323 [5];

3>	if configured grant resources for SDT are configured:

4>	configure the MAC entity with the configured grant resources for SDT and instruct MAC to start the cg-SDT-TimeAlignmentTimer;

2>	remove all the entries within VarConditionalReconfig, if any;

2>	for each measId, if the associated reportConfig has a reportType set to condTriggerConfig:

3>	for the associated reportConfigId:

4>	remove the entry with the matching reportConfigId from the reportConfigList within the VarMeasConfig;

3>	if the associated measObjectId is only associated to a reportConfig with reportType set to condTriggerConfig:

4>	remove the entry with the matching measObjectId from the measObjectList within the VarMeasConfig;

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	re-establish RLC entities for SRB1;

2>	if the RRCRelease message with suspendConfig was received in response to an RRCResumeRequest or an RRCResumeRequest1:

3>	stop the timer T319 if running;

3>	stop timer T319a if running;

3>	in the stored UE Inactive AS context:

4>	replace the KgNB and KRRCint keys with the current KgNB and KRRCint keys;

4>	replace the nextHopChainingCount with the value of nextHopChainingCount received in the RRCRelease message;

4>	replace the cellIdentity with the cellIdentity of the cell the UE has received the RRCRelease message;

4>	if the suspendConfig contains the sl-ServingCellInfo (i.e. the UE is a L2 U2N Remote UE):

5>	replace the physical cell identity with the value of the sl-PhysCellId;

5>	replace the C-RNTI with the value of the sl-UEIdentityRemote;

4> else:

5>	replace the C-RNTI with the C-RNTI used in the cell (see TS 38.321 [3]) the UE has received the RRCRelease message;

5>	replace the physical cell identity with the physical cell identity of the cell the UE has received the RRCRelease message;

3>	replace the nextHopChainingCount with the value associated with the current KgNB;

2>	else:

3>	store in the UE Inactive AS Context the nextHopChainingCount received in the RRCRelease message, the current KgNB and KRRCint keys, the ROHC state, the stored QoS flow to DRB mapping rules, the application layer measurement configuration, the C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, the spCellConfigCommon within ReconfigurationWithSync of the NR PSCell (if configured) and all other parameters configured except for:

-	parameters within ReconfigurationWithSync of the PCell;

-	parameters within ReconfigurationWithSync of the NR PSCell, if configured;

-	parameters within MobilityControlInfoSCG of the E-UTRA PSCell, if configured;

-	servingCellConfigCommonSIB;

-	sl-L2RelayUEConfig, if configured;

-	sl-L2RemoteUEConfig, if configured;

3>	store any previously or subsequently received application layer measurement reports for which no segment, or full message, has been submitted to lower layers for transmission;

NOTE 2:	NR sidelink communication related configurations and logged measurement configuration are not stored as UE Inactive AS Context, when UE enters RRC_INACTIVE.

2>	suspend all SRB(s) and DRB(s) and multicast MRB(s), except SRB0;

2>	indicate PDCP suspend to lower layers of all DRBs and multicast MRBs;

2>	if the t380 is included:

3>	start timer T380, with the timer value set to t380;

2>	if the RRCRelease message is including the waitTime:

3>	start timer T302 with the value set to the waitTime;

3>	inform upper layers that access barring is applicable for all access categories except categories '0' and '2';

2>	if T390 is running:

3>	stop timer T390 for all access categories;

3>	perform the actions as specified in 5.3.14.4;

2>	indicate the suspension of the RRC connection to upper layers;

2>	enter RRC_INACTIVE and perform cell selection as specified in TS 38.304 [20];

1>	else

2>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with the release cause 'other'.

End of the modified section