3GPP TSG-RAN2 Meeting #118-e	R2-2205878

# Online, 9th May 2022 - 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.304 | CR | 0848 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | RILZ312, Z313, Z316 CEL-based paging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh4_LTE_eMTC6-Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For coverage-based paging feature for R17 NB-IoT, network provides coverage-based paging configuration in broadcast signalling and also provides coverage related information to UE via dedicated signaling. Accordingly, UE in idle mode would perform CEL-based paging carrier selection. In [R2-2205162], the name of some parameters in CEL-based paging configuration are changed. Accordingly, the parameters used in paging carrier selection procedure also need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 7.7, to modify the names of some parameters in CEL-based paging carrier selection procedure so that they are consistent with the parameter names in TS 36.331. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The parameters used in CEL-based paging carrier selection procedure in TS 36.304 are not consistent with the parameter in TS 36.331. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 36.331 CR4789 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

7.7	Coverage based paging carrier selection

Coverage-based paging carrier selection is only used in the cell in which the UE most recently entered RRC-IDLE triggered by:

-	reception of RRCEarlyDataComplete or RRCConnectionRelease;

-	and the message includes cbp-Index.

Coverage-based carrier selection is enabled when at least one DL carrier in dl-CarrierConfigList is configured with cbp-Index.

When coverage-based carrier selection is used, the UE shall:

-	if cbp-HystTimer is not running:

-	if Srxlev > nrsrpMin in cbp-ConfigList indexed by value ofcbp-Index:

-	use the list of DL carriers in dl-CarrierConfigList configured with pcch-Config-r17 and with cbp-Index equal to the value of cbp-Index received from RRCEarlyDataComplete or RRCConnectionRelease.

-	else:

-	use the list of DL carriers in dl-CarrierConfigList configured with pcch-Config-r14 for carrier selection as described in clause 7.1.

-	else:

-	continue using the same DL carriers as previously selected as described in clause 7.1.

-	when UE switches between paging carriers configured with pcch-Config-r14 and paging carriers configured with pcch-Config-r17 for carrier selection:

-	start cbp-HystTimer;