3GPP TSG-RAN WG2 Meeting #118-e	R2-2205885

# Electronic, 9th May – 20th May 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3123 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | On DAPS handover failure handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2022-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The DAPS handover related failure enhancement, i.e., RLF report, was not included in Rel-16. However, when the UE is configured with DAPS handover, then the following actions are performed by the UE. - Upon declaring T304 expiry while performing DAPS HO, if the UE has not declared RLF in the source cell, then the UE does not store anything in the RLF report and performs the fallback. - Upon declaring T304 expiry while performing DAPS HO, if the UE has already declared RLF in the source cell, then the UE stores handover failure related information in the RLF report and performs the reestablishment. 2> if any DAPS bearer is configured, and radio link failure is not detected in the source PCell, according to subclause 5.3.10.3: 3> reset MAC for the target PCell and release the MAC configuration for the target PCell; 3> for each DAPS bearer: 4> release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the target PCell; 4> reconfigure the PDCP entity to release DAPS as specified in TS 38.323 [5]; 3> for each SRB: 4> if the masterKeyUpdate was not received: 5> configure the PDCP entity for the source PCell with state variables continuation as specified in TS 38.323 [5]; 4> release the PDCP entity for the target PCell; 4> release the RLC entity as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the target PCell; 4> trigger the PDCP entity for the source PCell to perform SDU discard as specified in TS 38.323 [5]; 4> re-establish the RLC entity for the source PCell; 3> release the physical channel configuration for the target PCell; 3> discard the keys used in target PCell (the KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key), if any; 3> resume suspended SRBs in the source PCell; 3> for each non-DAPS bearer: 4> revert back to the UE configuration used for the DRB in the source PCell, includes PDCP, RLC states variables, the security configuration and the data stored in transmission and reception buffers in PDCP and RLC entities ; 3> revert back to the UE measurement configuration used in the source PCell; 3> initiate the failure information procedure as specified in subclause 5.7.5 to report DAPS handover failure. 2> else: 3> revert back to the UE configuration used in the source PCell; 3> store the handover failure information in VarRLF-Report as described in the subclause 5.3.10.5; 3> initiate the connection re-establishment procedure as specified in subclause 5.3.7. The behaviour as specified in 1) is the correct UE behaviour as per the agreements. However, the UE behaviour in 2) is incorrect as the UE stores HOF related information for a DAPS handover in the RLF report. This has never been agreed in Rel-16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Procedural text is modified to remove the updation of RLF report creation upon declaring DAPS HOF and source RLF. Impact analysis Impacted functionality: RLF report Inter-operability: No inter-operability issues |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE would end up including DAPS HOF related RLF report in Rel-16 which could mess up network HO parameters configurations as the network thinks that this RLF reprot is associated to a normal HO and might end up tuning normal HO related parameters. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5.8.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/*start of changes*/

##### 5.3.5.8.3	T304 expiry (Reconfiguration with sync Failure)

The UE shall:

1>	if T304 of the MCG expires:

2>	release dedicated preambles provided in rach-ConfigDedicated if configured;

2>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated if configured;

2>	if any DAPS bearer is configured, and radio link failure is not detected in the source PCell, according to subclause 5.3.10.3:

3>	reset MAC for the target PCell and release the MAC configuration for the target PCell;

3>	for each DAPS bearer:

4>	release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the target PCell;

4>	reconfigure the PDCP entity to release DAPS as specified in TS 38.323 [5];

3>	for each SRB:

4>	if the masterKeyUpdate was not received:

5>	configure the PDCP entity for the source PCell with state variables continuation as specified in TS 38.323 [5];

4>	release the PDCP entity for the target PCell;

4>	release the RLC entity as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the target PCell;

4>	trigger the PDCP entity for the source PCell to perform SDU discard as specified in TS 38.323 [5];

4>	re-establish the RLC entity for the source PCell;

3>	release the physical channel configuration for the target PCell;

3>	discard the keys used in target PCell (the KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key), if any;

3>	resume suspended SRBs in the source PCell;

3>	for each non-DAPS bearer:

4>	revert back to the UE configuration used for the DRB in the source PCell, includes PDCP, RLC states variables, the security configuration and the data stored in transmission and reception buffers in PDCP and RLC entities ;

3>	revert back to the UE measurement configuration used in the source PCell;

3>	initiate the failure information procedure as specified in subclause 5.7.5 to report DAPS handover failure.

2>	else:

3>	if DAPS bearer is not configured:

4>	store the handover failure information in VarRLF-Report as described in the subclause 5.3.10.5;

3>	revert back to the UE configuration used in the source PCell;

3>	initiate the connection re-establishment procedure as specified in subclause 5.3.7.

NOTE 1:	In the context above, "the UE configuration" includes state variables and parameters of each radio bearer.

1>	else if T304 of a secondary cell group expires:

2>	if MCG transmission is not suspended:

3>	release dedicated preambles provided in rach-ConfigDedicated, if configured;

3>	initiate the SCG failure information procedure as specified in subclause 5.7.3 to report SCG reconfiguration with sync failure, upon which the RRC reconfiguration procedure ends;

2>	else:

3>	if the UE is in NR-DC:

4>	initiate the connection re-establishment procedure as specified in subclause 5.3.7;

3>	else (the UE is in (NG) EN-DC):

4>	initiate the connection re-establishment procedure as specified in TS 36.331 [10], subclause 5.3.7;

1>	else if T304 expires when RRCReconfiguration is received via other RAT (HO to NR failure):

2>	reset MAC;

2>	perform the actions defined for this failure case as defined in the specifications applicable for the other RAT.

NOTE 2:	In this clause, the term 'handover failure' has been used to refer to 'reconfiguration with sync failure'.

/*end of changes*/