3GPP RAN WG2 Meeting #118-e	R2-2205958

eMeeting May 9th – May 20th, 2022

Agenda Item:	6.10.3.2.1

Source:	InterDigital

Title:	Configuration of Timing Advance reporting in TS 38.331

Document for:	Discussion, Decision

# Introduction

In RAN2#117e it was agreed that if a TA report is triggered and there are no available UL-SCH resources, the network may optionally configure UE to trigger an SR. In MAC specification this is configured by RRC via timingAdvanceSR, however the RRC CR currently does not support this parameter.

This document discusses inclusion of timingAdvanceSR in the RRC specification, and alignement of Timing Advance reporting configuration with legacy procedures.

# Configuration of Timing Advance reporting in TS 38.331

The Timing Advance reporting procedure is introduced in Rel-17 to support timing synchronization in non-terrestrial networks. RRC controls Timing Advance reporting by configuring the following parameters: ta-Report, offsetThresholdTA, and timingAdvanceSR. timingAdvanceSR is currently not captured in TS 38.331, and offsetThresholdTA is captured under IE MAC-CellGroupConfig.

To better align with configuration of legacy reporting procedures such as Buffer Status reporting and Power Headroom reporting, it is suggested that a new IE tar-Config be introduced and included within MAC-CellGroupConfig. tar-Config would contain parameters used for configuration of the Timing Advance reporting, including offsetThresholdTA, and timingAdvanceSR.

Proposal 1:	Introduce a new IE tar-Config within MAC-CellGroupConfig to contain parameters used for configuration of Timing Advance Reporting.

Proposal 2:	offsetThresholdTA and timingAdvanceSR are included within tar-Config.

An exemplary text proposal for TS 38.331 capturing the above changes is provided in the Annex. It is suggested that this text proposal be agreed and included within Rapporteur’s CR.

Proposal 3:	Agree to the Text Proposal included within the Annex for TS 38.331.

# Conclusion

In this contribution the following proposals were made concerning configuration of the Timing Advance reporting procedure in RRC:

Proposal 1:	Introduce a new IE tar-Config within MAC-CellGroupConfig to contain parameters used for configuration of Timing Advance Reporting.

Proposal 2:	offsetThresholdTA and timingAdvanceSR are included within tar-Config.

Proposal 3:	Agree to the Text Proposal included within the Annex for TS 38.331.

# References

3GPP TS 38.331: Radio Resource Control (RRC) protocol specification NR v17.0.0

3GPP TS 38.321: Medium Access Control (MAC) protocol specification NR v17.0.0

# Annex: Text proposal for TS 38.331

<Beginning of Text proposal>

#### –	MAC-CellGroupConfig

The IE MAC-CellGroupConfig is used to configure MAC parameters for a cell group, including DRX.

MAC-CellGroupConfig information element

-- ASN1START

-- TAG-MAC-CELLGROUPCONFIG-START

MAC-CellGroupConfig ::=             SEQUENCE {

drx-Config                          SetupRelease { DRX-Config }                                     OPTIONAL,   -- Need M

schedulingRequestConfig             SchedulingRequestConfig                                         OPTIONAL,   -- Need M

bsr-Config                          BSR-Config                                                      OPTIONAL,   -- Need M

tag-Config                          TAG-Config                                                      OPTIONAL,   -- Need M

phr-Config                          SetupRelease { PHR-Config }                                     OPTIONAL,   -- Need M

skipUplinkTxDynamic                 BOOLEAN,

...,

[[

csi-Mask                            BOOLEAN                                                         OPTIONAL,   -- Need M

dataInactivityTimer                 SetupRelease { DataInactivityTimer }                            OPTIONAL    -- Cond MCG-Only

]],

[[

usePreBSR-r16                       ENUMERATED {true}                                               OPTIONAL,   -- Need R

schedulingRequestID-LBT-SCell-r16   SchedulingRequestId                                             OPTIONAL,   -- Need R

lch-BasedPrioritization-r16         ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

schedulingRequestID-BFR-SCell-r16   SchedulingRequestId                                             OPTIONAL,   -- Need R

drx-ConfigSecondaryGroup-r16        SetupRelease { DRX-ConfigSecondaryGroup }                       OPTIONAL    -- Need M

]],

[[

enhancedSkipUplinkTxDynamic-r16     ENUMERATED {true}                                               OPTIONAL,   -- Need R

enhancedSkipUplinkTxConfigured-r16  ENUMERATED {true}                                               OPTIONAL    -- Need R

]],

[[

intraCG-Prioritization-r17          ENUMERATED {enabled}                        OPTIONAL,    -- Cond LCH-PrioWithReTxTimer

drx-ConfigSL-r17                    SetupRelease { DRX-ConfigSL }               OPTIONAL,    -- Cond Mode1AndDRX-Only

drx-ConfigExt-v1700                 SetupRelease { DRX-ConfigExt-v1700 }        OPTIONAL,    -- Cond DRX

schedulingRequestID-BFR-r17         SchedulingRequestId                         OPTIONAL,    -- Need R

schedulingRequestID-BFR2-r17        SchedulingRequestId                         OPTIONAL,    -- Need R

schedulingRequestConfig-v1700       SchedulingRequestConfig-v1700               OPTIONAL,    -- Need M

--Editor's note: extension done according to A4.3.5

tar-Config-r17                      TAR-Config                                  OPTIONAL,    -- Need M

g-RNTI-ConfigToAddModList-r17       SEQUENCE (SIZE (1..maxG-RNTI-r17)) OF Group-Config-r17           OPTIONAL,    -- Need N

g-RNTI-ConfigToReleaseList-r17      SEQUENCE (SIZE (1..maxG-RNTI-r17)) OF G-RNTI-ConfigId-r17        OPTIONAL,    -- Need N

g-CS-RNTI-ConfigToAddModList-r17    SEQUENCE (SIZE (1..maxG-CS-RNTI-r17)) OF Group-Config-r17        OPTIONAL,    -- Need N

g-CS-RNTI-ConfigToReleaseList-r17   SEQUENCE (SIZE (1..maxG-CS-RNTI-r17)) OF G-CS-RNTI-ConfigId-r17  OPTIONAL,    -- Need N

allowCSI-SRS-Tx-MulticastDRX-Active-r17   BOOLEAN

]]

}

DataInactivityTimer ::=         ENUMERATED {s1, s2, s3, s5, s7, s10, s15, s20, s40, s50, s60, s80, s100, s120, s150, s180}

Group-Config-r17 ::=                   SEQUENCE {

groupCommon-RNTI                       CHOICE {

g-RNTI                                 SEQUENCE {

g-RNTI-ConfigId-r17                    G-RNTI-ConfigId-r17,

g-RNTI-r17                             RNTI-Value

},

g-CS-RNTI                              SEQUENCE {

g-CS-RNTI-ConfigId-r17                 G-CS-RNTI-ConfigId-r17,

g-CS-RNTI-r17                          RNTI-Value

}

},

drx-ConfigPTM-r17                      SetupRelease { DRX-ConfigPTM-r17 }                          OPTIONAL,   -- Need M

harq-FeedbackEnablerMulticast-r17      ENUMERATED {dci-enabler, enabled}                           OPTIONAL,   -- Need S

harq-FeedbackOptionMulticast-r17       ENUMERATED {ack-nack, nack-only}                            OPTIONAL,   -- Cond HARQFeedback

pdsch-AggregationFactorMulticast-r17   ENUMERATED {n2, n4, n8}                                     OPTIONAL    -- Cond G-RNTI

}

G-RNTI-ConfigId-r17 ::= INTEGER (0..maxG-RNTI-1-r17)

G-CS-RNTI-ConfigId-r17 ::= INTEGER (0..maxG-CS-RNTI-1-r17)

-- TAG-MAC-CELLGROUPCONFIG-STOP

-- ASN1STOP

MAC-CellGroupConfig field descriptions

allowCSI-SRS-Tx-MulticastDRX-Active Used to control the CSI/SRS transmission during MBS multicast DRX active time, see TS 38.321 [3].

csi-Mask If set to true, the UE limits CSI reports to the on-duration period of the DRX cycle, see TS 38.321 [3].

dataInactivityTimer Releases the RRC connection upon data inactivity as specified in clause 5.3.8.5 and in TS 38.321 [3]. Value s1 corresponds to 1 second, value s2 corresponds to 2 seconds, and so on.

drx-Config, drx-ConfigExt Used to configure DRX as specified in TS 38.321 [3].

drx-ConfigSecondaryGroup Used to configure DRX related parameters for the second DRX group as specified in TS 38.321 [3]. The network does not configure secondary DRX group with DCP simultaneously nor secondary DRX group with a dormant BWP simultaneously.

drx-ConfigSL Used to configure additional DRX parameters for the UE performing sidelink operation with resource allocation mode 1, as specified in TS 38.321 [3].

g-RNTI-ToAddModList List of G-RNTI configurations to add or modify.

g-RNTI-ToReleaseList List of G-RNTI configurations to release.

g-CS-RNTI-ToAddModList List of G-CS-RNTI configurations to add or modify.

g-RNTI-ToReleaseList List of G-CS-RNTI configurations to release.

intraCG-Prioritization Used to enable HARQ process ID selection based on LCH-priority for one CG as sepcfied in TS 38.321 [3].

lch-BasedPrioritization If this field is present, the corresponding MAC entity of the UE is configured with prioritization between overlapping grants and between scheduling request and overlapping grants based on LCH priority, see TS 38.321 [3]. The network does not configure lch-BasedPrioritization with enhancedSkipUplinkTxDynamic simultaneously nor lch-BasedPrioritization with enhancedSkipUplinkTxConfigured simultaneously.

schedulingRequestID-BFR-SCell Indicates the scheduling request configuration applicable for BFR on SCell, as specified in TS 38.321 [3].

schedulingRequestID-BFR-r17 Indicates the scheduling request configuration (SchedulingRequestConfig) that the UE shall use upon detecting a beam failure on the detection resources configured in BFDset of a serving cell but not on resources configured in BFDset2 of the same serving cell. Editor's note: BFDset and BFDset2 configuration is pending on LS response from RAN1.

schedulingRequestID-BFR2-r17 Indicates the scheduling request configuration (SchedulingRequestConfig) that the UE shall use upon detecting a beam failure on the detection resources configured in BFDset2 of a serving cell but not on resources configured in BFDset of the same serving cell. Editor's note: BFDset and BFDset2 configuration is pending on LS response from RAN1.

schedulingRequestID-LBT-SCell Indicates the scheduling request configuration applicable for consistent uplink LBT recovery on SCell, as specified in TS 38.321 [3].

skipUplinkTxDynamic, enhancedSkipUplinkTxDynamic, enhancedSkipUplinkTxConfigured If set to true, the UE skips UL transmissions as described in TS 38.321 [3]. If the UE is configured with enhancedSkipUplinkTxDynamic or enhancedSkipUplinkTxConfigured with value true, REPETITION_NUMBER (as specified in TS 38.321 [3], clause 5.4.2.1) of the corresponding PUSCH transmission of the uplink grant shall be equal to 1.

tag-Config The field is used to configure parameters for a time-alignment group. The field is not present if any DAPS bearer is configured.

tar-Config Used to configure Timing Advance Reporting in non-terrestrial networks as specified in TS 38.321 [3].

usePreBSR If set to true, the MAC entity of the IAB-MT may use the Pre-emptive BSR, see TS 38.321 [3].

<Unchanged text ommited>

#### –	TAR-Config

The IE TAR-Config is used to configure Timing Advance reporting in non-terrestrial networks.

TAR-Config information element

-- ASN1START

-- TAG-TAR-CONFIG-START

TAR-Config ::=                      SEQUENCE {

offsetThresholdTA-r17               ENUMERATED {ms0dot5, ms1, ms2, ms3, ms4, ms5, ms6 ,ms7, ms8, ms9, ms10, ms11, ms12,

ms13, ms14, ms15}                OPTIONAL,    -- Need R

timingAdvanceSR-r17         		ENUMERATED {enabled}                        OPTIONAL,    -- Need R

...

}

-- TAG-TAR-CONFIG-STOP

-- ASN1STOP

TAR-Config field descriptions

offsetThresholdTA-r17 Offset for TA reporting as specified in TS 38.321 [3].

timingAdvanceSR-r17 Used to configure whether a Timing Advance report may trigger a Scheduling Request as specified in TS 38.321 [3].

<End of text proposal>