3GPP TSG-RAN WG2 Meeting #118-e	R2-2205999

Elbonia, 9th – 20th May, 2022

Agenda item:	6.10.2.2

Source: 	Sequans Communications

Title: 	Correction to NR NTN epoch time definition

Document for:	Discussion and Decision

# Introduction

The epoch time definition is currently incomplete, which may lead to wrong UE implementation.

# Discussion

From 38.331, epoch time is defined as follows:

NTN-Config field descriptions

epochTime Indicate the epoch time for assistance information (i.e. Serving satellite ephemeris in IE ephemerisInfo and Common TA parameters). When explicitly provided through SIB, or through dedicated signaling, EpochTime is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information.The reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point. If this field is absent, the epoch time is the end of SI window where this SIB19 is scheduled. This field is mandatory present when provided in dedicated configuration.

The highlighted excerpt refers to "uplink time synchronization reference point", however it is currently not defined in NR specifications.

Observation 1: Uplink time synchronization reference point definition is missing

In our understanding, this sentence means that the timing for epoch time is defined at the "uplink time synchronization reference point". That is to say, the epoch time is "the starting time of a DL sub-frame […]" when that DL sub-frame is transmitted from the "uplink time synchronization reference point". At the UE, the corresponding DL sub-frame is received with a delay, equal to 1/2 Common TA(at tRP=epoch time) + service link delay (at tSAT= 1/2  Common TA).

This impacts the calculations of timing (common TA, service link delay). Hence it is essential to have an unambiguous understanding of the "uplink time synchronization reference point".

This also impacts the validity duration. I.e., "instruct the lower layers to start or restart ntnUlSyncValidityDuration from the subframe indicated by epochTime;" from 38.331 should consider the delay already elapsed upon receiving that subframe at the UE from the RP.

We propose to update Stage 2 similarly to IoT NTN.

Proposal 1: Update Stage 2 as per TP in Annex

# Conclusion

In this contribution, we make the following observations and proposals:

Observation 1: Uplink time synchronization reference point definition is missing

Proposal 1: Update Stage 2 as per TP in Annex

# Annex – 38.300 TP

### 16.14.2	User Plane aspects

The UE may be configured to report the UE's Timing Advance:

-	during Random Access procedure in Idle/Inactive state;

-	in connected mode:

-	using event-triggered reporting;

-	for RRC re-establishment procedure, if an indication is broadcasted by the target cell's SI;

-	for handover, the UE should trigger TA report if the target cell indicates this in the handover command.

To accommodate the long propagation delay, Use Plane procedures are adapted as follow:

-	For downlink, HARQ feedback can be enabled or disabled per HARQ process;

-	For uplink, the UE can be configured with a HARQ mode A or B per HARQ process;

-	Maximum number of HARQ processes is extended to 32;

-	The value ranges of MAC (i.e. sr-ProhibitTimer and configuredGrantTimer), RLC (i.e. t-Reassembly) and PDCP (i.e. discardTimer and t-reordering) layer timers are extended.

NOTE:	It is up to network implementation to ensure proper configuration of HARQ feedback (i.e. enabled or disabled) for HARQ processes used by an SPS configuration and of HARQ mode for HARQ processes used by a CG configuration.

If a logical channel is configured with allowedHARQ-mode, it can only be mapped to a HARQ process with the same HARQ mode.

Impact on timing aspects:

To accommodate the long propagation delays, several NR timings involving DL-UL timing interaction are enhanced by the support of two scheduling offsets: Koffset and kmac as illustrated in Figure 16.14.2-x:

-	Koffset is the RTT between the UE and the uplink time synchronization reference point (RP). It corresponds to the sum of the service link RTT and the common TA if indicated.

-	Kmac is the RTT between the RP and the gNB.

DL and UL are frame aligned at the uplink time synchronization reference point with an offset given by NTA,offset.

Figure 16.14.2-x Timing relationship parameters

The timing relationships that need to be modified for NTN using Koffset are summarized as follows:

-	The transmission timing of DCI scheduled PUSCH, including channel state information (CSI) transmission on PUSCH;

-	The transmission timing of random access response (RAR) grant or fallbackRAR grant scheduled PUSCH;

-	The timing of the first PUSCH transmission opportunity in type-2 configured grant;

-	The transmission timing of HARQ-ACK on physical uplink control channel (PUCCH), including HARQ-ACK on PUCCH to message B (MsgB) in 2-step random access;

-	The transmission timing of PDCCH ordered physical random access channel (PRACH);

-	The timing of the adjustment of uplink transmission timing upon reception of a corresponding timing advance command;

-	The transmission timing of aperiodic sounding reference signal (SRS);

-	The CSI reference resource timing.

Figure 16.14.2-1 is an illustration of the transmission timing of DCI scheduled PUSCH, the slot allocated for the PUSCH can be modified to be n+K2+Koffset. Note for this example the subcarrier spacing (SCS) value of the downlink is supposed to be the same as that of the uplink.

For initial access, the information of Koffset is carried in system information. Update of Koffset after initial access is supported. The UE-specific Koffset can be provided and updated by the network with MAC CE.

Figure 16.14.2-1: Timing relationship between UL and DL for PUSCH transmission

kmac is a scheduling offset supported in NTN for MAC CE timing relationships enhancement. It is provided by the network if downlink and uplink frame timing are not aligned at gNB. It is needed for UE action and assumption on downlink configuration indicated by a MAC-CE command in PDSCH. The Kmac is also used in the beam failure recovery, where after a PRACH transmission in uplink slot n the UE monitors the corresponding PDCCH starting from downlink slot "n + kmac + 4" within a corresponding RAR window.

If a UE is provided with a kmac value, when the UE would transmit a PUCCH with HARQ-ACK information in uplink slot n corresponding to a PDSCH carrying a MAC CE command on a downlink configuration, the UE action and assumption on the downlink configuration shall be applied starting from the first slot that is after slot n+3Nslotsubframe,µ+kmac, where µ is the SCS configuration for the PUCCH. MAC CE timing relationship enhancement with kmac is illustrated in Figure 16.14.2-2.

Figure 16.14.2-2: MAC CE timing relationship enhancement with Kmac

Timing pre-compensation at the UE:

To accommodate the long propagation delays experienced in NTN on both service link and feeder link, the UE should be able to perform time pre-compensation for all its uplink transmissions; including PRACH preamble transmissions and uplink transmissions during the RRC_CONNECTED state. To do such pre-compensation, the UE is assisted by its GNSS and by the network which periodically broadcasts assistance information including serving satellite ephemeris as well as higher layer Common-TA-related parameters, where the latter may be used to calculate the common RTD e.g. delay on the feeder link.

Release-17 specified the following formula for TA calculation that shall be applied by NTN UEs for PRACH preamble transmission and in RRC_CONNECTED state:

- TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUE×Tc

Where:

-	NTA and NTA,offset were already specified in [TS 38.213] [TS 38.211] as part of the existing TA Control;

-	NTA,adjcommon is network-controlled common TA, and may include any timing offset considered necessary by the network (e.g. feeder link delay). It is derived from the higher-layer parameters TACommon, TACommonDrift, and TACommonDriftVariation if configured, otherwise NTA,adjcommon=0;

-	NTA,adjUE is UE self-estimated TA to pre-compensate for the service link delay. It is computed by the UE based on UE position and serving satellite-ephemeris-related higher-layers parameters if configured, otherwise NTA,adjUE=0;

-	Tc is the NR basic time unit [TS 38.211].

Figure 16.14.2-3: Uplink/Downlink Radio Frame Timing at the UE

Frequency pre-compensation at the UE:

The UE shall be capable of using its acquired GNSS position and serving satellite ephemeris information (when provided by the network) to calculate frequency pre-compensation to counter shift the instantaneous Doppler shift experienced on the service link.

While the pre-compensation of the instantaneous Doppler shift experienced on the service link is to be performed by the UE, the management of Doppler shift experienced over the feeder link as well as any transponder frequency error whether it is introduced in Downlink or Uplink is left to the network implementation without any specification impacts in Release 17.