3GPP TSG-RAN2 Meeting #118-e	R2-2206039

Online, 9 – 20 May 2022

Agenda Item:	7.1.2

Source:			Huawei, HiSilicon

Title:				RIL H111 (New): 16 QAM DL configuration

Document for:	Discussion and decision

# Introduction

The RRC CR for NB-IoT and eMTC rel-17 additional enhancements was approved at RAN2#117-e and included in the RRC Rel-17 specification [1].

In this document, we discuss the configuration of 16 QAM DL for NB-IoT

# Discussion

In the RRC specification, DL 16QAM is configured directly in PhysicalConfigDedicated as below:

PhysicalConfigDedicated-NB-r13 ::= SEQUENCE {

[[ npusch-ConfigDedicated-v1700 NPUSCH-ConfigDedicated-NB-v1700 OPTIONAL,   -- Need ON

npdsch-ConfigDedicated-v1700 SetupRelease {NPDSCH-16QAM-Config-NB-r17} OPTIONAL,   -- Need ON

uplinkPowerControlDedicated-v1700   UplinkPowerControlDedicated-NB-v1700     OPTIONAL -- Cond npusch-16QAM

]]

}

This means that there is no explicit field to refer to the DL 16 QAM specification. On the other hand, TS 36.213 refers explicitly npdsch-16QAM-Config:

section 16.2.2:

If a UE is configured with higher layer parameters npdsch-16QAM-Config and nrs-PowerRatio,

section 16.4.1.5:

To determine the modulation order in the NPDSCH, the UE shall

- if the UE is configured with higher layer parameter npdsch-16QAM-Config and the DCI is mapped onto the UE specific search space and the 4-bit "modulation and coding scheme" field () in the DCI is set to ‘1111’,

Proposal: Introduce an explicit parameter npdsch-16QAM-Config as follows.

PhysicalConfigDedicated-NB-r13 ::= SEQUENCE {

[[ npusch-ConfigDedicated-v1700 NPUSCH-ConfigDedicated-NB-v1700 OPTIONAL,   -- Need ON

npdsch-ConfigDedicated-v1700 NPDSCH-ConfigDedicated-NB-v1700SetupRelease {NPDSCH-16QAM-Config-NB-r17} OPTIONAL,   -- Need ON

uplinkPowerControlDedicated-v1700   UplinkPowerControlDedicated-NB-v1700     OPTIONAL -- Cond npusch-16QAM

]]

}

NPDSCH-Config-NB information element

-- ASN1START

NPDSCH-ConfigCommon-NB-r13 ::=	SEQUENCE {

nrs-Power-r13					INTEGER (-60..50)

}

NPDSCH-ConfigDedicated-NB-r16 ::=	SEQUENCE {

npdsch-MultiTB-Config-r16			NPDSCH-MultiTB-Config-NB-r16	 OPTIONAL	-- Cond twoHARQ

}

NPDSCH-MultiTB-Config-NB-r16 ::=	SEQUENCE {

multiTB-Config-r16					ENUMERATED {interleaved, nonInterleaved},

harq-AckBundling-r16				ENUMERATED {true}		OPTIONAL	-- Cond interleaved

}

NPDSCH-ConfigDedicated-NB-v1700	::= SEQUENCE {

npdsch-16QAM-Config-r17			 SetupRelease {NPDSCH-16QAM-Config-NB-r17}

}

NPDSCH-16QAM-Config-NB-r17 ::=SEQUENCE{

nrs-PowerRatio-r17			ENUMERATED {dB-6, dB-4dot77, dB-3, dB-1dot77, dB0, dB1, dB2, dB3}	OPTIONAL,

nrs-PowerRatioWithCRS-r17	ENUMERATED {dB-6, dB-4dot77, dB-3, dB-1dot77, dB0, dB1, dB2, dB3}	OPTIONAL	-- Cond InBand

}

-- ASN1STOP

# Conclusion

In this document, we have discussed the configuration of 16 QAM DL for NB-IoT and made the following proposal:

Proposal: RIL H112: Introduce an explicit parameter npdsch-16QAM-Config.

# Reference

3GPP TS 36.331 E-UTRAN Radio Resource Control (RRC) (Release 17)

3GPP TS 36.304 User Equipment (UE) procedures in idle mode (Release 17)