3GPP TSG-RAN WG2#118-e	R2-2206166

Online, 9 - 20 May 2022

Source: 	Fujitsu (rapporteur)

Title: 	[AT118-e] [223] [DCCA] BFD corrections for DCCA enhancements (Fujitsu)

Agenda Item:	6.2.2

Document for:	Discussion and decision

1	Introduction

This document is a summary of:

[AT118-e][223][DCCA] BFD corrections for DCCA enhancements (Fujitsu)

Scope: Discuss BFD corrections for R17 DCCA marked for this discussion.

Intended outcome: Discussion report in R2-2206166.

Deadline: Deadline 3

Company contact persons for this discussion are invited to fill one entry in the table below:

| Company | Contact details |
| --- | --- |
| Huawei, HiSilicon | David Lecompte (david.lecompte at huawei.com) |
| Apple | Naveen Palle(naveen.palle@apple.com) |
| Sharp | Takuma Kawano(kawano.takuma@sharp.co.jp) Kyosuke Inoue (kyosuke_inoue@sharp.co.jp) |
| Lenovo | Congchi Zhang (zhangcc16@lenovo.com) |
| vivo | Xiaodong Yang(yangxiaodong5g@vivo.com) |
| LGE | Hanul Lee (hanul.lee@lge.com) |
| Fujitsu | Takako Sanda (sanda.takako@fujitsu.com) Meiyi Jia (jiameiyi@fujitsu.com) |
| Nokia | Jarkko Koskela (jarkko.t.koskela@nokia.com) |
| Ericsson | Stefan Wager (stefan.wager@ericsson.com) |
| Futurewei | Jialin Zou (jialinzou88@yahoo.com) |
| Qualcomm | Punyaslok Purkayastha (punyaslo@qti.qualcomm.com) |
| CATT | Erlin Zeng (erlin.zeng@catt.cn) |

2	Discussion

2.1	Per TRP BFD at SCG deactivation

Support of beam failure detection on each BFD-RS set of deactivated PSCell

At previous RAN2 meetings, some agreements are reached on beam failure detection for the PSCell when SCG is deactivated. [1] raised an issue, i.e. whether these agreements can apply only to BFD at cell level, or also to BFD at TRP level. BFD at TRP level means that beam failure detection on each BFD-RS set of the serving cell. So, regarding this, the possible options are:

- Option 1: beam failure detection on each BFD-RS set of PSCell configured with two BFD-RS sets will not be performed while the SCG is deactivated

- Option 2: beam failure detection on each BFD-RS set of PSCell configured with two BFD-RS sets can be performed while the SCG is deactivated

- Option 3: SCG can only be deactivated with bfd-and-RLM configured to true if the PSCell is configured with a single BFD-RS set

- Option 4: beam failure detection on non-serving BFD-RS set of PSCell configured with two BFD-RS sets is not done while the SCG is deactivated

Q1-1: Which option(s) do companies prefer?

| Company | Preference(s) | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Option 2 or 3 | The UE behaviour for BFD on the PSCell should be the same regardless whether the SCG is activated or not, so option 1 should be excluded |
| Apple | Op3, but Op2, Op4 might be possible | We need to evaluate the operation of SCG deactivation with mTRP configured. Atleast the inter-node communication of mTRP now needs to include the SCG status info for PSCell. In our view we need to evaluate this, in FeMIMO session, it is agreed that RAN2 will view this in case of issues. Probably simpler option is to go with option 3, and leave it to NW to not configure. |
| Sharp | Option 3 | We think no special handling is needed. Option 2 is possible but we prefer simple UE operaation in Rel-17. |
| Lenovo | Option 2 | We dont foresee much problem to support BFD on each BFD-RS when SCG is deactivated. |
| vivo | OP3 is OK for not coupling SCG deactivation with MTRP. OP2 is OK if we support to couple SCG deactivation with MTRP. | Cell BFD RS can not be configured with MTRP BFD RS togeter. We are ok for not coupling SCG deactivation with MTRP. |
| LGE | Option 1 | We don’t see any benefit by using TRP-level beam failure detection in deactivated SCG. Network may configure cell-level BFD-RS(s) to UE for deactivated SCG, and the UE performs beam failure detection per cell-level. |
| Fujitsu (proponent) | Option 1 | Option 3 is acceptive, but one problem with Option 3 is that RLM cannot be performed for a deactivated PSCell configured with 2 BFD-RS sets. |
| Nokia | Option 2 or 3 | Agree with Huawei, we don’t see a reason to restrict such configuration. |
| Ericsson | Op2 | We agree with Huawei that the UE behaviour for BFD on the PSCell should be the same regardless whether SCG is activated or not. Since no reasons have been found why two BFD-RS sets could not be supported when SCG is deactivated, there seems no reason restrict it, as we understand op3 would effectively do? The network always has the option to configure bfd-and-RLM to false if it wants. |
| Futurewei | Option 2 or 3 | We should allow BFD at per TRP basis and make use of it with deactived SCG. |
| Qualcomm | Option 2 | Same view as Ericsson. |
| CATT | Option 1 | We think option 1 is the way to go, considering the extra specifcation effort to support UE monitoring two BFD RS sets for de-activated SCG, and also that we are at the end of the release. Just to clarity our understanding on Option 1: there is no need to define a lot of new UE behaviour, it is just that UE is not required to monitor two BFD RS sets if configured when SCG is deactivated, which means it is up to NW to confiugre only one BFD RS set for the UE if SCG is deactived, or set the flag bfd-and-RLM to false if two sets are configured for SCG. This is simplest way as we do not have to discuss the other enhancements as in the next questions. In the RRC spec, we don’t even touch the procedure text, but just add some restriction in the field description of bfd-and-RLM bfd-and-RLM When the SCG is deactivated, indicates whether the UE performs BFD and RLM. The network configures this field to false if the SCG is deactivated and two BFD RS sets are configured for the PSCell of this SCG. |

Summary:

6 companies prefer Option 3 and we think that additional 4 companies can accept this option; 7 companies prefer Option 2 and 1 company thinks that this option is possible; 3 companies prefer Option 1 while 1 company thinks that Option 4 is possible. So, option 1/4 is excluded first.

In addition, as one company mentioned, the following assumption is agreed during feMIMO session:

RAN2 assumes (for now) that Rel17 DC (Rel-17 DC is mainly SCG deactivation) and feMIMO may be configured at the same time (can revisit if issues are found).

so, from feMIMO point of view, Option 2 is acceptable.

It seems that further discussion on Option 2 and Option 3 is needed.

Proposal 1: Choose one from the following options:

- Option 2: beam failure detection on each BFD-RS set of PSCell configured with two BFD-RS sets can be performed while the SCG is deactivated

- Option 3: SCG can only be deactivated with bfd-and-RLM configured to true if the PSCell is configured with a single BFD-RS set

Required changes to MAC spec for Option 1

At RAN2#116bis-e meeting, the following agreements were reached:

2-2. If BFD is not configured for deactivated SCG, UE stops (if running) beamFailureDetectionTimer associated with PSCell upon SCG deactivation as a part of partial MAC reset. 4. UE resets BFI_COUNTER associated with PSCell if BFD is not configured for deactivated SCG, upon SCG deactivation as a part of partial MAC reset.

According to these agreements, as captured in TS 38.321, the MAC entity stops (if running) all timers, except beamFailureDetectionTimer associated with PSCell and timeAlignmentTimers if beam failure detection is configured for the deactivated SCG; also, the MAC entity resets BFI_COUNTER associated with PSCell if BFD is not configured for SCG with bfd-and-RLM.

In [2], it is proposed to exclude the beamFailureDetectionTimer for each BFD-RS set of the PSCell and BFI_COUNTER for each BFD-RS set of the PSCell, i.e. these timers will be stopped, and these counters will be reset, if RAN2 agreed that beam failure detection is not performed for each BFD-RS set of PSCell when the SCG is deactivated, i.e. Option 1 for Q1-1. The proposed changes are shown below:

The MAC entity shall: 1> stop (if running) all timers, except beamFailureDetectionTimer of PSCell and timeAlignmentTimers if beam failure detection is configured for the deactivated SCG; 1> set the NDIs for all uplink HARQ processes to the value 0; 1> set the NDIs for all HARQ process IDs to the value 0 for monitoring PDCCH in Sidelink resource allocation mode 1; 1> stop, if any, ongoing Random Access procedure; 1> flush Msg3 buffer; 1> flush MSGA buffer; 1> cancel, if any, triggered Scheduling Request procedure; 1> cancel, if any, triggered Buffer Status Reporting procedure; 1> cancel, if any, triggered Power Headroom Reporting procedure; 1> cancel, if any, triggered consistent LBT failure; 1> cancel, if any, triggered BFR; 1> cancel, if any, triggered Recommended bit rate query procedure; 1> cancel, if any, triggered Configured uplink grant confirmation; 1> flush the soft buffers for all DL HARQ processes; 1> for each DL HARQ process, consider the next received transmission for a TB as the very first transmission; 1> release, if any, Temporary C-RNTI; 1> reset BFI_COUNTER of each BFD-RS set for PSCell; 1> reset BFI_COUNTER of PSCell if BFD is not configured for SCG with bfd-and-RLM; 1> reset all LBT_COUNTERs; 1> discard explicitly signalled contention-free Random Access Resources for 4-step RA type and 2-step RA type, if any. Editor note: FFS if add new clause as MAC reset for SCG deactivation or change the existing MAC reset clause for SCG deactivation

Q1-2: Assuming that Option 1 for Q1-1 is agreed, would companies also agree to the proposed changes in 5.12a of TS 38.321 in [2]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Disagree with 1 (see above) | The intention of the changes is unclear. |
| vivo | No | No change is needed even we couple MTRP with SCG deactivation. |
| LGE | Yes, but | We agree with the intention. Upon SCG deactivation, all TRP-level behaviour should be stopped or reset. However, we wonder whether "beamFailureDetectionTimer of PSCell" and "BFI_COUNTER of PSCell" can be interpretated as cell-level beamFailureDetectionTimer/BFI_COUNTER. Further discussion may be needed. |
| Fujitsu (proponent) | Yes | The intention of the changes is to stop the BFD timer and reset the BFI counter for each BFD-RS set of the deactivated PSCell. Assume that BFD is performed on each BFD-RS set of the PSCell and then the SCG is deactivated. In this case, MAC entity should stop the BFD timer and reset the BFI counter. Otherwise, upon SCG activation, the MAC entity will perform BFD based on the value of BFI counter before the PSCell is deactivated. |
| Nokia | No |  |
| Ericsson | No | Same comment as Huawei |
| Futurewei | No |  |
| Qualcomm | No |  |
| CATT | No | Please see our comments to Q1-1. |

Summary:

In addition to proponent, only 1 company supports this change. So, we will not pursue these changes.

Proposal 2: We will not pursue the changes in R2-2204909.

Required changes to RRC spec for Option 1 [RIL F001]

As captured in 5.3.5.18 of TS 38.331, upon initiating the SCG deactivation procedure, if bfd-and-RLM is not configured to true, the UE indicates to lower layers to stop beam failure detection on the PSCell. [1] proposed that the RRC indicates the lower layers to stop the beam failure detection on each BFD-RS set of the PSCell upon SCG deactivation, if Option 1 for Q1-1 is agreed, as shown below.

#### 5.3.5.18 SCG deactivation Upon initiating the procedure, the UE shall: 1> consider the SCG to be deactivated; 1> reset SCG MAC; 1> indicate to lower layers that the SCG is deactivated; 1> indicate to lower layers to stop beam failure detection for each BFD-RS set of the PSCell; Editor's note: FFS whether to make the above statement conditional to the SCG being previously activated. 1> If bfd-and-RLM is not configured to true: 2> stop radio link monitoring on the SCG; 2> indicate to lower layers to stop beam failure detection on the PSCell; 2> stop timer T310 for this cell group, if running; 2> stop timer T312 for this cell group, if running; 2> reset the counters N310 and N311; 1> if the UE was in RRC_CONNECTED and the SCG was activated before receiving the message for which this procedure is initiated: 2> if SRB3 was configured before the reception of the RRCReconfiguration or of the RRCConnectionReconfiguration and SRB3 is not to be released according to any RadioBearerConfig included in the RRCReconfiguration or in the RRCConnectionReconfiguration as specified in TS 36.331[10]: 3> trigger the PDCP entity of SRB3 to perform SDU discard as specified in TS 38.323 [5]; 3> re-establish the RLC entity of SRB3 as specified in TS 38.322 [4].

Q1-3: Assuming that Option 1 for Q1-1 is agreed, would companies also agree to the proposed change to 5.3.5.18 of TS 38.331 in [1]?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | We see no relation between option 1 and this proposal, this should be a separate discussion. |
| vivo | No | Agree with huawei |
| LGE | Yes | Upon SCG deactivation, RRC indicates to lower layer (MAC and PHY) to stop all TRP-level behaviour. |
| Fujitsu (proponent) | Yes | The intention is to stop TRP specific BFD when SCG is deactivated. We agree with Huawei that the change should be a separate discussion. |
| Nokia | No |  |
| Ericsson | No |  |
| Futurewei | No |  |
| Qualcomm | No |  |
| CATT | No | Please see our comments to Q1-1. |

Summary:

In addition to proponent, only 1 company supports this change. So, we will not pursue these changes.

Proposal 3: We will not pursue the changes in R2-2204910.

Further considerations for Option 2

If Option 2 is agreed, i.e. beam failure detection for each BFD-RS set of PSCell configured with two BFD-RS sets can be performed while the SCG is deactivated, the UE behaviour when beam failure is detected at one or both BFD-RS set of the PSCell while the SCG is deactivated should be specified.

In our understanding, in case that beam failure is detected on only one BFD-RS set of the PSCell while the SCG is deactivated, the UE shall stop the beam failure detection on the BFD-RS set and the UE can perform SCG activation without RA procedure. When beam failure is detected on both BFD-RS sets of the PSCell while the SCG is deactivated, the MAC entity will indicate the beam failure to RRC layer so that a Random access procedure is initiated upon SCG activation.

Q1-4: Assuming that Option 2 for Q1-1 is agreed, would companies also agree to the following UE behaviors:

- the UE shall stop the beam failure detection on the BFD-RS set and the UE can perform SCG activation without RA procedure when beam failure is detected on only one BFD-RS set of the PSCell while the SCG is deactivated

- the MAC entity will indicate the beam failure to RRC layer so that a Random access procedure is initiated upon SCG activation when beam failure is detected on both BFD-RS sets of the PSCell while the SCG is deactivated

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | We see no relation with option 2 and these two proposals. |
| Lenovo | Partially yes | We suppose what rapporteur means is RACH-less is allowed if BFD detected in only one BFD-RS while the other one still works. 1) and 2) are a bit conflicting? If UE stop BFD after detecting beam failure on one BFD-RS as in 1) how can UE further detemine the BFD on second BFD-RS as indicated in 2)? |
| vivo | No | Agree with huawei |
| LGE | No | As long as beam failure is detected, regardless of one TRP or two TRP, the MAC entity indicates indicate beam failure of the PSCell to upper layers. |
| Fujitsu | Yes | If TRP specific BFD is performed for a deactivated PSCell configured with 2 BFD-RS sets, beam failure for only one TRP will not trigger a RA upon SCG activation considering that the other TRP works. Regarding comment from Lenovo, for 1), the UE only stops beam failure detection on the TRP with beam failure and the UE will continue the beam failure detection on the other TRP. |
| Nokia | No | We can just trigger our agreed procedures whenever one or both of the BFD-RS sets fail – no need to optimize. Anyway the NW would not know which DL to use if any of the BFD-RS sets have failed. |
| Ericsson | No |  |
| Qualcomm | No |  |
| CATT | No | Please see our comments to Q1-1. |

Summary:

2 companies support the UE behavior. During the discussion, some companies think that beam failure is indicated to the upper layers when beam failure is detected on one or two TRPs.

The intention of rapporteur to ask such question is to collect companies view on potential changes to specifications. It seems that we can directly propose RAN2 to discuss this.

Proposal 4: RAN2 further discusses whether specification change is necessary based on the selected option in Proposal 1.

Also, if Option 2 is agreed, RAN2 needs to discuss whether TRP specific BFD is configured by the same parameter, i.e. bfd-and-RLM or another parameter is introduced to configure TRP specific BFD.

Q1-5: Assuming that Option 2 for Q1-1 is agreed, would companies also agree that TRP specific BFD is configured by the same parameter, i.e. bfd-and-RLM? Please provide the details if company prefers a new parameter to configure whether TRP specific BFD is performed while SCG is deactivated.

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSililcon | Not sure what the question means. | We suggest that UE behaviour for BFD is the same regardless whether the SCG is activated or not. |
| Lenovo | Yes | We assume the question means if bfd-and-RLM is true, and two sets of BFD-RS are configured, that automatically means UE shall perform BFD on each of the BFD-RS |
| LGE | No | Same understanding with Huawei. |
| Fujitsu | No | In our understanding, RLM is at cell level while BFD can be cell level or TRP level. So, we need to introduce another parameter to configure the TRP level BFD. |
| Nokia | Same view with Huawei |  |
| Ericsson | The question is unclear | Our understanding is that if bfd-and-RLM is configured to „true“, then the UE continues RLM/BFD as when SCG was activated, regardless the number of TRP, unless a reconfiguration is performed. |
| Futurewei |  | If we understand the question correctly, we think that TRP specific BFD configurations per feMIMO will not be affected by the bfd-and-RLM setting for SCG activation. |
| Qualcomm |  | The same parameter „bfd-and-RLM“ should be used to indicate whether UE performs BFD with one or two BFD-RS sets. No new parameter is needed. |
| CATT | No | Please see our comments to Q1-1. |

Summary:

It seems that most companies think that the same parameter “bfd-and-RLM” should be used to indicate whether the UE performs RLM and BFD for PSCell with/without 2 BFD-RS sets.

Proposal 5: If Option 2 is selected in Proposal 1, it is proposed to confirm that:

The same parameter “bfd-and-RLM” should be used to indicate whether the UE performs RLM and BFD for PSCell with/without 2 BFD-RS sets.

2.2	BFD stop/resumption

About the agreements reached at RAN2#117-e meeting:

- Agreement

2: Agree UE behaviours for PSCell beam failure while the SCG is deactivated:

- a) at PSCell beam failure, TA timer is not stopped

- b) at PSCell beam failure, stop BFD

- c) resume BFD upon reconfiguration of BFD RS (RadioLinkMonitoringConfig or tci-Info)

Regarding step b) and c), [5] and [7] think that these agreements are not captured by current specifications and RAN2 should capture these agreements in TS 38.331.

Q2-1: Would companies like to capture above step b) and c) in TS 38.331?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | b): if there was Beam Failure, BFI_COUNTER >= beamFailureInstanceMaxCount and according to 38.321 the UE does nothing. Then there is no need to send an indication. c) is already covered by existing procedure text in 38.321: 2> if beamFailureDetectionTimer, beamFailureInstanceMaxCount, or any of the reference signals used for beam failure detection is reconfigured by upper layers associated with this BFD-RS set of the Serving Cell: 3> set BFI_COUNTER of the BFD-RS set to 0. So this will resume UE actions and we don't see the need to specify anything in 38.331 for this. |
| Sharp | Yes (proponent) | In case that bfd-and-RLM is configured to false, RRC indicates to stop BFD to lower layer. Therefore, to allign with this, in case that bfd-and-RLM is configured to true and beam failure is declared, RRC should indicate lower layer to stop BFD. In addition, we think RAN2 should specify MAC behaviour based on the BFD stop indication from upper layer. |
| Lenovo | No strong view | As Huawei commented, it is more or less reflected in MAC spec. We can modiy the wording in MAC to make it clear though, if needed. |
| vivo | NO | Agree with huawei |
| LGE | Yes | For b), in case of beam failure in legacy operation, since BFD does not stop, a procedure to inform the low layer is necessary. For c), it is necessary to more explicitly state that BFD is resumed upon reconfiguring of BFD RS. |
| Fujitsu | See comments | Regarding BFD, there can be diffrent understandings. For example, BFD means that beam failure detection by counting the number of BFIs by the MAC entity. Or, BFD can also include that the quaility of BFD RS is measured/evaluated and thus BFI can be provided to MAC layer. For the latter, we think that we need to capture above step b) and c). |
| Nokia | No | This would ne unnecessary complexity for the modelling whilst the spec already works as is. |
| Ericsson | Yes (proponent) | We prefer to capture this more explicitly, compared to just relying on excisting BFI_COUNTER statements, similarly as is done for RA handling during SCG activation/deactivation. If we don’t capture b) and c) in the specifications, we don’t see why the agreement was captured in the first place? Agree with Sharp that there should be the indication from RRC to MAC to stop BFD once the SCGFailureInformation is sent. There is no need for the UE to continue the BFD after the BFD has been reported to the network, and the UE can save battery by stopping the BFD until reconfiguration of BFD RS. |
| Futurewei | No | Agree with Huawei |
| Qualcomm | Yes | Agree with Ericsson on this. |
| CATT | No | Agree with Huawei. |

Summary:

5 companies think that we will not further capture BFD stop/resumption in TS 38.331 while 4 companies prefer to capture them. 1 company has no strong view on this but thinks that we can modify the wording in MAC if needed. Since there is no sufficient support to capture BFD stop/resumption in TS 38.331, we propose to not capture it in TS38.331.

Proposal 6: RAN2 will not capture BFD stop/resumption in TS 38.331.

Regarding BFD stop, i.e. abovementioned step b), both [5] and [7] propose to capture in 5.7.3.2 of TS 38.331 (i.e., initiation of SCG failure information) that the RRC indicates to lower layers to stop BFD if the SCG failure information procedure is initiated due to beam failure on the PSCell while the SCG is deactivated.

Q2-2: Would companies like to capture above step b) in 5.7.3.2 of TS 38.331 that the RRC indicates to lower layers to stop BFD if the SCG failure information procedure is initiated due to beam failure on the PSCell while the SCG is deactivated?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | See comments above. |
| Sharp | Yes (proponent) | See comments above. |
| vivo | NO |  |
| LGE | Yes | In case of beam failure in legacy operation, since BFD does not stop, a procedure to inform the low layer is necessary. |
| Fujitsu | Conditional yes | See our comments to Q2-1. |
| Nokia | No |  |
| Ericsson | Yes (proponent) | See comments above. |
| Futurewei | No |  |
| Qualcomm | Yes |  |
| CATT | No |  |

Summary:

5 companies think that we will not further capture BFD stop in TS 38.331 that the RRC indicates to lower layers to stop BFD if the SCG failure information procedure is initiated due to beam failure on the PSCell while the SCG is deactivated; while 5 companies prefer to capture them.

This question depends on Q2-1. So, no proposal is made before RAN2 decides on Proposal 6.

Regarding BFD resumption, i.e. abovementioned step c), [5] proposed to capture in 5.3.5.5.7 of TS 38.331 (i.e., SpCell Configuration) that the UE indicates to lower layers to resume beam failure detection on the PSCell if stopped when any of the reference signal(s) that are used for beam failure detection are reconfigured by the received spCellConfigDedicated and the SCG is deactivated, and to capture in 5.17 of TS 38.321 that BFD resumption should include the process of BFI_COUNTER resetting and BFD restarting.

The proposed changes to 5.3.5.5.7 of TS 38.331 in [5] are shown as below:

##### 5.3.5.5.7 SpCell Configuration The UE shall: 1> if the UE is connected with a L2 U2N Relay UE via the PC5-RRC connection (i.e. the UE is a L2 U2N Remote UE): 2> use values for timers T300, T301 and T319 as included in ue-TimersAndConstants-RemoteUE received in SIB1; 2> use value for timers T311, as included in ue-TimersAndConstants received in SIB1; 1> else 2> if the SpCellConfig contains the rlf-TimersAndConstants: 3> configure the RLF timers and constants for this cell group as specified in 5.3.5.5.6; 2> else if rlf-TimersAndConstants is not configured for this cell group: 3> if any DAPS bearer is configured: 4> use values for timers T301, T310, T311 and constants N310, N311 for the target cell group, as included in ue-TimersAndConstants received in SIB1; 3> else 4> use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1; 2> if the SpCellConfig contains spCellConfigDedicated: 3> configure the SpCell in accordance with the spCellConfigDedicated; 3> consider the bandwidth part indicated in firstActiveUplinkBWP-Id if configured to be the active uplink bandwidth part; 3> consider the bandwidth part indicated in firstActiveDownlinkBWP-Id if configured to be the active downlink bandwidth part or the bandwidth part for Radio Link Monitoring, Beam Failure Detection and measurements if the SpCellConfig is included in an RRCReconfiguration message contained in an NR or E-UTRA RRC message indicating that the SCG is deactivated; 3> if any of the reference signal(s) that are used for radio link monitoring are reconfigured by the received spCellConfigDedicated: 4> stop timer T310 for the corresponding SpCell, if running; 4> stop timer T312 for the corresponding SpCell, if running; 4> reset the counters N310 and N311. 3> if any of the reference signal(s) that are used for beam failure detection are reconfigured by the received spCellConfigDedicated, and if the SCG is deactivated: 4> indicate to lower layers to resume beam failure detection on the PSCell if stopped; 1> if the SpCellConfig contains the lowMobilityEvaluationConnected: 2> the UE may perform the evaluation of the low mobility criterion for this cell group as specified in 5.7.13.1; 1> if the SpCellConfig contains the goodServingCellEvaluationRLM: 2> the UE may perform the evaluation of the good serving cell quality criterion for this cell group as specified in 5.7.13.2; 1> if the SpCellConfig contains the goodServingCellEvaluationBFD: 2> the UE may perform the evaluation of the good serving cell quality criterion for this serving cell as specified in 5.7.13.2;

While [7] proposed to capture in 5.3.5.5.7 of TS 38.331 (i.e., SpCell Configuration) that the UE restarts BFD (if configured) if the network reconfigures the reference signals used for beam failure detection on the PSCell after SCG beam failure was detected, as shown below.

##### 5.3.5.5.7 SpCell Configuration The UE shall: 1> if the UE is connected with a L2 U2N Relay UE via the PC5-RRC connection (i.e. the UE is a L2 U2N Remote UE): 2> use values for timers T300, T301 and T319 as included in ue-TimersAndConstants-RemoteUE received in SIB1; 2> use value for timers T311, as included in ue-TimersAndConstants received in SIB1; 1> else 2> if the SpCellConfig contains the rlf-TimersAndConstants: 3> configure the RLF timers and constants for this cell group as specified in 5.3.5.5.6; 2> else if rlf-TimersAndConstants is not configured for this cell group: 3> if any DAPS bearer is configured: 4> use values for timers T301, T310, T311 and constants N310, N311 for the target cell group, as included in ue-TimersAndConstants received in SIB1; 3> else 4> use values for timers T301, T310, T311 and constants N310, N311, as included in ue-TimersAndConstants received in SIB1; 2> if the SpCellConfig contains spCellConfigDedicated: 3> configure the SpCell in accordance with the spCellConfigDedicated; 3> consider the bandwidth part indicated in firstActiveUplinkBWP-Id if configured to be the active uplink bandwidth part; 3> consider the bandwidth part indicated in firstActiveDownlinkBWP-Id if configured to be the active downlink bandwidth part or the bandwidth part for Radio Link Monitoring, Beam Failure Detection and measurements if the SpCellConfig is included in an RRCReconfiguration message contained in an NR or E-UTRA RRC message indicating that the SCG is deactivated; 3> if any of the reference signal(s) that are used for radio link monitoring are reconfigured by the received spCellConfigDedicated: 4> stop timer T310 for the corresponding SpCell, if running; 4> stop timer T312 for the corresponding SpCell, if running; 4> reset the counters N310 and N311. 1> if the SpCellConfig contains the lowMobilityEvaluationConnected: 2> the UE may perform the evaluation of the low mobility criterion for this cell group as specified in 5.7.13.1; 1> if the SpCellConfig contains the goodServingCellEvaluationRLM: 2> the UE may perform the evaluation of the good serving cell quality criterion for this cell group as specified in 5.7.13.2; 1> if the SpCellConfig contains the goodServingCellEvaluationBFD: 2> the UE may perform the evaluation of the good serving cell quality criterion for this serving cell as specified in 5.7.13.2; 1> if the SCG is deactivated and the beam failure detection was previously stopped due to SCG RLF: 2> if the SpCellConfig for the secondaryCellGroup contains spCellConfigDedicated including a radioLinkMonitoringConfig or tci-Info: 3> if the bfd-and-RLM in deactivatedSCG-Config is configured to TRUE: 4> indicate to lower layers that radio link monitoring and beam failure detection is restarted on the PSCell;

So, regarding step c), the following conditions can be considered when the UE determines whether to resume/restart BFD:

- the SCG is deactivated (as in [5] [7])

- the RS for BFD is reconfigured by the received spCellConfigDedicated (as in [5])

- the SpCellConfig for the secondaryCellGroup contains spCellConfigDedicated including a radioLinkMonitoringConfig or tci-Info (as in [7])

- bfd-and-RLM in deactivatedSCG-Config is configured to TRUE (as in [7])

- the BFD was previously stopped due to SCG RLF (as in [7])

Q2-3: Which conditions should be considered for BFD resumption?

| Company | Condition(s) | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | None | a) was discussed above already. b) and c) are already covered by existing procedure text in 38.321: 2> if beamFailureDetectionTimer, beamFailureInstanceMaxCount, or any of the reference signals used for beam failure detection is reconfigured by upper layers associated with this BFD-RS set of the Serving Cell: 3> set BFI_COUNTER of the BFD-RS set to 0. e) is not very useful because TA timer was stopped For d), we think the field description of d) is sufficient. However, we could capture in 38.321 to say what it means that "the UE does not perform BFD". |
| Sharp | a/b/d | 3> if any of the reference signal(s) that are used for beam failure detection are reconfigured by the received spCellConfigDedicated, and if bfd-and-RLM is configured to TRUE, and if the SCG is deactivated: 4> indicate to lower layers to resume beam failure detection on the PSCell if stopped; In our contribution [5], Condition d is lacked. So we added above condition . (Additional comments): Condition c should include the condition which RS for BFD is reconfigured, otherwise UE may resume BFD based on the reconfiguration of RLM-RS. |
| vivo | NO | Agree with huawei |
| LGE | a), c), e) | For d), we wonder if there is a case where only the bfd-and-RLM value is changed while scg deactivation has already been done. In the description, the bfd-and-RLM seems to be set upon scg deactivation. bfd-and-RLM When the SCG is deactivated, indicates whether the UE performs BFD and RLM. |
| Fujitsu | a), b), d), e) | See our comments to Q2-1. |
| Nokia | None |  |
| Ericsson | a/b/c/d/e | We agree with the Sharp formulation above, which is a bit cleaner than ours. With this formulation, we assume „any of the reference signal(s) that are used for beam failure detection are reconfigured by the received spCellConfigDedicated,“ includes also TCI state update using tci-Info field. - was a typo in our TP. Instead of „SCG RLF“ it should read „SCG BFD“. Regarding the Huawei comment on b) and c), we don’t see how BFI_COUNTER reset alone would resume the BFD, if it is stopped at beam failure? |
| Futurewei | None |  |
| Qualcomm | a, b, d, e |  |
| CATT | None | Agree with Huawei. |

Summary:

Same as Q2-2, no proposal is made before RAN2 decides on Proposal 6.

Regarding BFD resumption, i.e. abovementioned step c), [5] also proposed to capture in 5.17 of TS 38.321 (i.e., Beam Failure Detection and Recovery procedure) the following text.

The following UE variables are used for the beam failure detection procedure: - BFI_COUNTER (per Serving Cell or per BFD-RS set of Serving Cell configured with two BFD-RS sets): counter for beam failure instance indication which is initially set to 0. - set BFI_COUNTER to 0 upon resumption of this procedure by BFD RS reconfiguration.

Q2-4: Would companies like to capture in 5.17 of TS 38.321 that BFD resumption should include the process of BFI_COUNTER resetting as in [5], in addition to BFD resumption indication in TS 38.331?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | As mentioned before, this is already covered in 38.321. If anything would be added, it would be to clarify what the UE does when bfd-and-RLM is not configured to true. |
| Sharp | Yes (proponent) | As mentioned above our comments, MAC stops BFD procedure based on the upper layer indication. In the case, BFI_COUNTER is not set to 0 even if RS is changed. We think resetting is needed. |
| LGE | No | According to the agreement, BFD is resumed upon the reconfiguration of BFD-RS, and in the current specification, BFI_COUNTER is reset upon the reconfiguration of BFD-RS. 2> if beamFailureDetectionTimer, beamFailureInstanceMaxCount, or any of the reference signals used for beam failure detection is reconfigured by upper layers associated with this Serving Cell: 3> set BFI_COUNTER to 0. |
| Fujitsu | No | Agree with LGE. |
| Nokia | No |  |
| Ericsson | Yes | We partially agree with Huawei that the case resetting BFI_COUNTER upon reference signal reconfiguration is covered in 38.321, but is it clear that it also includes tci-Info? |
| Futurewei | No |  |
| Qualcomm | No | Agree with LGE. |
| CATT | No |  |

Summary:

2 companies support this change while 7 companies don’t support it. So, we will not pursue these changes.

Proposal 7: We will not pursue the changes in R2-2205280.

2.3	Initiation of RA procedure due to beam failure

2.3.1	when SCG is deactivated

According to current MAC specification,

The MAC entity shall for each Serving Cell configured for beam failure detection: 1> if beam failure instance indication has been received from lower layers: 2> start or restart the beamFailureDetectionTimer; 2> increment BFI_COUNTER by 1; 2> if BFI_COUNTER >= beamFailureInstanceMaxCount: 3> if the Serving Cell is SCell: 4> trigger a BFR for this Serving Cell; 3> else if the Serving Cell is PSCell, the SCG is deactivated and beam failure of the PSCell was not indicated to upper layers since the SCG was deactivated: 4> indicate beam failure of the PSCell to upper layers; 3> else 4> initiate a Random Access procedure (see clause 5.1) on the SpCell.

[3] points out that a Random Access procedure will be initiated on the deactivated PSCell if beam failure on the PSCell is detected after the previous beam failure on this PSCell was indicated to upper layers (i.e. BFD has been resumed due to BFD RS reconfiguration), which contradicts the behaviours of deactivated SCG in Activation/Deactivation of SCG section. So, it is proposed to modify the BFD indication in the current MAC CR in order not to initiate Random Access on PSCell in deactivated SCG.

Q3-1: Do companies agree that the problem raised in [3] should be fixed?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Apple | Yes |  |
| Sharp | Yes (Proponent) |  |
| Lenovo | Yes |  |
| vivo | YES |  |
| LGE | No | UE recieves the first SCG deactivation command. Beam failure is detected on SCG deactivation and SCG failure information is transmitted. Then, the network may reconfigure BFD-RS within the second SCG deactivation command, i.e., SCG is deactivated again. If beam failure is detected again, this failure is a first beam failure detection after the reception of the second deactivation command. Thus, MAC entity indicates beam failure of the PSCell to RRC and RA procedure is not initiated. |
| Fujitsu | See comments | The current text is doable. We have the similar view as LGE. If the NW reconfigure BFD RS while keeping SCG deactivation state, the NW also include scg-state in the RRCReconfiguration. Everytime UE receive RRCReconfiguration including scg-state, MAC entity receives indication that SCG is deactivated. So, BF is indicated to RRC rather than RA when BF is detected during the SCG is deactivated. In our understanding, if we interprit “since the SCG was deactivated“ as “since SCG deactivation indication was received“, there is no issue. |
| Nokia | Yes |  |
| Ericsson | Yes |  |
| Futurewei | Yes with modification | It appears with current CR, even beam failure is detected at the deactivated SCG, random access is still skipped. Consider to add a sentence after indicate beam failure to upper layer: 4> else if the Serving Cell is PSCell and the SCG is deactivated; 5> if beam failure of the PSCell has not been indicated to upper layers since the last time BFI_COUNTER >= beamFailureInstanceMaxCount was fullfiled; 6> indicate beam failure of the PSCell to upper layers; 6> initiate a Random Access procedure (see clause 5.1) on the PSCell upon SCG activation. 4> else 5> initiate a Random Access procedure (see clause 5.1) on the SpCell. |
| Qualcomm | Yes |  |
| CATT | See comments | We have similar view as LGE and Fujitsu. If clarification is needed, “ since latest SCG deactivation indication was received“ could be better. |

Summary:

9 companies agree to fix the problem while 3 companies think that the current text is doable.

Proposal 8: The problem raised in R2-2205273 should be fixed.

[4] provides the suggested changes to fix the problem, as below:

The MAC entity shall for each Serving Cell configured for beam failure detection: 1>... 1> else: 2> if beam failure instance indication has been received from lower layers: 3> start or restart the beamFailureDetectionTimer; 3> increment BFI_COUNTER by 1; 3> if BFI_COUNTER >= beamFailureInstanceMaxCount: 4> if the Serving Cell is SCell: 5> trigger a BFR for this Serving Cell; 4> else if the Serving Cell is PSCell and the SCG is deactivated; 5> if beam failure of the PSCell has not been indicated to upper layers since the last time BFI_COUNTER >= beamFailureInstanceMaxCount was fullfiled; 6> indicate beam failure of the PSCell to upper layers. 4> else 5> initiate a Random Access procedure (see clause 5.1) on the SpCell. …

Q3-2: Do companies think that the CR in [4] is agreeable?

| Company | Yes or No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Apple | Ok |  |
| Sharp | Yes (proponent) |  |
| Lenovo | Yes |  |
| vivo | YES |  |
| LGE | No | See the answer in Q3-1. |
| Fujitsu | See comments | If text change is necessary for clarification, we suggest: 2> if BFI_COUNTER >= beamFailureInstanceMaxCount: 3> if the Serving Cell is SCell: 4> trigger a BFR for this Serving Cell; 3> else if the Serving Cell is PSCell, the SCG is deactivated and beam failure of the PSCell was not indicated to upper layers since the SCG deactivation was indicated: 4> indicate beam failure of the PSCell to upper layers; 3> else 4> initiate a Random Access procedure (see clause 5.1) on the SpCell. |
| Nokia | Intention OK | However the „since the last time BFI_COUNTER >= beamFailureInstanceMaxCount was fullfiled“ is not correct as this is fulfilled anytime the new BFI is indicated by L1. Proper text would be: 5> if beam failure of the PSCell has not been indicated to upper layers since the SCG was deactivated: 6> indicate beam failure of the PSCell to upper layers. |
| Ericsson | Yes |  |
| Futurewei | Yes with modification | See our answer to question Q3-1. |
| Qualcomm | Yes |  |
| CATT | No | See the comment in Q3-1. |

Summary:

7 companies think that the CR is agreeable and additional 2 companies agree with the intention but think that modification is needed. One company don’t think that there is a problem needs to be fixed. Two companies prefer another solution to fix the problem.

Proposal 9: RAN2 discusses how to fix the issue raised in R2-2205273 based on the CR in R2-2205274. This discussion is covered in [AT118-e][222].

2.3.2	upon SCG activation

According to current RRC/MAC specification, [8] understands that the UE will activate SCG with a Random Access procedure in case that BFD RS is changed at the time reception of SCG activation command.

Q3-3: Do companies agree to the above understanding that the UE will activate SCG with a Random Access procedure in case that BFD RS is changed at the time reception of SCG activation command according to current RRC/MAC specification?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| Apple | Yes |  |
| Sharp | Yes |  |
| Lenovo | Maybe no | Not sure why NW will reconfigure a new beam if no BFD on the old beam. If BFD has been detected, then UE will perform RACH upon SCG activation. |
| vivo | YES |  |
| LGE | See comments | We think this is a rare case. If the network wants rach-less activation, the network can control the configuration order to make it rach-less. |
| Nokia | Unclear | Lenovo has a point, we don’t fully understand why NW would reconfigure without any information if such would be required. |
| Ericsson | Yes |  |
| Futurewei | Unclear | We agree with the principle if there is a change of BFD RS change. But not clear the use case in practice. |
| Qualcomm | Yes |  |
| CATT | Yes |  |

Summary:

7 companies agree that the UE will activate SCG with a RA procedure in case that BFD RS is changed at the time reception of SCG activation command according to current RRC/MAC specification. 4 companies have some concerns on this use case.

To fix the problem, [9] proposed to take the case where BFD-RS is changed at the time reception of SCG activation command into account when the UE determines whether to initiate a RA procedure upon SCG activation. I.e., if reference signal for beam failure detection of the SpCell of the SCG was not changed since the last time that beam failure was indicated from lower layer after deactivating the SCG, the UE initiates the Random Access procedure on the PSCell.

In addition, in [9] and [10], it is proposed that whether to initiate a RA procedure due to beam failure of the deactivated PSCell upon SCG activation is determined by RRC rather than MAC entity.

The proposed changes to 5.3.5.3 of TS 38.331 (i.e. Reception of an RRCReconfiguration by the UE) in [9] are shown below:

1> if the UE is configured with E-UTRA nr-SecondaryCellGroupConfig (UE in (NG)EN-DC): 2> if the RRCReconfiguration message was received via E-UTRA SRB1 as specified in TS 36.331 [10]; or 2> if the RRCReconfiguration message was received via E-UTRA RRC message RRCConnectionReconfiguration within MobilityFromNRCommand (handover from NR standalone to (NG)EN-DC); 3> if the RRCReconfiguration is applied due to a conditional reconfiguration execution for CPC which is configured via conditionalReconfiguration contained in nr-SecondaryCellGroupConfig specified in TS 36.331 [10]: 4> submit the RRCReconfigurationComplete message via the E-UTRA MCG embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10], clause 5.6.2a. 3> else if the RRCReconfiguration message was included in E-UTRA RRCConnectionResume message: 4> submit the RRCReconfigurationComplete message via E-UTRA embedded in E-UTRA RRC message RRCConnectionResumeComplete as specified in TS 36.331 [10], clause 5.3.3.4a; 3> else: 4> submit the RRCReconfigurationComplete via E-UTRA embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10], clause 5.3.5.3/5.3.5.4/5.4.2.3; 3> if the scg-State is not included in the E-UTRA RRCConnectionReconfiguration message containing the RRCReconfiguration message: 4> if reconfigurationWithSync was included in spCellConfig of an SCG: 5> initiate the Random Access procedure on the SpCell, as specified in TS 38.321 [3]; 4> if the SCG was deactivated before the reception of the E-UTRA RRC message containing the RRCReconfiguration message: 5> if lower layers consider that a Random Access procedure is needed for SCG activation; or 5> if reference signal for beam failure detection of the SpCell of the SCG was not changed since the last time that beam failure was indicated from lower layer after deactivating the SCG: 6> initiate the Random Access procedure on the SpCell, as specified in TS 38.321 [3]; 4> else: 5> the procedure ends; 3> else: 4> the procedure ends; 2> if the RRCReconfiguration message was received within nr-SecondaryCellGroupConfig in RRCConnectionReconfiguration message received via SRB3 within DLInformationTransferMRDC: 3> submit the RRCReconfigurationComplete via E-UTRA embedded in E-UTRA RRC message RRCConnectionReconfigurationComplete as specified in TS 36.331 [10], clause 5.3.5.3/5.3.5.4; 3> if reconfigurationWithSync was included in spCellConfig of an SCG: 4> initiate the Random Access procedure on the SpCell, as specified in TS 38.321 [3]; 3> else: 4> the procedure ends; NOTE 1: The order the UE sends the RRCConnectionReconfigurationComplete message and performs the Random Access procedure towards the SCG is left to UE implementation. 2> else (RRCReconfiguration was received via SRB3) but not within DLInformationTransferMRDC: 3> submit the RRCReconfigurationComplete message via SRB3 to lower layers for transmission using the new configuration; NOTE 2: In (NG)EN-DC and NR-DC, in the case RRCReconfiguration is received via SRB1 or within DLInformationTransferMRDC via SRB3, the random access is triggered by RRC layer itself as there is not necessarily other UL transmission. In the case RRCReconfiguration is received via SRB3 but not within DLInformationTransferMRDC, the random access is triggered by the MAC layer due to arrival of RRCReconfigurationComplete. 1> else if the RRCReconfiguration message was received via SRB1 within the nr-SCG within mrdc-SecondaryCellGroup (UE in NR-DC, mrdc-SecondaryCellGroup was received in RRCReconfiguration or RRCResume via SRB1): 2> if the RRCReconfiguration is applied due to a conditional reconfiguration execution for CPC which is configured via conditionalReconfiguration contained in nr-SCG within mrdc-SecondaryCellGroup: 3> submit the RRCReconfigurationComplete message via the NR MCG embedded in NR RRC message ULInformationTransferMRDC as specified in clause 5.7.2a.3. 2> if the scg-State is not included in the RRCReconfiguration or RRCResume message containing the RRCReconfiguration message: 3> if reconfigurationWithSync was included in spCellConfig in nr-SCG: 4> initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3]; 3> if the SCG was deactivated before the reception of the NR RRC message containing the RRCReconfiguration message: 4> if lower layers consider that a Random Access procedure is needed for SCG activation; or 4> if reference signal for beam failure detection of the SpCell of the SCG was not changed since the last time that beam failure was indicated from lower layer after deactivating the SCG: 5> initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3]; 3> else: 4> the procedure ends; 2> else 3> the procedure ends; NOTE 2a: The order in which the UE sends the RRCReconfigurationComplete message and performs the Random Access procedure towards the SCG is left to UE implementation.

The proposed change to 5.29 of TS 38.321 (i.e. Activation/Deactivation of SCG) in [10] is shown below:

The network may activate and deactivate the configured SCG. The MAC entity shall for the configured SCG: 1> if upper layers indicate that activation of the SCG: 2> if the timeAlignmentTimer associated with PTAG is not running: 3> indicate to upper layers that a Random Access Procedure (as specified in clause 5.1.1) is needed for SCG activation; 2> else if a Random Access Procedure is not triggered for SCG activation: 3> activate the SCG according to the timing defined in TS 38.133 [11] for direct SCG activation;

Q3-4: Do companies think that the CRs in [9][10] are agreeable?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | 38.331: there are proposals to notify MAC only after everything was processed in the SCG configuration. In this case, BFI_COUNTER will be set to 0 by the reception of the RS reconfiguration, so MAC will not trigger RACH and the problem is solved. 38.321: no need for any change. |
| Sharp | Yes (proponent) | We assume BFD procedure is stopped during SCG deactivated in case that beam failure is declared or BFD is not configured. So we think changes are needed. (Additional explanation): In the current RRC spec, it is captured explicitly that BFD is stopped in deactivated SCG if BFD is not configured. 1> If bfd-and-RLM is not configured to true: 2> stop radio link monitoring on the SCG; 2> indicate to lower layers to stop beam failure detection on the PSCell; Also, it was agreed at RAN2#117-e that: - Agreement 2: Agree UE behaviours for PSCell beam failure while the SCG is deactivated: - b) at PSCell beam failure, stop BFD However, “stop beam failure detection (stop BFD)” is unclear as Fujitsu mentioned in Q2-1, so we’d like to confirm that whether “stop beam failure detection (stop BFD)” means that “stop beam failure detection procedure (see clause 5.17 on MAC spec)” or not. If “stop beam failure detection (stop BFD)” means that “stop beam failure detection procedure”, we think the changes in [9][10] are needed. |
| vivo | NO |  |
| Nokia | No |  |
| Ericsson | No | We agree with Huawei that it is better to move the activation of SCG to a later point in the 5.3.5.3, which also avoids the issue of activating SCG which is not yet configured, for the SCG addition case. |
| Futurewei | No |  |
| Qualcomm | No |  |
| CATT | No | Agree with Huawei. |

Summary:

Only one company (proponent) supports the changes. So, we will not pursue these changes.

Proposal 10: We will not pursue the changes in R2-2205278 and in R2-2205279.

2.4 	BFR information in the SCGFailureInformation

In current TS 38.331, the UE sends SCGFailureInformation message to network with the failureType is set to be beamFailure when beam failure is detected while SCG is in deactivated state. [6] proposed that the UE provides some assistance information, i.e. the suitable beam index information in SCGFailureInformation signalling to the network so that the NW can update the TCI state(s) and thus the UE can perform SCG activation without RA.

Q4-1: Would companies like to include the suitable beam index information in SCGFailureInformation signalling to the network?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes but | the UE already reports beams of the serving cell (and other cells) in the SCG failure information procedure which is reused, so no change is needed. |
| Apple | No, but | As Huawei mentioned, UE already reports. |
| Sharp |  | We also think the information is already included. |
| Lenovo | Yes | Not sure if existing IEs can be reused for the same purpose, e.g., failedPSCellId-r17 + failureType, otherwise it looks reasonable to modify as [6] proposed. |
| vivo | No | Exsting beam index report can be used. |
| LGE | No | In legacy SCG Failure Information, the UE can include the measurement result with beam index in measResultFreqList. measResultFreqList The field contains available results of measurements on NR frequencies the UE is configured to measure by measConfig. |
| Nokia | Yes | Agree with Huawei |
| Ericsson | Yes/No | As mentioned by others, the beam measurement information is already included, no update is needed. |
| Futurewei |  | We agree with above companies‘ points that network should know the information and it is specified. |
| Qualcomm | No | Existing signalling already seems to report this. |
| CATT | Yes | Proponent. In our understanding, the current exsiting measurement results in SCGFailureInformation are based on L3 measurmenet configuration, and those are with L3 filtering, which may or maynot relfect the real time beam information. We think it is meaningful to report the real-time beam related information when beam failure occured, to help the network to re-configure the BFD RS or TCI-info for fast SCG activation without RACH procedure. |

Summary:

All companies think that the suitable beam index information should be reported to the network. 2 companies see the need to update the information while 9 companies think that the existing signaling can be reused. So, it seems that no proposal is made for this issue.

[6] provided the text proposals for TS 38.331 and for TS 37.340.

For 5.7.3.5 of TS 38.331 (i.e., Actions related to transmission of SCGFailureInformation message)

The UE shall set the contents of the SCGFailureInformation message as follows: … 1> else if the UE initiates transmission of the SCGFailureInformation message due to detection of beam failure of the PSCell while the SCG is deactivated: 2> set the failureType as other and set failureType-v1610 as beamFailure. 2> include candidateRsIndex in SCGFailureInformation message if available. …

For 6.3.2 of TS 38.331 (i.e., Radio resource control information elements)

SCGFailureInformation message -- ASN1START -- TAG-SCGFAILUREINFORMATION-START SCGFailureInformation ::= SEQUENCE { criticalExtensions CHOICE { scgFailureInformation SCGFailureInformation-IEs, criticalExtensionsFuture SEQUENCE {} } } SCGFailureInformation-IEs ::= SEQUENCE { failureReportSCG FailureReportSCG OPTIONAL, nonCriticalExtension SCGFailureInformation-v1590-IEs OPTIONAL } … FailureReportSCG ::= SEQUENCE { [[ previousPSCellId-r17 SEQUENCE { physCellId-r17 PhysCellId, carrierFreq-r17 ARFCN-ValueNR } OPTIONAL, failedPSCellId-r17 SEQUENCE { physCellId-r17 PhysCellId, carrierFreq-r17 ARFCN-ValueNR } OPTIONAL, timeSCGFailure-r17 INTEGER (0..1023) OPTIONAL, perRAInfoList-r17 PerRAInfoList-r16 OPTIONAL, candidateRsIndex-r17 CHOICE { ssb-Index-r17 SSB-Index, csi-RS-Index-r17 NZP-CSI-RS-ResourceId } ]] } … SCGFailureInformation field descriptions. candidateRsIndex The field is used to indicate suitable beam index to network when beam failure is detected while in deactivation SCG. The field is set to the index of an SSB with SS-RSRP above rsrp-ThresholdSSB amongst the SSBs in candidateBeamRSList or to the index of a CSI-RS with CSI-RSRP above rsrp-ThresholdSSB amongst the CSI-RSs in candidateBeamRSList.. ….

For 7.7 of TS 37.340 (i.e., SCG/MCG failure handling)

RLF is declared separately for the MCG and for the SCG. … In case of deactivation SCG, upon transmission of the SCGFailureInformation message to the MN when beam failure is detected during SCG deactivation, the suitable beam index could be included in SCGFailureInformation message to SN via MN, SN may response the UE by sending the updated activated TCI state via MN to UE to perform BFR without triggering RACH when receiving the suitable beam index information. …

Q4-2: If the answer to Q4-1 is Yes, would companies also agree to the text proposals for TS 38.331 and for TS 37.340 in [6]?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | This is legacy procedure so there is no need to specify anything. |
| Apple | No | Same view as Huawei |
| Sharp | No | Same view as Huawei. |
| Lenovo | Yes | As commented in Q4-1. Actually, wouldn’t it be also benefitial to report other available beams for NW to better reconfigure? |
| Nokia | No |  |
| Ericsson | No | Same view as Huawei. |
| Futurewei | No |  |
| Qualcomm | No |  |
| CATT | Yes | Proponent. See comment in Q4-1. |

Summary:

2 companies agree to the TPs while 7 companies think that the existing signaling can be reused. So, we will not pursue these changes.

Proposal 11: We will not pursue the changes in R2-2205422.

3	Conclusion

Based on companies view, the following proposals are provided:

(a) Proposals for not pursuing/capturing CRs/TPs:

<per TRP BFD at SCG deactivation>

Proposal 2: We will not pursue the changes in R2-2204909.

Proposal 3: We will not pursue the changes in R2-2204910.

<BFD stop/resumption>

Proposal 6: RAN2 will not capture BFD stop/resumption in TS 38.331.

Proposal 7: We will not pursue the changes in R2-2205280.

<Initiation of RA procedure upon SCG activation>

Proposal 10: We will not pursue the changes in R2-2205278 and in R2-2205279.

<BFR information>

Proposal 11: We will not pursue the changes in R2-2205422.

(b) Proposals for requesting further discussions

<Per TRP BFD at SCG deactivation>

Proposal 1: Choose one from the following options:

- Option 2: beam failure detection on each BFD-RS set of PSCell configured with two BFD-RS sets can be performed while the SCG is deactivated

- Option 3: SCG can only be deactivated with bfd-and-RLM configured to true if the PSCell is configured with a single BFD-RS set

Proposal 4: RAN2 further discusses whether specification change is necessary based on the selected option in Proposal 1.

Proposal 5: If Option 2 is selected in Proposal 1, it is proposed to confirm that:

The same parameter “bfd-and-RLM” should be used to indicate whether the UE performs RLM and BFD for PSCell with/without 2 BFD-RS sets.

<Initiation of RA when SCG is deactivated>

Proposal 8: The problem raised in R2-2205273 should be fixed.

Proposal 9: RAN2 discusses how to fix the issue raised in R2-2205273 based on the CR in R2-2205274. This discussion is covered in [AT118-e][222].

4	References

- R2-2204910 [F001] Beam failure detection upon SCG deactivation Fujitsu

- R2-2204909 Beam failure detection upon SCG deactivation	Fujitsu	draftCR Rel-17 38.321	17.0.0	F

- R2-2205273 Remaining issues for BFD indication in deactivated SCG Sharp

- R2-2205274 CR on 38.321 for Remaining issues for BFD indication in deactivated SCG Sharp CR	Rel-17	38.321	17.0.0	1267	- F

- R2-2205280 [J006] Correction of BFD procedure Sharp

- R2-2205422 Discussion on Beam Failure Information for Deactivated SCG CATT

- R2-2205797 [E129] Stop/resume BFD at beam failure for deactivated SCG	Ericsson

- R2-2205277 RACH-less SCG activation by SCG activation command with BFD RS change	Sharp

- R2-2205278 CR on 38.331 for RACH-less SCG activation by SCG activation command with BFD RS change Sharp CR	Rel-17	38.331	17.0.0	3062	- F

- R2-2205279 CR on 38.321 for RACH-less SCG activation by SCG activation command with BFD RS change Sharp CR	Rel-17	38.321	17.0.0	1269	- F