3GPP TSG RAN WG2 Meeting #118-e                                         	R2-2206384

# Electronic meeting, 9th-20th May 2022	Revision of R2-2205824

Agenda item:	6.11

Source:	Intel Corporation, Ericsson

Title:	[I512] [I010] SRS Positioning configuration provided for SDT

Work Item:	NR_pos_enh-Core

Document for:	Discussion and decision

# Introduction

This document discusses the RIL [I512] and provides the corresponding suggested TP to address it.

- [I512] The configuration provided by SRS-PosRRC-InactiveConfig-r17 might require some update to work with CU/DU split where the information from the DU needs to be in a container and to allow delta signaling between SDT sessions (similarly as it was done for SDT configuration e.g. sdt-MAC-PHY-CG-Config-r17). Note that the delta part is inter-related to previous comment I010..

This issue was discussed as part of the offline discussion:

[AT118-e][501][Sdata] CP Open issues and CR to 38.331 (ZTE)

and the rapporteur suggestion based on the company comments is (See Annex 4.2 for the copy paste of the company comments from Offline 501):

Proposal 10:   I512 should be discussed as part of CB in positioning session considering the   comments made about the RAN3 impacts

The WI code   for I512 is changed to positioning and moved to positioning session

Hence this document is provided to the Positioning session.

# Discussion

## Background on other SDT configurations

UE configuration provided by RRC (such as RRCReconfiguration or RRCResume messages) can come originate from the CU or DU in a split gNB architecture.  Configuration originating in the DU is carried within a container in the RRC message (as CellGroupConfig) such that the DU configuration is transparent to the CU.

RRCReconfiguration-v1530-IEs ::=            SEQUENCE {

masterCellGroup                         OCTET STRING (CONTAINING CellGroupConfig)                              OPTIONAL, -- Need M

[..]

}

For CG SDT, a similar container solution was agreed such that the CG-SDT configuration can be provided by DU, transparent to the CU:

SDT-Config-r17 ::=                  SEQUENCE {

sdt-DRB-List-r17            SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity  OPTIONAL, -- Need M

sdt-SRB2-Indication-r17     ENUMERATED {allowed}                         OPTIONAL, -- Need R

sdt-MAC-PHY-CG-Config-r17   SetupRelease {SDT-CG-Config-r17}             OPTIONAL, -- Need M

sdt-DRB-ContinueROHC-r17    ENUMERATED { cell, rna }                     OPTIONAL   -- Need R

}

SDT-CG-Config-r17 ::= OCTET STRING (CONTAINING SDT-MAC-PHY-CG-Config-r17)

ASN.1 structure of sdt-MAC-PHY-CG-Config-r17 defines SDT-CG-Config-r17 as an OCTET string that is created by DU and conveyed from DU to CU. This container is used transparently by CU when creating the RRC message that configure or release the corresponding CG-SDT configuration. Similar discussion/exercise should be done for SRS positioning configuration defined for SDT (i.e. SRS-PosRRC-InactiveConfig-r17).

## Node to provide the SRS Positioning configuration

Firstly RAN2/RAN3 should discuss whether the configuration provided as part of SRS-PosRRC-InactiveConfig-r17 is provided by DU or by CU. During positioning discussion, our understanding is that this topic was not discussed, and for CG-SDT, it had minimal discussion i.e. without checking all the information of the actual configuration.

SuspendConfig ::=                   SEQUENCE {

fullI-RNTI                          I-RNTI-Value,

shortI-RNTI                         ShortI-RNTI-Value,

ran-PagingCycle                     PagingCycle,

ran-NotificationAreaInfo            RAN-NotificationAreaInfo   OPTIONAL,   -- Need M

t380                                PeriodicRNAU-TimerValue    OPTIONAL,   -- Need R

nextHopChainingCount                NextHopChainingCount,

...,

[[

sl-ServingCellInfo-r17              SL-ServingCellInfo-r17     OPTIONAL, -- Cond L2RemoteUE

sdt-Config-r17                      SetupRelease { SDT-Config-r17 }   OPTIONAL,   -- Need M

srs-PosRRC-InactiveConfig-r17       SRS-PosRRC-InactiveConfig-r17     OPTIONAL,   -- Need M

ran-ExtendedPagingCycle-r17         ExtendedPagingCycle-r17           OPTIONAL    -- Need R

]]

}

SRS-PosRRC-InactiveConfig-r17 ::=       SEQUENCE {

srs-PosConfig-r17                       SRS-PosConfig-r17,

bwp-r17                                 BWP                         OPTIONAL,    -- Need S

srs-TimeAlignmentTimer-r17              TimeAlignmentTimer          OPTIONAL,    -- Need R

inactivePosSRS-RSRP-changeThresh-r17    RSRP-ChangeThresh-r17       OPTIONAL,    -- Need R

srs-NrofSS-BlocksToAverage-r17          INTEGER (1..ffsUpperLimit)  OPTIONAL,    -- Need R

-- FFS upper limit

inactivePosSRS-AbsThreshSS-BlocksConsolidation-r17  RSRP-Range      OPTIONAL     -- Need R

}

--Editor’s Note: Following temporary constant is introduced only for ASN.1 syntax purposes. Actual upper limit of the ranges using this constant throughout the specification are FFS.

ffsUpperLimit INTEGER ::= 9999

RSRP-ChangeThresh-r17 ::= ENUMERATED {dB4, dB6, dB8, dB10, dB14, dB18, dB22, dB26, dB30, dB34, spare6, spare5, spare4, spare3, spare2, spare1}

--Editor’s Note: To be updated to align with SDT, to further update SUL/NUL and BWP--

SRS-PosConfig-r17 ::=               SEQUENCE {

srs-PosResourceSetToReleaseList-r17 SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF

SRS-PosResourceSetId-r16 OPTIONAL,-- Need N

srs-PosResourceSetToAddModList-r17  SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF

SRS-PosResourceSet-r16  OPTIONAL,-- Need N

srs-PosResourceToReleaseList-r17    SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF

SRS-PosResourceId-r16      OPTIONAL,-- Need N

srs-PosResourceToAddModList-r17     SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF

SRS-PosResource-r16        OPTIONAL -- Need N

}

For this exercise, we suggest start checking how similar R15/16 configuration is defined considering CU/DU split.

| IEs defined as part of SRS-PosRRC-InactiveConfig-r17 | Node to set the config. | CU/DU handling for similar R15/16 configuration |
| --- | --- | --- |
| srs-PosConfig-r17 | DU | srs-PosConfig-r17 includes SRS-PosResourceSetId-r16 configuration which previously was generated by DU and conveyed to CU via CellGroupConfig. |
| bwp-r17 | DU | bwp-r17 includes BWP configuration which previously was generated by DU and conveyed to CU via CellGroupConfig |
| srs-TimeAlignmentTimer-r17 | ??DU | The behaviour of this SRS-TAT is different than legacy TAT |
| inactivePosSRS-RSRP-changeThresh-r17 | ?? DU |  |
| srs-NrofSS-BlocksToAverage-r17 | ?? DU |  |
| inactivePosSRS-AbsThreshSS-BlocksConsolidation-r17 | ?? DU |  |

### Parity with connected mode

For connected mode, the DU generates the SRS positioning config.  As shown below, it is included in the SRS-Config:

SRS-Config ::=                          SEQUENCE {

[..]

srs-PosResourceSetToReleaseList-r16     SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSetId-r16

OPTIONAL, -- Need N

srs-PosResourceSetToAddModList-r16      SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSet-r16        OPTIONAL,-- Need N

srs-PosResourceToReleaseList-r16        SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResourceId-r16            OPTIONAL,-- Need N

srs-PosResourceToAddModList-r16         SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResource-r16              OPTIONAL -- Need N

[..]

}

And the SRS-Config is included in the BWP-UplinkDedicated which is included within the CellGroupConfig container.

Hence including the SRS-PosRRC-InactiveConfig-r17 also in the container generated by the DU makes it aligned with the corresponding configuration in CONNECTED mode.

Including the SRS-PosRRC-InactiveConfig-r17 for INACTIVE in the container generated by the DU makes it aligned with the corresponding configuration in CONNECTED mode where it is generated by DU.

- RAN2 assumes that DU can set the configuration associated to SRS-PosRRC-InactiveConfig-r17 and should be provided in a container as part of the corresponding ASN.1.

- If Proposal 1 is agreed, to inform RAN3 via an LS about this agreement highlighting the following:

  - RAN3 is requested to define a new F1-AP container to convey the container SRS-PosRRC-InactiveConfig-r17 from DU to CU.

  - RAN3 is asked to confirm that the configuration of the new SDT related parameters defined as part of RRCRelease message can be set by DU (i.e.,  SRS-PosRRC-InactiveConfig-r17 and SDT-MAC-PHY-CG-Config). This includes for example, srs-TimeAlignmentTimer-r17, inactivePosSRS-RSRP-changeThresh-r17, srs-NrofSS-BlocksToAverage-r17, inactivePosSRS-AbsThreshSS-BlocksConsolidation-r17, cg-SDT-TimeAlignmentTimer, cg-SDT-RSRP-ThresholdSSB-r17 and cg-SDT-RSRP-ChangeThreshold-r17 .

- If Proposal 1 is agreed, to adopt the TP available in section 4.1 of Annex.

# Conclusion

The observations captured are the following:

Observation 1.	ASN.1 structure of sdt-MAC-PHY-CG-Config-r17 defines SDT-CG-Config-r17 as an OCTET string that is created by DU and conveyed from DU to CU. This container is used transparently by CU when creating the RRC message that configure or release the corresponding CG-SDT configuration. Similar discussion/exercise should be done for SRS positioning configuration defined for SDT (i.e. SRS-PosRRC-InactiveConfig-r17).

Observation 2.	Including the SRS-PosRRC-InactiveConfig-r17 for INACTIVE in the container generated by the DU makes it aligned with the corresponding configuration in CONNECTED mode where it is generated by DU.

The proposals captured are the following:

Proposal 1.	RAN2 assumes that DU can set the configuration associated to SRS-PosRRC-InactiveConfig-r17 and should be provided in a container as part of the corresponding ASN.1.

Proposal 2.	If Proposal 1 is agreed, to inform RAN3 via an LS about this agreement highlighting the following:

Proposal 2.1.	RAN3 is asked to define a new F1-AP container to convey the container SRS-PosRRC-InactiveConfig-r17 from DU to CU.

Proposal 2.2.	RAN3 is requested to confirm that the configuration of the new SDT related parameters defined as part of RRCRelease message can be set by DU (i.e.,  SRS-PosRRC-InactiveConfig-r17 and SDT-MAC-PHY-CG-Config). This includes for example, srs-TimeAlignmentTimer-r17, inactivePosSRS-RSRP-changeThresh-r17, srs-NrofSS-BlocksToAverage-r17, inactivePosSRS-AbsThreshSS-BlocksConsolidation-r17, cg-SDT-TimeAlignmentTimer, cg-SDT-RSRP-ThresholdSSB-r17 and cg-SDT-RSRP-ChangeThreshold-r17 .

Proposal 3.	If Proposal 1 is agreed, to adopt the TP available in section 4.1 of Annex.

# Annex:

Latest draft TS are used from https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/Specifications/202203_draft_specs_after_RAN_95/

- TP to 38.331 v17.0.0 (2022-03) i.e. version -h00-v6

## Suggested Text Proposals (TPs)

- Modified section

### 6.2.2	Message definitions

<*** omitted text ***>

#### –	RRCRelease

The RRCRelease message is used to command the release of an RRC connection or the suspension of the RRC connection.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

RRCRelease message

-- ASN1START

-- TAG-RRCRELEASE-START

RRCRelease ::=                      SEQUENCE {

rrc-TransactionIdentifier           RRC-TransactionIdentifier,

criticalExtensions                  CHOICE {

rrcRelease                          RRCRelease-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

RRCRelease-IEs ::=                  SEQUENCE {

redirectedCarrierInfo               RedirectedCarrierInfo                                                       OPTIONAL,   -- Need N

cellReselectionPriorities           CellReselectionPriorities                                                   OPTIONAL,   -- Need R

suspendConfig                       SuspendConfig                                                               OPTIONAL,   -- Need R

deprioritisationReq                 SEQUENCE {

deprioritisationType                ENUMERATED {frequency, nr},

deprioritisationTimer               ENUMERATED {min5, min10, min15, min30}

}                                                                                                               OPTIONAL,   -- Need N

lateNonCriticalExtension                OCTET STRING                                                        OPTIONAL,

nonCriticalExtension                    RRCRelease-v1540-IEs                                                OPTIONAL

}

RRCRelease-v1540-IEs ::=            SEQUENCE {

waitTime                           RejectWaitTime                OPTIONAL, -- Need N

nonCriticalExtension               RRCRelease-v1610-IEs          OPTIONAL

}

RRCRelease-v1610-IEs ::=            SEQUENCE {

voiceFallbackIndication-r16        ENUMERATED {true}                             OPTIONAL, -- Need N

measIdleConfig-r16                 SetupRelease {MeasIdleConfigDedicated-r16}    OPTIONAL, -- Need M

nonCriticalExtension               RRCRelease-v1650-IEs                          OPTIONAL

}

RRCRelease-v1650-IEs ::=            SEQUENCE {

mpsPriorityIndication-r16          ENUMERATED {true}                             OPTIONAL, -- Cond Redirection2

nonCriticalExtension               SEQUENCE {}                                   OPTIONAL

}

RedirectedCarrierInfo ::=           CHOICE {

nr                                  CarrierInfoNR,

eutra                               RedirectedCarrierInfo-EUTRA,

...

}

RedirectedCarrierInfo-EUTRA ::=     SEQUENCE {

eutraFrequency                      ARFCN-ValueEUTRA,

cnType                              ENUMERATED {epc,fiveGC}                                             OPTIONAL    -- Need N

}

CarrierInfoNR ::=                   SEQUENCE {

carrierFreq                         ARFCN-ValueNR,

ssbSubcarrierSpacing                SubcarrierSpacing,

smtc                                SSB-MTC                                                             OPTIONAL,      -- Need S

...

}

SuspendConfig ::=                   SEQUENCE {

fullI-RNTI                          I-RNTI-Value,

shortI-RNTI                         ShortI-RNTI-Value,

ran-PagingCycle                     PagingCycle,

ran-NotificationAreaInfo            RAN-NotificationAreaInfo                                            OPTIONAL,   -- Need M

t380                                PeriodicRNAU-TimerValue                                             OPTIONAL,   -- Need R

nextHopChainingCount                NextHopChainingCount,

...,

[[

sl-ServingCellInfo-r17              SL-ServingCellInfo-r17                                              OPTIONAL, -- Cond L2RemoteUE

sdt-Config-r17                      SetupRelease { SDT-Config-r17 }                                     OPTIONAL,   -- Need M

srs-PosRRC-Inactive-r17       SetupRelease { SRS-PosRRC-Inactive-r17 }                      OPTIONAL,   -- Need M

ran-ExtendedPagingCycle-r17         ExtendedPagingCycle-r17                                             OPTIONAL    -- Need R

]]

}

PeriodicRNAU-TimerValue ::=         ENUMERATED { min5, min10, min20, min30, min60, min120, min360, min720}

CellReselectionPriorities ::=       SEQUENCE {

freqPriorityListEUTRA               FreqPriorityListEUTRA                                               OPTIONAL,       -- Need M

freqPriorityListNR                  FreqPriorityListNR                                                  OPTIONAL,       -- Need M

t320                                ENUMERATED {min5, min10, min20, min30, min60, min120, min180, spare1} OPTIONAL,     -- Need R

...,

[[

freqPriorityListNRSlicing-r17       FreqPriorityListNRSlicing-r17                                       OPTIONAL        -- Need M

]]

}

PagingCycle ::=                     ENUMERATED {rf32, rf64, rf128, rf256}

ExtendedPagingCycle-r17 ::=         ENUMERATED {rf256, rf512, rf1024, spare1}

FreqPriorityListEUTRA ::=           SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityEUTRA

FreqPriorityListNR ::=              SEQUENCE (SIZE (1..maxFreq)) OF FreqPriorityNR

FreqPriorityEUTRA ::=               SEQUENCE {

carrierFreq                         ARFCN-ValueEUTRA,

cellReselectionPriority             CellReselectionPriority,

cellReselectionSubPriority          CellReselectionSubPriority                                          OPTIONAL        -- Need R

}

FreqPriorityNR ::=                  SEQUENCE {

carrierFreq                         ARFCN-ValueNR,

cellReselectionPriority             CellReselectionPriority,

cellReselectionSubPriority          CellReselectionSubPriority                                          OPTIONAL        -- Need R

}

RAN-NotificationAreaInfo ::=        CHOICE {

cellList                            PLMN-RAN-AreaCellList,

ran-AreaConfigList                  PLMN-RAN-AreaConfigList,

...

}

PLMN-RAN-AreaCellList ::=           SEQUENCE (SIZE (1.. maxPLMNIdentities)) OF PLMN-RAN-AreaCell

PLMN-RAN-AreaCell ::=               SEQUENCE {

plmn-Identity                       PLMN-Identity                                                       OPTIONAL,   -- Need S

ran-AreaCells                       SEQUENCE (SIZE (1..32)) OF  CellIdentity

}

PLMN-RAN-AreaConfigList ::=         SEQUENCE (SIZE (1..maxPLMNIdentities)) OF PLMN-RAN-AreaConfig

PLMN-RAN-AreaConfig ::=             SEQUENCE {

plmn-Identity                       PLMN-Identity                                                       OPTIONAL,   -- Need S

ran-Area                            SEQUENCE (SIZE (1..16)) OF  RAN-AreaConfig

}

RAN-AreaConfig ::=                  SEQUENCE {

trackingAreaCode                    TrackingAreaCode,

ran-AreaCodeList                    SEQUENCE (SIZE (1..32)) OF  RAN-AreaCode                            OPTIONAL    -- Need R

}

SDT-Config-r17 ::=                  SEQUENCE {

sdt-DRB-List-r17                    SEQUENCE (SIZE (0..maxDRB)) OF DRB-Identity                         OPTIONAL,   -- Need M

sdt-SRB2-Indication-r17             ENUMERATED {allowed}                                                OPTIONAL,   -- Need R

sdt-MAC-PHY-CG-Config-r17           SetupRelease {SDT-CG-Config-r17}                                        OPTIONAL,   -- Need M

sdt-DRB-ContinueROHC-r17            ENUMERATED { cell, rna }                                            OPTIONAL    -- Need R

}

SDT-CG-Config-r17 ::= OCTET STRING (CONTAINING SDT-MAC-PHY-CG-Config-r17)

SDT-MAC-PHY-CG-Config-r17 ::=       SEQUENCE {

-- CG-SDT specific configuration

-- FFS on BSR configuration (e.g. i.e. for the FFS on the logicalChannelSR-DelayTimer)

-- FFS on delta signalling (We need to clarify how this works, for instance, whether initial BWP dedicated can be considered as

-- baseline to enable delta configuration or not etc).

cg-SDT-Config-LCH-restrictionToAddModList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF  CG-SDT-Config-LCH-restriction-r17 OPTIONAL,   -- Need N

cg-SDT-Config-LCH-restrictionToReleaseList-r17 SEQUENCE (SIZE(1..maxLC-ID)) OF  LogicalChannelIdentity  OPTIONAL,   -- Need N

cg-SDT-Config-Initial-BWP-NUL-r17       SetupRelease {BWP-Uplink-Dedicated-SDT-r17}                     OPTIONAL,   -- Need M

cg-SDT-Config-Initial-BWP-SUL-r17       SetupRelease {BWP-Uplink-Dedicated-SDT-r17}                     OPTIONAL,   -- Need M

cg-SDT-Config-Initial-BWP-DL-r17        BWP-Downlink-Dedicated-SDT-r17                                  OPTIONAL,   -- Need M

cg-SDT-TimeAlignmentTimer-r17           TimeAlignmentTimer                                              OPTIONAL,   -- Need M

cg-SDT-RSRP-ThresholdSSB-r17            RSRP-Range                                                      OPTIONAL,   -- Need M

cg-SDT-TA-ValiditationConfig-r17        SetupRelease { CG-SDT-TA-ValiditationConfig-r17 }               OPTIONAL,   -- Need M

...

}

CG-SDT-TA-ValiditationConfig-r17 ::= SEQUENCE {

cg-SDT-RSRP-ChangeThreshold-r17     RSRP-Range

}

BWP-Downlink-Dedicated-SDT-r17 ::=  SEQUENCE {

pdcch-Config-r17                    SetupRelease { PDCCH-Config }                                       OPTIONAL,   -- Need M

pdsch-Config-r17                    SetupRelease { PDSCH-Config }                                       OPTIONAL,   -- Need M

...

}

BWP-Uplink-Dedicated-SDT-r17 ::=    SEQUENCE {

pusch-Config-r17                    SetupRelease { PUSCH-Config }                                       OPTIONAL,   -- Need M

configuredGrantConfigToAddModList-r17                 ConfiguredGrantConfigToAddModList-r17             OPTIONAL,   -- Need N

configuredGrantConfigToReleaseList-r17                ConfiguredGrantConfigToReleaseList-r17            OPTIONAL,   -- Need N

...

}

ConfiguredGrantConfigToAddModList-r17 ::= SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfig

ConfiguredGrantConfigToReleaseList-r17 ::= SEQUENCE (SIZE (1..maxNrofConfiguredGrantConfig-r16)) OF ConfiguredGrantConfigIndex-r16

CG-SDT-Config-LCH-restriction-r17 ::= SEQUENCE {

logicalChannelIdentity-r17          LogicalChannelIdentity,

configuredGrantType1Allowed-r17     ENUMERATED {true}                                                   OPTIONAL,   -- Need R

allowedCG-List-r17              SEQUENCE (SIZE (0.. maxNrofConfiguredGrantConfigMAC-1-r16)) OF ConfiguredGrantConfigIndexMAC-r16

OPTIONAL    -- Need R

}

SRS-PosRRC-Inactive-r17 ::= OCTET STRING (CONTAINING SRS-PosRRC-InactiveConfig-r17)

SRS-PosRRC-InactiveConfig-r17 ::=       SEQUENCE {

srs-PosConfig-r17                       SRS-PosConfig-r17,

bwp-r17                                 BWP                                                                 OPTIONAL,    -- Need S

srs-TimeAlignmentTimer-r17              TimeAlignmentTimer                                                  OPTIONAL,    -- Need R

inactivePosSRS-RSRP-changeThresh-r17    RSRP-ChangeThresh-r17                                               OPTIONAL,    -- Need R

srs-NrofSS-BlocksToAverage-r17          INTEGER (1..ffsUpperLimit)                                          OPTIONAL,    -- Need R

-- FFS upper limit

inactivePosSRS-AbsThreshSS-BlocksConsolidation-r17  RSRP-Range                                              OPTIONAL     -- Need R

}

--Editor’s Note: Following temporary constant is introduced only for ASN.1 syntax purposes. Actual upper limit of the ranges using this constant throughout the specification are FFS.

ffsUpperLimit INTEGER ::= 9999

RSRP-ChangeThresh-r17 ::= ENUMERATED {dB4, dB6, dB8, dB10, dB14, dB18, dB22, dB26, dB30, dB34, spare6, spare5, spare4, spare3, spare2, spare1}

--Editor’s Note: To be updated to align with SDT, to further update SUL/NUL and BWP--

SRS-PosConfig-r17 ::=               SEQUENCE {

srs-PosResourceSetToReleaseList-r17 SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSetId-r16 OPTIONAL,-- Need N

srs-PosResourceSetToAddModList-r17  SEQUENCE (SIZE(1..maxNrofSRS-PosResourceSets-r16)) OF SRS-PosResourceSet-r16  OPTIONAL,-- Need N

srs-PosResourceToReleaseList-r17    SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResourceId-r16      OPTIONAL,-- Need N

srs-PosResourceToAddModList-r17     SEQUENCE (SIZE(1..maxNrofSRS-PosResources-r16)) OF SRS-PosResource-r16        OPTIONAL -- Need N

}

-- TAG-RRCRELEASE-STOP

-- ASN1STOP

RRCRelease-IEs field descriptions

cnType Indicate that the UE is redirected to EPC or 5GC.

deprioritisationReq Indicates whether the current frequency or RAT is to be de-prioritised.

deprioritisationTimer Indicates the period for which either the current carrier frequency or NR is deprioritised. Value minN corresponds to N minutes.

measIdleConfig Indicates measurement configuration to be stored and used by the UE while in RRC_IDLE or RRC_INACTIVE.

mpsPriorityIndication Indicates the UE can set the establishment cause to mps-PriorityAccess for a new connection to a new RAT following a redirect to NR. If the target RAT is E-UTRA, see TS 36.331 [10]. The gNB sets the indication only for UEs authorized to receive MPS treatment as indicated by ARP and/or QoS characteristics at the gNB, and it is applicable only for this instance of release with redirection to carrier/RAT included in the redirectedCarrierInfo field in the RRCRelease message.

srs-PosRRCInactiveConfig SRS for positioning confifuration during RRC_INACTIVE State.

suspendConfig Indicates configuration for the RRC_INACTIVE state. The network does not configure suspendConfig when the network redirect the UE to an inter-RAT carrier frequency or if the UE is configured with a DAPS bearer.

redirectedCarrierInfo Indicates a carrier frequency (downlink for FDD) and is used to redirect the UE to an NR or an inter-RAT carrier frequency, by means of cell selection at transition to RRC_IDLE or RRC_INACTIVE as specified in TS 38.304 [20]. Based on UE capability, the network may include redirectedCarrierInfo in RRCRelease message with suspendConfig if this message is sent in response to an RRCResumeRequest or an RRCResumeRequest1 which is triggered by the NAS layer (see 5.3.1.4 in TS 24.501 [23]).

voiceFallbackIndication Indicates the RRC release is triggered by EPS fallback for IMS voice as specified in TS 23.502 [43].

CarrierInfoNR field descriptions

carrierFreq Indicates the redirected NR frequency.

ssbSubcarrierSpacing Subcarrier spacing of SSB in the redirected SSB frequency. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

smtc The SSB periodicity/offset/duration configuration for the redirected SSB frequency. It is based on timing reference of PCell. If the field is absent, the UE uses the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing.

RAN-NotificationAreaInfo field descriptions

cellList A list of cells configured as RAN area.

ran-AreaConfigList A list of RAN area codes or RA code(s) as RAN area.

PLMN-RAN-AreaConfig field descriptions

plmn-Identity PLMN Identity to which the cells in ran-Area belong. If the field is absent the UE not in SNPN access mode uses the ID of the registered PLMN. This field is not included for UE in SNPN access mode (for UE in SNPN access mode the ran-Area always belongs to the registered SNPN).

ran-AreaCodeList The total number of RAN-AreaCodes of all PLMNs does not exceed 32.

ran-Area Indicates whether TA code(s) or RAN area code(s) are used for the RAN notification area. The network uses only TA code(s) or both TA code(s) and RAN area code(s) to configure a UE. The total number of TACs across all PLMNs does not exceed 16.

PLMN-RAN-AreaCell field descriptions

plmn-Identity PLMN Identity to which the cells in ran-AreaCells belong. If the field is absent the UE not in SNPN access mode uses the ID of the registered PLMN. This field is not included for UE in SNPN access mode (for UE in SNPN access mode the ran-AreaCells always belongs to the registered SNPN).

ran-AreaCells The total number of cells of all PLMNs does not exceed 32.

SDT-Config field descriptions

sdt-DRB-ContinueROHC Indicates whether the PDCP entity for the radio bearers configured for SDT continues or resets the ROHC header compression protocol during PDCP re-establishment during SDT procedure, as specified in TS 38.323 [5]. Value cell indicates that ROHC header compression continues when the UE resumes for SDT in the same cell as the PCell when the RRCRelease message is received. Value rna indicates that ROHC header compression continues when the UE resumes for SDT in a cell belonging to the same RNA as the PCell when the RRCRelease message is received. If the field is absent PDCP entity for the radio bearers configured for SDT reset the ROHC header compression protocol during PDCP re-establishment during SDT procedure, as specified in TS 38.323 [5].

sdt-DRB-List Indicates the ID(s) of the DRB(s) that are configured for SDT. If size of the sequence is zero, then the UE assumes that none of the DRBs are configured for SDT. The network only configures MN terminated MCG bearers for SDT.

sdt-SRB2-Indication Indiates whether SRB2 is configured for SDT or not.

SDT-MAC-PHY-CG-Config field descriptions

cg-SDT-RSRP-ThresholdSSB An RSRP threshold configured for SSB selection for CG-SDT as specified in TS 38.321 [3].

CG-SDT-TA-ValiditationConfig Configuration for the RSRP based TA validation. If this IE is not configured, then the UE does not perform RSRP based TA validation.

cg-SDT-timeAlignmentTimer TAT value for CG-SDT as specified in TS 38.321 [3]. The network always configures this when sdt-MAC-PHY-CG-Config is configured.

CG-SDT-TA-ValiditationConfig field descriptions

cg-SDT-RSRP-ChangeThreshold The RSRP threshold for TA validation for CG-SDT as specified in TS 38.321 [3].

SRS-PosRRC-InactiveConfig field descriptions

bwp BWP configuration for SRS for Positioning during the RRC_INACTIVE state. If the field is absent UE is configured with an SRS for Positioning associated with the initial UL BWP and transmitted, during the RRC_INACTIVE state, inside the initial UL BWP with the same CP and SCS as configured for initial UL BWP.

inactivePosSRS-AbsThreshSS-BlocksConsolidation absolute RSRP threshold for determining the set of SSBs for derivation of downlink pathloss reference for TA validation.

inactivePosSRS-NrofSS-BlocksToAverage number of SSBs with highest RSRPs for derivation of downlink pathloss reference for TA validation

inactivePosSRS-RSRP-changeThresh RSRP threshold for the increase/decrease of RSRP for time alignment validation

srs-TimeAlignmnetTimer TA timer for SRS for positioning transmission during RRC_INACTIVE State. Editor’s Note: Range to be taken from SDT

| SuspendConfig field descriptions |  |
| --- | --- |
| ran-ExtendedPagingCycle The extended DRX (eDRX) cycle for RAN-initiated paging to be applied by the UE. Value rf256 corresponds to 256 radio frames, value rf512 corresponds to 512 radio frames and so on. The field is only included when the UE is configured with eDRX in RRC_IDLE, see TS 24.401 [23]. Value of the field indicates an eDRX cycle which is shorter or equal to the IDLE mode eDRX cycle configured for the UE. |
| ran-NotificationAreaInfo Network ensures that the UE in RRC_INACTIVE always has a valid ran-NotificationAreaInfo. |  |
| ran-PagingCycle Refers to the UE specific cycle for RAN-initiated paging. Value rf32 corresponds to 32 radio frames, value rf64 corresponds to 64 radio frames and so on. |  |
| t380 Refers to the timer that triggers the periodic RNAU procedure in UE. Value min5 corresponds to 5 minutes, value min10 corresponds to 10 minutes and so on. |  |
| sl-ServingCellInfo Indicates the Uu serving Cell related related information. |  |

| Conditional Presence | Explanation |
| --- | --- |
| L2RemoteUE | The field is mandatory present for L2 U2N Remote UE, need M; otherwise it is absent. |
| Redirection2 | The field is optionally present, Need R, if redirectedCarrierInfo is included; otherwise the field is not present. |

End of the modified section

## Company comments on I512 in [AT118-e][501][Sdata] CP Open issues and CR to 38.331 (ZTE)

| I512 | The configuration provided by SRS-PosRRC-InactiveConfig-r17 might require some update to work with CU/DU split where the information from the DU needs to be in a container and to allow delta signaling between SDT sessions (similarly as it was done for SDT configuration e.g. sdt-MAC-PHY-CG-Config-r17). Note that the delta part is inter-related to previous comment I010. | We will provide a TDoc to discuss the suggested update to SRS-PosRRC-InactiveConfig-r17 | Discuss in positionining session. [Rapp4] the positioning guys should review the CR it whereever it gets agreed. [AT meeting guidance]: Place holder for now. May be this should be moved to positioning session. We should clarify this first. It seems positioning experts should implement this change in any case?? Update from positioning session. The following plan was agreed in positioning session: Chair’s note: Companies’ attention is drawn to email discussion [AT118-e][501], where the discussion of document R2-2205824 for RRC RIL I512 is handled. The proposal is to change the ASN.1 structure around the positioning-related field srs-PosRRC-InactiveConfig-r17. It is assumed that the conclusion will be captured by the positioning RRC rapporteur. So, please do comment on this (especially if you donot agree with this change). The rapporteur thinks this is merely an alignment with SDT way of signalling and hence is technically correct. So, unless I see some negative comments, I will propose to agree this. | [Intel] We understand that this topic should be better discuss on SDT session as positioning session agreed to follow the same approach as agreed for SDT on the signaling handling (e.g. on the delta behaviour). Further justification details and TP available in R2-2205824. We propose: - Proposal1. RAN2 assumes that DU can set the configuration associated to SRS-PosRRC-InactiveConfig-r17 and should be provided in a container as part of the corresponding ASN.1. - Proposal 2. If Proposal 1 is agreed, to inform RAN3 via an LS (with details in TDoc) - Proposal 3. Update ASN.1 to define srs-PosRRC-InactiveConfig-r17 with a SetupRelease type. | ZTE | We agree with the changes but these should be discusssed in positioning session. |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Intel | Positioning session Chair has informed positioning experts to check this email discussion, and the conclusion will be captured in positioning RRC CR directly. We suggest agreeing on P1/P2. Note that P3 has already being updated by positioning RRC Rapp. | Y |
|  |  |  |  |  | Google | We agree with the changes because SRS-PosRRC-InactiveConfig-r17 should be generated by the DU. | Y |
|  |  |  |  |  | Huawei, HiSilicon | We agree with the proposal, but it is completely unclear why this is discussed within SDT WI and not positioning WI. | Y (for positioning) |
|  |  |  |  |  | Qualcomm | It seems this should be discussed in Positioning session. | N |
|  |  |  |  |  | CATT | Discuss it in positioning session. |  |
|  |  |  |  |  | Xiaomi | This can be discussed in the positioning session. |  |
|  |  |  |  |  | Apple | It may need to be discussed in positioning session first. |  |
|  |  |  |  |  | Interdigital | It should be discussed in positioning session. |  |
|  |  |  |  |  | vivo | SRS needs to be sent to the other serving cell rather than the anchor, which is different than CG-SDT. So, it is better to leave it to Pos session at first. | N |
|  |  |  |  |  | Ericsson | This should also be discussed in Positioning RAN3 as CU-DU interaction is part of RAN3 to decide; i.e where it should be transparent to CU or not. | N |
|  |  |  |  |  | Qualcomm2 | The P1/P2 are more about RAN3 Positioning (or SDT) scope. RAN3 may discuss how the CU acquires the SRS-PosRRC-InactiveConfig-r17, and RAN2 can decide whether a similar container needs to be defined in RRC, like SDT-MAC-PHY-CG-Config. RAN2 can just inform RAN3 that it has assumed that it would be useful to define a container in RRC and to confirm with RAN3. |  |
|  |  |  |  |  | Rapp Summary | - Companies think some discussion is needed in positioning session - Companies commented that there may be some differences here compared to SDT (see Ericsson and Qualcomm comments above) Proposal 10: I512 should be discussed as part of CB in positioning session considering the comments made about the RAN3 impacts The WI code for I512 is changed to positioning and moved to positioning session |  |