3GPP TSG-RAN WG2 Meeting #118 electronic	R2-2206507

Online, May 9 – 20th, 2022

Agenda Item:	6.10.1.1

Source: 	Thales

Title:  	Summary of [AT118-e][112][NTN] Stage-2 CR (Thales)

Document for:	Discussion and Decision

# Introduction

This document aims to summarize the following discussion.

[AT118-e][112][NTN] Stage-2 CR (Thales)

Scope: continue the discussion on the Stage-2 CR, also considering Stage-2 text proposals in submitted contributions

Intended outcome: Agreeable Stage-2 CR

Deadline (for companies' feedback):  Thursday 2022-05-19 12:00 UTC

Deadline (for final CR in R2-2206202):  Friday 2022-05-20 08:00 UTC

Status: On going

The below topic is also added in this thread

R2-2206211 [offline-108] UE capabilities – second round Intel discussion Rel-17 NR_NTN_solutions-Core List of proposals that require online discussions: Proposal 2: RAN2 to discuss whether to add a clarification in a note in stage 2 spec, i.e., dish type UE (static “VSAT” type NTN capable UE without GNSS module but with GNSS coordinates) can also be supported in Rel-17. Discuss in offline 112

This offline discussion aims at consolidating a CR for TS 38.300 as outcome of RAN2#118-e.

# 1st round discussion

## 2.1 Chapter 16.14.8 UE location aspects

In R2-2204627, it is proposed to resolve the current EN as follow:

Editor's note: Upon network request, after AS security in connected mode is established, a UE can report coarse UE location information (X most Significant Bits of its GNSS coordinates with accuracy around 2km level) to the NG-RAN without receiving any prior explicit user consent. if "user consent" is available at the UE, the UE will report the coarse UE location information. Else, the UE will respond "no coarse GNSS location available". Periodic location reporting can be configured by gNB to obtain UE location update of mobile UEs in RRC_CONNECTED. This proposed text may be updated upon SA3 feedback.

Upon network request, after AS security in connected mode is established, a UE can report coarse UE location information (the most Significant Bits of its GNSS coordinates corresponding to an accuracy around 2km level) to the NG-RAN without receiving any prior explicit user consent. if "user consent" is available at the UE, the UE will report the coarse UE location information. Else, the UE will respond "no coarse GNSS location available". Periodic location reporting can be configured by gNB to obtain UE location update of mobile UEs in RRC_CONNECTED.

Question 2.1: Do companies agree with the above proposed change or have further suggestions ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Thales | Agree |  |
| Qualcomm | Disagree | There is no need of coarse UE location. There is already mechanism for UE to report full GNSS coordinates to network after AS security is established. This is already supported by Rel-17 NTN UEs. That’s what matters from RAN2 perspective and RAN2 specification point of view. The user consent issue is not RAN2 issue. We should not be doing unnecessary changes until LS reply is received from SA3. |
| LG | Agree |  |
| Ericsson | Disagree | We agree with QC. We propose to remove the Editor’s note and 16.14.8. |
| Apple | Agree with some comments | We are mostly OK with the current text. Perhaps instead of saying user consent explicitly, we can reuse the text from the agreement in the last meeting and replace “If user consent is available at the UE…” with “If the UE does not or cannot provide coarse GNSS coordinates…” |

[Rapporteur summary]:

2 companies agree while 2 companies disagree (of which 1 suggests to not be doing unnecessary changes until LS reply is received from SA3).

Apple suggests to revise the text as follow:

- Upon network request, after AS security in connected mode is established, a UE can report coarse UE location information (the most Significant Bits of its GNSS coordinates corresponding to an accuracy around 2km level) to the NG-RAN without receiving any prior explicit user consent. If the UE does not or cannot provide coarse GNSS coordinates the UE will respond "no coarse GNSS location available". Else, the UE will report the coarse UE location information. if "user consent" is available at the UE, the UE will report the coarse UE location information. Else, the UE will respond "no coarse GNSS location available". Periodic location reporting can be configured by gNB to obtain UE location update of mobile UEs in RRC_CONNECTED.

Moderator suggests to wait LS reply is received.

QC may clarify what mechanism already exists (at AS level) for UE to report full GNSS coordinates to network after AS security is established.

## Satellite Access Node

In [3]	R2-2204628, it is proposed the following

Proposal 2.2.1: Add in clause “3.1	Abbreviations” the following definition:

- SAN	Satellite Access Node

Question 2.2.1: Do companies agree with the above proposal or have further suggestions ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Thales | Agree |  |
| Qualcomm | Agree |  |
| LG | Agree |  |
| Ericsson | Disagree | This is not needed in stage 2spec. SAN is not used in the stage 2, nor in RAN2 nor RAN3 specs. |
| Apple | Disagree | Since the WI is closed, we are really reluctant to add a new definition. |

Proposal 2.2.2: Add in clause “3.2	Definitions” the following definition:

- Satellite Access Node: node providing NR user plane and control plane protocol terminations towards NTN capable UE, and connected via the NG interface to the 5GC. It encompass a transparent NTN payload on board a NTN platform, a gateway and gNB functions.

Question 2.2.2: Do companies agree with the above proposal or have further suggestions ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Thales | Agree |  |
| LG | Agree |  |
| Ericsson | Disagree | This is not needed in stage 2spec. SAN is not used in the stage 2, nor in RAN2 nor RAN3 specs. |
| Apple | Disagree | Similar view as Ericsson |

Proposal 2.2.3 Replace the figure B.4-1: NTN based NG-RAN in clause “B.4	Example implementation of Non-Terrestrial Networks” by

Question 2.2.3: Do companies agree with the above proposal or have further suggestions ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Thales | Agree |  |
| Qualcomm | Agree |  |
| LG | Agree |  |
| Ericsson | Strongly disagree | SAN is used by RAN4 for specific purposes, they can define it as they like. With this change, the gNB disappears completely from stage 2 NTN, which is of course totally _not_ acceptable. Further the Informative annex B.4 shall be aligned with the normative 16.14. This figure was very difficult to agree to in the first place. |
| Apple | Disagree | We don’t really understand how this clarifies stage-2 spec. |

[Rapporteur summary]:

3 companies agree while 2 other disagree on the basis that SAN is used by RAN4 for specific purposes, they can define it as they like.

This change is rejected.

## 2.3 Chapter 16.14.2	User Plane aspects

In [4]	R2-2205999, it is proposed to add Uplink time synchronization reference point definition as follow

Impact on timing aspects:

To accommodate the long propagation delays, several NR timings involving DL-UL timing interaction are enhanced by the support of two scheduling offsets: Koffset and kmac as illustrated in Figure 16.14.2-x:

-	Koffset is the RTT between the UE and the uplink time synchronization reference point (RP). It corresponds to the sum of the service link RTT and the common TA if indicated.

-	Kmac is the RTT between the RP and the gNB.

DL and UL are frame aligned at the uplink time synchronization reference point with an offset given by NTA,offset.

Figure 16.14.2-x Timing relationship parameters

The timing relationships that need to be modified for NTN using Koffset are summarized as follows:

Proposal 2.3: Add Uplink time synchronization reference point in clause “16.14.2	User Plane aspects” the following definition:

Question 2.3: Do companies agree with the above proposal or have further suggestions ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Thales | Disagree | Some companies (i.e. Oppo) had raised concerns about this figure in a previous email discussion [POST117-e][109][NTN] Stage 2 CR (Thales). |
| Ericsson | Disagree | The figure is incorrect. We see these problems with it: • Koffset shall be replaced by TTA • Commnn TA shall be Common TA • Move RP closer to GW and make clear that common TA is between NTN payload and RP • Remove Kmac • Remove “=Common TA+K_mac” Like this figure being discussed for IoT NTN: Further, the text needs to be updated. We propose: DL and UL are frame aligned at the uplink time synchronization reference point (RP) with an offset given by NTA,offset. To accommodate the long propagation delays in NTN, several timing relationships are enhanced by Common TA and two scheduling offsets: Koffset and Kmac illustrated in Figure 23.21.2.1-1: - Common TA is a configured offset that corresponds to half the the RTT between the RP and the NTN payload. - Koffset is a configured scheduling offset that approximately corresponds to the sum of the service link RTT and the common. - Kmac is a configured offset that approximately corresponds to the RTT between the RP and the eNB. |
| Sequans | Agree (with update discussed in Stage2 IOT NTN) | (Proponent) The point is to have the uplink time synchronization reference point (RP) clearly defined (as it is referred to in RRC spec). We used latest agreed IOT NTN stage 2 as a baseline but indeed it is not accurate. We agree with Ericsson corrections except for * eNB which should be gNB. * sum of the service link RTT and the common TA If there is no consensus on figure/details, we think the bare minimum would be to keep the sentence " DL and UL are frame aligned at the uplink time synchronization reference point (RP) with an offset given by NTA,offset." to understand what the uplink time synchronization reference point (RP) means. |
| Apple | Agree with Sequans update | It is good to align the figure with the one used for IoT NTN |
| Qualcomm | Agree with Sequans update |  |

[Rapporteur summary]:

A fair majority of companies are interested to add these clarifications and align with stage 2 text of IoT-NTN. However the proposed illustration by Sequans should be simplified since it bears some errors. The intent would be to illustrate the Reference Point.

A possible next text proposal will be discussed in the 2nd round.

# 2nd round discussion

## 3.1 Timing aspects in Chapter 16.14.2	User Plane aspects

Based on 1st round discussion, the following TP is proposed for agreement:

“Impact on timing aspects:

DL and UL are frame aligned at the uplink time synchronization reference point (RP) with an offset given by NTA,offset.

To accommodate the long propagation delays, several NR timing relationships involving DL-UL timing interaction are enhanced by the support of a Common TA and two scheduling offsets: Koffset and kmac. illustrated in Figure 16.14.2-x:

-    Common TA is a configured offset that corresponds to the RTT between the Reference Point (RP) and the NTN payload.

-    Koffset is a configured scheduling offset that approximately corresponds to the sum of the service link RTT and the common TA.

-    Kmac is a configured offset that approximately corresponds to the RTT between the RP and the gNB.

The timing relationships that need to be modified for NTN using Koffset are summarized as follows:

Figure 16.14.2-x Illustration of the Reference point

The timing relationships that need to be modified for NTN using Koffset are summarized as follows:”

Question 3.1: Do companies agree with the above text proposal (changes in red wrt existing TS 38.300 text)

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Intel | partially agree | In Figure 16.14.2-x, we suggest to also show Kmac between RP and gNB. If it’s a common understanding, we suggest to add description of RP, i.e., RP can be considered as a gateway in figure B.4-1. |
| Apple | Agree but | We need to distinguish between configured offsets (like K_offset) and physical quantities (like service link RTT) in the figure. Our suggestion is to just show the physical quantities since the text clarifies how these are related to configured offsets. |
| Ericsson | Agree | We think Kmac shall not be in the figure, as it is a RRN configured on a subframe level, thus the RTT between RP and gNB is most likely not equal to Kmac. We think the current figure does show the physical quantities, as the RP is exactly defined by the Common TA. For TTA you could argue that there is a small offsets between the sum of Service link RTT and TTA, but we think the figure is sufficient for stage 2. |
| Sequans | Agree |  |
| OPPO | Agree with comment | We also suggest to show K_mac between RP and gNB in figure. And before using TTA in the figure, it would be better to add some wording to introduce the meaning of TTA firstly. Koffset is a configured scheduling offset that approximately corresponds to TTA, i.e. the sum of the service link RTT and the common TA. |
| Huawei, HiSilicon | Agree |  |
| Nokia | Agree |  |

Moderator’s summary

All companies are in agreement with the text proposal.

However 2 companies out of 7 would be keen to add Kmac and/or Koffset in the picture

Moderator would recommend to just show the physical quantities and not illustrate Kmac nor Koffset.

The figure may be enhanced as follow:

## 3.2 Clean-up of stage 3 details in the stage2 text

As requested by the TS 38.300 rapporteur, we are invited to remove Stage-3 level details.

Therefore, all companies are invited to highlight (with comments in bubble) all stage 3 details in the “draft R2-2206202_NR-NTN Stg2 CR_v00.docx” (separate document).

## 3.3 EN’s resolving in chapter 16.14 of TS 38.300

Based on RRC related email discussion outcomes

Question 3.3.1: In Chapter 16.14.3.1	Mobility in RRC_IDLE and RRC_INACTIVE, how should the EN be resolved

Editor's note: Non-NTN capable UE can be prevented from accessing an NTN cell in Rel-17. FFS how.

| Company | Comments/Suggestions |
| --- | --- |
| Intel | According to the latest agreement made in offlne-111, i.e., “NTN UE follows the legacy cellBarred for TN access and consider the cell is not allowed for NTN access if cellBarredNTN is not present.”, Non-NTN capable UE can be prevented from accessing an NTN cell by legacy cellBarred indication. |
| Ericsson | Agree with intel suggestion to remove EN and add “Non-NTN capable UE can be prevented from accessing an NTN cell by legacy cellBarred indication”. |
| OPPO | Agree to remove this EN |
| Huawei, HiSilicon | Agree to just remove this EN. But there is no clear agreement about which IE is used to bar non-NTN UE yet before adding anything. |
| Nokia | Just remove this EN. The barring details will be covered in 38.304 anyway. |

Question 3.3.2: In Chapter 16.14.3.2.1	Handover, how should the EN be resolved ?

Editor's note: FFS details for NTN-TN hand-over impact on RRC procedure.

| Company | Comments/Suggestions |
| --- | --- |
| Intel | Since it’s more of the stage-3 details, we can remove this EN directly. |
| Apple | Agree with Intel |
| Ericsson | Agree w Intel. |
| OPPO | Agree with Intel. |
| Huawei, HiSilicon | Agree with Intel |
| Nokia | Remove EN and do not capture anything in Stage-2. |

Question 3.3.3: In Chapter 16.14.3.2.2	Conditional Handover, , how should the EN be resolved ?

Editor's note: FFS details for NTN-TN CHO impact on RRC procedure.

| Company | Comments/Suggestions |
| --- | --- |
| Intel | Since it’s more of the stage-3 details, we can remove this EN directly. |
| Apple | Agree with Intel |
| Ericsson | Agree w Intel. |
| OPPO | Agree with Intel. |
| Huawei, HiSilicon | Agree with Intel |
| Nokia | Same comment as to Q3.3.2. |

Moderator summary’s

All companies agree to remove the following ENs:

- “Editor's note: FFS details for NTN-TN hand-over impact on RRC procedure.” in Chapter 16.14.3.2.1

- “Editor's note: FFS details for NTN-TN CHO impact on RRC procedure.” in Chapter 16.14.3.2.2

As per the following EN in Chapter 16.14.3.1	Mobility in RRC_IDLE and RRC_INACTIVE “Editor's note: Non-NTN capable UE can be prevented from accessing an NTN cell in Rel-17. FFS how.“

Some companies suggests to simply remove it while some others propose to convert into the following normative text “Non-NTN capable UE can be prevented from accessing an NTN cell by legacy cellBarred indication.”

- Moderator suggests to simply remove this EN since as Nokia indicated, the barring details will be covered in 38.304 anyway.

# 3rd round discussion

On the basis of suggestions provided by companies on the stage 2 CR, the following topics are further discussed.

## 4.1 Chapter 16.14.2 User plane aspects

It is proposed to replace the existing text by the following text

### 16.14.2	Timing and Synchronization

#### 16.14.2.1	Scheduling Timing aspects

To accommodate the propagation delay, User Plane procedures are adapted as follow:

-	For downlink, HARQ feedback can be enabled or disabled per HARQ process;

-	For uplink, the UE can be configured with a HARQmodeA or HARQmodeB per HARQ process;

NOTE:	It is up to network implementation to ensure proper configuration of HARQ feedback (i.e. enabled or disabled) for HARQ processes used by an SPS configuration and of HARQmode for HARQ processes used by a CG configuration.

To accommodate the propagation delay, several NR timings involving DL-UL timing interaction are enhanced by the support of a Common Timing Advance and two scheduling offsets: Koffset and kmac. illustrated in Figure 16.14.2-x:

-    Common TA is a configured offset that corresponds to the RTT between the Reference Point (RP) and the NTN payload.

-    Koffset is a configured scheduling offset that approximately corresponds to the sum of the service link RTT and the common TA.

-    Kmac is a configured offset that approximately corresponds to the RTT between the RP and the gNB.

Figure 16.14.2-x Illustration of the Reference point

#### 16.14.2.2	Precompensation by the UE

For the serving cell, the network broadcast ephemeris information and common Timing Advance (common TA) parameters. The UE shall acquire its GNSS position as well as the satellite ephemeris and common TA before connecting to an NTN cell. To achieve synchronisation, before performing random access, the UE shall autonomously pre-compensate the Timing Advance (TTA), see Figure 16.14.2.2-1, as well as the frequency Doppler shift by considering the common TA, UE position and the satellite position through the satellite ephemeris.

In connected mode, the UE shall continuously update the Timing Advance and frequency pre-compensation. Timers ensure that the UE does not perform any transmissions due to outdated satellite ephemeris, common TA or GNSS position. In connected mode, upon outdated satellite ephemeris and common Timing Advance, the UE re-acquires the broadcasted parameters.

The UEs may be configured to report Timing Advance during Random Access procedures or in connected mode. In connected mode triggered reporting of the Timing Advance is supported.

Figure 16.14.2.2-1: Ilustration of Uplink/Downlink Radio Frame Timing at the UE

While the pre-compensation of the instantaneous Doppler shift experienced on the service link is to be performed by the UE, the management of Doppler shift experienced over the feeder link as well as any transponder frequency error whether it is introduced in Downlink or Uplink is left to the network implementation

Question 4.1.1: Do companies agree with the above text proposal for chapter 16.14.2 (complete change wrt existing TS 38.300 text)

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Xiaomi | No | The sentence “The UE shall acquire its GNSS position as well as the satellite ephemeris and common TA before connecting to an NTN cell.” Is misleading, as it may be misunderstood as UE needs to reacquire GNSS and ephemeris and common TA right before RRC connection setup. Suggest to change to “The UE shall ensure have valid GNSS position as well as the satellite ephemeris and common TA before connecting to an NTN cell.” |
| Huawei，HiSilicon | Agree with suggestions | 1/ “16.14.2.1 Scheduling and Timing aspects” 2/ In connected mode, event triggered reporting of the Timing Advance is supported. |
| Samsung | Agree with wording suggestions | Suggestion on wording with some stage-3 details removed: - For uplink, the UE can be configured with HARQmodeA or HARQmodeB per HARQ process; - To accommodate the propagation delay, several DL-UL timing relationships are enhanced 16.14.2.2 Precompensation by the UE For the serving cell, the network broadcast ephemeris information and common TA parameters. The UE shall have valid UE position as well as the satellite ephemeris and common TA before connecting to an NTN cell. To achieve synchronisation, before performing random access, the UE shall autonomously pre-compensate the TA (TTA), see Figure 16.14.2-x, as well as the frequency Doppler shift by considering the common TA, UE position and the satellite position through the satellite ephemeris. In connected mode, the UE shall continuously update the Timing Advance and frequency pre-compensation. The UEs may be configured to report Timing Advance during Random Access procedures or in connected mode. In connected mode event-triggered reporting of the Timing Advance is supported. While the pre-compensation of the instantaneous Doppler shift experienced on the service link is to be performed by the UE, the management of Doppler shift experienced over the feeder link as well as any transponder frequency error is left to the network implementation |
| Sequans | Agree with suggestion | We think it is good to keep the sentence " DL and UL are frame aligned at the uplink time synchronization reference point (RP) with an offset given by NTA,offset. " that was in earlier TP (maybe it is just a mistake above as the sentence is in the proposal in section 4) |
| Qualcomm |  | Agree with Sequans’ and Xiaomi’s suggestion. Can we please not change the agreed text from 2nd round. Please use track change or color code. |

Moderator’s summary

- Upon the collected views/suggestions, the TP for Chapter 16.14.2 is revised as follow

### 16.14.2	Timing and Synchronization

#### 16.14.2.1	Scheduling and Timing aspects

To accommodate the propagation delay, User Plane procedures are adapted as follow:

-	For downlink, HARQ feedback can be enabled or disabled per HARQ process;

-	For uplink, the UE can be configured with a HARQmodeA or HARQmodeB per HARQ process;

NOTE:	It is up to network implementation to ensure proper configuration of HARQ feedback (i.e. enabled or disabled) for HARQ processes used by an SPS configuration and of HARQmode for HARQ processes used by a CG configuration.

To accommodate the propagation delay, several NR timings involving DL-UL timing relationships interaction are enhanced by the support of a Common Timing Advance and two scheduling offsets: Koffset and kmac. illustrated in Figure 16.14.2-x:

-    Common TA is a configured offset that corresponds to the RTT between the Reference Point (RP) and the NTN payload.

-    Koffset is a configured scheduling offset that approximately corresponds to the sum of the service link RTT and the common TA.

-    Kmac is a configured offset that approximately corresponds to the RTT between the RP and the gNB.

DL and UL are frame aligned at the uplink time synchronization reference point (RP) with an offset given by NTA,offset.

Figure 16.14.2-x Illustration of the Reference point

#### 16.14.2.2	Precompensation by the UE

For the serving cell, the network broadcast ephemeris information and common Timing Advance (common TA) TA parameters. The UE shall have valid acquire its GNSS UE position as well as the satellite ephemeris and common TA before connecting to an NTN cell. To achieve synchronisation, before performing random access, the UE shall autonomously pre-compensate the TA Timing Advance (TTA), see Figure 16.14.2.x2-1, as well as the frequency Doppler shift by considering the common TA, UE position and the satellite position through the satellite ephemeris.

In connected mode, the UE shall continuously update the Timing Advance and frequency pre-compensation. Timers ensure that the UE does not perform any transmissions due to outdated satellite ephemeris, common TA or GNSS position. In connected mode, upon outdated satellite ephemeris and common Timing Advance, the UE re-acquires the broadcasted parameters.

The UEs may be configured to report Timing Advance during Random Access procedures or in connected mode. In connected mode, event-triggered reporting of the Timing Advance is supported.

Figure 16.14.2.x2-1: Ilustration of Uplink/Downlink Radio Frame Timing at the UE

While the pre-compensation of the instantaneous Doppler shift experienced on the service link is to be performed by the UE, the management of Doppler shift experienced over the feeder link as well as any transponder frequency error whether it is introduced in Downlink or Uplink is left to the network implementation

## 4.1bis Chapter 16.14.2 User plane aspects

As part of the email discussion [Offline-108][NTN] UE caps (Intel), the following proposal could not be discussed.

Proposal 2: RAN2 to discuss whether to add a clarification in a note in stage 2 spec, i.e., dish type UE (static “VSAT” type NTN capable UE without GNSS module but with GNSS coordinates) can also be supported in Rel-17.

It is therefore propose to continue the discussion in this thread

Question 4.1.2: Do companies agree to add a clarification note in stage 2 spec (e.g. in clause 16.14.2) ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Xiaomi | Agree |  |
| Huawei，HiSilicon | Disagree | This is not in R17 scope. We prefer not to discuss this at this late stage. |
| MediaTek | Disagree | Out of scope in R-17 and should NOT be included in a Work-Item that has been already declared “closed”. |
| Samsung | Disagree | Maybe a note is not needed, just leave to UE implementation. |
| Qualcomm | Agree |  |

The text of the clarification note could be

“Note: static “VSAT” type NTN capable UE is expected to acquire its GNSS coordinates either with GNSS receiver or through configuration”

Question 4.1.3: Do companies agree with the wording of the above note or have different suggestion ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Xiaomi | Agree |  |
| Huawei，HiSilicon | Disagree | See above answer. |
| MediaTek | Disagree | Out of scope in R-17 and should NOT be included in a Work-Item that has been already declared “closed”. |
| Samsung | Disagree | If a note is really needed, we suggest “Note: static “VSAT” type NTN capable UE is expected to acquire its GNSS coordinates either with GNSS receiver or through configuration.” |
| Qualcomm | Agree | Samsung’s suggestion looks fine. |

Moderator’s summary

Given balanced views, the proposed clarification note should be discussed on line

Proposal: RAN2 to discuss on line the addition of a clarification note in clause 16.14.2 of stage 2

“Note: static “VSAT” type NTN capable UE is expected to acquire its GNSS coordinates either with GNSS receiver or through configuration”

## 4.2 Chapter 16.14.3.1	Mobility in RRC_IDLE and RRC_INACTIVE

The following text proposal is submitted for discussion by LGE

The same principles as described in 9.2.1 apply to mobility in RRC_IDLE for NTN and the same principles as described in 9.2.2 apply to mobility in RRC_INACTIVE for NTN unless hereunder specified.

The network may broadcast multiple Tracking Area Codes (TAC) per PLMN in a NR NTN cell. A TAC change in the System Information is under network control, i.e. it may not be exactly synchronised with real-time illumination of beams on ground.

The UE can determine the network type (Terrestrial or non-terrestrial) implicitly by the existence of scheduling information of SIB19 in SIB1.

The NTN ephemeris is divided into serving cell's satellite ephemeris and neighbouring cell's satellite ephemeris.

At least in the quasi-earth fixed cell scenario, UE can perform time-based and location-based cell selection /reselection.:

-	The timing and location information associated to a cell are provided via system information;

-	Timing information refers to the time when the serving cell is going to stop serving a geographical area;

-	Location information refers to the reference location of serving or neighbouring cells.

Location information may be used to assist cell reselection in NTN with for example a condition based on the distance between UE and the reference location of the serving cell and/or neighbour cells.

UE may support mobility between radio access technologies based on different orbit (GSO, NGSO at different altitude).

Question 4.2.1: Do companies agree with the above text proposal (changes in red wrt existing TS 38.300 text)

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Xiaomi | Disagree | It is not about the red text part, but regarding to the sentence “The UE can determine the network type (Terrestrial or non-terrestrial) implicitly by the existence of scheduling information of SIB19 in SIB1.” We think it may not be correct, as for HAPS, network may not broadcast SIB19. As we have agreed that “consider the cell is not allowed for NTN access if cellBarredNTN is not present”, thus we suggest to change to: “The UE can determine the network type (Terrestrial or non-terrestrial) implicitly by the existence of cellBarredNTN in SIB1.” |
| Huawei，HiSilicon | Agree |  |
| MediaTek | Agree |  |
| Samsung | Agree with wording suggestion | Location-based cell reselection should be removed as RAN2 just agreed not to support in Rel-17. Last sentence is confusing, does it mean between TN and NTN or between GSO and NGSO, if the latter isn’t it same RAT? Suggestion on wording: The NTN ephemeris includes serving cell's satellite ephemeris and neighbouring cell's satellite ephemeris. In the quasi-earth fixed cell scenario, UE can perform time-based and location-based cell selection /reselection. |
| Qualcomm | Partly agree | Ok with suggestion from Xiaomi. Its cleaner to use cellBarredNTN in SIB1 to determine NTN cell. |

Question 4.2.2: Do companies agree to delete the time-based cell selection/reselection since RAN2 has agreed that the service stop time of the quasi-earth fixed cell is not applied to cell ranking in determining the target cell for reselection at RAN2#116bis-e ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Xiaomi | Agree |  |
| Huawei，HiSilicon | Agree |  |
| MediaTek | Disagree | We think RAN2 has agreed to use “Time” to trigger the cell search for earth-fixed cells. Hence, even if service stop time of the quasi-earth fixed cell is not applied to cell ranking, it can still be used for triggering the cell-searching. |
| Samsung | Agree | Keep the part of service stop time triggering measurement |

Question 4.2.3: Do companies agree to delete location-based cell reselection ?

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Xiaomi | Agree |  |
| Huawei，HiSilicon | Agree |  |
| MediaTek | Agree |  |
| Samsung | Agree |  |

Moderator’s summary

- Upon the collected views/suggestions collected above, the TP for Chapter 16.14.3.1 is revised as follow

The same principles as described in 9.2.1 apply to mobility in RRC_IDLE for NTN and the same principles as described in 9.2.2 apply to mobility in RRC_INACTIVE for NTN unless hereunder specified.

The network may broadcast multiple Tracking Area Codes (TAC) per PLMN in a NR NTN cell. A TAC change in the System Information is under network control, i.e. it may not be exactly synchronised with real-time illumination of beams on ground.

The UE can determine the network type (Terrestrial or non-terrestrial) implicitly by the existence of cellBarredNTN scheduling information of SIB19 in SIB1.

The NTN ephemeris includes is divided into serving cell's satellite ephemeris and neighbouring cell's satellite ephemeris.

At least iIn the quasi-earth fixed cell scenario, UE can perform time-based and location-based cell selection /reselection:

-	The timing and location information associated to a cell are provided via system information;

-	Timing information refers to the time when the serving cell is going to stop serving a geographical area;

-	Location information refers to the reference location of serving or neighbouring cells.

Location information may be used to assist cell reselection in NTN with for example a condition based on the distance between UE and the reference location of the serving cell and/or neighbour cells.

UE may support mobility between radio access technologies based on different orbit (GSO, NGSO at different altitude).

## 4.3 Chapter 16.14.3.3 Measurements

The following text proposal is submitted for discussion by LGE

The same principle as described in 9.2.4 applies to measurements in NTN unless hereunder specified.

The network can configure:

-	multiple SMTCs in parallel per carrier and for a given set of cells depending on UE capabilities using propagation delay difference, feeder link delay as well as serving/neighbour satellite cell ephemeris;

-	measurement gaps using the same propagation delay difference as computed for SMTC.

The adjustment of SMTCs is possible under network control for connected mode and under UE control based on UE location information and ephemeris for idle/inactive modes.

At least in the quasi-earth fixed cell scenario, UE can perform time-based and location-based measurement in RRC_IDLE/RRC_INACTIVE.

-	The timing and location information associated to a cell are provided via system information;

-	Timing information refers to the time when the serving cell is going to stop serving a geographical area;

-	Location information refers to the reference location of serving cell.

Timing information is used to force the UE to strat the neighbor cell measurement before the serving cell stop serving a geographical area.

Location information may be used to relax the neighbor cell measurement under certain conditions specified in section 5.2.4.2 in 38.304.

Question 4.3: Do companies agree with the above text proposal (changes in red wrt existing TS 38.300 text)

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Xiaomi | Agree with comments | For how to use the timing information and location information, RAN2 didn’t discuss he ‘relax neighor cell measurement’, we suggest delete the following sentences and generally speak ”Measurement rules for cell re-selection with timing information and location information is specified in section 5.2.4.2 in 38.304.” Timing information is used to force the UE to strat the neighbor cell measurement before the serving cell stop serving a geographical area. Location information may be used to relax the neighbor cell measurement under certain conditions specified in section 5.2.4.2 in 38.304. Measurement rules for cell re-selection with timing information and location information is specified in section 5.2.4.2 in 38.304 |
| Huawei，HiSilicon | Agree | Agree with Xiaomi |
| MediaTek | Agree with comments | Agree with Xiaomi |
| Samsung | Agree with wording suggestion | Agree with Xiaomi, in addition The network can configure: - multiple SMTCs in parallel per carrier and for a given set of cells depending on UE capabilities using propagation delay difference and ephemeris information; - measurement gaps based on multiple SMTC. The adjustment of SMTCs is possible under network control for connected mode and under UE control for idle/inactive modes. In the quasi-earth fixed cell scenario, UE can perform time-based and location-based measurement in RRC_IDLE/RRC_INACTIVE. - The timing and location information associated to a cell are provided via system information; - Timing information refers to the time when the serving cell is going to stop serving a geographical area; - Location information refers to the reference location of the serving cell. |
| Qualcomm |  | Agree wih Xiaomi. |

Moderator’s summary

- Upon the collected views/suggestions, the TP for Chapter 16.14.3.3 is revised as follow

The same principle as described in 9.2.4 applies to measurements in NTN unless hereunder specified.

The network can configure:

-	multiple SMTCs in parallel per carrier and for a given set of cells depending on UE capabilities using propagation delay difference, feeder link delay as well as serving/neighbour satellite cell and ephemeris information;

-	measurement gaps using the same propagation delay difference as computed for based on multiple SMTC.

The adjustment of SMTCs is possible under network control for connected mode and under UE control based on UE location information and ephemeris for idle/inactive modes.

At least in the quasi-earth fixed cell scenario, UE can perform time-based and location-based measurement in RRC_IDLE/RRC_INACTIVE.

-	The timing and location information associated to a cell are provided via system information;

-	Timing information refers to the time when the serving cell is going to stop serving a geographical area;

-	Location information refers to the reference location of the serving cell.

Timing information is used to force the UE to strat the neighbor cell measurement before the serving cell stop serving a geographical area.

Location information may be used to relax the neighbor cell measurement under certain conditions specified in section 5.2.4.2 in 38.304.

Measurement rules for cell re-selection with timing information and location information is specified in section 5.2.4.2 in 38.304

## 4.4 Chapter 16.14.8 UE location aspects

During the on line session, it has been agreed that

“Upon network request, after AS security in connected mode is established, a UE can report its coarse UE location information (coarse GNSS coordinates) to the NG-RAN with a possible reported value referring to “no coarse GNSS location available” (which the UE can set if it cannot/does not want to provide its coarse GNSS coordinates). RAN2 puts this in the specs in this meeting. If/when SA3 will provide feedback that this is not possible, this above text would be removed. RAN2 will consider any other feedback from SA3 and adapt accordingly.”

A TP is therefore proposed to resolve the current EN of chapter 16.14.8 as follow:

Editor's note: Upon network request, after AS security in connected mode is established, a UE can report coarse UE location information (X most Significant Bits of its GNSS coordinates with accuracy around 2km level) to the NG-RAN without receiving any prior explicit user consent. if "user consent" is available at the UE, the UE will report the coarse UE location information. Else, the UE will respond "no coarse GNSS location available". Periodic location reporting can be configured by gNB to obtain UE location update of mobile UEs in RRC_CONNECTED. This proposed text may be updated upon SA3 feedback.

Upon network request, after AS security in connected mode is established, a UE can report its coarse UE location information (coarse GNSS coordinates) to the NG-RAN with a possible reported value referring to “no coarse GNSS location available” (which the UE can set if it cannot/does not want to provide its coarse GNSS coordinates).

Editor's note: If/when SA3 will provide feedback that this is not possible, this above text would be removed. RAN2 will consider any other feedback from SA3 and adapt accordingly.

Question 4.4: Do companies agree to the TP above for the Chapter 16.14.8 UE location aspects

| Company | Agree/Disagree | Comments/Suggestions |
| --- | --- | --- |
| Xiaomi | Agree with comment | First, we should mention that the accuracy is around 2km as indicated in the LS to SA3. Second, currently the signalling details of coarse location report are not clear, if UE doesn’t want to report coarse location or it is not available, instead of explicitly indication of “no coarse GNSS location available”, implicit indication can also work by “UE doesn’t report any GNSS location”. Thus, we suggest to change to: Upon network request, after AS security in connected mode is established, a UE can report its coarse UE location information (coarse GNSS coordinates with accuracy around 2km) to the NG-RAN, or provide an explicit/implicit indication of “no coarse GNSS location available” if UE cannot/does not want to provide its coarse GNSS coordinates. |
| Huawei，HiSilicon | Agree with comment | Fine with Xiaomi’s revision. Besides, we prefer to make it a little more open for the potential text update via the following change: Editor's note: If/when SA3 will provide feedback that this is not recommended possible, this above text would be removed. Otherwise, RAN2 will can further update the above text, e.g. considering any other feedback from SA3 and adapt accordingly. |
| MediaTek | Agree with comments | Fine with Huawei’s suggestions. |
| Samsung | Agree with comments | Agree with Huawei’s comment |
| Qualcomm | Agree with comments | Agree with Xiaomi and Huawei. In addition, it is not clear how to define “UE does not want”?, We think “if the UE cannot” is sufficient. We suggest Upon network request, after AS security in connected mode is established, a UE can report its coarse UE location information (coarse GNSS coordinates) to the NG-RAN with a possible reported value referring to “no coarse GNSS location available” (which the UE can set if it cannot/does not want to provide its coarse GNSS coordinates). |

Moderator’s summary

- Upon the collected views/suggestions, the TP for Chapter 16.14.8 is revised as follow

Editor's note: Upon network request, after AS security in connected mode is established, a UE can report coarse UE location information (X most Significant Bits of its GNSS coordinates with accuracy around 2km level) to the NG-RAN without receiving any prior explicit user consent. if "user consent" is available at the UE, the UE will report the coarse UE location information. Else, the UE will respond "no coarse GNSS location available". Periodic location reporting can be configured by gNB to obtain UE location update of mobile UEs in RRC_CONNECTED. This proposed text may be updated upon SA3 feedback.

Upon network request, after AS security in connected mode is established, a UE can report its coarse UE location information (coarse GNSS coordinates with accuracy around 2km) to the NG-RAN with a possible reported value referring to “no coarse GNSS location available” (which the UE can set if it cannot/does not want to provide its coarse GNSS coordinates).

Editor's note: If/when SA3 will provide feedback that this is not recommended, this above text would be removed. Otherwise RAN2 will can further update the above text, e.g. considering any other feedback from SA3 and adapt accordingly.

# 4. Summary and Proposals

Proposal 1: Remove the following ENs in TS 38.300:

- “Editor's note: FFS details for NTN-TN hand-over impact on RRC procedure.” in Chapter 16.14.3.2.1

- “Editor's note: FFS details for NTN-TN CHO impact on RRC procedure.” in Chapter 16.14.3.2.2

- “Editor's note: Non-NTN capable UE can be prevented from accessing an NTN cell in Rel-17. FFS how.“ in chapter 16.14.3.1

Proposal 2: Repla	ce all text in chapter 16.14.2 “User Plane aspects” of TS 38.300 by the following (red corrections wrt agreed text in 2nd round)

### 16.14.2	Timing and Synchronization

#### 16.14.2.1	Scheduling and Timing aspects

To accommodate the propagation delay, User Plane procedures are adapted as follow:

-	For downlink, HARQ feedback can be enabled or disabled per HARQ process;

-	For uplink, the UE can be configured with a HARQmodeA or HARQmodeB per HARQ process;

NOTE:	It is up to network implementation to ensure proper configuration of HARQ feedback (i.e. enabled or disabled) for HARQ processes used by an SPS configuration and of HARQmode for HARQ processes used by a CG configuration.

To accommodate the propagation delay, several NR timings involving DL-UL timing relationships interaction are enhanced by the support of a Common Timing Advance and two scheduling offsets: Koffset and kmac. illustrated in Figure 16.14.2.1-x:

-    Common TA is a configured offset that corresponds to the RTT between the Reference Point (RP) and the NTN payload.

-    Koffset is a configured scheduling offset that approximately corresponds to the sum of the service link RTT and the common TA.

-    Kmac is a configured offset that approximately corresponds to the RTT between the RP and the gNB.

DL and UL are frame aligned at the uplink time synchronization reference point (RP) with an offset given by NTA,offset.

Figure 16.14.2.1-x Illustration of the Reference point

#### 16.14.2.2	Precompensation by the UE

For the serving cell, the network broadcast ephemeris information and common TA parameters. The UE shall have valid UE position as well as the satellite ephemeris and common TA before connecting to an NTN cell. To achieve synchronisation, before performing random access, the UE shall autonomously pre-compensate the TA (TTA), see Figure 16.14.2.x, as well as the frequency Doppler shift by considering the common TA, UE position and the satellite position through the satellite ephemeris.

In connected mode, the UE shall continuously update the Timing Advance and frequency pre-compensation.

The UEs may be configured to report Timing Advance during Random Access procedures or in connected mode. In connected mode, event-triggered reporting of the Timing Advance is supported.

Figure 16.14.2.2.x: Ilustration of Uplink/Downlink Radio Frame Timing at the UE

While the pre-compensation of the instantaneous Doppler shift experienced on the service link is to be performed by the UE, the management of Doppler shift experienced over the feeder link as well as any transponder frequency error is left to the network implementation

Proposal 3: Change text in chapter 16.14.3.1 of TS 38.300 as follow

“The same principles as described in 9.2.1 apply to mobility in RRC_IDLE for NTN and the same principles as described in 9.2.2 apply to mobility in RRC_INACTIVE for NTN unless hereunder specified.

The network may broadcast multiple Tracking Area Codes (TAC) per PLMN in a NR NTN cell. A TAC change in the System Information is under network control, i.e. it may not be exactly synchronised with real-time illumination of beams on ground.

The UE can determine the network type (Terrestrial or non-terrestrial) implicitly by the existence of cellBarredNTN scheduling information of SIB19 in SIB1.

The NTN ephemeris includes is divided into serving cell's satellite ephemeris and neighbouring cell's satellite ephemeris.

At least iIn the quasi-earth fixed cell scenario, UE can perform time-based and location-based cell selection /reselection.:

-	The timing and location information associated to a cell are provided via system information;

-	Timing information refers to the time when the serving cell is going to stop serving a geographical area;

-	Location information refers to the reference location of serving or neighbouring cells.

Location information may be used to assist cell reselection in NTN with for example a condition based on the distance between UE and the reference location of the serving cell and/or neighbour cells.

UE may support mobility between radio access technologies based on different orbit (GSO, NGSO at different altitude).”

Proposal 4: Change text in chapter 16.14.3.3 of TS 38.300 as follow

The same principle as described in 9.2.4 applies to measurements in NTN unless hereunder specified.

The network can configure:

-	multiple SMTCs in parallel per carrier and for a given set of cells depending on UE capabilities using propagation delay difference, feeder link delay as well as serving/neighbour satellite cell and ephemeris information;

-	measurement gaps using the same propagation delay difference as computed for based on multiple SMTC.

The adjustment of SMTCs is possible under network control for connected mode and under UE control based on UE location information and ephemeris for idle/inactive modes.

At least in the quasi-earth fixed cell scenario, UE can perform time-based and location-based measurement in RRC_IDLE/RRC_INACTIVE.

-	The timing and location information associated to a cell are provided via system information;

-	Timing information refers to the time when the serving cell is going to stop serving a geographical area;

-	Location information refers to the reference location of the serving cell.

Timing information is used to force the UE to strat the neighbor cell measurement before the serving cell stop serving a geographical area.

Location information may be used to relax the neighbor cell measurement under certain conditions specified in section 5.2.4.2 in 38.304.

Measurement rules for cell re-selection with timing information and location information is specified in section 5.2.4.2 in 38.304

Proposal 5: Change text in chapter 16.14.8 of TS 38.300 as follow

Editor's note: Upon network request, after AS security in connected mode is established, a UE can report coarse UE location information (X most Significant Bits of its GNSS coordinates with accuracy around 2km level) to the NG-RAN without receiving any prior explicit user consent. if "user consent" is available at the UE, the UE will report the coarse UE location information. Else, the UE will respond "no coarse GNSS location available". Periodic location reporting can be configured by gNB to obtain UE location update of mobile UEs in RRC_CONNECTED. This proposed text may be updated upon SA3 feedback.

Upon network request, after AS security in connected mode is established, a UE can report its coarse UE location information (coarse GNSS coordinates with accuracy around 2km) to the NG-RAN with a possible reported value referring to “no coarse GNSS location available” (which the UE can set if it cannot/does not want to provide its coarse GNSS coordinates).

Editor's note: If/when SA3 will provide feedback that this is not recommended, this above text would be removed. Otherwise RAN2 can further update the above text, e.g. considering any other feedback from SA3.

Proposal 6: P1, P2, P3, P4, P5 are captured in the draft CR 38.300 in [R2-2206202] which is submitted for approval

Proposal 7: RAN2 to discuss on line the addition of a clarification note in clause 16.14.2 of stage 2

“Note: static “VSAT” type NTN capable UE is expected to acquire its GNSS coordinates either with GNSS receiver or through configuration”

# 5. References

- 3GPP TS 38.300 “NR; NR and NG-RAN Overall description; Stage-2”, v17.0.0

- R2-2204627	Support of UE location in Non-Terrestrial Networks	THALES

- R2-2204628	SAN for NTN based NG-RAN	THALES

- R2-2205999	Correction to NR NTN epoch time definition	Sequans Communications

# Contact information

| Company | Delegate contact |
| --- | --- |
| COMPANY_NAME | NAME (email@address.com) |
| Thales | nicolas.chuberre@thalesaleniaspace.com |
| Apple | Pavan.nuggehalli@apple.com |
| Intel | xun.tang@intel.com |
| Sequans | omarco@sequans.com |
| OPPO | lihaitao@oppo.com |