3GPP TSG RAN WG2#118-e	R2-2206813

e-Meeting, 9th - 20th May, 2022

3GPP TSG-SA3 Meeting #107-e 	S3-221268

e-meeting, 16 - 20 May 2022

Title:	LS reply on Reply LS on NTN specific User Consent and UE location in connected mode in NTN

Response to:	LS S3-220661/R2-2201754 on Reply LS on NTN specific User Consent and LS S3-220665/R2-2204257 on LS on UE location in connected mode in NTN from RAN2

Release:	Rel-17

Source:	SA3

To:	RAN2

Cc:	SA2, RAN3, CT1, CT4

Contact person:	Vlasios Tsiatsis

vlasios dot tsiatsis at ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

SA3 would like to thank RAN2 for the following two LS:

-	Reply LS on NTN specific User Consent (S3-220661/R2-2201754)

-	LS on UE location in connected mode in NTN (S3-220665/R2-2204257)

SA3 would like to respond to both these LSes with this single reply since they both include questions related to the topic of NTN specific User Consent.

The RAN2 LS R2-2201754 states the following:

RAN2 would like to thank SA3 for their reply LS. RAN2 would like to request SA3 to further work on introducing mechanism for user consent handling that can be used specifically for the NTN use case. RAN2 would like to ask whether it is possible to have NTN-specific user consent (e.g., based on subscription), in Rel-17.

The RAN2 LS R2-2204257 states the following:

RAN2 assumes a normative solution for user consent framework of TS 33.501 to specifically cover NTN will not be agreed upon in Rel-17 by SA3. In case other methods of obtaining user consent (e.g., subscription based) at NG-RAN would not be possible in Rel-17, RAN2 is then considering the solution where, upon network request, after AS security in connected mode is established, a UE can report a coarse UE location information (X most Significant Bits of its GNSS coordinates with accuracy around 2km level) to the NG-RAN without a new explicit "NTN specific" user consent.

Specifically for Rel-17, RAN2 is considering the implicit user consent approach where:

- in connected mode, the network can request the UE to provide its coarse GNSS coordinates without receiving any prior user consent

- then, if "user consent" is available at the UE, the UE will report the information (implicitly giving the consent). If it's not present, the UE will respond that "no coarse GNSS location available" (implicitly refusing the consent).

SA3 would like to observe the following:

1)	SA3 will study potential solutions for User Consent for the NTN use case in Rel-18. For Rel-17 SA3 would like to remind RAN2 that whether user consent is required would depend on local regulations. For Rel-17, in regions where user consent is required for NTN, SA3 recommends that the user consent requirement be met via provisional means, e.g. per gNB/NTN-GW configuration (consent granted for all UEs subscribing for NTN) based on the service-level agreement between the operator and its NTN subscribers.

2)	With respect to the coarse-grained GNSS location reporting by the UE with an accuracy of 2km without explicit consent, it is up to local regulations whether it is acceptable or not. In case explicit user consent is required by local regulation it is up to proprietary mechanisms for Rel-17.

3)	With respect to the implicit user consent approach which is considered by RAN2 LS R2-2204257 for Rel-17, SA3 would like to suggest that it shall be the network that decides whether there is a user consent for the aforementioned location request (based on subscription-based means or proprietary mechanisms) and not the UE. In other words, the network should request for the location if there is user consent (based on subscription-based or proprietary mechanisms) and the network should not request for the location if there is no user consent. The UE should provide the location information (if available) if the network requests for it.

SA3 is kindly requesting RAN2 to take the above information into account.

# 2	Actions

To RAN2

ACTION: 	SA3 is kindly requesting RAN2 to take the above information into account.

# 3	Dates of next TSG SA WG 3 meetings

SA3#107e-Bis	27 June - 1 July 2022	electronic meeting

SA3#108e	22 -26 August 2022	Goteborg, Sweden