3GPP TSG-RAN WG2 Meeting # 118-e 	R2-2206841

# Electronic, May 9th – 20th 2022                                                    Revision of R2-2206444

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0349 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NMEA GGA sentence info in high accuracy GNSS location estimates [HA-GNSS-NMEA] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ESA, Ericsson, Deutsche Telekom, T-Mobile USA, Swift Navigation, Hexagon, MediaTek Inc., u-blox |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2022-05-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Support for high accuracy GNSS was introduced in Rel 15 and has been enhanced in Rel 16 and 17. It was leveraged by NTRIP/RTCM distribution and representation, where NMEA GGA sentences are used to report high accuracy GNSS performance. The parts representing the location estimate and its uncertainty have already been represented in 3GPP LPP, but some parts are not yet represented – number of satellites used, dilution of precision, GNSS positioning fix quality indicator and age of used assistance data for HA GNSS – all attributes that are relevant for high accuracy GNSS devices to report to assess performance. In cases when it is legitimate for LMF to obtain position estimates based on high accuracy GNSS from the device, it is typically as part of an offered service in a use case. With these additional fields from the de facto standard NMEA GGA, the LMF is more precise in analyzing the provided positioning service. DOP provides information about the geometry of the positioning problem in terms of how the used satellites are lined up, and it is highly relevant to analyze the provided positioning performance accordingly. If the position estimates that are self-assessed as less accurate by the device also are provided with a poor DOP, then the underlying reason for poor performance is better understood. Same thing if the number of used satellites is low. It is also possible for the operator to analyze the overall situation in a region based on crowd sourced data to identify parts of a service area where high accuracy GNSS is not alone providing sufficient performance, and thereby indicate a part of the service area where outdoor 5G positioning build-out could be considered. The ability to maintain an integer or floating point ambiguity solution to the carrier measurements is also seen as an important quality and performance assessment that is an important and de factor standard performance metric |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding additional location source alternatives to CommonIEsProvideLocationInformation IE to represent the positioning fix quality indicator. Adding HA-GNSS-Metric IE to the GNSS-LocationInformation IE to represent the remaining fields of the NMEA GGA sentence - number of satellite vehicles used - dilution of precision - age of the most recent used assistance data for HA GNSS |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The GNSS industry standard performance metrics in terms to NMEA GGA information cannot be reported – parameters that are important for LMF to disclose the positioning performance with the provided A-GNSS assistance data |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.4.2, 6.5.2.5, 6.5.2.6, 6.5.2.7, 6.5.2.8, 6.5.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

#### […]

# 2	References

#### […]

[30]	RTCM Standard 10403.3: "Differential GNSS (Global Navigation Satellite Systems) Services" – Version 3, October 7, 2016.

[31]	IGS ANTEX: "The Antenna Exchanged Format" – version 1.4, September 15, 2010.

[32]	Federal Information Processing Standards Publication 197, "Specification for the ADVANCED ENCRYPTION STANDARD (AES)", November 26, 2001.

[33]	NIST Special Publication 800-38A, "Recommendation for Block Cipher Modes of Operation Methods and Techniques", 2001.

[34]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[35]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[36]	3GPP TS 38.215: "NR; Physical layer measurements".

[37]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[38]	IRNSS Signal-In-Space (SPS) Interface Control Document (ICD) for standard positioning service version 1.1, Aug 2017.

[39]	BDS-SIS-ICD-B1C-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1C (Version 1.0)", December, 2017.

[40]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[41]	3GPP TS 38.211: "3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Physical channels and modulation".

[42]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[43]	IS-QZSS-L6-001, Quasi-Zenith Satellite System Interface Specification – Centimetre Level Augmentation Service, Cabinet Office, November 5, 2018.

[44]	3GPP TR 38.901: "Technical Specification Group Radio Access Network; Study on channel model for frequencies from 0.5 to 100 GHz".

[45]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[46]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[47]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[48]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[49]	BDS-SIS-ICD-B2a-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B2a (Version 1.0)", December, 2017.

[50]	BDS-SIS-ICD-B3I-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B3I (Version 1.0)", December, 2017.

[xx]	NMEA standard 0183, Version 4.11, November 2018.

#### […]

### 6.4.2	Common Positioning

#### […]

#### –	CommonIEsProvideLocationInformation

The CommonIEsProvideLocationInformation carries common IEs for a Provide Location Information LPP message Type.

-- ASN1START

CommonIEsProvideLocationInformation ::= SEQUENCE {

locationEstimate			LocationCoordinates		OPTIONAL,

velocityEstimate			Velocity				OPTIONAL,

locationError				LocationError			OPTIONAL,

...,

[[	earlyFixReport-r12		EarlyFixReport-r12		OPTIONAL

]],

[[	locationSource-r13		LocationSource-r13		OPTIONAL,

locationTimestamp-r13	UTCTime					OPTIONAL

]],

[[

segmentationInfo-r14	SegmentationInfo-r14	OPTIONAL		-- Cond Segmentation

]],

[[

integrityInfo-r17		IntegrityInfo-r17		OPTIONAL

]]

}

LocationCoordinates ::= CHOICE {

ellipsoidPoint								Ellipsoid-Point,

ellipsoidPointWithUncertaintyCircle			Ellipsoid-PointWithUncertaintyCircle,

ellipsoidPointWithUncertaintyEllipse		EllipsoidPointWithUncertaintyEllipse,

polygon										Polygon,

ellipsoidPointWithAltitude					EllipsoidPointWithAltitude,

ellipsoidPointWithAltitudeAndUncertaintyEllipsoid

EllipsoidPointWithAltitudeAndUncertaintyEllipsoid,

ellipsoidArc								EllipsoidArc,

...,

highAccuracyEllipsoidPointWithUncertaintyEllipse-v1510

HighAccuracyEllipsoidPointWithUncertaintyEllipse-r15,

highAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-v1510

HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15,

ha-EllipsoidPointWithScalableUncertaintyEllipse-v1680																				HA-EllipsoidPointWithScalableUncertaintyEllipse-r16,

ha-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid-v1680

HA-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid-r16

}

Velocity ::= CHOICE {

horizontalVelocity							HorizontalVelocity,

horizontalWithVerticalVelocity				HorizontalWithVerticalVelocity,

horizontalVelocityWithUncertainty			HorizontalVelocityWithUncertainty,

horizontalWithVerticalVelocityAndUncertainty

HorizontalWithVerticalVelocityAndUncertainty,

...

}

LocationError ::= SEQUENCE {

locationfailurecause			LocationFailureCause,

...

}

LocationFailureCause ::= ENUMERATED {

undefined,

requestedMethodNotSupported,

positionMethodFailure,

periodicLocationMeasurementsNotAvailable,

...

}

EarlyFixReport-r12 ::= ENUMERATED {

noMoreMessages,

moreMessagesOnTheWay

}

LocationSource-r13 ::= BIT STRING {	a-gnss				(0),

wlan				(1),

bt					(2),

tbs					(3),

sensor				(4),

ha-gnss-v1510		(5),

motion-sensor-v1550 (6),

dl-tdoa-r16 		(7),

dl-aod-r16			(8) } (SIZE(1..16))

IntegrityInfo-r17 ::= SEQUENCE {

horizontalProtectionLevel-r17		INTEGER (0..50000),

verticalProtectionLevel-r17			INTEGER (0..50000)				OPTIONAL,

achievableTargetIntegrityRisk-r17	INTEGER (10..90)				OPTIONAL,	-- Need OP

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Segmentation | This field is optionally present, need OP, if lpp-message-segmentation-req has been received from the location server with bit 1 (targetToServer) set to value 1. The field shall be omitted if lpp-message-segmentation-req has not been received in this location session, or has been received with bit 1 (targetToServer) set to value 0. |

CommonIEsProvideLocationInformation field descriptions

locationEstimate This field provides a location estimate using one of the geographic shapes defined in TS 23.032 [15]. Coding of the values of the various fields internal to each geographic shape follow the rules in TS 23.032 [15]. The conditions for including this field are defined for the locationInformationType field in a Request Location Information message.

velocityEstimate This field provides a velocity estimate using one of the velocity shapes defined in TS 23.032 [15]. Coding of the values of the various fields internal to each velocity shape follow the rules in TS 23.032 [15].

locationError This field shall be included if and only if a location estimate and measurements are not included in the LPP PDU. The field includes information concerning the reason for the lack of location information. The LocationFailureCause 'periodicLocationMeasurementsNotAvailable' shall be used by the target device if periodic location reporting was requested, but no measurements or location estimate are available when the reportingInterval expired.

earlyFixReport This field shall be included if and only if the ProvideLocationInformation message contains early location measurements or an early location estimate. The target device shall set the values of this field as follows: - noMoreMessages: This is the only or last ProvideLocationInformation message used to deliver the entire set of early location information. - moreMessagesOnTheWay: This is one of multiple ProvideLocationInformation messages used to deliver the entire set of early location information (if early location information will not fit into a single message). If this field is included, the IE SegmentationInfo shall not be included.

locationSource This field provides the source positioning technology for the location estimate. NOTE 1: In this version of the specification, the entry 'tbs' is used only for TBS positioning based on MBS signals. NOTE 2: The entry 'sensor' is used only for positioning technology that uses barometric pressure sensor. The entry 'motion-sensor' is used for positioning technology that uses sensor(s) to detect displacement and movement, e.g. accelerometers, gyros, magnetometers.

locationTimestamp This field provides the UTC time when the location estimate is valid and should take the form of YYMMDDhhmmssZ.

segmentationInfo This field indicates whether this ProvideLocationInformation message is one of many segments, as specified in clause 4.3.5

integrityInfo This field provides the integrity result for the locationEstimate. - horizontalProtectionLevel provides the Horizontal Protection Level (HPL) for the locationEstimate along the semi-major axis of the error ellipse. Scale factor 0.01 metre; range 0 – 500 metres. - verticalProtectionLevel provides the Vertical Protection Level (VPL) for the locationEstimate. Scale factor 0.01 metre; range 0 – 500 metres. - achievableTargetIntegrityRisk indicates the achievable Target Integrity Risk (TIR) for which the HPL and VPL are provided. The achievable TIR is given by P=10-0.1n [hour-1] where n is the value of achievableTargetIntegrityRisk and the range is 10-1 to 10-9 per hour. If this field is absent, the achievable TIR is the same as the targetIntegrityRisk in IntegrityInformationRequest.

NOTE: 	The Protection Level (PL) is a statistical upper-bound of the Positioning Error (PE) that ensures that, the probability per unit of time of the true error being greater than the AL and the PL being less than or equal to the AL, for longer than the TTA, is less than the required TIR, i.e., the PL satisfies the following inequality: 
Prob per unit of time [((PE>AL) & (PL<=AL)) for longer than TTA] < required TIR
When the PL bounds the positioning error in the horizontal plane or on the vertical axis then it is called Horizontal Protection Level (HPL) or Vertical Protection Level (VPL) respectively.
A specific equation for the PL is not specified as this is implementation-defined. For the PL to be considered valid, it must simply satisfy the inequality above.

#### […]

#### 6.5.2.5	GNSS Location Information

#### –	A-GNSS-ProvideLocationInformation

The IE A-GNSS-ProvideLocationInformation is used by the target device to provide location measurements (e.g., pseudo-ranges, location estimate, velocity) to the location server, together with time information. It may also be used to provide GNSS positioning specific error reason.

-- ASN1START

A-GNSS-ProvideLocationInformation ::= SEQUENCE {

gnss-SignalMeasurementInformation	GNSS-SignalMeasurementInformation		OPTIONAL,

gnss-LocationInformation			GNSS-LocationInformation				OPTIONAL,

gnss-Error							A-GNSS-Error							OPTIONAL,

...

}

-- ASN1STOP

#### 6.5.2.6	GNSS Location Information Elements

#### […]

#### –	GNSS-LocationInformation

The IE GNSS-LocationInformation is included by the target device when location and optionally velocity information derived using GNSS or hybrid GNSS and other measurements is provided to the location server.

-- ASN1START

GNSS-LocationInformation ::= SEQUENCE {

measurementReferenceTime		MeasurementReferenceTime,

agnss-List						GNSS-ID-Bitmap,

... ,

[[

ha-GNSS-Metrics-r17			HA-GNSS-Metrics-r17		OPTIONAL

]]

}

-- ASN1STOP

GNSS-LocationInformation field descriptions

measurementReferenceTime This field specifies the GNSS system time for which the location estimate and optionally velocity are valid. It may also include GNSS-network time relationship, if requested by the location server and supported by the target device.

agnss-List This field provides a list of satellite systems used by the target device to calculate the location estimate and velocity estimate, if included. This is represented by a bit string in GNSS-ID-Bitmap, with a one-value at the bit position means the particular method has been used; a zero-value means not used.

ha-GNSS-Metrics This field provides high accuracy GNSS positioning metrics associated to the reported location estimate.

#### –	HA-GNSS-Metrics

The IE HA-GNSS-Metrics is included by the target device when high accuracy GNSS positioning metrics associated to a location estimate is provided to the location server. The parameters provided in IE HA-GNSS-Metrics are used as specified for sentence type GGA in [xx] and apply to all GNSSs and types of high accuracy GNSS assistance data.

-- ASN1START

HA-GNSS-Metrics-r17 ::= SEQUENCE {

nrOfUsedSatellites-r17		INTEGER (0..64),

hdopi-r17					INTEGER (1..256)   OPTIONAL,

pdopi-r17					INTEGER (1..256)   OPTIONAL,

age-r17						INTEGER (0..99)    OPTIONAL,

fixType-r17					ENUMERATED (carrier-phase-float,

carrier-phase-fix, ...},

...

}

-- ASN1STOP

HA-GNSS-Metrics

nrOfUsedSatellites This field specifies number of used GNSS satellites for the location estimate provided by the target device.

hdopi This field specifies the horizontal dilution of precision for the location estimate, scale factor 0.1.

pdopi This field specifies the 3D position dilution of precision, scale factor 0.1.

age This field, if supported by the device, specifies the age of the most recent used assistance data for high accuracy GNSS, scale factor 0.1 second.

fixType This field specifies the positioning fix type, based on the positioning fix quality indicators RTK float and RTK fix of [xx]. Specifically - carrier-phase-float - converged carrier phase integer ambiguity resolution - carrier-phase-fix - converging carrier phase floating point ambiguity resolution

#### 6.5.2.7	GNSS Location Information Request

#### –	A-GNSS-RequestLocationInformation

The IE A-GNSS-RequestLocationInformation is used by the location server to request location information from the target device using GNSS.

-- ASN1START

A-GNSS-RequestLocationInformation ::= SEQUENCE {

gnss-PositioningInstructions		GNSS-PositioningInstructions,

...

}

-- ASN1STOP

#### 6.5.2.8	GNSS Location Information Request Elements

#### –	GNSS-PositioningInstructions

The IE GNSS-PositioningInstructions is used to provide GNSS measurement instructions.

-- ASN1START

GNSS-PositioningInstructions ::= SEQUENCE {

gnss-Methods				GNSS-ID-Bitmap,

fineTimeAssistanceMeasReq	BOOLEAN,

adrMeasReq					BOOLEAN,

multiFreqMeasReq			BOOLEAN,

assistanceAvailability		BOOLEAN,

...,

[[

ha-GNSS-Req-r15			ENUMERATED { true }		OPTIONAL	-- Cond UEB

]] ,

[[

ha-GNSS-MetricsReq-r17	ENUMERATED { true }		OPTIONAL	-- Cond UEB

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| UEB | The field is optionally present, need OP, if the locationInformationType is set to locationEstimateRequired, locationEstimatePreferred, or locationMeasurementsPreferred; oltherwise it is not present. |

GNSS-PositioningInstructions field descriptions

gnssMethods This field indicates the satellite systems allowed by the location server. This is represented by a bit string in GNSS-ID-Bitmap, with a one-value at the bit position means the particular GNSS is allowed; a zero-value means not allowed. The target device shall not request assistance data or report or obtain measurements for systems that are not indicated in this bit map. At least one of the bits in this bit map shall be set to value one.

fineTimeAssistanceMeasReq This field indicates whether the target device is requested to report GNSS-network time association. TRUE means requested.

adrMeasReq This field indicates whether the target device is requested to include ADR measurements in GNSS-MeasurementList IE or not. TRUE means requested.

multiFreqMeasReq This field indicates whether the target device is requested to report measurements on multiple supported GNSS signal types in GNSS-MeasurementList IE or not. TRUE means requested.

assistanceAvailability This field indicates whether the target device may request additional GNSS assistance data from the server. TRUE means allowed and FALSE means not allowed.

ha-GNSS-Req This field, if present, indicates that any location estimate provided by the target device should be obtained using high accuracy RTK/PPP methods.

ha-GNSS-MetricsReq This field, if present, indicates that any location estimate provided by the target device should be reported with high accuracy GNSS positioning metrics.

#### 6.5.2.9	GNSS Capability Information

#### –	A-GNSS-ProvideCapabilities

The IE A-GNSS-Provide-Capabilities is used by the target device to indicate its capability to support A-GNSS and to provide its A-GNSS location capabilities (e.g., GNSSs and assistance data supported) to the location server.

-- ASN1START

A-GNSS-ProvideCapabilities ::= SEQUENCE {

gnss-SupportList			GNSS-SupportList				OPTIONAL,

assistanceDataSupportList	AssistanceDataSupportList		OPTIONAL,

locationCoordinateTypes		LocationCoordinateTypes			OPTIONAL,

velocityTypes				VelocityTypes					OPTIONAL,

...,

[[ periodicalReportingNotSupported-r14

PositioningModes				OPTIONAL,

idleStateForMeasurements-r14

ENUMERATED { required }			OPTIONAL

]],

[[ periodicAssistanceData-r15

BIT STRING { solicited 	 (0),

unsolicited (1)	} (SIZE (1..8))		OPTIONAL

]]

}

GNSS-SupportList ::= SEQUENCE (SIZE(1..16)) OF GNSS-SupportElement

GNSS-SupportElement ::= SEQUENCE {

gnss-ID							GNSS-ID,

sbas-IDs						SBAS-IDs					OPTIONAL,	-- Cond GNSS-ID-SBAS

agnss-Modes						PositioningModes,

gnss-Signals					GNSS-SignalIDs,

fta-MeasSupport					SEQUENCE {

cellTime	AccessTypes,

mode		PositioningModes,

...

}							OPTIONAL,	-- Cond fta

adr-Support						BOOLEAN,

velocityMeasurementSupport		BOOLEAN,

...,

[[

adrEnhancementsSupport-r15	ENUMERATED { true }			OPTIONAL,

ha-gnss-Modes-r15			PositioningModes			OPTIONAL

]] ,

[[

ha-gnss-MetricsSupport-r17	ENUMERATED { true }			OPTIONAL

]]

}

AssistanceDataSupportList ::= SEQUENCE {

gnss-CommonAssistanceDataSupport	GNSS-CommonAssistanceDataSupport,

gnss-GenericAssistanceDataSupport	GNSS-GenericAssistanceDataSupport,

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| GNSS-ID-SBAS | The field is mandatory present if the GNSS-ID = sbas; otherwise it is not present. |
| fta | The field is mandatory present if the target device supports the reporting of fine time assistance measurements; otherwise it is not present. |

A-GNSS-ProvideCapabilities field descriptions

gnss-SupportList This field specifies the list of GNSS supported by the target device and the target device capabilities associated with each of the supported GNSS. This field shall be present if the gnss-SupportListReq in the A-GNSS -RequestCapabilities IE is set to TRUE and if the target device supports the A-GNSS positioning method. If the IE A-GNSS-Provide-Capabilities is provided unsolicited, this field shall be included if the target device supports the assisted GNSS positioning method.

gnss-ID This field specifies the GNSS supported by the target device for which the capabilities in GNSS-SupportElement are provided.

sbas-IDs This field specifies the SBAS(s) supported by the target device. This is represented by a bit string, with a one-value at the bit position means the particular SBAS is supported; a zero-value means not supported.

agnss-Modes This field specifies the GNSS mode(s) supported by the target device for the GNSS indicated by gnss-ID. This is represented by a bit string, with a one-value at the bit position means the particular GNSS mode is supported; a zero-value means not supported.

gnss-Signals This field specifies the GNSS signal(s) supported by the target device for the GNSS indicated by gnss-ID. This is represented by a bit string, with a one-value at the bit position means the particular GNSS signal type is supported; a zero-value means not supported.

fta-MeasSupport This field specifies that the target device is capable of performing fine time assistance measurements (i.e., GNSS-cellular time association reporting). The cellTime field specifies for which cellular network(s) this capability is supported. This is represented by a bit string, with a one-value at the bit position means FTA measurements for the specific cellular network time is supported; a zero-value means not supported. The mode field specifies for which GNSS mode(s) FTA measurements are supported by the target device. This is represented by a bit string, with a one-value at the bit position means FTA measurements for the GNSS mode is supported; a zero-value means not supported.

adr-Support This field specifies whether the target device supports ADR measurement reporting. TRUE means supported.

velocityMeasurementSupport This field specifies whether the target device supports measurement reporting related to velocity. TRUE means supported.

assistanceDataSupportList This list defines the assistance data and assistance data choices supported by the target device. This field shall be present if the assistanceDataSupportListReq in the A-GNSS-RequestCapabilities IE is set to TRUE and if the target device supports GNSS assistance data. If the IE A-GNSS-Provide-Capabilities is provided unsolicited, this field shall be included if the target device supports any GNSS assistance data.

locationCoordinateTypes This parameter identifies the geographical location coordinate types that a target device supports for GNSS. TRUE indicates that a location coordinate type is supported and FALSE that it is not. This field shall be present if the locationVelocityTypesReq in the A-GNSS-RequestCapabilities IE is set to TRUE and if the target device supports UE-based or standalone GNSS positioning method. If the IE A-GNSS-Provide-Capabilities is provided unsolicited, this field shall be included if the target device supports UE-based or standalone GNSS positioning method.

velocityTypes This parameter identifies the velocity types that a target device supports for GNSS. TRUE indicates that a velocity type is supported and FALSE that it is not. FALSE for all velocity types indicates that velocity reporting is not supported. This field shall be present if the locationVelocityTypesReq in the A-GNSS-RequestCapabilities IE is set to TRUE and if the target device supports UE-based or standalone GNSS positioning method. If the IE A-GNSS-Provide-Capabilities is provided unsolicited, this field shall be included if the target device supports UE-based or standalone GNSS positioning method.

periodicalReportingNotSupported This field, if present, specifies the positioning modes for which the target device does not support periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is not supported; a zero-value means supported. If this field is absent, the location server may assume that the target device supports periodicalReporting in CommonIEsRequestLocationInformation for each supported positioning mode.

idleStateForMeasurements This field, if present, indicates that the target device requires idle state to perform GNSS measurements.

periodicAssistanceData This field identifies the periodic assistance data delivery procedures supported by the target device. This is represented by a bit string, with a one value at the bit position means the periodic assistance data delivery procedure is supported; a zero value means not supported. Bit 0 (solicited) represents the procedure according to clause 5.2.1a; bit (1) (unsolicited) represents the procedure according to clause 5.2.2a.

adrEnhancementsSupport This field, if present, indicates that the target device supports the fields adrMSB, adrSign, adrRMSerror, and delta-codePhase in IE GNSS-MeasurementList. This field may only be present if adr-Support is set to TRUE, and shall be absent if adr-Support is set to FALSE.

ha-gnss-Modes This field specifies the High-Accuracy GNSS mode(s) supported by the target device for the GNSS indicated by gnss-ID. This is represented by a bit string, with a one-value at the bit position means the particular GNSS mode is supported; a zero-value means not supported.

ha-gnss-MetricsSupport This field specifies that high accuracy GNSS positioning metrics are supported by the target device.

END OF CHANGE