3GPP TSG-RAN WG2#118-e	Draft R2-2206167

Online, 9 - 20 May 2022

Source: 	Huawei (rapporteur)

Title: 	[AT118-e][224][DCCA] RRC for SCG deactivation (Huawei)

Agenda Item:	6.2.2

Document for:	Discussion and decision

# 1	Introduction

This document is the summary of the following email discussion:

[AT118-e][224][DCCA] RRC for SCG deactivation (Huawei)

Scope: Discuss RRC corrections for SCG deactivation marked for this discussion.

Intended outcome: Discussion report in R2-2206167.

Comment deadline, 1st phase: Thursday W1, 0900 UTC (for collecting views)

Companies are invited to indicate the person providing input in the table below:

| Company | Name / Email |
| --- | --- |
| Huawei, HiSilicon | David Lecompte (david.lecompte@huawei.com) |
| LGE | Soo Kim (soo.kim@lge.com) |
| Ericsson | Stefan Wager (stefan.wager@ericsson.com) |
| Lenovo | Congchi Zhang (zhangcc16@lenovo.com) |
| Apple | Naveen Palle(naveen.palle@apple.com) |
| MediaTek | Felix Tsai (chun-fan.tsai@mediatek.com) |
| ZTE | LiuJing (liu.jing30@zte.com.cn) |
| vivo | wenjuan.pu@vivo.com |
| Nokia | Jarkko Koskela (jarkko.t.koskela@nokia.com) |
| Sharp | Takuma Kawano(kawano.takuma@sharp.co.jp) |
| Spreadtrum | Min Xu (Ellen.Xu@unisoc.com) |
| NEC | Hisashi Futaki (hisashi.futaki @ nec.com) |
| Futurewei | Jialin Zou (jialinzou88@yahoo.com) |
| CMCC | Xiaoxuan Tang (tangxiaoxuan@chinamobile.com) |
| OPPO | Shukun Wang (wangshukun@oppo.com) |
| Qualcomm | Punyaslok Purkayastha (punyaslo@qti.qualcomm.com) |
| NTT Docomo | Riki Okawa (riki.ookawa.rp@nttdocomo.com) |
| CATT | Erlin Zeng (erlin.zeng@catt.cn) |
| Fujitsu | Takako Sanda (sanda.takako @ Fujitsu.com) |

# 2	Discussion

## 2.1	TCI-Info

R2-2205800 is proposing to replace TCI-Info by a container including a MAC PDU that can contained MAC CEs:

- –	TCI-Info

The IE TCI-Info is used to transfer a MAC PDU from the network to the UE containing MAC CEs defined in TS 38.321 to activate (and deactivate) the configured TCI states for PDSCH and/or PDCCH of the PSCell. The RRC layer is transparent for this information.

TCI-Info information element

-- ASN1START

-- TAG-TCI-INFO-START

TCI-Info ::=        SEQUENCE {

mac-PduContainer      OCTET STRING

}

-- Editor's note: This IE is currently a starting point for discussion, details are FFS.

-- TAG-TCI-INFO-STOP

-- ASN1STOP

TCI-Info field descriptions

mac-PduContainer Includes a MAC PDU for controlling the TCI state for PDSCH and/or PDCCH of the PSCell. The MAC PDU may only contain the following MAC CEs defined in TS 38.321 for TCI state control: - Indication of TCI state for UE-specific PDCCH - Activation/Deactivation of UE-specific PDSCH TCI state - …

The motivations exposed are:

-	TCI state framework is enhanced in every release by RAN1, and the risk to define a signalling in RRC for the SCG deactivated is that it may require new IEs for every enhancement in the TCI state framework area

-	The existing signalling in RRC specifications repeats the signalling already defined for MAC CE and DCI, which is unnecessary specification work.

No change is proposed to the field that use TCI-Info, tci-Info in ServingCellConfig (per serving cell) or to its presence condition, which are defined as below:

tci-Info If configured for an SCell, or if configured for the PSCell when the SCG is being activated upon the reception of the containing message, the UE shall consider the indicated TCI states as the activated TCI states for PDCCH/PDSCH reception on this serving cell. If configured for the PSCell when the SCG is indicated as deactivated in the containing message: - the UE shall consider the indicated TCI states as the TCI states to be activated for PDCCH/PDSCH reception upon a later SCG activation in which tci-Info is absent - if bfd-and-RLM is configured and no RS is configured in RadioLinkMonitoringConfig for RLM, respectively for BFD, the UE shall use the indicated TCI states for PDCCH as RS for RLM, respectively for BFD. When this field is absent for the PSCell and the SCG is being deactivated: - the UE shall consider the previously activated TCI states as the TCI states to be activated for PDCCH/PDSCH reception upon a later SCG activation in which tci-Info is absent - if bfd-and-RLM is configured and no RS is configured in RadioLinkMonitoringConfig for RLM, respectively for BFD, the UE shall use the previously activated TCI states for PDCCH as RS for RLM, respectively for BFD.

| TCI_Info | This field is optional Need N for SCells if sCellState is configured, otherwise it is absent. This field is optional Need S for the PSCell when the SCG is indicated as deactivated or is being activated, otherwise it is absent. This field is absent for the PCell. |
| --- | --- |

The rapporteur notices that the current TCI-Info is for one serving cell only, can only be included at direct SCell activation of the corresponding SCell or for the deactivated PSCell and always includes information PDCCH and PDSCH, and for all PDDCH CORESETs while:

-	a MAC PDU can include MAC CEs for any serving cell

-	a MAC CE can affect multiple serving cells if simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2 are configured and the serving cell belongs to them

-	a MAC PDU may contain MAC CEs only for a subset of configured CORESETs for PDCCH reception

Therefore, it should be clarified whether the intention of the proposal is to specify such restrictions (and then a TP is needed) or not to specify any restriction and perhaps include a MAC PDU at cell group level instead.

In addition, the processing in 38.321 for MAC CEs is just "indicate to lower layers" the MAC CE contents, but in the case of the deactivated SCG the meaning is a little different, so some changes to L1 specification might be needed.

Question 1: Do company want to change TCI-Info to a container including a MAC PDU as proposed in R2-2205800? (if yes, please provide your suggestions on the above questions).

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | The proposal seems feasible but there is more work to be done than the current TP and RAN2 is supposed to freeze ASN.1 after this meeting. In addition, any extension (in later release or after ASN.1 freeze) requires defining new UE capabilities plus description of what can/cannot be included exactly depending on UE release and capability, and it is probably easier to do that unambiguously with new RRC fields (that have release or version suffixes) than with changes of allowed MAC PDU contents. |
| LGE | No | Agree with Huawei. |
| Ericsson | Yes (proponent) | We think using the MAC CE is the better approach to avoid duplicate functionality in RRC and MAC. It is true that the MAC PDU can include MAC CEs for any serving cell, but from the current field description of the tci-Info field, the usage of the content is already clear. For the SCG activation use case and the deactivated SCG use case, only TCI states indicated for the PSCell are used. Similar for the SCell activation use case, only TCI states indicated for the corresponding serving cell are used. But if companies think this is unclear, it can of course be further explained in field description. Regarding the possible L1 impact, we don’t see why the MAC CE approach would be different in this respect from the current approach of introducing new RRC IEs. If any changes are needed in the L1 specification due to SCG activation/deactivation, it is probably the same whether MAC CE or RRC IEs are used. If we decide to introduce new RRC IEs for the TCI state control, RAN2 should probably ask RAN1 if these are fine and if they cover the latest developments in the TCI state area. A drawback of adding new RRC IEs for TCI-state control is that the network implementation efforts are higher, as it needs to implement a new way to encode the TCI state information. Same goes probably for the UE. |
| Apple | No | We are also concerned with the handling of later extensions, and think we simply do not have enough time for considering this, as it requires further evaluation. In addition, this is not absolutely needed (things are not broken as of now). |
| MediaTek | No | We understand that current SPEC is okay and the proposal is just another way to implement the agreement. Although we tend to agree using a container also works but it brings the “extension” issue as mentioned by Huawei. So, we prefer to keep current approach. On implementation effort, in our view both approaches request some reasonable effort. It is not clear which one would be simpler to us. |
| ZTE | No | Agree with HW, Apple and MTK. |
| vivo | No | Agree with HW. |
| Nokia | No | Although proposal is intriguing and would likely be more future proof it seems to have few drawbacks as well as listed by Huawei. Additionally it seems bit late to do this change now and would require quite bit more discussions on technical details. |
| Sharp | No | Agree with Huawei. More general discussion may be able to be done in future Release. |
| Spreadtrum | No | Agree with Huawei. |
| Futurewei | No | Agree with Huawei |
| CMCC | No | Agree with HW and prefer to keep the current version which already implements the agreement. |
| OPPO | No |  |
| Qualcomm | No | It seems to us that the RRC-based signaling is simple and as the rapporteur mentions there are mechanisms available in the RRC specification (release and version suffixes) for extensions. |
| Docomo | No | Agree with Huawei, Apple and MediaTek. |
| CATT | No | Agree with Huawei. |
| Fujitsu | No | Agree with Huawei |

All companies except the proponent prefer not to include a MAC PDU in order to provide TCI state information.

Proposal 1: TCI-Info is not replaced by a container for a MAC PDU.

## 2.2	Cause of the UE preference for the SCG to be deactivated

There are proposals in R2-2204621 and R2-2205798 that the UE provides a cause value when it indicates that it prefers the SCG to be deactivated.

The proposed values are overheating and lowTraffic in R2-2204621 and batteryDrain and lowTraffic in R2-2205798.

The motivations are:

-	R2-2204621: (for overheating) UE may similarly request SCG deactivation if it determines that it can help alleviate an existing overheating problem

-	R2-2205798: the cause information can help the network decide on whether the SCG should be deactivated or released or whether there may be other actions that the network could perform

On the other hand, R2-2205062 argues that the network knows the traffic and for overheating or battery issue, the UE can already take actions as below:

NOTE 3: The UE can implicitly indicate a preference for NR SCG release by reporting the maximum aggregated bandwidth preference for power saving of the cell group, if configured, as zero for both FR1 and FR2, and by reporting the maximum number of secondary component carriers for power saving of the cell group, if configured, as zero for both uplink and downlink.

Besides, the rapporteur would like to remind that, in 36.331 and 38.331, the UE can only indicate that it prefers to deactivate the SCG if, since the UE was configured to indicate its preference, the UE did not send any indication or the last indication is that the UE did not prefer to deactivate the SCG and timer started at that time is expired.

As none of the companies proposing to add a cause value proposed any other change, it should be assumed that this remains true, e.g. the UE cannot indicate a change of reason for the preference of the SCG to be deactivated.

Question 2: Do companies want to add a cause value that UE includes when it indicates that it prefers the SCG to be deactivated? (if yes, please provide a list)

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | It seems a bit unclear what the network can do with that. |
| LGE | No | The network can implicitly know why the UE wants to deactivate SCG because the UE may send the preference for power saving or overheating information along with SCG deactivation preference in the UAI message. |
| Ericsson | Yes (proponent) | batteryDrain and lowTraffic. |
| Lenovo | No | We are also not sure about the usefulness and necessity of the new cause value. |
| Apple | No, but we are willing to discuss if necessary | It is also our view that what NW can do with this info, and if it means providing preference, then we prefer that NW try to honor UE request everytime . |
| MediaTek | No strong view |  |
| ZTE | No strong view | Currently, cause value is not provided if the UE indicates preference for RRC_IDLE or RRC_INACTIVE, so perhaps it’s ok to not have it for SCG deactivation. |
| vivo | No | The benefit is unclear. Similar view with LGE. |
| Nokia | No | Agrew with Huawei and infact there is already quite ab it of information possible to be include in the UE preference message. |
| Sharp | No | The benefit is unclear. Current cause value can be included and enough if needed. |
| Spreadtrum | No | The benefit is unclear. As mentioned by LG, the network may know the cause through the UAI message. |
| NEC | No (preferably) | We do not see additional gain from network point of view. |
| Futurewei | No | The benefit would be helping network to make the final decision on deactivation. But it seems very limited… |
| CMCC | Yes | Yes, the UAI in NOTE 3 above is to indicate its preference for SCG release, but not SCG deactivation. It may confuse the NW if we reuse the information from overheating or power saving UAI. Causes like batteryDrain and lowTraffic are fine for us. |
| OPPO | No |  |
| Qualcomm | Yes | We think that at least the “overheating” cause value can be useful. The NOTE 3 referred to in the discussion above is a request for SCG release, whereas we think a request for SCG deactivation can alleviate an overheating problem without having to give up the MR-DC operation altogether, i.e., SCG release. We agree with the rapporteur’s observation that “the UE cannot indicate a change of reason for the preference of the SCG to be deactivated”. |
| Docomo | No | The benefit is unclear. R2-2205798 provides an example that if the cause is low traffic the network can decide to release SCG, but we guess that in that case the network would release SCG whatever the cause is (e.g. low battery, overheating). |
| CATT | No | The benefit is unclear. |
| Fujitsu | No strong view |  |

3 companies support having a cause value, while 12 companies prefer not to have any. 4 companies have no strong view. Among the companies that suggest a cause value, there are divergencies on the values to indicate.

Proposal 2: No cause value is introduced in UAI to say why the UE prefers the SCG to be deactivated (unless there is consensus at a later meeting on a backward compatible CR to add a cause value).

## 2.3	Sending of " UL data" indication

R2-2205247 is proposing to capture in 5.7.4 that this is to be sent via SRB1:

The UE shall:

1>	if the procedure was triggered to provide configured grant assistance information for NR sidelink communication by an NR RRCReconfiguration message that was embedded within an E-UTRA RRCConnectionReconfiguration:

2>	submit the UEAssistanceInformation to lower layers via SRB1, embedded in E-UTRA RRC message ULInformationTransferIRAT as specified in TS 36.331 [10], clause 5.6.28;

1>	else if the procedure was triggered to provide an indication that the UE has uplink data related to a deactivated SCG:

2>	submit the UEAssistanceInformation message via SRB1 to lower layers for transmission;

1>	else if the UE is in (NG)EN-DC:

2>	if SRB3 is configured:

3>	submit the UEAssistanceInformation message via SRB3 to lower layers for transmission;

2>	else:

3>	submit the UEAssistanceInformation message via the E-UTRA MCG embedded in E-UTRA RRC message ULInformationTransferMRDC as specified in TS 36.331 [10].

The rapporteur agrees this is missing to proposes to agree this change (because in procedure text, if the UE is in EN-DC, the UE is supposed to use SRB3, which is not available

Question 3: Do companies agree with the above change to TS 38.331?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | but the wording should be modified to align it with the wording in other places. |
| LGE | No | At the 116bis-e meeting, RAN2 agreed that the UE sends an MN RRC message to indicate that there are UL data for SCG bearer. It means UAI will triggered in MN side. So, without change, the UE will send UAI message via SRB1 in MN side. #116bis-e Agreement As baseline, when the SCG is deactivated and there are UL data for one or more SCG bearer, the UE sends an MN RRC message to indicate that there are UL data for one or more SCG bearer. |
| Ericsson | No | UEAssistanceInformation informing the network about UL data on the deactivated SCG is MN configured, and is thus sent on SRB1. UEAssistanceInformation associated with SCG should be suspended, as proposed in section 2.6. |
| Lenovo | Yes | It helps clarifying. |
| Apple | Neutral | we raised this issue (that in EN-DC, LTE needs to handle it, if it’s MN who the UE should inform about UL data). We are ok to go with majority here. |
| MediaTek | No | It seems that the proposal is for EN-DC. In that case, we have same understanding as Ericsson that this will be triggered and handled in 36.331. It seems not related to 38.331 5.7.4. |
| ZTE | No | Same view as Ericsson. |
| vivo | No | Agree with Ericsson that this UAI configuration is configured by MN reconfiguration message, so the UE will send this UAI via SRB1 according to the current spec. |
| Nokia | Yes (proponent) | Not sure on LGE comment – they seem to support in the comment the proposal as it captures agreements which are not currently done properly? To LGE: implementations will be based on the specification more than on the agreements. The CR’s Reason for change does state: “RAN2#116bis agreed that it is an MN RRC message. However, this is not clear enough from the pre-existing condition” [in the specification]. |
| Sharp | No | Same view as Ericsson |
| Spreadtrum | Neutral | In EN-DC, the UAI will be triggered and handled in 36.331. But in 38.331, it is not clear as “SRB3 is configured” when SCG deactivated. |
| NEC | No | Based on the clarification from Ericsson, we understand the UAI for SCG deactivation is only captured in 36.331. |
| Futurewei | Neutral |  |
| CMCC | No | Share the same view with Ericsson. |
| OPPO | No | Agree with Ericsson. |
| Qualcomm | Yes, this change is needed |  |
| Docomo | No | Same view as Ericsson and vivo. |
| CATT | Maybe Yes | It seems the report or uplinkData in the UEAssistanceInformation is not configurable in the current spec, so not 100% clear how ue determines that it associates with SCG. So it seems useful to clarify. |
| Fujitsu | Yes | As RRC cannot know about UL data without receiving indication, specifying indication is necessary. However, as Huawei is pointing out, wording should be modified. For specifying “via SRB”, we have no strong view. |

There are mixed views on the correction (6 support, 10 oppose, 3 neutral).

The rapporteur understand that the procedure would be correct if the UE considers as false "if the UE assistance configuration that triggered this UE assistance information is associated with the SCG".

Some companies commented that "UEAssistanceInformation informing the network about UL data on the deactivated SCG is MN configured" but actually, what is configured is the SCG state, not the UE assistance information to indicate UL data.

Perhaps one could that since there is no "UE assistance configuration that triggered this UE assistance information", "if the UE assistance configuration that triggered this UE assistance information is associated with the SCG" is to be considered as false.

Proposal 3: No correction is done to send the UL data indication via SRB1 now. A clarification could be re-discussed later.

## 2.4	SRB3 handling while the SCG is deactivated

R2-2205247 is also proposing that measurement reports for SN are submitted to SRB3 and that this triggers sending the indication of UL data.

The rapporteur thinks this contradicts with previous agreement and would mean that, if SRB3 is established, no measurement report can be sent to the SN while the SCG is deactivated and SRB3 is established.

Question 4: Do companies agree that SN measurement reports should be submitted to SRB3 when the SCG is deactivated and that UL data on SRB3 triggers sending UAI on SRB1?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | This contradicts with previous agreements and would make SCG deactivation not usable if SRB3 is established, as the SCG would need to be activated to send any measurement report to the SN. |
| LGE | No | At the 112 meeting, RAN2 have already agreed that the UE sends MeasurementReport messages for measurement results of SN-configured measurements embedded in the NR (if the MCG is NR) ULInformationTransferMRDC message via SRB1. #112 agreement When the SCG is in deactivated state, the UE sends MeasurementReport messages for measurement results of SN-configured measurements embedded in the E-UTRA (if the MCG is EUTRA) or in the NR (if the MCG is NR) ULInformationTransferMRDC message via SRB1 |
| Ericsson | No | Our understanding is that the agreement in RAN2#116bis was for UL data, i.e. not including signalling. For signalling on SRB3, measurement reports will be sent on SRB1. |
| Lenovo | No |  |
| Apple | No | Atleast measurement reports should not trigger UE coming out of SCG deactivation. |
| MediaTek | No | Also understand that we agree to send SN measurement report via SRB1. |
| ZTE | No | We think this is already clear based on stage 2 spec. When SRB3 is not configured or the SCG is deactivated, reports for measurements configured by the SN are sent on SRB1. |
| vivo | No | No need to trigger SCG activation due to measurement report. |
| Nokia | Yes (proponent) | In our view this does not contradict with the agreements which state “As baseline, when the SCG is deactivated and there are UL data for one or more SCG bearer, the UE sends an MN RRC message to indicate that there are UL data for one or more SCG bearer”. SCG bearer is not limited to DRB. If companies think this is not the case then we would like ot have clear agreement that this is not the case and only concerns DRBs. The CR’s Reason for change does acknowledge the #112 agreement. Perhaps ironically we agree with the Huawei comment: the thinking behind the proposed change is that RRC procedures can be kept simpler if it is assumed that SRB3 should not be left configured (just like PDCP duplication applied to an SCG RLC entity, or a split bearer’s primaryPath set to SCG should not, as agreed!) when SCG is deactivated. |
| Sharp | No | In the current spec, SN measurement report is sent via SRB1 if SCG is deactivated. 3> if SRB3 is configured and the SCG is not deactivated: 4> submit the MeasurementReport message via SRB3 to lower layers for transmission, upon which the procedure ends; 3> else: 4> submit the MeasurementReport message via SRB1 embedded in NR RRC message ULInformationTransferMRDC as specified in 5.7.2a.3; |
| Spreadtrum | No |  |
| NEC | No | As clarified by ZTE, it seems already clear. |
| Futurewei | No |  |
| CMCC | No |  |
| OPPO | No |  |
| Qualcomm | No |  |
| Docomo | No |  |
| CATT | No |  |
| Fujitsu | No |  |

Only the proponent support the proposal.

Proposal 4: Confirm that measurement reporting for SN-configured measurements while the SCG is deactivated is sent via SRB1 (i.e. like in TS 38.331 v17.0.0, no change needed).

## 2.5	Support direct SCG SCell activation at SCG activation

R2-2204754 is proposing to support direct SCG SCell activation at SCG activation and has a TP.

The rapporteur understands that the presence condition for sCellState as in TS 38.331 v17.0.0 (identical to Rel-16) allows direct SCG SCell activation at SCG activation if the SCell is added in the SCG activation command or if reconfigurationWithSync is included for the SCG.

This means that direct SCell activation at RACH-less SCG activation is already possible, but it requires adding the SCell in the SCG activation command, which has some signalling overhead.

From RAN2 specification perspective, allowing activation of an already configured SCG SCell at RACH-less SCG activation seems rather simple. The TP in R2-22004754 is not sufficient, i.e. the presence condition of sCellState also needs some modification, but no other change seems needed.

Question 5: Do companies agree to allow direct SCG SCell activation (i.e. including sCellState), even if reconfigurationWithSync is not included for the SCG and the SCG SCell was configured before SCG activation?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes | Unless some issue is identified. |
| LGE | Yes |  |
| Ericsson | Yes | Unless some issue is identified. |
| Lenovo | Yes |  |
| Apple | Yes | Better to also get confirmation from RAN4 or atleast indicate to them about this scenario, if this gets agreed in RAN2. |
| MediaTek | Yes | Seems no issue in RAN2 so far. We are also fine to inform RAN4 with the understanding that whether to define corresponding requirement is up to RAN4. |
| ZTE | Yes |  |
| vivo | Yes |  |
| Nokia | Yes | We thought this was already possible 😊 but it is good that someone found out error in the text. We think this should be corrected |
| Sharp | Yes |  |
| Spreadtrum | Yes (proponent) |  |
| NEC | Yes |  |
| Futurewei | Yes |  |
| CMCC | Yes |  |
| OPPO | Yes |  |
| Qualcomm | Yes | The proposal is quite useful and should be also supported in the case reconfigurationWithSync is included in the SCG activation. It probably has impacts on the RAN4 timeline for SCG activation, so RAN4 may need to be informed if this proposal is accepted. |
| Docomo | Yes |  |
| CATT | Yes |  |
| Fujitsu | Yes |  |

All companies agree.

Proposal 5: Support direct SCG SCell activation (i.e. including sCellState), even if reconfigurationWithSync is not included for the SCG and the SCG SCell was configured before SCG activation.

## 2.6	UE assistance information to the SCG while the SCG is deactivated

R2-2205799 is proposing:

1)	to confirm that the UE does not send UAI via SRB3 while the SCG is deactivated.

2)	The UE report of SN configured UAI is stopped while the SCG is deactivated and resumed once the SCG is activated

There is a corresponding TP:

#### 5.3.5.13b	SCG deactivation

Upon initiating the procedure, the UE shall:

1>	consider the SCG to be deactivated;

1>	reset SCG MAC;

1>	indicate to lower layers that the SCG is deactivated;

1>	suspend all initiations of UE assistance information, as defined in clause 5.7.4.2, for fields configured via otherConfig associated with the SCG;

#### 5.3.5.13a	SCG activation

Upon initiating the procedure, the UE shall:

1>	if the UE is configured with an SCG after receiving the message for which this procedure is initiated:

2>	consider the SCG to be activated;

2>	resume all initiations of UE assistance information, as defined in clause 5.7.4.2, for fields configured via otherConfig associated with the SCG;

The motivation is that:

-	there is no particular use of the UE report to the SN

-	it may actually just prevent the UE from sending a new report (due to prohibit timer or because the UE report is the same) once the UE SCG is later activated

The rapporteur understands that current specification does not force the UE to send any UAI information, so without the TP, a UE implementation could do like what is described. The TP would simply forbid UEs from doing so.

Question 6: Do companies agree that the UE does not send UAI via SRB3 while the SCG is deactivated?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | Yes |  |
| LGE | No | We think the UE should follow the network configuration. The network can release or reconfigure UAI associated with the SCG whenever the SCG is deactivated or activated. |
| Ericsson | Yes (proponent) |  |
| Lenovo | Yes |  |
| Apple | Yes |  |
| MediaTek | Yes |  |
| ZTE | Yes |  |
| vivo | Yes |  |
| Nokia | No | P1 is covered by Q3 |
| Sharp | Yes |  |
| Spreadtrum | Yes |  |
| NEC | Yes |  |
| Futurewei | Yes |  |
| CMCC | Yes |  |
| OPPO | Yes |  |
| Qualcomm | Yes |  |
| Docomo | Yes |  |
| CATT | Yes |  |
| Fujitsu | Yes |  |

Question 7: Do companies agree with the proposal to specify that the UE shall suspend notifications of UAI towards the SCG while the SCG is deactivated and resume when the SCG is activated?

| Company | Yes/No | Comments |
| --- | --- | --- |
| Huawei, HiSilicon | No | It can be left to the UE to judge whether it wants to send a notification or not. |
| LGE | No | We think the UE should follow the network configuration. The network can release or reconfigure UAI associated with the SCG whenever the SCG is deactivated or activated. |
| Ericsson | Yes | We prefer to specify this in order to avoid unexpected UE behaviour. |
| Lenovo | No | Same view as LGE |
| Apple | No | MN can be the relay, if the UAI is configured. |
| MediaTek | Yes/No | Maybe a NOTE indicates some like that “the UE may suspend the SN configured UAI while SCG is deactivated” is sufficient. |
| ZTE | Yes | We understand the “resume” means after SCG activation the UE can send UAI towards the SCG if there is new trigger, it does not mean the UE will resend the previous UAI which was suspended. |
| vivo | No | Agree with HW. |
| Nokia | No |  |
| Sharp | No | UE should use SRB1 if needed. |
| Spreadtrum | No |  |
| NEC | Yes | We also slightly prefer to capture this in the spec. Also, agree with ZTE on the meaning of resume in Q7. |
| Futurewei | Yes | It is better to specify the UE behaviour, e.g. that the activation notification should be sent to MCG not to deactivated SCG. |
| CMCC | No | We have already agreed that SN reconfiguration can be sent during SCG deactivation. If unexpected UE behaviour for unsuccessful notifications of UAI to SN is identified, we prefer the note proposed by MTK. |
| OPPO | No |  |
| Qualcomm | No | UE should not be initiating transmission of UAI over the SCG while SCG is deactivated – there does not seem to be a scenario where it is needed. SRB1 should be used if there is any need for transmission of UAI, and we have agreements indicating when UE transmits UAI over SRB1 while in SCG deactivated. |
| Docomo | Yes, but | We have the same understanding of the meaning of “resume” as ZTE. But otherwise (if “resume” means resend suspended UAI), we disagree to capture in spec. |
| CATT | No |  |
| Fujitsu | No | Agree with LGE |

5 companies want to explicitly forbid the UE from sending UAI for the SCG on SRB1 while the SCG is deactivated, 13 companies are against specifying this, but some companies think the network should explicitly de-configure the corresponding features while other companies thing the UAI should be forward to the SN by the MN, and 2 companies suggests a note saying that “the UE may suspend the SN configured UAI while SCG is deactivated”.

The rapporteur understands that this topic needs further thinking.

Proposal 6: Confirm that the UE won't send UAI via SRB3 for the SCG while the SCG is deactivated.

Proposal 7: Further discuss whether the UE can send UAI via SRB1 for the SCG when the SCG is deactivated.

# 3	Conclusion

Based on the input from all companies, the rapporteur proposes that:

Proposal 1: TCI-Info is not replaced by a container for a MAC PDU.

Proposal 2: No cause value is introduced in UAI to say why the UE prefers the SCG to be deactivated (unless there is consensus at a later meeting on a backward compatible CR to add a cause value).

Proposal 3: No correction is done to send the UL data indication via SRB1 now. A clarification could be re-discussed later.

Proposal 4: Confirm that measurement reporting for SN-configured measurements while the SCG is deactivated is sent via SRB1 (i.e. like in TS 38.331 v17.0.0, no change needed).

Proposal 5: Support direct SCG SCell activation (i.e. including sCellState), even if reconfigurationWithSync is not included for the SCG and the SCG SCell was configured before SCG activation.

Proposal 6: Confirm that the UE won't send UAI via SRB3 for the SCG while the SCG is deactivated.

Proposal 7: Further discuss whether the UE can send UAI via SRB1 for the SCG when the SCG is deactivated.

# 4 References

- R2-2205800	[E035] Define the content of TCI-Info	Ericsson	discussion	LTE_NR_DC_enh2-Core

- R2-2204621	(TP for CR to TS 38.331) Efficient SCG deactivation/activation	Qualcomm Incorporated

- R2-2205246	38.331 corrections on deactivated SCG	Nokia, Nokia Shanghai Bell

- R2-2205247	Correction to deactivated SCG UL SRB3 handling	Nokia, Nokia Shanghai Bell

- R2-2204754	Discussion on SCG activation	Spreadtrum Communications

- R2-2205798	[E130] Cause values for UAI indicating preference for SCG deactivation	Ericsson

- R2-2205799	[E131] Handling of UAI for deactivated SCG	Ericsson

- R2-2205062	Discussion on whether cause value is needed in the SCG deactivation preference reporting	vivo