3GPP TSG-RAN WG2 Meeting #118-e                                                 R2-220xxxx

E-Meeting: 09 – 20 May 2022

Agenda item:	7.1.1

Source:	Qualcomm Incorporated

Title:	[draft] Report of ASN.1 R17 Review for NB-IoT/eMTC enhancements

Document for:	Discussion and Decision

# Introduction

This offline is to progress the solutions to ASN.1 review comments.

- [Pre118-e][301][NBIOT/eMTC R17] 36331 CR and rapporteur resolutions (Qualcomm)

# ASN.1 Review comments and propose conclusion

## RIL list

| ID | Class | TDoc | Status | Proposed Conclusion | Description | Proposed Change | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Z308 | 1 | None | ToDo | Change ‘“HD-FDD BL UE’ to ‘“HD-FDD Cat M1 UE’ Note: This change should also be done to TS 36.306. | ce-PDSCH-14HARQ-Config is applicable for HD-FDD Cat M1 UEs. The “HD-FDD BL UE” in field description is not accurate. | ce-PDSCH-14HARQ-Config Indicates whether 14-HARQ is enabled for HD-FDD Category M1BL UE, see TS 36.211 [21], TS 36.212 [22] and TS 36.213 [23]. E-UTRAN may set this field to setup only when DL multi-TB scheduling is not enabled and PUCCH repetition with HARQ-ACK bundling is not configured. |  |
| Z309 | 1 | None | ToDo | Change ‘“HD-FDD BL UE’ to ‘“HD-FDD Cat M1 UE in CE Mode A’ Note: This change should also be done to TS 36.306. | ce-PDSCH-maxTBS is applicable for HD-FDD Cat M1 UEs. The “HD-FDD BL UE” in field description is not accurate. | ce-PDSCH-maxTBS Indicates whether DL TBS of 1736 bits is enabled for HD-FDD Category M1BL UE in CE mode A, see TS 36.213 [23], clause TBD.. |  |
| H100 | 1 | No | ToDo | Replace ‘TBD’ with ‘7.1.7.2’ in description of ce-PDSCH-maxTBS | The reference is missing | replace TBD with 7.1.7.2 |  |
| H101 | 1 | R2-22xxxxx | ToDo | 1. Replace ‘cbpcg-‘, ‘coveragePagedPaging’ with ‘cbp-‘. 2. Add ‘cbp’ to section 3.2. | The identification of this feature in the specification is ambiguous as all ‘cbpcg-‘, ‘cbp-’ and ‘coveragePagedPaging’ are used. Also, the meaning of cbpcg and cbp is not defined. |  |  |
| Z312 | 1 | R2-22xxxxx | ToDo | 1. Change ‘cbpcg-Config-r17 ENUMERATED {pcg1, pcg2}’ to ‘cbp-Index-r17 INTEGER (1..2)‘ 2. Update field description. | Based on the field description, this parameter indicates the index to the coverage-based paging configuration. For easy understanding, we suggest to change “cbpcg-Config-r17” to “cbp-Index-r17”. Moreover, in order to avoid new abbreviation of “pcg”, we can just use integer. | cbpcg-Config-r17 ENUMERATED {pcg1, pcg2} OPTIONAL, -- Need OR cbp-Index-r17 INTEGER (1..2) OPTIONAL, -- Need OR cbpgc-ConfigIndex Index to the coverage-based paging configuration Value pcg1 corresponds to the first entry in cbpcg-ConfigList and pcg value 2 corresponds to the second entry in cbpcg-ConfigList in SystemInformationBlockType22-NB. | Huawei-v19: OK to change to Index (1..2 |
| Z313 | 1 | R2-22xxxxx | ToDo | See proposed conclusion for Z312. | Based on the field description, this parameter indicates the index to the coverage-based paging configuration. For easy understanding, we suggest to change “cbpcg-Config-r17” to “cbp-Index-r17”. Moreover, in order to avoid new abbreviation of “pcg”, we can just use Integer. | cbpcg-Config-r17 ENUMERATED {pcg1, pcg2} OPTIONAL, -- Need OR cbp-Index-r17 INTEGER (1..2) OPTIONAL, -- Need OR cbpgc-ConfigIndex Index to the coverage-based paging configuration. Value pcg1 corresponds to the first entry in cbpcg-ConfigList and, pcg value 2 corresponds to the second entry in cbpcg-ConfigList in SystemInformationBlockType22-NB. | Huawei-v19: O |
| B002 | 1 | None | ToDo | Add following sentence to description for s-MeasureInter: ‘If the field is not present, the UE applies the value of s-MeasureIntra.’ | Need code s-MeasureInter-r17 is set to Need OP but in the description the UE behaviour when the field is absent is missing. | Replace “Need OP” with “Need OR”. | Huawei0v19: Disagree. Should be OP and behaviour in absence (use value of s-MeasureIntra) specified in filed decsriptio |
| H102 | 1 | No | ToDo | Add description for cbpcg-ConfigList. Note: ‘cbpcg-‘ will be changed to ‘cbp-‘ as per H101. | Field description is missing | cbpcg-ConfigList List of of coverage-based paging groups and associated configuration |  |
| Z316 | 1 | R2-22xxxxx | ToDo | See proposed conclusion for H101. | We have concern on this naming. The meaning of abbreviation of“cbpc-” or “cbpcg-” are ambiguous. As mentioned in our response to H101, we are fine to use “cbp-” everywhere. Moreover, it’s clear that CBPCG-Config-NB-r17 is for providing several (2) coverage-level configurations. | CoverageBasedPagingConfig-NB-r17 ::= SEQUENCE { cbpc-HystTimer-r17 ENUMERATED {ms2560, ms7680, ms12800, ms17920, ms23040, ms28160, ms33280, ms40960}, cbpcg-ConfigList-r17 SEQUENCE (SIZE (1.. 2)) OF CBPCG-Config-NB-r17 } |  |
| Z317 | 1 | R2-22xxxxx | ToDo | See proposed conclusion for Z312. | As mentioned in our response to H101, we are fine to use “cbp-” everywhere. | cbpcg-Index-r17 INTEGER (1..2), |  |
| H103 | 1 | No | ToDo | Update field description to: ‘The minimum serving cell NRSRP applicable to the coverage-based paging carrier group, see TS 36.304 [4].’ | There is redundancy of meaning between ‘minimum RSRP’ and ‘threshold’. A reference to 36.304 would also be beneficial | The minimum serving cell NRSRP threshold applicable to the coverage-based paging carrier group, see TS 36.304 [4]. |  |
| H104 | 1 | No | ToDo | Update description to: ‘Minimum UE specific DRX cycle for the coverage-based paging carrier group, see TS 36.304 [4].’ | the parameter is defined per paging carrier group not per paging carrier. | Minimum UE specific DRX cycle for the coverage-based paging carrier group, see TS 36.304 [4]. [Comments]: |  |
| H105 | 1 | No | ToDo | Add ‘Need OR’. | Need code is missing | Add Need OR – see H106 |  |
| H106 | 1 | No | ToDo | Delete: “If this field is absent then legacy power ratio of NPDSCH EPRE to NRS EPRE applies.” | it is unclear what ‘ legacy’ means and the behaviour in absence is specified in TS 36.213. so we propose need OR and remove the behaviour in absence. | If this field is absent then legacy power ratio of NPDSCH EPRE to NRS EPRE applies |  |
| H107 | 1 | No | ToDo | Add description for pur-DL-16QAM-Config. | Field description for DL is missing | pur-DL-16QAM-Config Activation of 16QAM for downlink, see TS 36.213 [23] |  |
| H108 | 1 | No | ToDo | Replace: ‘In case of 16-QAM UL for PUR is configured, multiTone index is used, for the guardband and standalone modes the 16-QAM MCS index is equal to the value of multiTone + 14, for the inband mode the 16-QAM MCS index is equal to the value of multiTone + 11.’ with ‘If 16QAM UL for PUR is configured, the MCS is derived as follows: - For guardband and standalone: Actual value = multiTone field value + 14 - For inband: Actual value = multiTone field value + 11.’ | we normally refer to the ‘actual value’ and ‘field value’ [Proposed Change]: In case of 16-QAM UL for PUR is configured, multiTone index is used, for the guardband and standalone modes the 16-QAM MCS index is equal to the value of multiTone + 14, for the inband mode the 16-QAM MCS index is equal to the value of multiTone + 11. If 16QAM UL for PUR is configured, the MCS is derived as follows: - For guardband and standalone: Actual value = multiTone field value + 14 - For inband: Actual value = multiTone field value + 11. | In case of 16-QAM UL for PUR is configured, multiTone index is used, for the guardband and standalone modes the 16-QAM MCS index is equal to the value of multiTone + 14, for the inband mode the 16-QAM MCS index is equal to the value of multiTone + 11. If 16QAM UL for PUR is configured, the MCS is derived as follows: - For guardband and standalone: Actual value = multiTone field value + 14 - For inband: Actual value = multiTone field value + 11. |  |
| H109 | 1 | No | ToDo | Replace ‘Configures 16-QAM for uplink.’ with ‘Activation of 16QAM for uplink, see TS 36.213 [23]’ | the wording is different from other place (16-QAM instead of 16QAM, configures instead of Activation) | Configures 16-QAMActivation of 16QAM for uplink, see TS 36.213 [23]. |  |
| H110 | 1 | No | PropAgree | 1. Add IE ‘ measParameters-v1700 MeasParameters-NB-v1700 OPTIONAL,’ 2. move IEs‘connModeMeasIntraFreq-r17’ and ‘connModeMeasInterFreq-r17’ to be members of MeasParameters-NB-v1700. | should be a subset of measParameters | UE-Capability-NB-v1700-IEs ::= SEQUENCE { connModeMeasIntraFreq-r17 ENUMERATED {supported} OPTIONAL, connModeMeasInterFreq-r17 ENUMERATED {supported} OPTIONAL, coverageBasedPaging-r17 ENUMERATED {supported} OPTIONAL, phyLayerParameters-v1700 PhyLayerParameters-NB-v1700 OPTIONAL, measParameters-r17 MeasParameters-NB-r17 OPTIONAL, ntn-Parameters-r17 NTN-Parameters-NB-r17 OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } MeasParameters-NB-r17 ::= SEQUENCE { connModeMeasIntraFreq-r17 ENUMERATED {supported} OPTIONAL, connModeMeasInterFreq-r17 ENUMERATED {supported} OPTIONAL } |  |
| H111 | 1 | No | ToDo | Change ‘Stop performing neighbour cell measurement while in RRC_CONNECTED.’ to ‘Stop performing connected mode neighbour cell measurement.’ | this seems to imply that the UE will no longer perform measurements in this connection, which is not true | Reword: Stop performing connected mode neighbour cell measurement while in RRC_CONNECTED. |  |
| Z318 | 1 | R2-22xxxxx | ToDo | See proposed conclusion for Z312. | With similar reason, we suggest to change “cbpcg-Index-r17” to “cbp-Index-r17”. Moreover, in order to avoid new abbreviation of “pcg”, we can just use integer. And, we see no reason to mention paging carrier group in the field description of this IE. It’s better to keep consistence with other related IEs. | cbpcg-Index-r17 ENUMERATED {pcg1, pcg2} OPTIONAL, -- Cond CB-Paging cbp-Index-r17 INTEGER (1..2) OPTIONAL, -- Cond CB-Paging cbpcg-Index Index to the coverage-based paging carrier group configuration signalled to the UE during RRC connection release. Value pcg1 corresponds to the first paging carrier group entry in cbp-ConfigList, pcg value 2 corresponds to the second paging carrier group entry in cbp-ConfigList in SystemInformationBlockType22-NB. | Huawei-v19: O |

## Class 0 issues list

| Issue number | Copied existing specification text. Text should be unique, so that it can be easily found in the specification. If needed, add also the new text. | Comment/description/ correction | Email address | Proposed Conclusion |  |
| --- | --- | --- | --- | --- | --- |
| Insert issues from here |  |  |  |  |  |
| 2 | SystemInformationBlockType22-NB cbpc-HystTimer The minimum duration, in milliseconds, a UE configured with coverage-based paging carrier uses the same carrier for paging, see TS 36.304 [4]. Value ms2560 corresponds to 2560ms, value ms7680 corresponds to 7680ms, and so on. | ‘carrier’ should be removed | odile.rollinger@huawei.com | Delete ‘carrier’ |  |
| 3 | NPDSCH-Config-NB Activativation of 16QAM for DL, see TS 36.213 [23]. | Typo: Activativation | odile.rollinger@huawei.com | Change ‘Activativation’ to ‘Activation’ |  |
| 9 | 5.5.8 Measurements in NB-IoT …………. if (NRSRPRef – (NRSRP– PowerOffsetNonAnchor)) > s-MeasureDeltaP | PowerOffsetNonAnchor is typo. PowerOffsetNonAnchor => nrs-PowerOffsetNonAnchor | lu.ting@zte.com.cn | Change ‘PowerOffsetNonAnchor’ to ‘nrs-PowerOffsetNonAnchor’ |  |
| 13 | section 5.5.8 1> if neighCellMeasCriteria is present in SystemInformationBlockType3-NB: 2> if (NRSRPRef – (NRSRP– PowerOffsetNonAnchor)) > s-MeasureDeltaP: 3> set NRSRPRef = (NRSRP – nrs-PowerOffsetNonAnchor); 3> start or restart T326 with the value t-MeasureDeltaP; | Should nrs-PowerOffsetNonAnchor - | odile.rollinger@huawei.com | Change ‘PowerOffsetNonAnchor’ to nrs-‘PowerOffsetNonAnchor’ |  |
| 15 | 7.3.1: The timers in the table out of order. | Sort the timers in order. | hchoi5@lenovo.com | Spec Rapportuer can fix this. |  |

# Summary

This documents proposes solutions to the issues identified during ASN.1 review and the corresponding CR to implement the changes is provided in [2].

Proposal 1	xx

# References

- R2-201306, 17Additional enhancements for NB-IoT and LTE-MTC, RAN#88e, Reno, June 2020.

- R2-220xxxx, Correction to R17 NB-IoT/eMTC enhancements, RAN2#118e, May 2022.

# Participants

| Company | Name | e-mail address |
| --- | --- | --- |