3GPP TSG RAN2 Meeting #118-e 	R2-2206608

# Online, 09 – 20 May 2022

Title:	LS on rate matching patterns and CORESET configuration for MBS

Response to:	-

Release:	Rel-17

Work Item:	NR_MBS-Core

Source:	RAN2

To:	RAN1

Cc:	-

Contact Person:

#### Name:	Dawid Koziol

E-mail:	dawid.koziol@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN2 has discussed some details of configuration of rate matching resources and additional common CORESET for MBS broadcast in RRC Connected mode and has made agreements to clarify that:

- If the rate match pattern with the same RateMatchPatternId is configured in both the CFR and in the associated BWP, network should include the same resource configuration and they shall be counted as a single pattern in the total number of configured rate matching patterns at the UE.

- If the rate match pattern with the same RateMatchPatternId is configured in both ServingCellConfig/ServingCellConfigCommon and in SIB20/MCCH, network should include the same resource configuration and they shall be counted as a single pattern in the total number of configured rate matching patterns at the UE.

- Similar to the case of commonControlResourceSet, for MBS broadcast reception in RRC_CONNECTED, the network may configure via dedicated signaling the control resource set with the same ControlResourceSetId as used for commonControlResourceSetExt in CFR configuration in SIB20, and the configuration from PDCCH-Config takes precedence and should not be updated by the UE based on SIB20.

2. Actions:

To RAN1 group:

ACTION: 	RAN2 respectfully asks RAN1 to take the above information into account.

3. Dates of next TSG-RAN WG2 meetings:

RAN2#119	15 – 26 August 2022			Online

RAN2#119-bis-e	10 – 19 October 2022			Online