3GPP TSG RAN2 Meeting #119-e	R2-2207011

# Electronic, 17th – 26th August, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR |  | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Miscellaneous Corrections for eDCCA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Issue 1: In section 5.9, conditions for applying SCell deactivation procedures are - a) if an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received deactivating the SCell - b) if the sCellDeactivationTimer associated with the activated SCell expires The condition ‘if the SCG associated with the activated SCell is deactivated’ is missing. Issue 2: In section 5.12, reset operation are defined only for the case reset of the MAC entity is requested by upper layers. However, MAC itself may initate MAC reset (see section 5.29). Issue 3: Section 5.15.1 specifies that for active BWP, UE transmit on UL-SCH on the BWP; transmit on RACH on the BWP, if PRACH occasions are configured; monitor the PDCCH on the BWP; transmit PUCCH on the BWP, if configured; report CSI for the BWP; transmit SRS on the BWP, if configured; On the other hand section 5.29 specifies that when SCG is deactivated, for PSCell, UE does not transmit on UL-SCH; does not transmit on RACH; monitor the PDCCH on the BWP; does not transmit PUCCH on the BWP; does not report CSI; does not transmit SRS. This is contradictory operation for active BWP of PSCell. Note that when SCG is deactivated BWP is not deactivated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For issue 1 condition ‘if the SCG associated with the activated SCell is deactivated’ is added. For issue 2, it is clarified in section 5.12 that reset operation are also applied when reset is initiated by MAC itself upon SCG deactivation. For issue 3 calrified that for activated BWP of PSCell, UL-SCH/RACH/PDCCH/PUCCH/SRS/CSI report related operation are performed only if SCG is activated. Impacted functionality: SCG activation/deactivation Inter-operability: - If the network is implemented according to the CR and the UE is not, there are no interoperability issues. - If the UE is implemented according to the CR and the network is not, there are no interoperability issues. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.9, 5.12, 5.15.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

## 5.9	Activation/Deactivation of SCells

If the MAC entity is configured with one or more SCells, the network may activate and deactivate the configured SCells. Upon configuration of an SCell, the SCell is deactivated unless the parameter sCellState is set to activated for the SCell by upper layers.

The configured SCell(s) is activated and deactivated by:

-	receiving the SCell Activation/Deactivation MAC CE described in clause 6.1.3.10;

-	receiving the Enhanced SCell Activation/Deactivation MAC CE described in clause 6.1.3.55;

-	configuring sCellDeactivationTimer timer per configured SCell (except the SCell configured with PUCCH, if any): the associated SCell is deactivated upon its expiry;

-	configuring sCellState per configured SCell: if configured, the associated SCell is activated upon SCell configuration;

-	receiving scg-State: the SCells of SCG are deactivated.

The MAC entity shall for each configured SCell:

1>	if an SCell is configured with sCellState set to activated upon SCell configuration, or an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received activating the SCell:

2>	if the SCell was deactivated prior to receiving this Enhanced SCell Activation/Deactivation MAC CE and a TRS is selected for SCell activation:

3>	indicate to lower layers the information regarding the TRS.

2>	if the SCell was deactivated prior to receiving this SCell Activation/Deactivation MAC CE or this Enhanced SCell Activation/Deactivation MAC CE; or

2>	if the SCell is configured with sCellState set to activated upon SCell configuration:

3>	if firstActiveDownlinkBWP-Id is not set to dormant BWP:

4>	activate the SCell according to the timing defined in TS 38.213 [6] for MAC CE activation and according to the timing defined in TS 38.133 [11] for direct SCell activation; i.e. apply normal SCell operation including:

5>	SRS transmissions on the SCell;

5>	CSI reporting for the SCell;

5>	PDCCH monitoring on the SCell;

5>	PDCCH monitoring for the SCell;

5>	PUCCH transmissions on the SCell, if configured.

3>	else (i.e. firstActiveDownlinkBWP-Id is set to dormant BWP):

4>	stop the bwp-InactivityTimer of this Serving Cell, if running.

3>	activate the DL BWP and UL BWP indicated by firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id respectively.

2>	start or restart the sCellDeactivationTimer associated with the SCell according to the timing defined in TS 38.213 [6] for MAC CE activation and according to the timing defined in TS 38.133 [11] for direct SCell activation;

2>	if the active DL BWP is not the dormant BWP:

3>	(re-)initialize any suspended configured uplink grants of configured grant Type 1 associated with this SCell according to the stored configuration, if any, and to start in the symbol according to rules in clause 5.8.2;

3>	trigger PHR according to clause 5.4.6.

1>	else if an SCell Activation/Deactivation MAC CE or an Enhanced SCell Activation/Deactivation MAC CE is received deactivating the SCell; or

1>	if the sCellDeactivationTimer associated with the activated SCell expires;

1> if the SCG associated with the activated SCell is deactivated:

2>	deactivate the SCell according to the timing defined in TS 38.213 [6];

2>	stop the sCellDeactivationTimer associated with the SCell;

2>	stop the bwp-InactivityTimer associated with the SCell;

2>	deactivate any active BWP associated with the SCell;

2>	clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively;

2>	clear any PUSCH resource for semi-persistent CSI reporting associated with the SCell;

2>	suspend any configured uplink grant Type 1 associated with the SCell;

2>	flush all HARQ buffers associated with the SCell;

2>	cancel, if any, triggered consistent LBT failure for the SCell.

1>	if PDCCH on the activated SCell indicates an uplink grant or downlink assignment; or

1>	if PDCCH on the Serving Cell scheduling the activated SCell indicates an uplink grant or a downlink assignment for the activated SCell; or

1>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers; or

1>	if a MAC PDU is received in a configured downlink assignment:

2>	restart the sCellDeactivationTimer associated with the SCell.

1>	if the SCell is deactivated:

2>	not transmit SRS on the SCell;

2>	not report CSI for the SCell;

2>	not transmit on UL-SCH on the SCell;

2>	not transmit on RACH on the SCell;

2>	not monitor the PDCCH on the SCell;

2>	not monitor the PDCCH for the SCell;

2>	not transmit PUCCH on the SCell.

HARQ feedback for the MAC PDU containing SCell Activation/Deactivation MAC CE or Enhanced SCell Activation/Deactivation MAC CE shall not be impacted by PCell, PSCell and PUCCH SCell interruptions due to SCell activation/deactivation in TS 38.133 [11].

### When SCell is deactivated, the ongoing Random Access procedure on the SCell, if any, is aborted.

<End of Change 1>

<Start of Change 2>

## 5.12	MAC Reset

If a reset of the MAC entity is requested by upper layers or the reset of the MAC entity is triggered due to SCG deactivation as defined in clause 5.29, the MAC entity shall:

1>	if the MAC reset is not due to SCG deactivation:

2>	initialize Bj for each logical channel to zero;

1>	initialize SBj for each logical channel to zero if Sidelink resource allocation mode 1 is configured by RRC;

1>	if upper layers indicate SCG deactivation and bfd-and-RLM with value true is configured for the deactivated SCG:

2>	stop (if running) all timers except beamFailureDetectionTimer associated with PSCell and timeAlignmentTimers.

1>	else:

2>	stop (if running) all timers, except MBS broadcast DRX timers;

2>	consider all timeAlignmentTimers, inactivePosSRS-TimeAlignmentTimer, and cg-SDT-TimeAlignmentTimer, if configured, as expired and perform the corresponding actions in clause 5.2;

1>	set the NDIs for all uplink HARQ processes to the value 0;

1>	sets the NDIs for all HARQ process IDs to the value 0 for monitoring PDCCH in Sidelink resource allocation mode 1;

1>	stop, if any, ongoing Random Access procedure;

1>	discard explicitly signalled contention-free Random Access Resources for 4-step RA type and 2-step RA type, if any;

1>	flush Msg3 buffer;

1>	flush MSGA buffer;

1>	cancel, if any, triggered Scheduling Request procedure;

1>	cancel, if any, triggered Buffer Status Reporting procedure;

1>	cancel, if any, triggered Power Headroom Reporting procedure;

1>	cancel, if any, triggered consistent LBT failure;

1>	cancel, if any, triggered BFR;

1>	cancel, if any, triggered Sidelink Buffer Status Reporting procedure;

1>	cancel, if any, triggered Pre-emptive Buffer Status Reporting procedure;

1>	cancel, if any, triggered Timing Advance Reporting procedure;

1>	cancel, if any, triggered Recommended bit rate query procedure;

1>	cancel, if any, triggered Configured uplink grant confirmation;

1>	cancel, if any, triggered configured sidelink grant confirmation;

1>	cancel, if any, triggered Desired Guard Symbol query;

1>	cancel, if any, triggered Positioning Measurement Gap Activation/Deactivation Request procedure;

1>	cancel, if any, triggered SDT procedure;

1>	flush the soft buffers for all DL HARQ processes, except for the DL HARQ process being used for MBS broadcast;

1>	for each DL HARQ process, consider the next received transmission for a TB as the very first transmission;

1>	release, if any, Temporary C-RNTI;

1>	if upper layers indicate SCG deactivation and bfd-and-RLM with value true is not configured; or

1>	if the MAC reset is not due to SCG deactivation:

2>	reset all BFI_COUNTERs;

1>	reset all LBT_COUNTERs.

If a Sidelink specific reset of the MAC entity is requested for a PC5-RRC connection by upper layers, the MAC entity shall:

1>	flush the soft buffers for all Sidelink processes for all TB(s) associated to the PC5-RRC connection;

1>	consider all Sidelink processes for all TB(s) associated to the PC5-RRC connection as unoccupied;

1>	cancel, if any, triggered Scheduling Request procedure only associated to the PC5-RRC connection;

1>	cancel, if any, triggered Sidelink Buffer Status Reporting procedure only associated to the PC5-RRC connection;

1>	cancel, if any, triggered Sidelink CSI Reporting procedure associated to the PC5-RRC connection;

1>	stop (if running) all timers associated to the PC5-RRC connection;

1>	reset the numConsecutiveDTX associated to the PC5-RRC connection;

1>	initialize SBj for each logical channel associated to the PC5-RRC connection to zero.

<End of Change 2>

<Start of Change 3>

### 5.15.1	Downlink and Uplink

In addition to clause 12 of TS 38.213 [6], this clause specifies requirements on BWP operation.

A Serving Cell may be configured with one or multiple BWPs, and the maximum number of BWP per Serving Cell is specified in TS 38.213 [6].

The BWP switching for a Serving Cell is used to activate an inactive BWP and deactivate an active BWP at a time. The BWP switching is controlled by the PDCCH indicating a downlink assignment or an uplink grant, by the bwp-InactivityTimer, by RRC signalling, or by the MAC entity itself upon initiation of Random Access procedure or upon detection of consistent LBT failure on SpCell. Upon RRC (re-)configuration of firstActiveDownlinkBWP-Id and/or firstActiveUplinkBWP-Id for SpCell except for PSCell when SCG is deactivated (see clause 5.29) or activation of an SCell, the DL BWP and/or UL BWP indicated by firstActiveDownlinkBWP-Id and/or firstActiveUplinkBWP-Id respectively (as specified in TS 38.331 [5]) is active without receiving PDCCH indicating a downlink assignment or an uplink grant. Upon RRC (re-)configuration of firstActiveDownlinkBWP-Id for PSCell when SCG is deactivated, the DL BWP is switched to the firstActiveDownlinkBWP-Id as specified in TS 38.331 [5]. The active BWP for a Serving Cell is indicated by either RRC or PDCCH (as specified in TS 38.213 [6]). For unpaired spectrum, a DL BWP is paired with a UL BWP, and BWP switching is common for both UL and DL.

For each SCell a dormant BWP may be configured with dormantBWP-Id by RRC signalling as described in TS 38.331 [5]. Entering or leaving dormant BWP for SCells is done by BWP switching per SCell or per dormancy SCell group based on instruction from PDCCH (as specified in TS 38.213 [6]). The dormancy SCell group configurations are configured by RRC signalling as described in TS 38.331 [5]. Upon reception of the PDCCH indicating leaving dormant BWP, the DL BWP indicated by firstOutsideActiveTimeBWP-Id or by firstWithinActiveTimeBWP-Id (as specified in TS 38.331 [5] and TS 38.213 [6]) is activated. Upon reception of the PDCCH indicating entering dormant BWP, the DL BWP indicated by dormantBWP-Id (as specified in TS 38.331 [5]) is activated. The dormant BWP configuration for SpCell or PUCCH SCell is not supported.

For each activated Serving Cell configured with a BWP, the MAC entity shall:

1>	if a BWP is activated and the active DL BWP for the Serving Cell is not the dormant BWP:

2> if the Serving Cell is SCell; or

2> if the Serving Cell is PSCell and, SCG is activated:

3>	transmit on UL-SCH on the BWP;

3>	transmit on RACH on the BWP, if PRACH occasions are configured;

3>	monitor the PDCCH on the BWP;

3>	transmit PUCCH on the BWP, if configured;

3>	report CSI for the BWP;

3>	transmit SRS on the BWP, if configured;

2>	receive DL-SCH on the BWP;

2>	(re-)initialize any suspended configured uplink grants of configured grant Type 1 on the active BWP according to the stored configuration, if any, and to start in the symbol according to rules in clause 5.8.2;

2>	if lbt-FailureRecoveryConfig is configured:

3>	stop the lbt-FailureDetectionTimer, if running;

3>	set LBT_COUNTER to 0;

3>	monitor LBT failure indications from lower layers as specified in clause 5.21.2.

1>	if a BWP is activated and the active DL BWP for the Serving Cell is dormant BWP:

2>	stop the bwp-InactivityTimer of this Serving Cell, if running.

2>	not monitor the PDCCH on the BWP;

2>	not monitor the PDCCH for the BWP;

2>	not receive DL-SCH on the BWP;

2>	not report CSI on the BWP, report CSI except aperiodic CSI for the BWP;

2>	not transmit SRS on the BWP;

2>	not transmit on UL-SCH on the BWP;

2>	not transmit on RACH on the BWP;

2>	not transmit PUCCH on the BWP;

2>	clear any configured downlink assignment and any configured uplink grant Type 2 associated with the SCell respectively;

2>	suspend any configured uplink grant Type 1 associated with the SCell;

2>	if configured, perform beam failure detection and beam failure recovery for the SCell if beam failure is detected.

1>	if a BWP is deactivated:

2>	not transmit on UL-SCH on the BWP;

2>	not transmit on RACH on the BWP;

2>	not monitor the PDCCH on the BWP;

2>	not transmit PUCCH on the BWP;

2>	not report CSI for the BWP;

2>	not transmit SRS on the BWP;

2>	not receive DL-SCH on the BWP;

2>	clear any configured downlink assignment and configured uplink grant of configured grant Type 2 on the BWP;

2>	suspend any configured uplink grant of configured grant Type 1 on the inactive BWP.

A RedCap UE may be configured with a RedCap-specific initial UL BWP in initialUplinkBWP-RedCap, as specified in TS 38.331 [5].

Upon initiation of the Random Access procedure on a Serving Cell, after the selection of carrier for performing Random Access procedure as specified in clause 5.1.1, the MAC entity shall for the selected carrier of this Serving Cell:

1>	if PRACH occasions are not configured for the active UL BWP:

2>	if the UE is a RedCap UE; and

2>	if initialUplinkBWP-RedCap is configured:

3>	switch the active UL BWP to BWP configured by initialUplinkBWP-RedCap.

2>	else:

3>	switch the active UL BWP to BWP indicated by initialUplinkBWP.

2>	if the Serving Cell is an SpCell:

3>	if the UE is a RedCap UE; and

3>	if initialDownlinkBWP-RedCap is configured:

4>	switch the active DL BWP to BWP configured by initialDownlinkBWP-RedCap.

3>	else:

4>	switch the active DL BWP to BWP indicated by initialDownlinkBWP.

1>	else:

2>	if the Serving Cell is an SpCell:

3>	if the active DL BWP does not have the same bwp-Id as the active UL BWP:

4>	switch the active DL BWP to the DL BWP with the same bwp-Id as the active UL BWP.

1>	stop the bwp-InactivityTimer associated with the active DL BWP of this Serving Cell, if running.

1>	if the Serving Cell is SCell:

2>	stop the bwp-InactivityTimer associated with the active DL BWP of SpCell, if running.

1>	perform the Random Access procedure on the active DL BWP of SpCell and active UL BWP of this Serving Cell.

If the MAC entity receives a PDCCH for BWP switching of a Serving Cell, the MAC entity shall:

1>	if there is no ongoing Random Access procedure associated with this Serving Cell; or

1>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in clauses 5.1.4, 5.1.4a, and 5.1.5):

2>	cancel, if any, triggered consistent LBT failure for this Serving Cell;

2>	perform BWP switching to a BWP indicated by the PDCCH.

If the MAC entity receives a PDCCH for BWP switching for a Serving Cell(s) or a dormancy SCell group(s) while a Random Access procedure associated with that Serving Cell is ongoing in the MAC entity, it is up to UE implementation whether to switch BWP or ignore the PDCCH for BWP switching, except for the PDCCH reception for BWP switching addressed to the C-RNTI for successful Random Access procedure completion (as specified in clauses 5.1.4, 5.1.4a, and 5.1.5) in which case the UE shall perform BWP switching to a BWP indicated by the PDCCH. Upon reception of the PDCCH for BWP switching other than successful contention resolution, if the MAC entity decides to perform BWP switching, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure after performing the BWP switching; if the MAC decides to ignore the PDCCH for BWP switching, the MAC entity shall continue with the ongoing Random Access procedure on the Serving Cell.

Upon reception of RRC (re-)configuration for BWP switching for a Serving Cell while a Random Access procedure associated with that Serving Cell is ongoing in the MAC entity, the MAC entity shall stop the ongoing Random Access procedure and initiate a Random Access procedure after performing the BWP switching.

Upon reception of RRC (re-)configuration for BWP switching for a Serving Cell, cancel any triggered LBT failure in this Serving Cell.

The MAC entity shall for each activated Serving Cell configured with bwp-InactivityTimer:

1>	if the defaultDownlinkBWP-Id is configured, and the active DL BWP is not the BWP indicated by the defaultDownlinkBWP-Id, and the active DL BWP is not the BWP indicated by the dormantBWP-Id if configured; or

1>	if the defaultDownlinkBWP-Id is not configured, and if the UE is not a RedCap UE, and the active DL BWP is not the initialDownlinkBWP, and the active DL BWP is not the BWP indicated by the dormantBWP-Id if configured; or

1>	if the defaultDownlinkBWP-Id is not configured and if the UE is a RedCap UE, and initialDownlinkBWP-RedCap is not configured, and the active DL BWP is not the initialDownlinkBWP, and the active DL BWP is not the BWP indicated by the dormantBWP-Id if configured; or

1>	if the defaultDownlinkBWP-Id is not configured and if the UE is a RedCap UE, and initialDownlinkBWP-RedCap is configured, the active DL BWP is not the initialDownlinkBWP-RedCap, and the active DL BWP is not the BWP indicated by the dormantBWP-Id if configured:

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received on the active BWP; or

2>	if a PDCCH addressed to G-RNTI or G-CS-RNTI configured for multicast indicating downlink assignment is received on the active BWP; or

2>	if a PDCCH addressed to C-RNTI or CS-RNTI indicating downlink assignment or uplink grant is received for the active BWP; or

2>	if a MAC PDU is transmitted in a configured uplink grant and LBT failure indication is not received from lower layers; or

2>	if a MAC PDU is received in a configured downlink assignment for unicast or MBS multicast:

3>	if there is no ongoing Random Access procedure associated with this Serving Cell; or

3>	if the ongoing Random Access procedure associated with this Serving Cell is successfully completed upon reception of this PDCCH addressed to C-RNTI (as specified in clauses 5.1.4, 5.1.4a and 5.1.5):

4>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

2>	if the bwp-InactivityTimer associated with the active DL BWP expires:

3>	if the defaultDownlinkBWP-Id is configured:

4>	perform BWP switching to a BWP indicated by the defaultDownlinkBWP-Id.

3>	else:

4>	if the UE is a RedCap UE; and

4>	if initialDownlinkBWP-RedCap is configured:

5>	perform BWP switching to the initialDownlinkBWP-RedCap.

4>	else:

5>	perform BWP switching to the initialDownlinkBWP.

NOTE:	If a Random Access procedure is initiated on an SCell, both this SCell and the SpCell are associated with this Random Access procedure.

1>	if a PDCCH for BWP switching is received, and the MAC entity switches the active DL BWP:

2>	if the defaultDownlinkBWP-Id is configured, and the MAC entity switches to the DL BWP which is not indicated by the defaultDownlinkBWP-Id and is not indicated by the dormantBWP-Id if configured; or

2>	if the defaultDownlinkBWP-Id is not configured, and the MAC entity switches to the DL BWP which is not the initialDownlinkBWP and is not indicated by the dormantBWP-Id if configured:

3>	start or restart the bwp-InactivityTimer associated with the active DL BWP.

Upon initiation of the Random Access procedure, after selection of the carrier for performing Random Access procedure as specified in clause 5.1.1, if the UE is a RedCap UE in RRC_IDLE or RRC_INACTIVE mode, the MAC entity shall:

1>	if initialUplinkBWP-RedCap is configured:

2>	perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP-RedCap.

1>	else:

2>	perform the Random Access procedure as specified in clause 5.1 by using the BWP configured by initialUplinkBWP.

1>	if initialDownlinkBWP-RedCap is configured:

2>	monitor the PDCCH on the BWP configured by initialDownlinkBWP-RedCap.

1>	else:

2>	monitor the PDCCH on the BWP configured by initialDownlinkBWP.

<End of Change 3>