3GPP TSG RAN WG1 #119-e			R2- 2207042

e-Meeting, August 17 – 26, 2022

Title:	Reply LS on UE Power Saving for XR and Media Services

Response to:	R2-2204523 (S2-2203418) LS on UE Power Saving for XR and Media Services

Release:	Rel-18

Work Item:	FS_XRM, FS_NR_XR_enh

Source:	Qualcomm [To be RAN2]

To:	SA2

Cc:	RAN1

Contact Person:

Name:	 Linhai He

E-mail:	 linhaihe@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1. Overall Description:

RAN2 would like to thank SA2 for the LS R2-2204523/S2-2203418 on UE Power Saving for XR and Media Services.

RAN2 discussed the question asked by SA2 and identified the following parameters of a QoS flow as being useful for RAN in enabling UE power saving enhancements for XR applications:

For flows with PDU bursts:

- Periodicity of bursts, start offset, range of jitter of start time of a burst, average and maximum size of a burst. This information is needed by RAN in DRX configurations, and it can be semi-statically provided to RAN (e.g., at PDU/QoS flow establishment or modification).

- End of burst indication. This information is needed by RAN in early termination of DRX active time or indication of PDCCH skipping, and it can be dynamically provided via user-plane signaling.

- Burst sequence number for identifying a burst. This information is needed in enabling above enhancements, and it can be dynamically provided via user-plane signaling.

For other flows:

- Periodicity, start offset, range of jitter. This information is needed by RAN in DRX configurations, and it can be semi-statically provided to RAN (e.g., at PDU/QoS flow establishment or modification).

Please note that RAN2’s discussion was based on the following understandings on the structure of XR traffic:

- Traffic generated by an XR application may comprise multiple QoS flows.

- A QoS flow may comprise a periodic sequence of single PDUs or a burst of PDUs (e.g. audio vs video).

- Each burst comprises one or multiple PDU Sets.

- Each PDU Set comprises one or multiple PDUs.

# 2. Actions:

To: SA2

ACTION: RAN2 respectfully asks SA2 to take the above information into account in the relevant work.

# 3. Date of Next RAN2 Meetings:

TSG-RAN WG2 Meeting #119bis-e    			10 – 10 October, 2022			E-Meeting

TSG-RAN WG2 Meeting #120     			14 – 18 November, 2022			E-Meeting