3GPP TSG-RAN2 Meeting #119	R2-2207167

# E-meeting, August 17-29, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3226 | rev |  | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SFN-DFN offset and PosSIB request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2022-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | A Remote UE served by a Relay UE can have a requirement for positioning like any other UE. The remote UE may initiate an emergency call, and its user may invoke a service requiring location. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In 5.8.9.1.2, the text in first level bullet is added to allow Relay UE to set the sl-SFN-DFN-Offset within the RRCReconfigurationSidelink message - In 5.8.9.8.2 and 5.8.9.8.3, the action is added for Relay UE if the RemoteUEInformationSidelink includes the sl-RequestedPosSIB-List - In 5.8.9.9.2, the action is added for Remote UE when posSIB(s) is acquired. - In 6.6.2, the IE structure in RRCReconfigurationSidelink is updated to include sl-SFN-DFN-Offset. the IE structure in RemoteUEInformationSidelink is updated to include sl-RequestedPosSIB-List. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Positioning for remote UEs is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.9.1.2, 5.8.9.8.2, 5.8.9.8.3, 5.8.9.9.2, and 6.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start Change

##### 5.8.9.1.2	Actions related to transmission of RRCReconfigurationSidelink message

The UE shall set the contents of RRCReconfigurationSidelink message as follows:

1>	for each sidelink DRB that is to be released, according to clause 5.8.9.1a.1.1, due to configuration by sl-ConfigDedicatedNR, SIB12, SidelinkPreconfigNR or by upper layers:

2>	set the SLRB-PC5-ConfigIndex included in the slrb-ConfigToReleaseList corresponding to the sidelink DRB;

1>	for each sidelink DRB that is to be established or modified, according to clause 5.8.9.1a.2.1, due to receiving sl-ConfigDedicatedNR, SIB12 or SidelinkPreconfigNR:

2>	set the SLRB-Config included in the slrb-ConfigToAddModList, according to the received sl-RadioBearerConfig and sl-RLC-BearerConfig corresponding to the sidelink DRB;

1>	set the sl-MeasConfig as follows:

2>	If the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12:

3>	if UE is in RRC_CONNECTED:

4>	set the sl-MeasConfig according to stored NR sidelink measurement configuration information for this destination;

3>	if UE is in RRC_IDLE or RRC_INACTIVE:

4>	set the sl-MeasConfig according to stored NR sidelink measurement configuration received from SIB12;

2>	else:

3>	set the sl-MeasConfig according to the sl-MeasPreconfig in SidelinkPreconfigNR;

1>	set the sl-LatencyBoundIUC-Report;

1>	start timer T400 for the destination;

1>	set the sl-CSI-RS-Config;

1>	set the sl-LatencyBoundCSI-Report,

NOTE 1:	How to set the parameters included in sl-CSI-RS-Config and sl-LatencyBoundCSI-Report is up to UE implementation.

1>	set the sl-DRX-ConfigUC-PC5 as follows:

2>	If the frequency used for NR sidelink communication is included in sl-FreqInfoToAddModList in sl-ConfigDedicatedNR within RRCReconfiguration message or included in sl-ConfigCommonNR within SIB12:

3>	if UE is in RRC_CONNECTED and if sl-ScheduledConfig is included in sl-ConfigDedicatedNR within RRCReconfiguration:

4>	set the sl-DRX-ConfigUC-PC5 according to stored NR sidelink DRX configuration information for this destination.

NOTE 2:	If UE is in RRC_IDLE or in RRC_INACTIVE or out of coverage, or in RRC_CONNECTED and sl-UE-SelectedConfig is included in sl-ConfigDedicatedNR within RRCReconfiguration, it is up to UE implementation to set the sl-DRX-ConfigUC-PC5.

1>	for each PC5 Relay RLC channel that is to be released due to configuration by sl-ConfigDedicatedNR:

2>	set the SL-RLC-ChannelID corresponding to the PC5 Relay RLC channel in the sl-RLC-ChannelToReleaseListPC5;

1>	for each PC5 Relay RLC channel that is to be established or modified due to receiving sl-ConfigDedicatedNR:

2>	set the SL-RLC-ChannelConfigPC5 included in the sl-RLC-ChannelToAddModListPC5, according to the received sl-RLC-ChannelConfig corresponding to the PC5 Relay RLC channel;

1>	if the UE is operating as a L2 U2N Relay UE:

2>	if the SFN-DFN offset has changed since a previous transmission of the RRCReconfigurationSidelink message, or no previous transmission of the RRCReconfigurationSidelink message has occurred:

3>	set the sl-SFN-DFN-Offset according to the relation between the SFN timeline of the PCell and the DFN timeline;

The UE shall submit the RRCReconfigurationSidelink message to lower layers for transmission.

Next Change

##### 5.8.9.8.2	Actions related to transmission of RemoteUEInformationSidelink message

When entering RRC_IDLE or RRC_INACTIVE, or upon change in any of the information in the RemoteUEInformationSidelink while in RRC_IDLE or RRC_INACTIVE, the L2 U2N Remote UE shall:

1>	if the UE has not stored a valid version of a SIB, in accordance with clause 5.2.2.2.1, of one or several required SIB(s) in accordance with clause 5.2.2.1 and the requested SIB has not been indicated in RemoteUEInformationSidelink message to the L2 U2N Relay UE before:

2>	include sl-RequestedSI-List in the RemoteUEInformationSidelink to indicate the requested SIB(s);

1>	if the UE has not sent sl-PagingInfo-RemoteUE in the RemoteUEInformationSidelink message to the L2 U2N Relay UE before, set sl-PagingInfo-RemoteUE as follows:

2>	if the L2 U2N Remote UE is in RRC_IDLE:

3>	include ng-5G-S-TMSI in the sl-PagingIdentityRemoteUE;

3>	if the UE specific DRX cycle is configured by upper layer, set sl-PagingCycleRemoteUE to the value of UE specific Uu DRX cycle configured by upper layer;

2>	else if the L2 U2N Remote UE is in RRC_INACTIVE:

3>	include ng-5G-S-TMSI and fullI-RNTI in the sl-PagingIdentityRemoteUE;

3>	if the UE specific DRX cycle is configured by upper layer,

4>	set sl-PagingCycleRemoteUE to the minimum value of UE specific Uu DRX cycles (configured by upper layer and configured by RRC);

3>	else:

4>	set sl-PagingCycleRemoteUE to the value of UE specific DRX cycle configured by RRC;

1>	submit the RemoteUEInformationSidelink message to lower layers for transmission;

When entering RRC_CONNECTED, if L2 U2N remote UE had sent sl-RequestedSIB-List,sl-RequestedPosSIB-List, and/or sl-PagingInfo-RemoteUE, the L2 U2N Remote UE shall:

1>	set the sl-RequestedSIB-List to the value release if requested before;

1>	set the sl-RequestedPosSIB-List to the value release if requested before;

1>	set the sl-PagingInfo-RemoteUE to the value release if sent before;

1>	submit the RemoteUEInformationSidelink message to lower layers for transmission;

##### 5.8.9.8.3	Reception of RemoteUEInformationSidelink message by the L2 U2N Relay UE

The L2 U2N Relay UE shall:

1>	if the RemoteUEInformationSidelink includes the sl-PagingInfo-RemoteUE:

2>	if the UE is in RRC_CONNECTED on an active BWP with common search space configured including pagingSearchSpace; or

2>	if the UE is in RRC_IDLE or RRC_INACTIVE:

3>	if the sl-PagingInfo-RemoteUE is set to setup:

4>	monitor the Paging message at the L2 U2N Remote UE's paging occasion calculated according to sl-PagingIdentityRemoteUE and sl-PagingCycleRemoteUE included in sl-PagingInfo-RemoteUE;

3>	else (the sl-PagingInfo-RemoteUE is set to release):

4>	stop monitoring the Paging message at the L2 U2N Remote UE's paging occasion;

4>	release the received paging information in sl-PagingInfo-RemoteUE;

2>	else (the UE is in RRC_CONNECTED on an active BWP without pagingSearchSpace configured):

3>	if the sl-PagingInfo-RemoteUE is set to setup:

4>	include the received sl-PagingIdentityRemoteUE in SidelinkUEInformationNR message and perform Sidelink UE information transmission in accordance with 5.8.3;

3>	else (the sl-PagingInfo-RemoteUE is set to release):

4>	initiate transmission of the SidelinkUEInformationNR message to release the sl-PagingIdentityRemoteUE in SidelinkUEInformationNR message in accordance with 5.8.3;

4>	release the received paging information in sl-PagingInfo-RemoteUE;

1>	if the RemoteUEInformationSidelink includes the sl-RequestedSIB-List:

2>	if the sl-RequestedSIB-List is set to setup:

3> if the L2 U2N Relay UE has not stored a valid version of SIB(s) indicated in sl-RequestedSIB-List:

4>	perform acquisition of the system information indicated in sl-RequestedSIB-List in accordance with 5.2.2;

3>	else:

4>	perform the Uu message transfer procedure in accordance with 5.8.9.9;

2>	if the sl-RequestedSIB-List is set to release:

3>	release received SIB request in sl-RequestedSIB-List.

1>	if the RemoteUEInformationSidelink includes the sl-RequestedPosSIB-List:

2>	if the sl-RequestedPosSIB-List is set to setup:

3> if the L2 U2N Relay UE has not stored a valid version of posSIB(s) indicated in sl-RequestedPosSIB-List:

4>	perform acquisition of the positioning system information indicated in sl-RequestedPosSIB-List in accordance with 5.2.2;

3>	else:

4>	perform the Uu message transfer procedure in accordance with 5.8.9.9;

2>	if the sl-RequestedPosSIB-List is set to release:

3>	release received posSIB request in sl-RequestedPosSIB-List.

Next Change

##### 5.8.9.9.2	Actions related to transmission of UuMessageTransferSidelink message

The L2 U2N Relay UE initiates the Uu message transfer procedure when at least one of the following conditions is met:

1>	upon receiving Paging message related to the connected L2 U2N Remote UE from network (including Paging message within RRCReconfiguration message);

1>	upon acquisition of the SIB(s) requested by the connected L2 U2N Remote UE (as indicated in sl-RequestedSIB-List in the RemoteUEInformationSidelink) or upon receiving the updated SIB(s) from network which has been requested by the connected L2 U2N Remote UE;

1>	upon acquisition of the posSIB(s) requested by the connected L2 U2N Remote UE (as indicated in sl-RequestedPosSIB-List in the RemoteUEInformationSidelink) or upon receiving the updated posSIB(s) from network which has been requested by the connected L2 U2N Remote UE;

1>	upon unsolicited SIB1 forwarding to the connected L2 U2N Remote UE or upon receiving the updated SIB1 from network;

The L2 U2N Relay UE shall set the contents of UuMessageTransferSidelink message as follows:

1>	include sl-PagingDelivery if the Paging message received from network containing the associated ue-Identity of the L2 U2N Remote UE;

1>	include sl-SystemInformationDelivery if any of the conditions for initiating Uu message transfer procedure related to System Information are met;

1>	submit the UuMessageTransferSidelink message to lower layers for transmission.

NOTE:	The L2 U2N Relay UE may perform unsolicited forwarding of SIB1 to the L2 U2N Remote UE based on UE implementation.

Next Change

6.6.2	Message definitions

[...]

#### RemoteUEInformationSidelink

The RemoteUEInformationSidelink message is used to request SIB(s) or provide paging related information as specified in clause 5.8.9.8.1.

Signalling radio bearer: SL-SRB3

RLC-SAP: AM

Logical channel: SCCH

Direction: L2 U2N Remote UE to L2 U2N Relay UE

RemoteUEInformationSidelink message

-- ASN1START

-- TAG-REMOTEUEINFORMATIONSIDELINK-START

RemoteUEInformationSidelink-r17 ::=           SEQUENCE {

criticalExtensions                            CHOICE {

remoteUEInformationSidelink-r17               RemoteUEInformationSidelink-r17-IEs,

criticalExtensionsFuture                      SEQUENCE {}

}

}

RemoteUEInformationSidelink-r17-IEs ::=       SEQUENCE {

sl-RequestedSIB-List-r17                     SetupRelease { SL-RequestedSIB-List-r17}          OPTIONAL, -- Need M

sl-PagingInfo-RemoteUE-r17                    SetupRelease { SL-PagingInfo-RemoteUE-r17}         OPTIONAL, -- Need M

lateNonCriticalExtension                      OCTET STRING                                       OPTIONAL,

nonCriticalExtension                          RemoteUEInformationSidelink-v18xy-IEs                                        OPTIONAL

}

RemoteUEInformationSidelink-v18xy-IEs ::=	SEQUENCE {

sl-RequestedPosSIB-List-r18	SetupRelease {SL-RequestedPosSIB-List-r18}	OPTIONAL,	-- Need M

nonCriticalExtension						SEQUENCE {}				OPTIONAL

}

SL-RequestedSIB-List-r17 ::=                 SEQUENCE (SIZE (maxSIB-MessagePlus1-r17)) OF SL-SIB-ReqInfo-r17

SL-PagingInfo-RemoteUE-r17 ::=                SEQUENCE {

sl-PagingIdentityRemoteUE-r17                 SL-PagingIdentityRemoteUE-r17,

sl-PagingCycleRemoteUE-r17                    PagingCycle                                        OPTIONAL  -- Need M

}

SL-SIB-ReqInfo-r17 ::=                   ENUMERATED { sib1, sib2, sib3, sib4, sib5, sib6, sib7, sib8, sib9, sib10, sib11, sib12, sib13,

sib14, sib15, sib16, sib17, sib18, sib19, sib20, sib21, spare11, spare10, spare9,

spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1, ... }

SL-RequestedPosSIB-List-r18 ::=       SEQUENCE (SIZE (maxSIB-MessagePlus1-r17)) OF SL-PosSIB-ReqInfo-r18

SL-PosSIB-ReqInfo-r18 ::=                ENUMERATED { posSibType1-1, posSibType1-2, posSibType1-3, posSibType1-4, posSibType1-5, posSibType1-6, posSibType1-7, posSibType1-8, posSibType2-1, posSibType2-2, posSibType2-3, posSibType2-4, posSibType2-5, posSibType2-6, posSibType2-7, posSibType2-8, posSibType2-9, posSibType2-10, posSibType2-11, posSibType2-12, posSibType2-13, posSibType2-14, posSibType2-15, posSibType2-16, posSibType2-17, posSibType2-18, posSibType2-19, posSibType2-20, posSibType2-21, posSibType2-22, posSibType2-23, posSibType3-1, posSibType4-1,

posSibType5-1,posSibType6-1, posSibType6-2, posSibType6-3,... }

-- TAG-REMOTEUEINFORMATIONSIDELINK-STOP

-- ASN1STOP

RemoteUEInformationSidelink-IEs field descriptions

sl-RequestedSIB-List Contains a list of requested SIBs.

sl-PagingInfo-RemoteUE Indicates the paging information used by L2 U2N Relay UE to perform the connected L2 U2N Remote UE's paging monitoring.

sl-PagingIdentityRemoteUE Indicates the L2 U2N Remote UE's paging UE ID.

sl-PagingCycleRemoteUE Indicates the L2 U2N Remote UE's UE specific DRX cycle as the minimum value of the one provided by upper layers (if configured) and the one provided by RRC layer (if configured). Value rf32 corresponds to 32 radio frames, value rf64 corresponds to 64 radio frames and so on.

#### –	RRCReconfigurationSidelink

The RRCReconfigurationSidelink message is the command to AS configuration of the PC5 RRC connection. It is only applied to unicast of NR sidelink communication.

Signalling radio bearer: SL-SRB3

RLC-SAP: AM

Logical channel: SCCH

Direction: UE to UE

RRCReconfigurationSidelink message

-- ASN1START

-- TAG-RRCRECONFIGURATIONSIDELINK-START

RRCReconfigurationSidelink ::=          SEQUENCE {

rrc-TransactionIdentifier-r16           RRC-TransactionIdentifier,

criticalExtensions                      CHOICE {

rrcReconfigurationSidelink-r16          RRCReconfigurationSidelink-r16-IEs,

criticalExtensionsFuture                SEQUENCE {}

}

}

RRCReconfigurationSidelink-r16-IEs ::=  SEQUENCE {

slrb-ConfigToAddModList-r16             SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SLRB-Config-r16             OPTIONAL, -- Need N

slrb-ConfigToReleaseList-r16            SEQUENCE (SIZE (1..maxNrofSLRB-r16)) OF SLRB-PC5-ConfigIndex-r16    OPTIONAL, -- Need N

sl-MeasConfig-r16                       SetupRelease {SL-MeasConfig-r16}                                    OPTIONAL, -- Need M

sl-CSI-RS-Config-r16                    SetupRelease {SL-CSI-RS-Config-r16}                                 OPTIONAL, -- Need M

sl-ResetConfig-r16                      ENUMERATED {true}                                                   OPTIONAL, -- Need N

sl-LatencyBoundCSI-Report-r16           INTEGER (3..160)                                                    OPTIONAL, -- Need M

lateNonCriticalExtension                OCTET STRING                                                        OPTIONAL,

nonCriticalExtension                    RRCReconfigurationSidelink-v1700-IEs                                OPTIONAL

}

RRCReconfigurationSidelink-v1700-IEs ::= SEQUENCE {

sl-DRX-ConfigUC-PC5-r17                 SetupRelease { SL-DRX-ConfigUC-r17 }                                OPTIONAL, -- Need M

sl-LatencyBoundIUC-Report-r17           SetupRelease { SL-LatencyBoundIUC-Report-r17 }                      OPTIONAL, -- Need M

sl-RLC-ChannelToReleaseListPC5-r17      SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-ChannelID-r17         OPTIONAL, -- Need N

sl-RLC-ChannelToAddModListPC5-r17       SEQUENCE (SIZE (1..maxSL-LCID-r16)) OF SL-RLC-ChannelConfigPC5-r17  OPTIONAL, -- Need N

nonCriticalExtension                    SEQUENCE {}                                                         OPTIONAL

}

RRCReconfigurationSidelink-v18xy-IEs ::= SEQUENCE {

sl-SFN-DFN-Offset-r18					SL-SFN-DFN-Offset-r18				OPTIONAL, -- Need N

nonCriticalExtension                    SEQUENCE {} 				        OPTIONAL

}

SL-LatencyBoundIUC-Report-r17::=            INTEGER (3..160)

SLRB-Config-r16::=                      SEQUENCE {

slrb-PC5-ConfigIndex-r16                SLRB-PC5-ConfigIndex-r16,

sl-SDAP-ConfigPC5-r16                   SL-SDAP-ConfigPC5-r16                                               OPTIONAL, -- Need M

sl-PDCP-ConfigPC5-r16                   SL-PDCP-ConfigPC5-r16                                               OPTIONAL, -- Need M

sl-RLC-ConfigPC5-r16                    SL-RLC-ConfigPC5-r16                                                OPTIONAL, -- Need M

sl-MAC-LogicalChannelConfigPC5-r16      SL-LogicalChannelConfigPC5-r16                                      OPTIONAL, -- Need M

...

}

SLRB-PC5-ConfigIndex-r16 ::=            INTEGER (1..maxNrofSLRB-r16)

SL-SDAP-ConfigPC5-r16 ::=               SEQUENCE {

sl-MappedQoS-FlowsToAddList-r16         SEQUENCE (SIZE (1.. maxNrofSL-QFIsPerDest-r16)) OF SL-PQFI-r16      OPTIONAL, -- Need N

sl-MappedQoS-FlowsToReleaseList-r16     SEQUENCE (SIZE (1.. maxNrofSL-QFIsPerDest-r16)) OF SL-PQFI-r16      OPTIONAL, -- Need N

sl-SDAP-Header-r16                      ENUMERATED {present, absent},

...

}

SL-PDCP-ConfigPC5-r16 ::=               SEQUENCE {

sl-PDCP-SN-Size-r16                     ENUMERATED {len12bits, len18bits}                                   OPTIONAL, -- Need M

sl-OutOfOrderDelivery-r16               ENUMERATED { true }                                                 OPTIONAL,  -- Need R

...

}

SL-RLC-ConfigPC5-r16 ::=                CHOICE {

sl-AM-RLC-r16                           SEQUENCE {

sl-SN-FieldLengthAM-r16                 SN-FieldLengthAM                                                OPTIONAL, -- Need M

...

},

sl-UM-Bi-Directional-RLC-r16            SEQUENCE {

sl-SN-FieldLengthUM-r16                 SN-FieldLengthUM                                                OPTIONAL, -- Need M

...

},

sl-UM-Uni-Directional-RLC-r16           SEQUENCE {

sl-SN-FieldLengthUM-r16                 SN-FieldLengthUM                                                OPTIONAL, -- Need M

...

}

}

SL-LogicalChannelConfigPC5-r16 ::=      SEQUENCE {

sl-LogicalChannelIdentity-r16           LogicalChannelIdentity,

...

}

SL-PQFI-r16 ::=                         INTEGER (1..64)

SL-CSI-RS-Config-r16 ::=                SEQUENCE {

sl-CSI-RS-FreqAllocation-r16            CHOICE {

sl-OneAntennaPort-r16                   BIT STRING (SIZE (12)),

sl-TwoAntennaPort-r16                   BIT STRING (SIZE (6))

}                                                                                                           OPTIONAL, -- Need M

sl-CSI-RS-FirstSymbol-r16               INTEGER (3..12)                                                     OPTIONAL, -- Need M

...

}

SL-RLC-ChannelConfigPC5-r17::=          SEQUENCE {

sl-RLC-ChannelID-PC5-r17                SL-RLC-ChannelID-r17,

sl-RLC-ConfigPC5-r17                    SL-RLC-ConfigPC5-r16                                                OPTIONAL, -- Need M

sl-MAC-LogicalChannelConfigPC5-r17      SL-LogicalChannelConfigPC5-r16                                      OPTIONAL, -- Need M

...

}

SL-SFN-DFN-Offset-r18 ::=					SEQUENCE {

sl-FrameOffset								INTEGER (1..1023)			OPTIONAL, -- Need S

sl-SubframeOffset							INTEGER (1..9)				OPTIONAL, -- Need S

sl-SlotOffset								INTEGER (1..31)				OPTIONAL  -- Need S

}

-- TAG-RRCRECONFIGURATIONSIDELINK-STOP

-- ASN1STOP

RRCReconfigurationSidelink field descriptions

sl-CSI-RS-FreqAllocation Indicates the frequency domain position for sidelink CSI-RS.

sl-CSI-RS-FirstSymbol Indicates the position of first symbol of sidelink CSI-RS.

sl-DRX-ConfigUC-PC5 Indicates the NR sidelink DRX configuration for unicast communication, as specified in TS 38.321 [3]

sl-FrameOffset Indicates the offset in units of radio frames. If the field is absent, the UE assumes an offset of 0 frames.

sl-LatencyBoundCSI-Report Indicates the latency bound of SL CSI report from the associated SL CSI triggering in terms of number of slots.

sl-LatencyBoundIUC-Report Indicates the latency bound of SL Inter-UE coordination report from the associated SL Inter-UE coordination explicit request triggering in terms of number of slots.

sl-LogicalChannelIdentity Indicates the identity of the sidelink logical channel.

sl-MappedQoS-FlowsToAddList Indicate the QoS flows to be mapped to the configured sidelink DRB. Each entry is indicated by the SL-PQFI, which is used between UEs, as defined in TS 23.287 [55].

sl-MappedQoS-FlowsToReleaseList Indicate the QoS flows to be released from the configured sidelink DRB. Each entry is indicated by the SL-PQFI, which is used between UEs, as defined in TS 23.287 [55].

sl-MeasConfig Indicates the sidelink measurement configuration for the unicast destination.

sl-OutOfOrderDelivery Indicates whether or not outOfOrderDelivery specified in TS 38.323 [5] is configured. This field should be either always present or always absent, after the sidelink radio bearer is established.

sl-PDCP-SN-Size Indicates the PDCP SN size of the configured sidelink DRB.

sl-Resetconfig Indicates that the full configuration should be applicable for the RRCReconfigurationSidelink message.

sl-SDAP-Header Indicates whether or not a SDAP header is present on this sidelink DRB.

sl-SFN-DFN-Offset Indicates the offset between DFN#0 according to the timeline provided in the MasterInformationBlockSidelink message and SFN#0 in the UE’s serving cell.

sl-SlotOffset Indicates the offset in units of slots. If the field is absent, the UE assumes an offset of 0 slots.

sl-SubframeOffset Indicates the offset in units of subframes. If the field is absent, the UE assumes an offset of 0 subframes.

End of Change