3GPP TSG-RAN WG2 Meeting #119-e                                                 	R2-2207519

Electronic, 17th - 26th August, 2022

Source:	CATT

Title:	Subscription-based Aerial-UE Identification for NR

Agenda Item:	8.8.3

Document for:	Discussion and Decision

Introduction

In RAN#94-e, one new WID on NR support for Uncrewed Aerial Vehicles (UAV) [1] was agreed. As one of the scope of the WID the following was agreed:

2. Specify the signaling to support subscription-based aerial-UE identification [RAN3/SA2 interaction/RAN2]

Note: Work done in LTE is a starting point for this objective. NR-specific enhancements can be considered, if needed, while overall the LTE and NR solutions should be harmonized as much as possible.

In this contribution, we discuss this question and propose our view.

Discussion

In [2], it already captured that how the subscription based identification is transferred from CN to the eNB for LTE.

- 23.17.2	Subscription based identification of Aerial UE function

Support of Aerial UE function is stored in the user's subscription information in HSS. HSS transfers this information to the MME during Attach, Service Request and Tracking Area Update procedures.

The subscription information can be provided from the MME to the eNB via the S1-AP Initial Context Setup Request during Attach, Tracking Area Update and Service Request procedures. The subscription information can also be updated via the S1-AP UE Context Modification Request message. In addition, for X2-based handover, the source eNodeB can include the subscription information in the X2-AP Handover Request message to the target eNodeB.

For the intra and inter MME S1 based handover, the MME provides the subscription information to the target eNB after the handover procedure.

For NR UAV, the already outputted spec is TS 23.256[3].

### 4.3.4	AMF

In addition to the functionality defined in TS 23.501, the AMF:

-	may trigger the UUAA-MM procedure for a UE requiring UAV authentication and authorization by a USS when registering with 5GS when the UE has Aerial UE subscription information and based on local operator policy, or when the USS that authenticated the UAV triggers a re-authentication, or when AMF itself determines to re-authentication the UAV after the initial registration.

Considering the same logical for 4G MME and 5G AMF, the subscription based information can be signalled from the AMF to the gNB for NR UAV. Similar with LTE case, it is also identified that conveying this information during handover is beneficial so that the target gNB will also be aware of the UAV UE subscription information.

Proposal 1: For NR, the subscription based information is signaled from the AMF to the gNB.

Proposal 2: RAN2 sends LS to SA2 for the above proposal and cc RAN3.

Proposal 3: For NR, the subscription based information is signaled from the source gNB to the target gNB.

For Proposal 3, it is more related with RAN3 and can be discussed together with RAN3.

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: For NR, the subscription based information is signaled from the AMF to the gNB.

Proposal 2: RAN2 sends LS to SA2 for the above proposal and cc RAN3.

Proposal 3: For NR, the subscription based information is signaled from the source gNB to the target gNB.

- Reference

- RP-213600 New WID on NR support for UAV (Uncrewed Aerial Vehicles) Nokia

- TS 36.300 V16.6.0 Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2 (Release 16)

- TS 23.256 V17.3.0 Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2 (Release 17)