3GPP TSG-RAN WG2 Meeting #119-e	R2-2207524

# Electronic meeting, 17th - 29th August, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 3275 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the transmission of SidelinkUEInformationNR message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - In subclause 5.8.3.3, the description on SL DRX on/off indication is missed in paragraphs of how UE shall set the contents of the SidelinkUEInformationNR message . - In subclause 5.8.3.3, by the ASN.1 structure, the sl-RxInterestedQoS-InfoList is not included in the sl-RxDRX-ReportList but in the sl-RxInterestedGC-BC-DestList, and the associated destination ID for sl-RxInterestedQoS-InfoList is also missed. - In subclause 5.8.3.3, the sl-TxResourceReqList and/or sl-TxResourceReqListCommRelay-r17 is used for sidelink transmission but not for sidelink reception. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In subclause 5.8.3.3, add the description of SL DRX on/off indication for SL groupcast transmission in the associated paragraphs. - In subclause 5.8.3.3, add the description of “include sl-RxInterestedGC-BC-DestList and set its fields (if needed) as follows for each Destination Layer-2 ID for which it reports to network” before the content related sl-RxInterestedQoS-InfoList in order to set the sl-RxInterestedQoS-InfoList in sl-RxInterestedGC-BC-DestList, and add the description of destination ID information in the sl-RxInterestedGC-BC-DestList. - In subclause 5.8.3.3, change “sidelink reception” to “sidelink transmission”. Impact analysis Impacted functionality: NR sidelink communication Inter-operability: If the NW implements the CR but the UE does not, the UE cannot set the SidelinkUEInformationNR message including related SL DRX on/off information, sl-RxInterestedGC-BC-DestList and sl-TxResourceReqList, the network will have no information on the SL DRX on/off, sl-RxInterestedGC-BC-DestList and sl-TxResourceReqList, then network cann’t allocate the SL grant for the enabled SL DRX, give the correct Uu DRX configuration for alignement of SL DRX and Uu DRX, or configure the SL DRX for SL communication. If the UE implements the CR but the NW does not, the NW cannot have correct understanding to the SL DRX on/off, sl-RxInterestedGC-BC-DestList and sl-TxResourceReqList, then cann’t allocate the SL grant for the enabled SL DRX, give the correct Uu DRX configuraiton for alignement of SL DRX and Uu DRX, or configure the SL DRX for SL communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For change1 to change 3, the network may have no information or wrong understanding to SL DRX information including SL DRX on/off sl-RxInterestedGC-BC-DestList and sl-TxResourceReqList, then network cann’t allocate the SL grant for the enabled SL DRX over SL groupcast communication, give the correct Uu DRX configuration for alignement of SL DRX and Uu DRX, or configure the SL DRX for SL communication. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

5.8.3.3	Actions related to transmission of SidelinkUEInformationNR message

The UE shall set the contents of the SidelinkUEInformationNR message as follows:

1>	if the UE initiates the procedure to indicate it is (no more) interested to receive NR sidelink communication or to request (configuration/ release) of NR sidelink communication transmission resources or to report to the network that a sidelink radio link failure or sidelink RRC reconfiguration failure has been declared or to report to the network the sidelink DRX configuration for NR sidelink unicast reception or to report to the network the sidelink DRX assistance information or the sidelink DRX configuration reject information for NR sidelink unicast transmission or to report to the network the Destination Layer-2 ID and QoS profile associated with its interested services that sidelink DRX is applied for NR sidelink groupcast or broadcast reception or to report to the network the sidelink DRX on/off indication for NR sidelink groupcast transmission or to indicate it is (no more) interested to receive NR sidelink discovery messages or to request (configuration/ release) of NR sidelink discovery messages transmission resources or to request (configuration/ release) of NR sidelink U2N relay communication transmission resources (i.e. UE includes all concerned information, irrespective of what triggered the procedure):

<skip>

3>	if sl-DRX-ConfigCommon-GC-BC is included in SIB12-IEs:

4>	if configured by upper layers to perform NR sidelink reception:

5>	include sl-RxDRX-ReportList and set its fields (if needed) as follows for each destination for which it reports to network:

6>	set sl-DRX-ConfigFromTx to include the accepted sidelink DRX configuration of the associated destination for NR sidelink unicast communication, if received from the associated peer UE;

5>	include sl-RxInterestedGC-BC-DestList and set its fields (if needed) as follows for each Destination Layer-2 ID for which it reports to network:

6>	set sl-RxInterestedQoS-InfoList to include the QoS profile of its interested service for the associated destination for NR sidelink groupcast or broadcast communication;

6> set sl-DestinationIdentity to include the associated destination for which UE is interested in reception to the network for NR sidelink groupcast and broadcast communication;

4>	if configured by upper layers to perform NR sidelink transmission and configured with sl-ScheduledConfig:

5>	include sl-TxResourceReqList and/or sl-TxResourceReqListCommRelay-r17 and set its fields (if needed) as follows for each destination for which it reports to network:

6>	set sl-DRX-InfoFromRx to include the sidelink DRX assistance information of the associated destination, if any, received from the associated peer UE;

4>	if configured by the upper layers to perform NR sidelink groupcast transmission:

5>	include the sl-TxResourceReqList and set its fields (if needed) as follows for each destination for which it reports to network:

6>set sl-DRX-Indication to include the DRX on/off indication for the associated destination;

Editor's Note: FFS on the message used for Tx UE to report DRX configuration reject information.

1>	if the UE initiates the procedure while connected to an E-UTRA PCell:

2>	submit the SidelinkUEInformationNR to lower layers via SRB1, embedded in E-UTRA RRC message ULInformationTransferIRAT as specified in TS 36.331 [10], clause 5.6.28;

1>	else:

2>	submit the SidelinkUEInformationNR message to lower layers for transmission.

END OF CHANGE