3GPP TSG-RAN WG2 Meeting #119-e	R2-2207998

Online, 15th – 26th August, 2022

Agenda Item:	8.5.2

Source:	MediaTek Inc.

Title:	On RAN awareness of XR traffic characteristics

Document for:	Discussion and decision

# 1 Introduction

- Objectives on XR-awareness in RAN (RAN2):

  - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of.

  - Study how the above information aids XR-specific traffic handling.

The SID on enhancements to support XR traffic [1] includes the following objective:

In this TDoc, we take a closer look at XR traffic characteristics whose knowledge can be beneficial to the gNB scheduler. This discussion is tied to the remaining XR objectives on power savings and capacity, which is introduced in companion TDocs [2][3].

# 2 Discussion

This paper is split into two halves. In the first half, we take a look at XR traffic characteristics and how they can be useful to the gNB scheduler. In the latter half, we discuss how these different XR traffic characteristics can be provided to the gNB. We base this discussion on the Rel-17 TR 38.838 [4] as well as ongoing discussions in other 3GPP WGs.

## 2.1 Useful traffic characteristics

The traffic model in TR 38.838 [4] largely splits XR traffic into video and pose/control streams. Figure 1 illustrates how the video component of XR traffic is modelled.

Figure 1: Characteristics of XR video traffic

Video traffic is periodic in nature and the periodicity is a function of the refresh rate of the video stream (typically expressed in FPS, i.e. Frames Per Second). Video frame rendering, encoding and network transfer results in a jitter associated with frame arrival at RAN. This jitter is modelled as a truncated Gaussian distribution with a σ of 2ms and truncation range of ±4ms. Variations in video frame size is also modelled as a truncated Gaussian distribution with a σ of 10.5% of the mean frame size, and the truncation range as ±50% of the mean.

Figure 2 illustrates how UL pose/control traffic is modelled in the TR [4]. The traffic is periodic in nature (4ms period assumed) with no associated jitter. Size of the traffic is small (100 bytes assumed) with a PER of 99% to be met within a PDB of 10ms.

Figure 2: Characteristics of XR pose/control traffic

As explained in our companion paper on C-DRX enhancements [2], significant power savings (and associated capacity gains) for XR traffic can be realised by:

- Aligning the C-DRX cycle with XR traffic periodicity

- Reduced monitoring during DRX on-duration (which needs to be at least as long as jitter)

Similarly, our companion paper on capacity enhancements [3] highlights the importance of the use of configured grants to meet UL traffic requirements with the following enhancements:

- Aligning CGs with UL pose/control traffic and with UL video in the case of AR

- Hybrid use of CG and DG, such that CG is used for (latency-sensitive) late data arrival and DG for (latency-tolerant) early data arrival

- CG resource size adaptation to deal with video-frame size variations

Figure 3: Uses of XR traffic characteristics by the gNB

These enhancements only work when the gNB has some awareness of the traffic characteristics. For example, the gNB needs to be aware of:

- Traffic periodicity: to appropriately configure C-DRX cycle and CG periodicity

- Traffic arrival time: to correctly configure DRX wake-up occasions and CG occasions

- Jitter associated with traffic: to configure an appropriate length of the DRX on duration timer, and to configure CG at an appropriate time

- Frame-size variation: to configure CG resource adaptation and DRX inactivity timer correctly

Proposal 1: To enable power savings and capacity enhancements, the following XR traffic characteristics are useful to the gNB: periodicity, arrival time, jitter and frame-size variations.

Excerpt 1:  Definition of a PDU set (source S2-2201851)

PDU Set: A PDU Set is composed of one or more PDUs carrying the payload of one unit of information generated at the application level (e.g., a frame or video slice for XRM Services), which are of same importance requirement at application layer. All PDUs in a PDU Set are needed by the application layer to use the corresponding unit of information. In some cases, the application layer can still recover parts of the information unit, when some PDUs are missing.

In the discussions in SA2 on XR [5], the concept of a PDU set has been defined as captured in Excerpt 1 below. The applicability of this PDU set definition to XR applications is being currently discussed in SA4 with an initial response provided in [6].

Once SA2 and SA4 conclude on the applicability of the PDU set characteristics to XR traffic, the gNB could potentially take advantage of this information to aid the scheduler. For example, the definition above indicates that all PDUs in a PDU set are needed by the application layer to use the unit of information. In this case, RAN could prioritise the transmission of new PDU sets if some PDUs from an older PDU set have not been succesfully transmitted, i.e. some older PDU sets can be dropped to enable the transmission of newer PDU sets.

Proposal 2: RAN2 to investigate the impact of PDU set dropping on XR traffic latency and capacity.

For now, SA4 have indicated in their response [6] that such PDU set dropping behaviour may not be always preferable as applications may try to recreate the scene using partial PDU set information. Such behaviour would vary from application to application. We need to wait on the SA4/SA2 discussions to progress further before determining the set of cases where PDU set information can be used. We expect that the CN will configure the gNB with policies around the use of PDU set information which would govern gNB behaviour vis-à-vis handling PDU sets.

Proposal 3: The CN configures how PDU set information can be used by the gNB.

## 2.2 Delivery of traffic characteristics to the gNB

Aforementioned traffic characteristics can be classified as semi-static and dynamically changing. Semi-static traffic characteristics are those that apply to all packets that belong to a QoS flow that change infrequently over time. Dynamic traffic characteristics are those change from PDU to PDU or across PDU sets. Table 1 below provides examples of different traffic characteristics and how they can be classified.

| Semi-static traffic characteristics | Dynamic traffic characteristics |
| --- | --- |
| Periodicity | Packets that belong to a PDU-set |
| Arrival time | PDU-set importance |
| Jitter | Cross PDU-set priorities |
| Frame-size variations |  |

Table 1: Classification of XR traffic characteristics

While SA2 are discussing how such information can be derived, we need to discuss how this information can be passed on to the gNB. The means by which these different characteristics can be signalled to the gNB depends on whether they are semi-static or dynamic in nature. It also depends on whether this information can be derived by the CN (e.g. by the application function in case of DL traffic) or by the UE (e.g. in case of UL traffic).

In case of DL traffic, we find similarities between XR traffic characteristics and IIoT traffic characteristics that have been discussed in past releases. The TSC assistance information framework in IIoT already provide some of the semi-static information discussed in this paper, as shown in Table 2.

| Assistance Information | Description |
| --- | --- |
| Flow Direction | The direction of the TSC flow (uplink or downlink). |
| Periodicity | It refers to the time period between start of two bursts. |
| Burst Arrival time (Optional) | The latest possible time when the first packet of the data burst arrives at either the ingress of the RAN (downlink flow direction) or egress interface of the UE (uplink flow direction). |
| Survival Time (Optional) | It refers to the time period an application can survive without any burst, as defined in clause C.2.3 of TS 22.104 [105]. |

Table 2: TSC Assistance Information (source TS 23.501)

We propose to reuse and extend this framework to provide the gNB with semi-static DL XR traffic characteristics. In case of UL traffic, this information can be derived by UE implementation. Once derived, the UE assistance information framework can be extended to pass this information on to the gNB to aid the scheduler.

Proposal 4: TSC assistance information framework can be used to provide the gNB with DL XR traffic characteristics that do not vary from PDU to PDU.

Proposal 5: UE assistance information framework can be used to provide the gNB with UL XR traffic characteristics that do not vary from PDU to PDU.

Dynamically changing traffic characteristics such as PDU set identification and associated information such as their importance (following SA2/SA4 conclusions) change from PDU to PDU. Therefore, this information ought to be signalled in-band with the actual PDU itself. GTP-U headers are well-placed to provide the gNB with this information.

Proposal 6: GTP-U headers can be used to provide the gNB with DL XR traffic characteristics that could vary from PDU to PDU.

# 3 Conclusion

In this contribution we propose the following:

Proposal 1: To enable power savings and capacity enhancements, the following XR traffic characteristics are useful to the gNB: periodicity, arrival time, jitter and frame-size variations.

Proposal 2: RAN2 to investigate the impact of PDU set dropping on XR traffic latency and capacity.

Proposal 3: The CN configures how PDU set information can be used by the gNB.

Proposal 4: TSC assistance information framework can be used to provide the gNB with DL XR traffic characteristics that do not vary from PDU to PDU.

Proposal 5: UE assistance information framework can be used to provide the gNB with UL XR traffic characteristics that do not vary from PDU to PDU.

Proposal 6: GTP-U headers can be used to provide the gNB with DL XR traffic characteristics that could vary from PDU to PDU.

# 4 References

- RP-220285 - Study on XR Enhancements for NR (Nokia)

- R2-2207999 - C-DRX enhancements for XR (MediaTek)

- R2-2207719 - XR-specific capacity improvements (MediaTek)

- 3GPP TR 38.838 V17.0.0 - Study on XR (Extended Reality) Evaluations for NR (Release 17)

- S2-2201803 - LS on QoS support with PDU Set granularity (SA2)

- S4-220505 - LS Reply on QoS support with PDU Set granularity (SA4)

- R2-2206966 - LS on UE Power Saving for XR and Media Services (SA2)