3GPP TSG RAN WG2 Meeting #119-e                                                       R2-2208183

# Electronic meeting, August 2022

Agenda item:		6.15.2

Source:			Intel Corporation

Title:		Open issue on SL DRX

Document for:	 	Discussion, Decision

# Introduction

In RAN2#118-e meeting, several open issues related to SL DRX procedure were discussed and closed. One of the few open issues that need to be addressed, particularly related to the SL DRX configuration procedure is related to the SR configuration for SL DRX command MAC CE. In this contribution, we seek to address this open aspect and present our view.

# Discussion

We have the following agreement regarding the definition of a new SR configuration for SL DRX MAC CE:

RAN2 does not define a separate SR configuration for SL DRX Command MAC CE.

Then, the question of how to handle the case when the UE needs to trigger an SR for transmission of a SL DRX command MAC CE was discussed. Based on company views, two possible solutions were proposed but there was no consensus, and it was agreed to discuss this in the following meeting.

Proposal 5a RAN2 aims at down-selection of SR configuration for SL DRX Command MAC CE between the following 2 options in next meeting: • Option 1: Reuse the SR configuration for SL logical channel, and which logical channel triggered a Scheduling Request for SL DRX command indication is up to UE implementation. • Option 2: Reuse the SR configuration for CSI report.

In our view, the intention of the agreement to not define a separate SR configuration does not necessarily imply that no SR is allowed to be triggered for SL DRX command MAC CE. Rather, we need to decide which SR configuration is utilized for this purpose. Then, given the marked similarities between the SL CSI report MAC CE and the SL DRX command MAC CE, including having the same SL priority naturally lends to the second option, i.e. the same SR configuration as that defined for CSI report can be reused for the DRX command MAC CE. It is the easiest and most feasible solution and does not require big specification impact at this stage. Therefore, we prefer to go with option 2 above, i.e. reuse the SR configuration for CSI report for SL DRX command MAC CE.

Proposal:	RAN2 is proposed to agree that the SR configuration for SL CSI Report is reused for SL DRX command MAC CE.

# Conclusion

This contribution discusses the open issue related to SR configuration for SL DRX command MAC CE and makes the following proposal:

Proposal:	RAN2 is proposed to agree that the SR configuration for SL CSI Report is reused for SL DRX command MAC CE.

# References

| [1] | “RAN2#118 e-Meeting, Chairman Notes”. |
| --- | --- |