3GPP TSG-RAN WG2 Meeting #119-e	R2-2208512

# Electronic, 17th August – 26th August, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | - | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for triggered Positioning MG Req MAC CE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Procedure of MAC entity when it received an indication from upper layer that the triggered Positioning Measurement Gap Activation/Deactivation Request MAC CE should be cancelled are captured in TS 38.321 v17.1.0. However the procedural text may be misled after cancelling the triggered Positioning Measurement Gap Activation/Deactivation Request MAC CE by indication from upper layer. Following the procedure, the MAC entity may instruct the Multiplexing and Assembly procedure to generate the cancelled MAC CE, or may trigger a Scheduling Request for the cancelled MAC CE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add condition to clarify operation after receiving an indication from upper layer that the triggered Positioning Measurement Gap Activation/Deactivation Request MAC CE should be cancelled. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | MAC entity may instruct the Multiplexing and Assembly procedure to generate the cancelled Positioning Measurement Gap Activation/Deactivation Request MAC CE or trigger a Scheduling Request for the cancelled Positioning Measurement Gap Activation/Deactivation Request MAC CE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.25 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

5.25	Positioning Measurement Gap Activation/Deactivation Request

If the UE is configured with pre-configured measurement gap, the UE may request the network to activate or deactivate the Positioning measurement gap with UL MAC CE for Positioning Measurement Gap Activation/Deactivation Request in clause 6.1.3.40.

The MAC entity shall, when triggered by the upper layer to send Positioning Measurement Gap Activation/Deactivation Request, cancel the triggered Positioning Measurement Gap Activation/Deactivation Request, if any and trigger another Positioning Measurement Gap Activation/Deactivation Request according to the upper layer's request.

The MAC entity shall,

1>if Positioning Measurement Gap Activation/Deactivation Request MAC CE has been triggered, and not cancelled:

2>	if indication from upper layer has been received that the triggered Positioning Measurement Gap Activation/Deactivation Request MAC CE should be cancelled:

3>	cancel the triggered Positioning Measurement Gap Activation/Deactivation Request MAC CE.

2> else:

3>	if UL-SCH resources are available for a new transmission and these UL-SCH resources can accommodate the Positioning Measurement Gap Activation/Deactivation Request MAC CE plus its subheader as a result of logical channel prioritization:

4>	instruct the Multiplexing and Assembly procedure to generate the Positioning Measurement Gap Activation/Deactivation Request MAC CE according to the upper layer's request;

4>	cancel triggered Positioning Measurement Gap Activation/Deactivation Request MAC CE.

3>	else:

4>	trigger a Scheduling Request for Positioning Measurement Gap Activation/Deactivation Request MAC CE.

END OF CHANGE