3GPP TSG-RAN WG2 Meeting #119-e	R2-2208609

Electronic, 17th Aug – 26th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | xxx | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on SubgroupID for UE-ID based subgrouping |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi, ZTE Corporation,Vivo, Ericsson, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | R17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clarifications on SubgroupID for UE-ID based subgrouping. In RAN2#118-e meeting, we have agreed the following for iPO determination to solve the mismatching problem by using different paging cycles for different RRC states: In RRC_INACTIVE state, if the UE supports inactiveStatePO-Determination and the network broadcasts ranPagingInIdlePO with value "true", the UE shall use the same iPO as for RRC_IDLE state. Otherwise, the UE determines the iPO based on the formula defined in clause 10.4a in TS 38.213 [4]. The same issue exists on how to calculate the subgroup ID for UE-id based subgrouping when the configured T is different in idle and inactive in the following formular (N is related to T): SubgroupID = (floor(UE_ID/(N*Ns)) mod subgroupsNumForUEID) + (subgroupsNumPerPO - subgroupsNumForUEID) It is better to make some clarifications on SubgroupID for UE-ID based subgrouping in TS 38.304.The simplest way is that for the UE to use one UE_ID based subgroup ID, no matter the UE is in idle or inactive state which is also more aligned with what has been captured in TS 38.300:” The RRC state (RRC_IDLE or RRC_INACTIVE state) does not impact which subgroup the UE belongs to”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in TS 38.304 that In RRC-INACTIVE state, the UE uses the paging cycle in RRC-IDLE state to calculate the SubgroupID for PEI monitoring. Impact analysis Impacted 5G architecture options: NR SA, NE-DC,NR-DC Impacted functionality PEI Inter-operability: - If the network is implemented according to the CR and the UE is not, there could be a mismatch between UE and the network regarding the UE-ID based subgroup of the UE. - If the UE is implemented according to the CR and the network is not, there could be a mismatch between UE and the network regarding the UE-ID based subgroup of the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Caculations on SubgroupID for UE-ID based subgrouping is not correctly captured. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

7.3.2	UE_ID based subgrouping

Paging with UE_ID based subgrouping is used in the cell which supports UE_ID based subgrouping, as described in clause 7.3.0.

If the UE is not configured with a CN assigned subgroup ID, or if the UE configured with a CN assigned subgroup ID is in a cell supporting only UE_ID based subgrouping, the subgroup ID of the UE is determined by the formula below:

SubgroupID = (floor(UE_ID/(N*Ns)) mod subgroupsNumForUEID) + (subgroupsNumPerPO - subgroupsNumForUEID),

where:

N: number of total paging frames in T, which is the DRX cycle of RRC_IDLE state as specified in clause 7.1

Ns: number of paging occasions for a PF

UE_ID: 5G-S-TMSI mod X, where X is 32768, if eDRX is applied; otherwise, X is 8192

subgroupsNumForUEID: number of subgroups for UE_ID based subgrouping in a PO, which is broadcasted in system information

The UE belonging to the SubgroupID monitors its associated PEI which indicates the paged subgroup(s) as specified in clause 7.2.

End of change