3GPP TSG-RAN WG2 Meeting #119-e	R2-2208681

Elbonia, 17th – 26th August, 2022

Agenda item:	7.2.3

Source: 	Sequans Communications

Title: 	NTN Configuration at CHO

Document for:	Discussion and Decision

# Introduction

This contribution discusses a possible issue related to NTN configuration at CHO.

# Discussion

From 36.331v17.1.0, the SystemInformationBlockType31 is provided in RRCConnectionReconfiguration, as a mandatory present field in case of handover to an NTN cell.

The SIB31 mainly contains ephemeris and common TA information that are related to the indicated Epoch Time. They are required to access the target cell.

In the general CHO case, the NW has no knowledge of when the actual handover will take place. It may then be impossible to provide the SystemInformationBlockType31 directly in the HO message.

Observation 1: In the general CHO case, SystemInformationBlockType31cannot be provided by the NW in the HO message

A possible solution would be to let the UE acquire the target SystemInformationBlockType31 when executing the CHO.

Proposal 1: RAN2 to discuss whether in case of CHO, SystemInformationBlockType31 may not be provided by the NW, in which case the UE acquires SystemInformationBlockType31 in the target cell

# Conclusion

In this contribution, we make the following observations and proposals:

Observation 1: In the general CHO case, SystemInformationBlockType31cannot be provided by the NW in the HO message

Proposal 1: RAN2 to discuss whether in case of CHO, SystemInformationBlockType31 may not be provided by the NW, in which case the UE acquires SystemInformationBlockType31 in the target cell