# 3GPP TSG-RAN WG2 Meeting #119 Electronic										    R2-2208686

# Online Meeting,  August 17th – 29th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | Draft | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on PPW for positioning enhancement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | During RAN1 #108-e, the following agreements have been made regarding PPW : - The PRS processing window is configured per DL BWP. - The maximum number of activated PRS processing windows per DL BWP is 1. - The maximum number of activated PRS processing windows across all active DL BWPs is 4. - The maximum number of preconfigured PRS processing window per DL BWP is 4. 1) According to TS 38.331, BWP-DownlinkDedicated -> dl-PPW-PreConfigToAddModList-r17 has been defined to indicate the number of PPW configurations for the dedicated DL BWP, and maxNrofPPW-Config-r17 is equal to 4. DL-PPW-PreConfigToAddModList-r17 ::= SEQUENCE (SIZE (1..maxNrofPPW-Config-r17)) OF DL-PPW-PreConfig-r17 Since the value range of DL-PPW-PreConfig-r17 -> dl-PPW-ID-r17 is 0 to 15, and the length of the PPW ID field in PPW Activation/Deactivation Command MAC CE is 2 bits, the PPW ID field should indicate the index of an entry in dl-PPW-PreConfigToAddModList-r17 but not directly mapped to dl-PPW-ID-r17. 2) Some editorial changes to the numEntry field make it more readable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the definition of the PPW ID and numEntry fields in PPW Activation/Deactivation Command MAC CE. Impacted functionality: PPW activation/deactivation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description of PPW ID and numEntry fields may be unclear and cause misunderstanding. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.42 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.1.3.42	PPW Activation/Deactivation Command MAC CE

<Unchanged part omitted>

The PPW Activation/Deactivation Command MAC CE is identified by MAC subheader with eLCID as specified in Table 6.2.1-1b.

It has variable size defined as follows (Figure 6.1.3.42-1):

-	numEntry: This field indicates the number of entries N in the MAC CE. 00 indicates that N equals to 1; 01 indicates that N equals to 2 and so on. The length of the field is 2 bits;

-	Serving Cell ID: This field indicates the identity of the Serving Cell for which the MAC CE applies. The length of the field is 5 bits;

-	PPW ID: This field is set to the index of a PPW configuration in dl-PPW-PreConfigToAddModList, as specified in TS 38.331 [5]. Index of a PPW configuration is the index of an entry in the dl-PPW-PreConfigToAddModList which is configured on the active DL BWP of the Serving Cell identified by the above Serving Cell ID. Index 0 corresponds to the first entry in the dl-PPW-PreConfigToAddModList, index 1 corresponds to the second entry in the list and so on. The length of the field is 2 bits;

-	A/D: This field indicates the activation or deactivation of the PPW. The field is set to 1 to indicate activation, otherwise it indicates deactivation. The length of the field is 1 bit;

-	R: Reserved bit, set to 0.

Figure 6.1.3.42-1: PPW Activation/Deactivation Command MAC CE

<Unchanged part omitted>