3GPP TSG-RAN WG2 Meeting #119-e	R2-2209180

# E-meeting, 17. - 29.08.2022	was R3-225190

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR | 0558 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Completion of the missing descripition of SCG MRO handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | R3 (Nokia, Nokia Shanghai Bell, ZTE) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_SON_MDT_enh-Core |  |  |  |  |  | Date: |  |  | 2022-08-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The procedure to enable analysis of the SCG failure cases is based on possible UE reporting, collecting and storing SN change information and using 2 separate procedures. Therefore, to enable proper interpretation, it has to be described well. Currently, the description is distributed in 3 specifications: TS 38.423, TS 37.340 and TS 38.300. While the stage-3 is quite clear, both stages-2 are very vague and say roughly the same. Also, the use of the SCG Failure Transfer procedure is not mentioned in any stage-2! Some references in the text are wrong. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the description of the PSCell change failure handling, the use of the SCG Failure Transfer procedure is mentioned. Impact assessment towards the previous version of the specification (same release): The impact can be considered isolated because the change only clarifies the intended use of the procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | References in the stage-3 are not addressed in the referred specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.5.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR was endorsed by RAN3 in R3-225190. |  |  |  |  |  |  |  |  |

First change, ommited text not changed

#### 15.5.2.6	PSCell change failure

For analysis of PSCell change failures, the UE makes the SCG Failure Information available to the MN. If the MN can perform an initial analysis, it transfers the SCG Failure Information together with the analysis results to the relevant SN which is responsible for the PSCell change failures. Otherwise, the MN transfers the SCG Failure Information to the last serving SN, which may respond using the SCG Failure Transfer procedure to inform the MN it is not responsible for the SCG failure. If needed, the MN transfer the SCG Failure Information to the source SN.

Remaining text not changed